SV3H-E servo drives

User Manual

● Millions of thanks for buying SINSEGYE servo drives.

● Please use this product correctly and safely upon reading this manual carefully.

● Please keep this manual carefully.

● If there are some changes of this manual,it wouldn't inform users.

- Preface

Thank you for utilizing our products. This manual provides information on SV3H servo drives and the corresponding motors.

Content

◼ Installation and inspection of servo drives & motors

◼ servo architecture and the wiring diagrams

◼ Steps of commissioning operation

◼ Training on servo tuning

◼ Description on parameters

◼ Description on communication protocol

◼ Abnormal alarm clear

◼ Inspection and maintenance

SV3H features

SV3H servo is general AC servo product independently developed by SINSEGYE, which provides inertia identification, automatic gain adjustment and other functions to make the drive simple and easy to use. With the latest development of servo motors, it achieves rapid and accurate control with cost-effective solutions in electronic manufacturing, robot, packaging, lathe and other industries of automation. The latest improved design for servo drive structure can save the inner space of cabinet. The new generation of motor design can meet the need on miniaturization of equipment structure and lightweight.

How to use this manual

This manual will tell you how to install, configure, use and maintain the product. Prior to tuning, read Chapter 1 to Chapter 8.

Technical service

If you still have problems with the application, please contact the distributor or our customer service center.

Version change log

| Date of release | Version | Change |
| --- | --- | --- |
| 2024.7 | V1.0 | Initial release |

Copyright statement

All the right  reserved by SINSEGYE. Do not to reproduce or disseminate any part of this manual in any format without the Company's permission.

Disclaimer

This product documentation is accurate and reliable at the time of release, and SINSEGYE reserves the right to change this manual without any additional notice.

About the manual

This manual is not attached to the product package. If you need to get electronic PDF files, download on SINSEGYE website (https://www.sinsegye.com.cn/). If you need consulting or assistance, please contact our company.

- Contents

Preface	i

Contents	II

Safety precautions	I

Chapter 1 Model selection and installation	1-1

1.1 Model Definition for Servo Drives	1-1

1.2 Model Definition for Motors	1-2

1.2.1 SM3-M2 Motor	1-2

1.2.2 SM3-M3 Motor	1-2

1.3 Specifications	1-3

1.3.1 Basic Parameters of the Model	1-3

1.3.2 Specifications of EtherCAT Communication 	1-4

1.3.3 Electrical Parameters of Models	1-4

1.4 Description on Components	1-5

1.4.1 SIZE A Model	1-5

1.4.2 SIZE B Model	1-6

1.4.3 SIZE C Model	1-7

1.4.4 SIZE D Model	1-8

1.5 Installation of Drives 	1-9

1.5.1 Installation Site	1-9

1.5.2 Environmental Conditions	1-9

1.5.3 Installation Dimensions	1-10

1.5.4 Installation Precautions	1-12

1.5.5 Grounding	1-13

1.5.6 Wiring Requirements	1-13

1.6 Motor Installation	1-14

1.6.1 Installation Site	1-14

1.6.2 Environment Conditions	1-14

1.6.3 Installation Methods	1-14

1.6.4 Protection Countermeasures for Oil and Water 	1-15

1.6.5 Cable stress	1-15

1.7 Wiring for Cable Carrier Guide Rail	1-15

Chapter 2 Wiring	2-1

2.1 Description on System Wiring	2-1

2.1.1 SIZE A System wiring diagram	2-1

2.1.2 SIZE B System wiring diagram	2-2

2.1.3 SIZE C System wiring diagram	2-3

2.1.4 SIZE D System wiring diagram	2-4

2.2 Servo Drive Port Definition	2-5

2.3 Definition and wiring description of power terminals	2-6

2.3.1 Definition of power supply and motor terminals	2-6

2.3.2 Wiring of circlip terminals	2-9

2.3.3 Sample of Main Circuit Wiring	2-10

2.3.4 Specification of Main Loop Cable	2-14

2.4 Common DC Bus Cable Wiring	2-15

2.5 Description of regenerative resistance wiring	2-16

2.5.1 Connecting regenerative resistance wiring	2-16

2.5.2 Specifications of regenerative resistors	2-17

2.5.3 Selection and calculation of regenerative resistance	2-18

2.6 Wiring of band-type brake	2-19

2.7 Description of Control Signal CN5 Port Wiring	2-20

2.7.1 Definition of Control Signal CN5 Port	2-20

2.7.2 Description of Digital Input Wiring	2-21

2.7.3 Description of Digital Output Wiring 	2-23

2.7.4 Description of Frequency Division Output Wiring	2-26

2.8 Encoder interface definition and wiring description	2-26

2.8.1 Motor encoder	2-26

2.8.2 Full closed-loop encoder	2-27

2.8.3 Input of motor temperature sensor	2-29

2.9 Definition of Communication Ports	2-29

2.9.1 Serial communication port	2-29

2.9.2 EtherCAT Communication Port	2-30

2.10 Definition and Wiring Description of Functional Safety Terminals	2-31

2.10.1 Definition of Functional Security Terminals	2-31

2.10.2 Function Safety Wiring Description	2-31

2.10.3 Relationship of STO I/O Signals	2-31

2.11 Grounding and anti-interference measures	2-32

2.11.1 Grounding Measures	2-32

2.11.2 Anti-interference measures	2-32

2.11.3 Noise filter	2-33

2.11.4 Leakage protection circuit breaker	2-36

2.11.5 Cable and wiring Requirements	2-36

2.11.6 I/O magnetic ring selection	2-38

2.11.7 Solutions to common EMC problems	2-39

2.12  General Wiring Diagram	2-40

Chapter 3 Commissioning	3-1

3.1 Settings of Basic Operation 	3-1

3.1.1 Motor Tuning	3-1

3.1.2 Lock Setting	3-3

3.1.3 Selection of Running Direction	3-7

3.1.4 Absolute Value Function	3-8

3.1.5 Setting of Electronic Gear Ratio 	3-10

3.1.6 Time sequence diagram	3-11

3.1.7 Operation	3-17

3.1.8 Stop	3-18

3.2 Panel Control Operation	3-21

3.2.1 Introduction of Panel 	3-21

3.2.2 Jog Operation	3-25

3.3 Use the Servo3 Designer	3-26

3.3.1 Overview	3-27

3.3.2 Operating Environment	3-27

3.3.3 Parameter Management	3-27

3.3.4 Oscilloscope	3-29

3.3.5 Initialization	3-31

3.3.6 Commissioning	3-33

3.3.7 Tuning	3-37

3.3.8 Troubleshooting	3-41

3.4 EtherCAT control operation	3-44

3.4.1 Running Status Control	3-44

3.4.2 PDO Configuration	3-49

3.4.3 Setting of Operation Mode	3-52

3.4.4 Application Functions	3-106

Chapter 4 Gain Adjustment	4-1

4.1 Purpose	4-1

4.2 Tuning Method	4-2

4.2.1 Mechanical load identification	4-3

4.2.2  Manual gain adjustment	4-6

4.2.3 Gain Switching	4-16

4.2.4 Automatic gain adjustment	4-21

4.3 Vibration suppression	4-25

4.3.1 Notch Filter	4-25

4.3.2 Vibration Suppression	4-26

4.4 Common Application Scenarios	4-30

4.4.1 Load of ball screw	4-30

4.4.2 Synchronous Belt Load	4-31

4.4.3 Rack and pinion load	4-32

4.4.4 Inertia disk load	4-33

4.4.5 Long cantilever load	4-34

Chapter 5 Troubleshooting and Warning	5-1

5.1 Display and Review	5-1

5.2 Fault Code Overview	5-2

5.3 Warning Code Overview	5-3

5.4 Troubleshooting	5-4

5.5 Warning Handling	5-11

5.6 Resetting Methods	5-12

Chapter 6 EtherCAT Communication	6-1

6.1 Overview	6-1

6.1.1 Overview of EtherCAT	6-1

6.1.2 Host/Slave System Composition	6-1

6.1.3 ESC Overview	6-2

6.1.4 Structure of EtherCAT Application Layer Protocol	6-3

6.2 EtherCAT Specification	6-4

6.2.1 EtherCAT Frame Structure	6-4

6.2.2 Addressing Mode	6-7

6.2.3 Frame Processing Sequence	6-8

6.2.4 ESC Register	6-10

6.2.5  ESM(EtherCAT State Machine)	6-12

6.2.6  SII(Slave Information Interface) EEPROM	6-13

6.2.7 Synchronization Mode	6-14

6.2.8 MailBox Mailbox Structure	6-15

6.2.9  PDO(Process Data Object)	6-20

Chapter 7 Object Dictionary	7-1

7.1 Allocation List of Object Group 1000h	7-1

7.2 Allocation List of Object Groups 2000h	7-9

7.3 Allocation List of Object Group 6000h	7-33

Chapter 8 Appendix	8-1

8.1 Definition of DI/DO functions	8-1

8.2 SDO Transmission Stop Code	8-3

8.3 Application Cases of Adapting Codesys Host Station	8-4

8.4 Adapting TwinCAT Operation Guide	8-6

8.4.1 TwinCAT Jog Operation	8-6

8.4.2 TwinCAT3-PDO Control Operation	8-15

8.5 Samples for Various Mode Operation	8-19

8.5.1 Samples for CSP Mode	8-19

8.5.2 Samples for CSV Mode	8-20

8.5.3 Samples for CST Mode	8-21

8.5.4 Samples for PP Mode	8-22

8.5.5 Samples for PV Mode	8-23

8.5.6 Samples for PT mode	8-24

8.5.7 Return to Zero Operation Sample	8-25

- Safety precautions

Safety Statement

- ●	This chapter describes the safety precautions for proper use of the product. Before using this product, please read the instruction manual and correctly understand the relevant safety precautions. Failure to comply with the safety precautions may result in death, serious injury, or equipment damage.

- ●	"Danger", "warning" and "attention" in the manual do not represent all the safety matters that shall be followed, but only supplement all the safety precautions.

- ●	This product should be used in an environment that meets the requirements of the design specifications, otherwise it may cause failure, and functional abnormalities or component damage caused by failure to comply with the relevant regulations are not within the scope of product quality assurance.

- ●	Our company will not bear any legal responsibility for personal safety accidents and property losses caused by non-compliance with the contents of this chapter and illegal operation of products.

Note for safety levels

|  | Danger | The sign refers to "To result in death or serious injury". |
| --- | --- | --- |
|  | Warning | The sign refers to "The most probably to result in death or serious injury". |
|  | Attention | The sign refers to "The most probablyy to result in injury or damage to property". |

Note for safety precautions

| Unpacking acceptance |  |
| --- | --- |
|  | Warning |
| - ● Do not install if product and the accessories are found while unpacking with damage, rust, trace of use,etc. - ● Do not install if there is any water inside product, or missing part, or damanged part. - ● Please check the packing list carefully. If the packing list does not match product name, do not install! |  |
|  | Attention |
| - ● Before unpacking, check if outer package of the device is intact, damaged, soaked, damp, or deformed. - ● Please open the package in order. Do not knock it hard! - ● Before unpacking, please check the surface of the equipment and accessories for damage, rust or damage. - ● After unpacking, please check if quantity and information of equipment and accessories are complete in accordance with the packing list. |  |
| Storage and transport |  |
|  | Warning |
| - ● Be sure to use professional lifting equipment and have qualified personnel operate large/heavy products. Otherwise, there is the risk of injury or product damage! - ● Before lifting the product vertically, ensure that the front cover, terminal block and other components of product are firmly secured with screws. Otherwise, components might fall off, that results in personal injury or product damage. - ● When the product is being lifted by lifting equipment, it is forbidden to stand or stay under the product. - ● When lifting the product with wire rope, please lift it at a steady and uniform speed, prevent products from vibration or impact; Do not turn products over, nor make products in lifting state for a long time, or there is the risk of personal injury or product damage! |  |
|  | Attention |
| - ● While moving the product, be sure to handle it gently and always pay attention to any object under feet to prevent tripping or falling, or it may result in personal injury or product damage! - ● When handling the product manually, be sure to grasp product shell firmly to avoid any component falling, or it might result in personal injury! - ● Please store and transport in strict accordance with the required storage and transportation conditions of product, or there is a risk of product damage. - ● Avoid storage and transportation in such places as water splashing and rain falling, direct sunlight, strong electric field, strong magnetic field, and strong vibration. - ● Avoid product storage time above 3 months, if the period was too long, please take more stringent protection and necessary inspection. - ● Please strictly package the product prior to vehicle transportation; For long distance transportation, it must use the enclosed cargo van. - ● It's strictly prohibited to transport this product together with equipment or articles that may affect or damage this product. |  |
| Installation |  |
|  | Danger |
| - ● Operators must obtain electrical knowledge and have been trained on electrical equipment. Non-professional personnel are strictly prohibited for any operation! |  |
|  | Warning |
| - ● Please read the user's manual and safety precautions carefully prior to installation! - ● Do not install this product in places in strong electric field or with strong electromagnetic wave interference! - ● Before installation, please make sure that the mechanical strength of installation positions is sufficient to support device weight. Otherwise, it might casue mechanical hazards. - ● Do not wear loose clothing or accessories during installation, otherwise there might be a risk of electric shock! - ● When installing the product in the enclosed environment (e.g., cabinet or chassis), please use the cooling device (e.g., cooling fan or cooling AC.) to cool the product sufficiently in order to meet the environment requirement on installation. Otherwise, it might cause product overheart or even a fire. - ● It's strictly prohibited to modify this product! - ● It's strictly forbidden to screw the fixing bolts and red marked bolts of parts and components of product! - ● When the product is installed in a cabinet or terminal equipment which shall be provided with fire protection shell, electrical protection shell and mechanical protection shell. The protection level shall comply with relevant IEC standards and local rules and regulations. - ● When it's necessary to install the equipment with strong electromagnetic interference such as transformer, please install shielding protection device to avoid misoperation of this product! - ● Please install the product on flame-retardant metal, do not make flammable substances contact the product or attach flammable substances to the product, otherwise there might be a risk of fire. |  |
|  | Attention |
| - ● During installation, please cover the top of product with cloth or paper to prevent foreign articles, such as scrap metal, oil and water, from entering the product that could result in faults. After operation, please remove the cover to avoid blocking the ventilation and affecting heat radiation, that results in abnormal heating of product. - ● Resonance may occur when a machine at a constant speed runs at variable speeds. Here, it can effectively weaken the resonance to mount the anti-vibration rubber under motor frame or to utilize vibration suppression function. |  |
| Wiring |  |
|  | Danger |
| - ● It's strictly prohibited for non-professional personnel to perform equipment installation, wiring, maintenance, inspection or part replacement! - ● Before wiring, please power off all devices. As the power-off devices have built-in capacitors which have residual voltage, please wait at least the period specified in the warning label before wiring. Measure DC voltage of main loop and confirm that it's under the safe voltage, otherwise there is a risk of electric shock. - ● Please perform wiring operation, remove the cover of product, or touch circuit board when it powers off. Otherwise, there is a risk of electric shock. - ● Please ensure that devices and products are properly grounded; Otherwise, electric shocks might occur. |  |
|  | Warning |
| - ● It's strictly prohibited to connect the input power to output of device or product; Otherwise, device might be damaged or even it starts a fire. - ● When drive is connected to the motor, please ensure that the phase sequence of product and motor terminals are accurate and consistent so as to avoid reversing motor rotation. - ● Cables used for wiring must meet the requirement on diameters and shielding, correspondingly. Shielding layer of shielded cables shall be reliably grounded at single end. - ● Tighten terminal screws according to tightening torque specified in the manual. Insufficient or excessive tightening torque may cause overheating or damage to the connection, and even start a fire. - ● After wiring operation ends, ensure that all cables are properly connected and no screw, gasket or exposed wire are found inside the product. Otherwise, there might be a risk of electric shock or product damage. |  |
|  | Attention |
| - ● To avoid damage to the equipment or built-in circuit of product, follow the steps specified in ESD preventive measures and wear a wrist strap to handle wiring operation. - ● For wiring of control loop, use double-stranded shielding cables to connect the shielding layer to grounding terminal of the product. Otherwise, it might cause abnormal actions of the product. |  |
| Power-on |  |
|  | Danger |
| - ● Before power-on, please make sure that the product is installed properly, the wiring is firm, and the motor is allowed to re-start. - ● Before power-on, please make sure that power supply meets the product requirement to avoid product damage or starting a fire. - ● It is strictly forbidden to open cabinet doors or protective cover plates of product, to touch any terminal of product, to disassemble any device or component of product in the power-on state. Otherwise, there is a risk of electric shock! |  |
|  | Warning |
| - ● After wiring and parameter setting ends, please do a test run to ensure that the machine can operate safely. Otherwise, it might result in human injury or device damage. - ● Before power-on, please make sure that the rated voltage of product is consistent with power supply voltage. If power supply voltage used was incorrect, there is a risk of fire. - ● Before power-on, please make sure that no one is around the product, motor or motor. Otherwise, it might result in personal injury or death. |  |
| Operation |  |
|  | Danger |
| - ● It is strictly forbidden for non-professional personnel to operate the product, otherwise it might lead to personal injury or death! - ● It is strictly forbidden to touch any terminal of the equipment, to disassemble any device or component of the equipment and product during operation, otherwise there is a risk of electric shock! |  |
|  | Warning |
| - ● Do not touch the device shell, fan or resistor to test the temperature, otherwise it may cause burns! - ● During operation, prvent other articles or metal objects from falling into the equipment, otherwise it might start a fire or cause product damage! |  |
| Maintenance |  |
|  | Danger |
| - ● It's strictly prohibited for non-professional personnel to perform equipment installation, wiring, maintenance, inspection or part replacement! - ● It is strictly prohibited to perform equipment maintenance in the power-on state, otherwise there is the risk of electric shock! - ● After powering off all the equipment, wait at least the period specified on the warning label before maintenance. - ● When using a motor, even if the product is powered off, induced voltage can be generated on motor terminals during motor rotation. Do not touch any terminal of motors, otherwise there may be a risk of electric shock. |  |
|  | Warning |
| - ● Please perform daily and routine check and maintenance of the equipment and products according to the maintenance requirement, and make the maintenance records. |  |
| Repair |  |
|  | Danger |
| - ● It's strictly prohibited for non-professional personnel to perform equipment installation, wiring, maintenance, inspection or part replacement! - ● It is strictly prohibited to perform maintenance in power-on state, otherwise there is a risk of electric shock! - ● After powering off all devices, please wait at least the period specified in the warning label before checking or maintaining. |  |
|  | Warning |
| - ● Please repair the equipment according to the product warranty. - ● When fuse burn-out, circuit breaker trip or ELCB trip, wait at least the period specified on the warning label before powering on or operating the machine. Otherwise, it may cause personal injury or equipment damage. - ● When the equipment is faulty or damaged, it is necessary for professional to troubleshoot and repair the equipment and products according to the maintenance guidance, and make a repair record. - ● Please replace in accordance with the instructions for replacing consumable parts. - ● Do not use the machine damaged, otherwise it might cause casualties or further damage to products. - ● After replacing the device, be sure to check the wiring and set parameters again. |  |
| Scrap |  |
|  | Warning |
| - ● Please scrap the equipment and products in accordance with the relevant national regulations and standards to avoid property losses or human casualties! - ● Scrapped equipment and products should be recycled in accordance with industrial waste treatment standards to avoid environmental pollution. |  |

- Chapter 1 Model selection and installation

1.1 Model Definition for Servo Drives

| SV3H | - | E | S | 3R5 | - | S | G |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ① |  | ② | ③ | ④ |  | ⑤ | ⑥ |

| ① | Product series | ④ | Rated current |  | ⑤ | Safety function |
| --- | --- | --- | --- | --- | --- | --- |
|  | SV3H:SV3H series high |  | Single-phase/three-phase 220V | 1R6:1.6A |  | Vacancy: Standard model |
|  | Performance servo drive |  |  | 2R8:2.8A |  | S: Safe torque Off (STO) *2 |
|  |  |  |  | 5R5:5.5A |  |  |
| ② | Communication method |  |  | 7R6:7.6A | ⑥ | Custom function |
|  | P: Pulse |  |  | 012:12A |  | Vacancy: Standard model |
|  | E:EtherCAT |  |  | 014:14A*1 |  | C: Full closed-loop function |
|  | C:CANopen |  | Three-phase 220V | 018:18A*1 |  | G: Gantry function |
|  | F:Profinet |  |  | 022:22A*1 |  | A: Analog interface *3 |
|  | R:RS485 |  |  | 027:27A*1 |  |  |
|  |  |  | Three-phase 380V | 3R5:3.5A |  |  |
|  |  |  |  | 5R4:5.4A |  |  |
|  |  |  |  | 8R4:8.4A | …… |  |
| ③ | Voltage level |  |  | 012:11.9A |  |  |
|  | S: Single-phase/three-phase 220V |  |  | 017:16.5A |  |  |
|  | U: Three-phase 220V |  |  | 021:20.8A |  |  |
|  | T: Three-phase 380V |  |  | 026:26A |  |  |

Note *1: S014, U018, U022, U027 models are coming soon.

*2: STO is not supported for CANopen and RS485

*3: Only Canopen and RS485 models are supported

1.2 Model Definition for Motors

1.2.1 SM3-M2 Motor

| SM3-M2 | H | 130 | - | S | 85B | 15C | - | N | H | 1 | B | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ① | ② | ③ |  | ④ | ⑤ | ⑥ |  | ⑦ | ⑧ | ⑨ | ⑩ | ⑪ |

| ① | Product series | ⑤ | Rated power /W | ⑨ | Keyway type |
| --- | --- | --- | --- | --- | --- |
|  | SM3-M2:SM3-M2 motors |  | B:×10 C:×100 Example: 85B:850W |  | 0: optical axis 1: with keyway |
| ② | Class of inertia | ⑥ | Rated speed /rpm | ⑩ | Band brake |
|  | A: Low inertia M: Medium inertia H: High inertia |  | B:×10 C:×100 Example: 15C:1500rpm |  | N: No band brake B: With band brake |
| ③ | Flange size /mm | ⑦ | Encoder type | ⑪ | Oil seal |
|  | 130:130 flange 180:180 flange |  | M:17Bit absolute value of single turn N:17Bit absolute value of multiple turns P:23Bit absolute value of multiple turns |  | 0: No oil seal 1: With oil seal |
| ④ | Voltage level | ⑧ | Interface type | …… |  |
|  | S:AC220V |  | H: Aviation plug type |  |  |

1.2.2 SM3-M3 Motor

| SM3-M3 | H | 80 | - | S | 75B | 30C | - | M | T | 1 | N | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ① | ② | ③ |  | ④ | ⑤ | ⑥ |  | ⑦ | ⑧ | ⑨ | ⑩ | ⑪ |

| ① | Product series | ⑤ | Rated power /W | ⑨ | Keyway type |
| --- | --- | --- | --- | --- | --- |
|  | SM3-M3:SM3-M3 motors |  | B:×10 C:×100 Example: 75B:750W |  | 0: optical axis 1: with keyway |
| ② | Class of inertia | ⑥ | Rated speed /rpm | ⑩ | Band brake |
|  | A: Low inertia M: Medium inertia H: High inertia |  | B:×10 C:×100 For example, 30C:3000rpm |  | N: No band brake B: With band brake |
| ③ | Flange size /mm | ⑦ | Encoder type | ⑪ | Oil seal |
|  | 40:40 Flange 60:60 flange 80:80 flange |  | M:17Bit absolute value of single turn N:17Bit absolute value of multiple turns P:23Bit absolute value of multiple turns |  | 0: No oil seal 1: With oil seal |
| ④ | Voltage level | ⑧ | Interface type | …… |  |
|  | S:AC220V |  | T: connector type |  |  |

1.3 Specifications

1.3.1 Basic Parameters of the Model

Table 1-1 Basic parameters of the model

| Item |  |  | Description |
| --- | --- | --- | --- |
| Basic specification | Control mode |  | IGBT SVPWM control, sine wave current drive mode. 220V, 380V: single-phase or three-phase full-wave rectification. |
|  | Use Conditions | Usage/storage temperature *1 | 0~+40 ℃/-20~+70℃ |
|  |  | Use/store humidity | Less than 90%RH (no condensation) |
|  |  | Vibration strength | 4.9m/s2 |
|  |  | Impact strength | 19.6m/s2 |
|  |  | Class of protection | IP20 |
|  |  | Class of contamination | Class PD2 |
|  |  | Altitude | The highest altitude is 5,000m. Derating is unnecessary for 1,000m or below; Derating rate is 1% per 100m rise above 1,000m. Please contact the manufacturer in case of altitudes over 2,000m. |
| Position control Mode | Performance | Feedforward compensation | Support speed feedforward (0~100.0%) setting to eliminate following deviation |
|  |  | Command shaping | Position instruction low-pass filtering, average filtering |
|  | Frequency division Output | Output pattern | Phase A, Phase B, Phase Z: differential output |
|  |  | Frequency division range | The motor rotates one circle, and the frequency can be divided into any pulse from 140 to 1,048,576. |
| Speed Torque Control mode | Performance | Dynamic characteristics of current loop | Step response: 187.5us(0~100%) Frequency response: -3dB amplitude attenuation bandwidth, 2000Hz(command signal: ±25%) -90° phase shift bandwidth, 3500Hz(command signal: ±25%); |
|  |  | Speed control range | from 0 to 12000rpm. In case of the requirement over 6000rpm, please contact the manufacturer. |
|  |  | Dynamic characteristics of speed loop | Step response: 562.5us(0~1000rpm) Frequency response: -3dB amplitude attenuation bandwidth, 1000Hz(command signal: ±500rpm) -90° phase shift bandwidth, 630Hz(command signal: ±500rpm); |
|  |  | Torque control accuracy | ±2% |
| I/O | Digital input signal |  | Functions can be configured: forward overrange switch, reverse overrange switch, origin switch, etc. |
|  | Digital output signal |  | Functions can be configured: servo ready, zero speed signal, speed arrival, position arrival, positioning approach signal, torque limit, warning, servo fault, etc. |
| Support function | Electronic gear ratio |  | Built-in two sets of electronic gear ratio, support gear ratio switching function |
|  | Position limitation protection |  | Stop immediately while forward/reverse overrange switch operates. |
|  | Fault detection |  | Overcurrent, overvoltage, undervoltage, overload, main circuit detection abnormal, radiator overheating, overspeed, encoder abnormal, parameter abnormal, etc. |
|  | Display function |  | 5-digit LED display, power indicator CHARGE |
|  | Vibration suppression |  | With 4 notch filters, 50Hz~5000Hz, all 4 notch filters can be adaptive setting. |
|  | Ease of use |  | Self-tuning, speed observer, model tracking |
|  | Debugging interface |  | MiniUSB |
|  | Other |  | Status display, alarm logging, JOG running, etc. |
| Attention |  |  |  |
| Note *1: Install or store servo drives within this temperature range. |  |  |  |

1.3.2 Specifications of EtherCAT Communication

Table 1-2 EtherCAT communication specification

| Item |  | Specification |
| --- | --- | --- |
| EtherCAT Basic performance of slave station | Communication protocol | EtherCAT protocol |
|  | Support service | CoE（PDO、SDO） |
|  | Synchronous mode | DC-distributed clock |
|  | Physical layer | 100BASE-TX |
|  | Transmission rate | 100 MBit/s (100BASE-TX) |
|  | Duplex mode | Full duplex |
|  | Topological structure | Circular, linear |
|  | Transmission medium | Shielded Category 5E cable or higher category |
|  | Transmission distance | Less than 100m between two nodes (good environment, good cable) |
|  | Number of slave stations | Protocol support up to 65,535, the actual quantity is not more than 100 sets |
|  | EtherCAT frame length | from 44 to 1498 bytes |
|  | Process data | A single Ethernet frame has a maximum of 1,486 bytes. |
|  | Synchronous jitter of 2 slave stations | < 1μs |
|  | Refresh time | I/O of 1000 switching quantity is about 30μs; 100 servo shafts about 100μs; Define different refresh time for different interfaces. |
|  | Communication bit error rate | 10-10 Ethernet standard |
| EtherCAT Configuration unit | Fieldbus memory management unit | 8 |
|  | Storage synchronization management unit | 8 |
|  | Process data RAM | 8K bytes |
|  | Distributed clock,DC | 64-bit |
|  | EEPROM capacity | 32kBit Initialization data is written by EtherCAT master station |

1.3.3 Electrical Parameters of Models

Table 1-3 Electrical specifications of 220V servo drives

| Structural dimension | SIZE Type A |  | SIZE Type B | SIZE Type C |  |  | SIZE Type D |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Model SV3H | S1R6 | S2R8 | S5R5 | S7R6 | S012 | S014*1 | U018*1 | U022*1 | U027*1 |
| Rated output current Arms | 1.6 | 2.8 | 5.5 | 7.6 | 11.6 | 14.0 | 18 | 22 | 27 |
| Maximum output current Arms | 5.8 | 10.1 | 16.9 | 23.0 | 32.0 | 42.0 | 45.0 | 55.0 | 72.0 |
| Rated input current Arms | Single-phase 2.3/ Three-phase 1.4 | Single-phase 4.0/ Three-phase 2.6 | Single-phase 7.9/ Three-phase 4.4 | Single-phase 9.6/ Three-phase 5.6 | Single-phase 12.8/ Three-phase 8.0 | Single-phase 16.0/ Three-phase 10.2 | 18.7 | 20.7 | 24.4 |
| Main circuit power supply | Single-phase/three-phase AC 200V ~ 240V, -10%～+10%，50/60Hz |  |  |  |  |  | Three-phase AC 200V ~ 240V, -10%～+10%，50/60Hz |  |  |
| Power supply of control loop | Single-phase AC200V~240V, -10%~+10%, 50/60Hz |  |  |  |  |  |  |  |  |
| Regenerative resistance | No built-in regenerative resistance as standard |  | 50Ω/50W | 25Ω/80W |  |  | 20Ω/100W |  |  |

Note *1: S014/U018/U022/U027 models are coming soon.

Table 1-4 Electrical specifications of 380V servo drives

| Structural dimension | SIZE Type C |  |  |  | SIZE Type D |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Model SV3H | T3R5 | T5R4 | T8R4 | T012 | T017 | T021 | T026 |
| Rated output current Arms | 3.5 | 5.4 | 8.4 | 11.9 | 16.5 | 20.8 | 26.0 |
| Maximum output current Arms | 11.0 | 14.0 | 20.0 | 29.8 | 41.3 | 52.1 | 65.0 |
| Rated input current Arms | 2.4 | 3.6 | 6.6 | 8.0 | 12.0 | 16.0 | 21.0 |
| Main circuit power supply | Three-phase AC380V~440V, -10%~+10%, 50/60Hz |  |  |  |  |  |  |
| Power supply of control loop | Single-phase AC380V~440V, -10%~+10%, 50/60Hz |  |  |  |  |  |  |
| Regenerative resistance | 100Ω/80W |  | 50Ω/80W |  | 35Ω/100W |  |  |

1.4 Description on Components

1.4.1 SIZE A Model

| ID | Name | Remarks |
| --- | --- | --- |
| ① | Nixie tube display | 5 digit 7 segment LED digital tube |
| ② | Key-type operator | Operation mode, with settings of functions, parameters and monitoring |
| ③ | CHARGE(Bus voltage indicator) | Power indicator light |
| ④ | L1C, L2C(control power input terminal) | Power supply of control loop, connected to single-phase power supply (AC200~240V,50/60Hz power supply) |
|  | L1, L2, L3(input terminal of main power) | Power supply of main loop, connected to single/three-phase power supply (AC200~240V,50/60Hz power supply) |
| ⑤ | P/B(regenerative resistance connection terminal) | P/B are connected to external regenerative resistors |
|  | P/N(common DC bus terminal) | Common DC bus for multiple servos |
|  | U/V/W(servo motor connection terminal) | Servo drive output, connected to motor power connector (U/V/W) |
| ⑥ | CN1(Mini USB) | Mini USB port, connected to PC |
| ⑦ | CN2(Connector for safety function) | STO interface, only 'P/E/F' models support this option |
| ⑧ | CN3/CN4(Communication terminal) | EtherCAT high-speed communication port |
| ⑨ | CN5(I/O Connector) | I/O signals are connected to programmable controller (PLC) or control I/O with connectors |
| ⑩ | CN6(ENC encoder connector) | Encoder interface, connected to the encoder on servo motor |
| ⑪ | CN7(Fully Closed Loop Encoder Connector) | Fully closed loop interface, connected to th external second encoder. This function is supported by '-*C/*G' models |
| ⑫ | CN8(Reserved) | Reserved interface |
| ⑬ | Ground screw | Connect to earth wires of power supply and motors |

1.4.2 SIZE B Model

| ID | Name | Remarks |
| --- | --- | --- |
| ① | Nixie tube display | 5 digit 7 segment LED digital tube |
| ② | Key-type operator | Operation mode, with settings of functions, parameters and monitoring |
| ③ | CHARGE(Bus voltage indicator) | Power indicator light |
| ④ | L1C, L2C(control power input terminal) | Power supply of control loop, connected to single-phase power supply (AC200~240V,50/60Hz power supply) |
|  | L1, L2, L3(input terminal of main power) | Power supply of main loop, connected to single/three-phase power supply (AC200~240V,50/60Hz power supply) |
| ⑤ | P, RB, B(regenerative resistance connection terminal) | Use external regenerative resistor(remove the shorting cap of P and RB; Connect it to both ends of P and B) |
|  | P/N(common DC bus terminal) | Common DC bus for multiple servos |
|  | U/V/W(servo motor connection terminal) | Servo drive output, connected to motor power connector (U/V/W) |
| ⑥ | CN1(Mini USB) | Mini USB port, connected to PC |
| ⑦ | CN2(Connector for safety function) | STO interface, only 'P/E/F' models support this option |
| ⑧ | CN3/CN4(Communication terminal) | EtherCAT high-speed communication port |
| ⑨ | CN5(I/O Connector) | I/O signals are connected to programmable controller (PLC) or control I/O with connectors |
| ⑩ | CN6(ENC encoder connector) | Encoder interface, connected to the encoder on servo motor |
| ⑪ | CN7(Fully Closed Loop Encoder Connector) | Fully closed loop interface, connected to th external second encoder. This function is supported by '-*C/*G' models |
| ⑫ | CN8(Reserved) | Reserved interface |
| ⑬ | Ground screw | Connect to earth wires of power supply and motors |

1.4.3 SIZE C Model

| ID | Name | Remarks |
| --- | --- | --- |
| ① | Nixie tube display | 5 digit 7 segment LED digital tube |
| ② | Key-type operator | Operation mode, with settings of functions, parameters and monitoring |
| ③ | CHARGE(Bus voltage indicator) | Power indicator light |
| ④ | L1C, L2C(control power input terminal) | Control power supply of loop, connect to single phase power supply(depending on model 200~240VAC or 380~440VAC, 50/60Hz power supply) |
|  | L1, L2, L3(input terminal of main power) R/S/T(main power input terminal) | Main loop power supply, connect to single/three phase power supply(depending on model 200~240VAC or 380~440VAC, 50/60Hz power supply) |
|  | P, RB, B(regenerative resistance connection terminal) | Use external regenerative resistor(remove the shorting cap of P and RB; Connect it to both ends of P and B) |
|  | P/N(common DC bus terminal) | Common DC bus for multiple servos |
| ⑤ | U/V/W/PE(servo motor connection terminal) | Servo drive output, connected to motor power connectors(U/V/W/PE) |
| ⑥ | CN1(Mini USB) | Mini USB port, connected to PC |
| ⑦ | CN2(Connector for safety function) | STO interface, only 'P/E/F' models support this option |
| ⑧ | CN3/CN4(Communication terminal) | EtherCAT high-speed communication port |
| ⑨ | CN5(I/O Connector) | I/O signals are connected to programmable controller (PLC) or control I/O with connectors |
| ⑩ | CN6(ENC encoder connector) | Encoder interface, connected to the encoder on servo motor |
| ⑪ | CN7(Fully Closed Loop Encoder Connector) | Fully closed loop interface, connected to th external second encoder. This function is supported by '-*C/*G' models |
| ⑫ | Ground screw | Connect to earth wires of power supply and motors |

1.4.4 SIZE D Model

| ID | Name | Remarks |
| --- | --- | --- |
| ① | Nixie tube display | 5 digit 7 segment LED digital tube |
| ② | Key-type operator | Operation mode, with settings of functions, parameters and monitoring |
| ③ | CHARGE(Bus voltage indicator) | Power indicator light |
| ④ | L1C, L2C(control power input terminal) | Control power supply of loop, connect to single phase power supply(depending on model 200~240VAC or 380~440VAC, 50/60Hz power supply) |
|  | R/S/T(main power input terminal) | Main loop power supply, connect to single/three phase power supply(depending on model 200~240VAC or 380~440VAC, 50/60Hz power supply) |
|  | P, RB, B(regenerative resistance connection terminal) | Use external regenerative resistor(remove the shorting cap of P and RB; Connect it to both ends of P and B) |
|  | P/N1/N2(Common DC bus terminals) | Do not remove the shorting cap between N1 and N2. Connect it between P and N1 for common DC bus of multiple servos |
|  | U/V/W(servo motor connection terminal) | Servo drive output, connected to motor power connector (U/V/W) |
| ⑤ | CN1(Mini USB) | Mini USB port, connected to PC |
| ⑥ | CN2(Connector for safety function) | STO interface, only 'P/E/F' models support this option |
| ⑦ | CN3/CN4(Communication terminal) | EtherCAT high-speed communication port |
| ⑧ | CN5(I/O Connector) | I/O signals are connected to programmable controller (PLC) or control I/O with connectors |
| ⑨ | CN6(ENC encoder connector) | Encoder interface, connected to the encoder on servo motor |
| ⑩ | CN7(Fully Closed Loop Encoder Connector) | Fully closed loop interface, connected to th external second encoder. This function is supported by '-*C/*G' models |
| ⑪ | Ground screw | Connect to earth wires of power supply and motors |

1.5 Installation of Drives

1.5.1 Installation Site

Table 1-5 Drive installation sites

Please install in an electric control cabinet free from sunshine and rain

Do not use this product in corrosive environment with hydrogen sulfide, chlorine gas, ammonia, sulfur, chlorinated gas, acid, alkali, salt

Do not use this product in the presence of flammable gases or near the combustible

Do not install in an environment with high temperature, humidity, dust, or metal dust

Vibration-free site

Installation site contamination level: PD2

1.5.2 Environmental Conditions

Table 1-6 Environment conditions of drive installation

| Item | Description |
| --- | --- |
| Operating ambient temperature | 0 ~ +40 ℃ |
| Ambient humidity | Less than 90%RH (no condensation) |
| Storage temperature | -20~70℃(non-freezing) |
| Storage humidity | Less than 90%RH (no condensation) |
| Vibration | Below 4.9m/s2 |
| Impulse | Below 19.6m/s2 |
| Class of protection | IP20 Note: Except Terminal (IP00) |
| Altitude | The highest altitude is 5,000m. Derating is unnecessary for 1,000m or below; Derating rate is 1% per 100m rise above 1,000m. Please contact the manufacturer in case of altitudes over 2,000m. |

1.5.3 Installation Dimensions

1.5.3.1 SIZE A Dimensions:

Approx. weight: 0.77kg.

Front view

Fixed screw：M4*2pcs

Recommended torque：1.2N·m

Top view

Rear view

Left view

Figure 1-1 Appearance of SV3H SIZE A

1.5.3.2 SIZE B Dimensions:

Approx. weight: 1.10kg.

Top view

Front view

Rear view

Fixed screw：M4*2pcs

Recommended torque：1.2N·m

Left view

Figure 1-2 Appearance of SV3H SIZE B

1.5.3.3 SIZE C Dimensions:

Approx. weight: 1.75kg.

Front view

Top view

Rear view

Left view

Fixed screw：M4*3pcs

Recommended torque：1.2N·m

figure 1–3  SV3H SIZE CExternal drawing

1.5.3.4 SIZE D Size diagram:

Approx. weight: 2.58kg.

Rear view

Left view

Front view

Top view

Fixed screw：M4*4pcs

Recommended torque：1.2N·m

Figure 1-4 SV3H SIZE D Appearance

1.5.4 Installation Precautions

Table 1-7 Notes for drive installation

| Installation requirements | Ensure that mounting direction is perpendicular to the wall(drive mounting face is 90° from the bottom surface, vertically upward). Use natural convection or fan to cool servo drives. It is firmly fixed on mounting face through mounting holes of servo drive. Screws and torque used for installation see the above figure. While installing, the front of drive faces operational personnel for easy operation and maintenance. |
| --- | --- |
| The requirement for heat radiation | To ensure the heat radiation of drives, please design the heat radiation solution of control cabinet according to the following figure. Please install a cooling fan on the top of servo drive to ensure that temperature of servo drive is uniform without local overheat. |
| Space requirement | For installation with the spacing reserved, it's recommended to leave the transverse spacing more than 10mm on both sides of the drive and the longitudinal spacing more than 50mm on both sides of the drive. For compact installation, it's recommended to leave the transverse spacing more than 1mm on both sides of the drive and the longitudinal spacing more than 50mm on both sides of the drive. Here, please derate the rated load ratio to 75%. |

Air inlet

Air inlet

Air inlet

Air outlet

Air outlet

Air outlet

Figure 1-5 Servo drive installation diagram (for the spacing is reserved)

Air inlet

Air inlet

Air outlet

Air outlet

Air outlet

Air inlet

Figure 1-6 Servo drive installation diagram (for compact installation)

1.5.5 Grounding

Please ensure to ground the grounding terminal, otherwise there might be a risk of electric shock or misoperation from intreference.

For details on electrical ground, see 2.9 Grounding and anti-interference Measures.

1.5.6 Wiring Requirements

When connecting cables to the driver, route the cables downward (see the following figure) to prevent any liquid from flowing into drives that might cause damage.

Figure 1-7 Requirement for servo drive routing

1.6 Motor Installation

1.6.1 Installation Site

Table 1-8 Installation site of motors

Please install motors in a room free from rain and direct sunlight.

Do not use this product in corrosive environment with hydrogen sulfide, chlorine gas, ammonia, sulfur, chlorinated gas, acid, alkali, salt,etc.

Do not use this product in the presence of flammable gases or near the combustible.

Places without cutting fluid, oil mist, iron powder, and iron filings.

Places in good ventilation, no moisture or oil or water intrusion, away from furnace and other heat sources.

Vibration-free place.

Place for easy inspection and cleaning.

Do not use a motor in the enclosed environment which will cause high temperature of motor and shorten the service life.

1.6.2 Environment Conditions

Table 1-9 Environmental conditions for motor use

| Item | Description |  |
| --- | --- | --- |
| Operating ambient temperature | 0 ~ +40 ℃ |  |
| Ambient humidity | Less than 90%RH (no condensation) |  |
| Storage temperature | -20~70℃(non-freezing) |  |
| Storage humidity | Less than 90%RH (no condensation) |  |
| Vibration | Only for motor | Below 49 m/s2 when rotating, below 24.5m /s2 when stopping |
| Impulse | Only for motor | Below 98 m/s2 |
| Class of protection | Connector-type motor | IP67(The cable used is specified, except for connection pins of output shaft rotation, motor connector, encoder connector) |
|  | Wire-type motor | IP65(except for connection pins of output shaft rotation, motor connector, encoder connector) |
| Altitude | The highest altitude is 5,000m. Derating is unnecessary for 1,000m or below; Derating rate is 1% per 100m rise above 1,000m. Please contact the manufacturer in case of altitudes over 2,000m. |  |

1.6.3 Installation Methods

Motor can be installed vertically or horizontally, but the following requirements must be followed.

| Installation mode | Notes |
| --- | --- |
| Horizontal installation | Put cable outlet facing down to avoid oil and water infiltration inside motor. |
| Vertical installation | When a motor with reducer is installed axially, please use the motor with oil seal to avoid reducer oil seeping inside motor. |

Motor

Cable

Oil & Water

1.6.4 Protection Countermeasures for Oil and Water

Do not immerse the cable into oil or water, or use the special oil-proof cable

Set the cable outlet downwards.

Do not use in the environment where oil/water often splashes on motor body.

When equipped with a reducer, please use the motor with oil seal so as to avoid the oil seeping from shaft extension into motor.

1.6.5 Cable stress

① Do not stress the lead-in part and connection part of cable due to bending and self-weight.

② Particularly, when move motors and use a trunk cable that can be stored in cable tray, it shall minimize the bending stress of cable.

③ Try to increase the bending radius of cable.

1.7 Wiring for Cable Carrier Guide Rail

High-flex rag chain dcable plays an important role in signal, control and power transmission of the equipment. High-flex drag chain cable must be synchronized with other components in carrier. Furthermore, it must take care of installation and protection of high-flex drag chain cable in carrier particularly. Stable and effective use with a long service life depends on accurate installation.

1. Ensure that cable is completely free to move within the bending radius, i.e., the cable can move relative to each other and to guide device without any forced movement. When high-flex cable is routed in carrier, it can neither be too loose, nor too tight: the former might cause cable to bend and twist in carrier so as to affect its service life; The latter might lead in greater friction between the cable and inner wall of carrier so as to cause cable sheath wear and tear, and increase radial force of cable, as well as cable distortion that affects the service life.

Cable fixture must be mounted at both ends of cable carrier, and the fixing point can't be moved; The distance from the end of bending curve to the fixture shall be as large as possible, in general, greater than 20 to 30 times  cable diameter.

It's forbidden to fix the high-flex drag chain cable in any moving part of cable carrier, or bundle the cables in cable carrier together, that would hinder the absorption and dispersion capability of cable for bending stress, and would affect the service life.

Appropriate tension

Cable fixed to the moving part of the drag chain

Too tight

Too loose

Figure 1-8 Distribution status of guide duct cable

2. As for wiring of the high-flex cable in carrier, the cable should be laid side by side in the support of carrier as far as possible. There must be a certain gap between two adjacent cables; The gap betwee cables in carrier shall be at least 10% of cable diameter.

Avoid multi-layer routing of  cable(i.e., avoid arranging one cable on another without using a spacer). If the space is limited and it need place one cable on another, it must utilize a spacer or shelves!

If there is a large difference in diameter of cables, and the diameter difference of cables is greater than 20% wiring size, it is necessary to utilize a partition between  two cables to avoid the play of cables or winding each other.

The filling coefficient of cables in carrier should be controlled below 60% (less than 30% is more ideal).

In order to ensure the balance of cable carrier in motion, the weight of the cable distributed on carrier should be as balanced as possible; It's recommended that the heavier cable be located on both sides and the lighter one be located in the middle.

Note: A:	After a period of operation, periodically check the position of cable; The check must be performed after any push/pull movement; If there is any displacement, it shall be improved and adjusted in time.

B: For vertically suspended cable carrier, there must be more free space inside the bracket, because the cable will be stretched during operation due to gravity. After a period of operation and use, regularly check the length and position of cable; The check must be performed after any push/pull movement; If there is any displacement, it shall be improved and adjusted in time.

Error                              Error                                    Error

Right                                              Right

3. If cable carrier is damaged or broken, the cable also needs to be replaced, because damages from excessive stretching can't be prevented or repaired.

4. Bending radius of the flexible cable used in cable carrier should conform to technical parameters in the product specification.

5. After a period of operation, regularly check the position of cable, and often confirm that the cable can follow the carrier movement without any force; The inspection must be performed after push-pull movement; If any displacement or force exists, it shall be improved and adjusted in time.

6. To lay multi-core high-flexible cables with a diameter less than 10mm, it's recommended to use a guide duct.

7. For freely moving pipes, guide duct or partition should be mounted.

8. Adjacent cables can't cross over each other, so the gap over cable should not be greater than 50% of the diameter of adjacent cables.

The rules below must be followed:

Rule 1: If D1+D2>1.2 times the inner height of cable carrier, it's unnecessary to separate the two cables. The cables can't cross over or wrap each other (shown as  below).

Rule 2: If D1+D2<1.2 times the inner height of cable carrier, separator must be used to reduce the gap (shown as below).

- Chapter 2 Wiring

2.1 Description on System Wiring

2.1.1 SIZE A System wiring diagram

Electromagnetic contactor

Control servo motor band-type brake ON/OFF by servo DO port.

Safety Device

Regenerative Resistor

Absorbs the regenerative feedback energy from the drive. When using an external regenerative resistor, please secure it to non-flammable materials such as metal heat sinks.

Slave station

Connect to the next servo drive or other slave station devices.

Sevo motor

CN6 Servo encoder cable

Servo power line

CN7 full closed-loop

encoder cable

CN4 EtherCAT communication cable

CN3 EtherCAT communication cable

Expandable IO

Master station controller

Residual Current

Circuit Breaker

Power

Switching Power Supply

A dedicated DC 24V power supply for the brake. Do not connect other electrical devices.

Servo drive

Servo holding brake line

CN5伺服IO线

PC

Connects to a personal computer, allowing debugging of the servo drive through upper computer debugging software.

Line Reactor (L)

To reduce higher harmonic currents in the power input.

Magnetic Contactor (MC)

For connecting and disconnecting the main power supply to the servo drive. Use after installing a coil surge absorber.

It is strictly prohibited to use the magnetic contactor for motor run/stop operations.

Noise Filter (NF)

To prevent interference from power lines and reduce the impact of the servo drive on other sensitive equipment.

Molded Case Circuit Breaker (MCCB)

To prevent damage to the servo drive due to excessive instantaneous current during power switching or short circuits, please select a circuit breaker that matches the power capacity as an over-current protection device.

Figure 2-1 SV3H SIZE A system wiring diagram

	As for single-phase power input, power supply can be connected to any 2 terminals of L1/L2/L3.

	Pleae use a circuit breaker with leakage protection and noise filter between power supply and the main power terminal;

	Voltage and power of band-type brake power supply should meet the requirement on parameters of motor band-type brake.

	SIZE A has no built-in regenerative resistor, in case of the application with external regenerative resistor, please select the appropriate resistance. It can't be less than the allowable minimum external resistance, otherwise it might cause damage to the driver.

	CN3 is the EtherCAT communication input, connected to the controller or the previous servo; CN4 is the EtherCAT communication output, connected to the next servo.

2.1.2 SIZE B System wiring diagram

Power

Residual Current

Circuit Breaker

Molded Case Circuit Breaker (MCCB)

To prevent damage to the servo drive due to excessive instantaneous current during power switching or short circuits, please select a circuit breaker that matches the power capacity as an over-current protection device.

CN5 servo IO cable

CN7 full closed-loop encoder cable

CN3 EtherCAT communication cable

Slave station

Connect to the next servo drive or other slave station devices.

Sevo motor

Servo power line

Servo holding brake line

Regenerative resistance

Absorb the regenerative feedback energy of drive. When using an external regenerative resistor, please fix it on  non-combustible material such as metal cooling plate.

Electromagnetic contactor

Control servo motor band-type brake ON/OFF by servo DO port.

CN6 Servo encoder cable

CN4 EtherCAT communication cable

Safety Device

Servo drive

Switching Power Supply

A dedicated DC 24V power supply for the brake. Do not connect other electrical devices.

PC

Connects to a personal computer, allowing debugging of the servo drive through upper computer debugging software.

Line Reactor (L)

To reduce higher harmonic currents in the power input.

Magnetic Contactor (MC)

For connecting and disconnecting the main power supply to the servo drive. Use after installing a coil surge absorber.

It is strictly prohibited to use the magnetic contactor for motor run/stop operations.

Noise Filter (NF)

To prevent interference from power lines and reduce the impact of the servo drive on other sensitive equipment.

Expandable IO

Master station controller

Figure 2-2 SV3H SIZE B system wiring diagram

	As for single-phase power input, power supply can be connected to any 2 terminals of L1/L2/L3.

	Pleae use a circuit breaker with leakage protection and noise filter between power supply and the main power terminal;

	Voltage and power of band-type brake power supply should meet the requirement on parameters of motor band-type brake.

	SIZE B has the built-in regenerative resistor, in case of the application with external regenerative resistor, please select the appropriate resistance. It can't be less than the allowable minimum external resistance, otherwise it might cause damage to the driver. Remove jumper bar between P and RB, connect it to both ends of P and B.

	CN3 is the EtherCAT communication input, connected to the controller or the previous servo; CN4 is the EtherCAT communication output, connected to the next servo.

2.1.3 SIZE C System wiring diagram

Power

Master station controller

Expandable IO

Slave station

Connect to the next servo drive or other slave station devices.

PC

Connects to a personal computer, allowing debugging of the servo drive through upper computer debugging software.

Switching Power Supply

A dedicated DC 24V power supply for the brake. Do not connect other electrical devices.

Electromagnetic contactor

Control servo motor band-type brake ON/OFF by servo DO port.

Residual Current

Circuit Breaker

Line Reactor (L)

To reduce higher harmonic currents in the power input.

Molded Case Circuit Breaker (MCCB)

To prevent damage to the servo drive due to excessive instantaneous current during power switching or short circuits, please select a circuit breaker that matches the power capacity as an over-current protection device.

Noise Filter (NF)

To prevent interference from power lines and reduce the impact of the servo drive on other sensitive equipment.

Servo power line

CN3 EtherCAT communication cable

CN4 EtherCAT communication cable

CN7 full closed-loop encoder cable

Regenerative resistance

Absorb the regenerative feedback energy of drive. When using an external regenerative resistor, please fix it on  non-combustible material such as metal cooling plate.

Magnetic Contactor (MC)

For connecting and disconnecting the main power supply to the servo drive. Use after installing a coil surge absorber.

It is strictly prohibited to use the magnetic contactor for motor run/stop operations.

Sevo motor

CN6 Servo encoder cable

Servo drive

Servo holding brake line

CN5 servo IO cable

Safety Device

Figure 2-3 SV3H SIZE C system wiring diagram

	SIZE-C has 2 models: 220VAC and 380VAC. Please select the power supply voltage upon the actual model and specification. Wrong power supply may cause drive damage.

	As for single-phase power input, power supply can be connected to any 2 terminals of L1/L2/L3.

	Pleae use a circuit breaker with leakage protection and noise filter between power supply and the main power terminal;

	Voltage and power of band-type brake power supply should meet the requirement on parameters of motor band-type brake.

	SIZE C has the built-in regenerative resistor, in case of the application with external regenerative resistor, please select the appropriate resistance. It can't be less than the allowable minimum external resistance, otherwise it might cause damage to the driver. Remove jumper bar between P and RB, connect it to both ends of P and B.

	CN3 is the EtherCAT communication input, connected to the controller or the previous servo; CN4 is the EtherCAT communication output, connected to the next servo.

2.1.4 SIZE D System wiring diagram

Power

Residual Current

Circuit Breaker

Master station controller

Expandable IO

PC

Connects to a personal computer, allowing debugging of the servo drive through upper computer debugging software.

Servo holding brake line

Servo drive

Servo power line

Sevo motor

CN6 Servo encoder cable

CN7 full closed-loop encoder cable

CN5 servo IO cable

安全装置

Slave station

Connect to the next servo drive or other slave station devices.

Regenerative resistance

Absorb the regenerative feedback energy of drive. When using an external regenerative resistor, please fix it on  non-combustible material such as metal cooling plate.

Switching Power Supply

A dedicated DC 24V power supply for the brake. Do not connect other electrical devices.

Electromagnetic contactor

Control servo motor band-type brake ON/OFF by servo DO port.

Line Reactor (L)

To reduce higher harmonic currents in the power input.

Magnetic Contactor (MC)

For connecting and disconnecting the main power supply to the servo drive. Use after installing a coil surge absorber.

It is strictly prohibited to use the magnetic contactor for motor run/stop operations.

Noise Filter (NF)

To prevent interference from power lines and reduce the impact of the servo drive on other sensitive equipment.

Molded Case Circuit Breaker (MCCB)

To prevent damage to the servo drive due to excessive instantaneous current during power switching or short circuits, please select a circuit breaker that matches the power capacity as an over-current protection device.

Figure 2-4 SV3H SIZE D system wiring diagram

	Pleae use a circuit breaker with leakage protection and noise filter between power supply and the main power terminal;

	Voltage and power of band-type brake power supply should meet the requirement on parameters of motor band-type brake.

	SIZE D has the built-in regenerative resistor, in case of the application with external regenerative resistor, please select the appropriate resistance. It can't be less than the allowable minimum external resistance, otherwise it might cause damage to the driver. Remove jumper bar between P and RB, connect it to both ends of P and B.

	CN3 is the EtherCAT communication input, connected to the controller or the previous servo; CN4 is the EtherCAT communication output, connected to the next servo.

2.2 Servo Drive Port Definition

| CN1 | Serial communication port |  |
| --- | --- | --- |
|  | Pin No. | Signal Name |
| MINI USB | 1 | VBUS |
|  | 2 | D- |
|  | 3 | D+ |
|  | 4 | - |
|  | 5 | GND |
| CN6 | Encoder port |  |
|  | Pin No. | Signal Name |
| Recommended model 3M:36210-0100PL | 1 | 5V |
|  | 2 | GND |
|  | 3 | - |
|  | 4 | - |
|  | 5 | - |
|  | 6 | - |
|  | 7 | - |
|  | 8 | - |
|  | 9 | SD+ |
|  | 10 | SD- |
| CN7 | Second encoder port |  |
|  | Pin No. | Signal Name |
| Recommended model 3M:36210-0100PL | 1 | 5V |
|  | 2 | GND |
|  | 3 | SEC_A+ |
|  | 4 | SEC_A- |
|  | 5 | SEC_B+ |
|  | 6 | SEC_B- |
|  | 7 | SEC_Z+ |
|  | 8 | SEC_Z- |
|  | 9 | - |
|  | 10 | MTR_TEMP |

| CN2 | Security port |  |  |  |
| --- | --- | --- | --- | --- |
|  | Pin No. | Signal name |  |  |
| Recommended model TE: 2013595-1 | 1 | Internal power negative |  |  |
|  | 2 | Internal power positive |  |  |
|  | 3 | STO1- |  |  |
|  | 4 | STO1+ |  |  |
|  | 5 | STO2- |  |  |
|  | 6 | STO2+ |  |  |
|  | 7 | STO_OUT- |  |  |
|  | 8 | STO_OUT+ |  |  |
| EtherCAT communication port |  |  |  |  |
| Recommended model RJ45 | CN3（IN） |  | CN4(OUT) |  |
|  | Pin No. | Signal Name | Pin No. | Signal Name |
|  | 1 | TX+ | 1 | TX+ |
|  | 2 | TX- | 2 | TX- |
|  | 3 | RX+ | 3 | RX+ |
|  | 4 | - | 4 | - |
|  | 5 | - | 5 | - |
|  | 6 | RX- | 6 | RX- |
|  | 7 | - | 7 | - |
|  | 8 | - | 8 | - |
| CN5 | Control signal port |  |  |  |
|  | Pin No. | Signal Name | Pin No. | Signal Name |
| Recommended model SCSI-20P | 1 | DO1+ | 11 | DI6 |
|  | 2 | DO1- | 12 | HDI1 |
|  | 3 | DO3+ | 13 | HDI2 |
|  | 4 | DO3- | 14 | DO2+ |
|  | 5 | DI1 | 15 | DO2- |
|  | 6 | DI_COM | 16 | GND |
|  | 7 | DI2 | 17 | PAO+ |
|  | 8 | DI3 | 18 | PAO- |
|  | 9 | DI4 | 19 | PBO+ |
|  | 10 | DI5 | 20 | PBO- |

Figure 2–5  Servo Drive Port Definition

2.3 Definition and wiring description of power terminals

2.3.1 Definition of power supply and motor terminals

Table 2-1 Definition of SIZE A main loop ports

| Port classification | Connector | Port No. | Port Code | Port function | Remarks |
| --- | --- | --- | --- | --- | --- |
| Power input |  | 1 | L1C | Control power input | Single-phase AC200V ~ 240V, -10 ~ +10%, 50/60Hz |
|  |  | 2 | L2C |  |  |
|  |  | 3 | L1 | Main power input | Single-phase/three-phase AC200V ~ 240V, -10 ~ +10%, 50/60Hz |
|  |  | 4 | L2 |  |  |
|  |  | 5 | L3 |  |  |
| Motor dynamics |  | 1 | P | Regenerative function and DC bus terminals | When the regeneration function is used, use Port P/B, as detailed in Section 2.5. |
|  |  | 2 | B |  | For common DC bus applications, use P (DC+)/N (DC-) terminals, as described in Section 2.4. |
|  |  | 3 | N |  |  |
|  |  | 4 | U | Motor drive | Connect U/V/W three phases of the servo motor. |
|  |  | 5 | V |  |  |
|  |  | 6 | W |  |  |

Table 2-1 Definition of SIZE B main loop ports

| Port classification | Connector | Port No. | Port Code | Port function | Remarks |
| --- | --- | --- | --- | --- | --- |
| Power input |  | 1 | L1C | Control power input | Single-phase AC200V ~ 240V, -10 ~ +10%, 50/60Hz |
|  |  | 2 | L2C |  |  |
|  |  | 3 | L1 | Main power input | Single-phase/three-phase AC200V ~ 240V, -10 ~ +10%, 50/60Hz |
|  |  | 4 | L2 |  |  |
|  |  | 5 | L3 |  |  |
| Motor dynamics |  | 1 | P | Regenerative function and DC bus terminals | When the regeneration function is used, use Port P/RB/B, as detailed in Section 2.5. |
|  |  | 2 | RB |  |  |
|  |  | 3 | B |  | For common DC bus applications, use P (DC+)/N (DC-) terminals, as described in Section 2.4. |
|  |  | 4 | N |  |  |
|  |  | 5 | U | Motor drive | Connect U/V/W three phases of the servo motor. |
|  |  | 6 | V |  |  |
|  |  | 7 | W |  |  |

Table 2-1 Definition of SIZE C main loop ports

| Port classification | Connector | Port No. | Port Code | Port Functions | Remarks |
| --- | --- | --- | --- | --- | --- |
| Power input |  | 1 | L1C | Control power supply Input | 220V model: single-phase 200V~240VAC, -10 ~ +10%, 50/60Hz 380V model: single-phase 380V~440VAC, -10 ~ +10%, 50/60Hz |
|  |  | 2 | L2C |  |  |
|  |  | 3 | L1/R | Main power input | 220V model: single-phase/three-phase 200V~240VAC, -10 ~ +10%, 50/60Hz 380V model: three-phase 380V~440VAC, -10 ~ +10%, 50/60Hz |
|  |  | 4 | L2/S |  |  |
|  |  | 5 | L3/T |  |  |
|  |  | 6 | P | Regenerative function and DC bus terminals | When the regeneration function is used, use Port P/RB/B, as detailed in Section 2.5. |
|  |  | 7 | RB |  |  |
|  |  | 8 | B |  | For common DC bus applications, use P (DC+)/N (DC-) terminals, as described in Section 2.4. |
|  |  | 9 | N |  |  |
| Motor dynamics |  | 1 | U | Motor drive | Connect U/V/W three phases and earth PE terminal of servo motor. |
|  |  | 2 | V |  |  |
|  |  | 3 | W |  |  |
|  |  | 4 | PE |  |  |

Table 2-1 Definition of SIZE D main loop ports

| Connector | Port No. | Port Code | Port function | Remarks |
| --- | --- | --- | --- | --- |
|  | 1 | L1C | Control power input | Single-phase 380V~440VAC, -10 ~ +10%, 50/60Hz |
|  | 2 | L2C |  |  |
|  | 3 | R | Main power input | Three-phase 380V~440VAC, -10 ~ +10%, 50/60Hz |
|  | 4 | S |  |  |
|  | 5 | T |  |  |
|  | 6 | U | Motor drive | Connect U/V/W three phases of the servo motor. |
|  | 7 | V |  |  |
|  | 8 | W |  |  |
|  | 9 | N2 | Regeneration function, Common DC bus port, external reactance | When the regeneration function is used, use Port P/RB/B, as detailed in Section 2.5. |
|  | 10 | N1 |  | For common DC bus applications, use P (DC+)/N2 (DC-) terminals, as described in Section 2.4. |
|  | 11 | P |  |  |
|  | 12 | RB |  | N1/N2 are shorted by default. It shall suppress high-order harmonics of power supply, remove the short-circuit wire and connect DC reactance between N1 and N2. |
|  | 13 | B |  |  |

2.3.2 Wiring of circlip terminals

Power terminal is circlip connector for quick wiring. During wiring, perform the following process to ensure reliable connection.

Press operating lever to press down the internal spring.

Peel off the insulation layer of wires, and the bare wire length is 10mm.

Release operating lever, gently pull the wire, and confirm that the connection is firm, then the wiring is completed.

Insert all bare wires into the connector.

Figure 2-6 Circlip terminal wiring

2.3.3 Sample of Main Circuit Wiring

Contactor

Single/three-phase 220V power input

Surge absorber

Alarm indicatro

Run button

Stop button

Noise filter

Figure 2-7 Sample for SIZE A main circuit wiring

	Do not connect power supply input (L1/L2/L3) to output terminal (U/V/W) for motor;

	Wiring of mootor output terminal(U/V/W) is consistent with motor (U/V/W) wiring; The sequence can't be wrong;

	Do not put power cable and signal cable together, and keep a distance more than 30cm.

	Do not switch servo power supply frequently; Otherwise, capacitor will be charged frequently inside the servo, and pre-charging circuit will be overloaded, resulting in performance degradation. Please keep switching frequency below 1 time per minute;

	After the servo is powered off, there might still be residual high voltage inside servo. Only after power-off for 15 minutes and wait until the power indicator is off,   can the wiring be executed.

Contactor

Single/three-phase 220V power input

Alarm indicatro

Surge absorber

Run button

Stop button

Noise filter

Figure 2-8 Sample for SIZE B main circuit wiring

	Do not connect power supply input (L1/L2/L3) to output terminal (U/V/W) for motor;

	Wiring of mootor output terminal(U/V/W) is consistent with motor (U/V/W) wiring; The sequence can't be wrong;

	Do not put power cable and signal cable together, and keep a distance more than 30cm.

	Do not switch servo power supply frequently; Otherwise, capacitor will be charged frequently inside the servo, and pre-charging circuit will be overloaded, resulting in performance degradation. Please keep switching frequency below 1 time per minute;

	After the servo is powered off, there might still be residual high voltage inside servo. Only after power-off for 15 minutes and wait until the power indicator is off,   can the wiring be executed.

Contactor

Run button

Surge absorber

Alarm indicatro

Stop button

Single/three-phase  power input

Noise filter

Figure 2-9 Sample for SIZE C main circuit wiring

	Do not connect power supply input (L1/R, L2/S, L3/T) to  output terminal (U/V/W) of motor;

	Wiring of mootor output terminal(U/V/W) is consistent with motor (U/V/W) wiring; The sequence can't be wrong;

	Do not put power cable and signal cable together, and keep a distance more than 30cm.

	Do not switch servo power supply frequently; Otherwise, capacitor will be charged frequently inside the servo, and pre-charging circuit will be overloaded, resulting in performance degradation. Please keep switching frequency below 1 time per minute;

	After the servo is powered off, there might still be residual high voltage inside servo. Only after power-off for 15 minutes and wait until the power indicator is off,   can the wiring be executed.

Alarm indicatro

Surge absorber

Contactor

Noise filter

Stop button

Run button

Three-phase 380V Power Input

Figure 2-10 SIZE D main circuit wiring sample

	Do not connect the power input (R, S, T) to the output terminal (U, V, W) of motor;

	Wiring of mootor output terminal(U/V/W) is consistent with motor (U/V/W) wiring; The sequence can't be wrong;

	Do not put power cable and signal cable together, and keep a distance more than 30cm.

	Do not switch servo power supply frequently; Otherwise, capacitor will be charged frequently inside the servo, and pre-charging circuit will be overloaded, resulting in performance degradation. Please keep switching frequency below 1 time per minute;

	After the servo is powered off, there might still be residual high voltage inside servo. Only after power-off for 15 minutes and wait until the power indicator is off,   can the wiring be executed.

2.3.4 Specification of Main Loop Cable

Table 2-5 Recommended cable specifications for main loop

| Series | Drive Model | L1C、L2C |  | L1、L2、L3 (R、S、T) |  | P、B |  | U、V、W |  | PE |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | mm2 | AWG | mm2 | AWG | mm2 | AWG | mm2 | AWG | mm2 | AWG |
| Single-phase 220V power supply |  |  |  |  |  |  |  |  |  |  |  |
| SIZE A | S1R6 | 2x0.5 | 20 | 2x0.5 | 20 | 2x2.0 | 14 | 3x0.5 | 20 | 0.5 | 20 |
|  | S2R8 | 2x0.5 | 20 | 2x0.5 | 20 | 2x2.0 | 14 | 3x0.5 | 20 | 0.5 | 20 |
| SIZE B | S5R5 | 2x0.75 | 18 | 2x0.75 | 18 | 2x2.0 | 14 | 3x0.75 | 18 | 0.75 | 18 |
| SIZE C | S7R6 | 2x0.75 | 18 | 2x1.5 | 15 | 2x2.0 | 14 | 3x1.5 | 15 | 1.5 | 15 |
|  | S012 | 2x0.75 | 18 | 2x1.5 | 15 | 2x2.0 | 14 | 3x1.5 | 15 | 1.5 | 15 |
|  | S014 | 2x0.75 | 18 | 2x2.0 | 14 | 2x2.0 | 14 | 3x2.0 | 14 | 2.0 | 14 |
| Three-phase 220V power supply |  |  |  |  |  |  |  |  |  |  |  |
| SIZE A | S1R6 | 2x0.5 | 20 | 3x0.5 | 20 | 2x2.0 | 14 | 3x0.5 | 20 | 0.5 | 20 |
|  | S2R8 | 2x0.5 | 20 | 3x0.5 | 20 | 2x2.0 | 14 | 3x0.5 | 20 | 0.5 | 20 |
| SIZE B | S5R5 | 2x0.75 | 18 | 3x0.75 | 18 | 2x0.75 | 18 | 3x0.75 | 18 | 0.75 | 18 |
| SIZE C | S7R6 | 2x0.75 | 18 | 3x1.00 | 17 | 2x1.00 | 17 | 3x1.0 | 17 | 1.00 | 17 |
|  | S012 | 2x0.75 | 18 | 3x1.5 | 15 | 2x2.0 | 14 | 3x1.5 | 15 | 1.5 | 15 |
|  | S014 | 2x0.75 | 18 | 3x2.0 | 14 | 2x2.0 | 14 | 3x2.0 | 14 | 2.0 | 14 |
| SIZE D | U018 | 2x0.75 | 18 | 3x2.50 | 13 | 3x2.5 | 13 | 3x2.5 | 13 | 2.50 | 13 |
|  | U022 | 2x0.75 | 18 | 3x4.0 | 11 | 3x4.0 | 11 | 3x4.0 | 11 | 4.00 | 11 |
|  | U027 | 2x0.75 | 18 | 3x6.0 | 9 | 2x6.0 | 9 | 3x6.0 | 9 | 6.0 | 9 |
| Three-phase 380V power supply |  |  |  |  |  |  |  |  |  |  |  |
| SIZE C | T3R5 | 2x0.75 | 18 | 3x0.75 | 18 | 2x0.75 | 18 | 3x0.75 | 18 | 0.75 | 18 |
|  | T5R4 | 2x0.75 | 18 | 3x0.75 | 18 | 2x0.75 | 18 | 3x0.75 | 18 | 0.75 | 18 |
|  | T8R4 | 2x0.75 | 18 | 3x0.75 | 18 | 2x1.0 | 17 | 3x1.0 | 17 | 1.0 | 17 |
|  | T012 | 2x0.75 | 18 | 3x1.0 | 17 | 2x1.5 | 15 | 3x1.5 | 15 | 1.5 | 15 |
| SIZE D | T017 | 2x0.75 | 18 | 3x1.5 | 15 | 3x2.0 | 14 | 3x2.0 | 14 | 2.0 | 14 |
|  | T021 | 2x0.75 | 18 | 3x4.0 | 11 | 2x4.0 | 11 | 3x4.0 | 11 | 2.5 | 11 |
|  | T026 | 2x0.75 | 18 | 3x6.0 | 9 | 2x6.0 | 9 | 3x6.0 | 9 | 6.0 | 9 |

2.4 Common DC Bus Cable Wiring

In some multi-axis application scenarios, the motion state of each axis is different, some servo motors work in the braking power-generation state, and other servo motors work in the energy-consumption state. Use the common DC bus technology for energy interaction, the energy generated by braking motor can be supplied to the energy-consuming motors, other than dissipating by regenerative resistors, so as to realize the energy-saving effect.

Figure 2-11 Sample of Servo Common Bus

	Only those drivers of the same specification are allowed to share bus in this way. To utilize common bus with drivers of different specifications may burn the drivers directly. DC bus P and N of the servo are connected in parallel so as to realize energy transfer among drives;

	All drives must use the same power input. It's forbidden to utilize the power supply of different voltage and phase number. Otherwise, it might cause drive damage;

	All drives mu-st be powered ON/OFF at the same time. Otherwise, it might cause drive damage;

	Voltage level and phase sequence of all drives with common bus shall be consistent. Otherwise, it might cause drive damage and start a fire.

	While using common bus, only after completing the configuration of common bus for all drives, can it be allowed for servo drive operation;

	After the drive is powered on, when drive Rdy state occurs, drive DO outputs the ready signal, control the relay swich-on so as to realize the common bus connection;

While drive PN is connected to bus, in order to avoid excessive current in case of single-drive exception, please connect a fast fuse between bus bars.

2.5 Description of regenerative resistance wiring

2.5.1 Connecting regenerative resistance wiring

No built-in regenerative resistor, external regenerative resistor

PRB disconnected, external resistor connected between PB, configured as external regenerative resistor

PRB short-circuited, configured as built-in regenerative resistor

Figure 2-12 Regenerating resistor wiring

	For models without built-in regenerative resistor, external regenerative resistors are are connected between P-B;

	Do not connect any regenerated resistor between bus P and N, otherwise it might cause damage to drives and start a fire;

	For models with built-in regenerative resistor, short-circuit P-RB terminals while using the built-in resistor. While using the external resistor, remove terminal jumper between P-RB and connect the external resistor between P-B;

	In case of external regenerative resistor, select an appropriate resistor according to Section 2.5.3. Do not use any resitor below Min. external resistance allowed in Table 2-6. Otherwise, it might cause drive damage.

	Before using the servo, please confirm that the parameters related to external regeneration resistor have been properly set: P02.34 (regeneration resistor selection), P02.39 (power of external regeneration resistor), P02.40 (external regeneration resistance).

2.5.2 Specifications of regenerative resistors

Table 2-6 Specifications of regenerative resistors

| Servo drive Rated voltage, current |  | Specifications on internal regenerative resistance |  |  | Minimum external resistance | Maximum braking energy absorption of capacitor/Ec |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Resistance value | Resistance power | Applicable power/Pr |  |  |
| Single/three-phase 220V | 1.6A | - | - | - | 50Ω | 10J |
|  | 2.8A | - | - | - | 45Ω | 15J |
|  | 5.5A | 50Ω | 50W | 25W | 40Ω | 23J |
|  | 7.6A | 25Ω | 80W | 40W | 20Ω | 32J |
|  | 12.0A | 25Ω | 80W | 40W | 15Ω | 47J |
|  | 14.0A | 25Ω | 80W | 40W | 15Ω | 57J |
| Three-phase 220V | 18.0A | 20Ω | 100W | 50W | 10Ω | 64J |
|  | 22.0A | 20Ω | 100W | 50W | 10Ω | 71J |
|  | 27.0A | 20Ω | 100W | 50W | 10Ω | 95J |
| Three-phase 380V | 3.5A | 100Ω | 80W | 40W | 80Ω | 39J |
|  | 5.4A | 100Ω | 80W | 40W | 60Ω | 39J |
|  | 8.4A | 50Ω | 80W | 40W | 45Ω | 57J |
|  | 12.0A | 50Ω | 80W | 40W | 45Ω | 57J |
|  | 17.0A | 35Ω | 100W | 50W | 35Ω | 114J |
|  | 21.0A | 35Ω | 100W | 50W | 25Ω | 114J |
|  | 26.0A | 35Ω | 100W | 50W | 25Ω | 114J |

2.5.3 Selection and calculation of regenerative resistance

7.Calculate regenerative resistor power

6.Eb(51J)>EC(26J), regenerative energy greater than capacitor absorption energy, need to use braking resistor.

Use built-in regenerative resistor

9.Regenerative resistor power

P=Pb/70% =68.5W, select 70W resistor.

8.Compare Pb(50W)>Pr(0W), need to use external regenerative resistor.

Pr value refer to above regenerative resistor specification table.

Need to use regenerative resistor

Calculate required braking resistor power Pb

Use external regenerative resistor

Select regenerative resistor with 70% derating

End

1.Confirm motor reciprocating motion cycle 1s.

3.Confirm load inertia ratio 5.

5.Refer to above regenerative resistor specification table, EC=26J.

Example: Drive rated current 5.5A, reciprocating motion with 1s cycle, speed 3000rpm, rotor inertia 1.72kg.cm2, load inertia ratio 5.

Calculate regenerative energy

Eb(J)

Determine load to motor inertia ratio N

Determine motor speed V（rpm）

Determine reciprocating motion cycle T(s)

Start

4.Calculate regenerative energy

Determine maximum braking energy EC that capacitor can absorb, EC value refer to above braking resistor specification table.

J is motor rotor moment of inertia （kg.cm2）;V is motor speed（rpm）

Refer to inertia identification content in adjustment chapter.

Determined based on actual operating conditions.

Determined based on actual operating conditions.

2.Confirm motor speed 3000rpm

Yes

Yes

No

No

No need to use regenerative resistor

Determine capacitor absorption energy Ec(J)

Figure 2-13 Illustration of regenerative resistor selection

2.6 Wiring of band-type brake

Prevent movement due to external force

Motor with brake

Prevent movement due to self-weight when servo power off

Motor with brake

Figure 2-14 Schematic diagram of band-type brake

Band-type brake is used to stop the unexpected movement of moving load(e.g., falling under gravity) when servo system is not activated (e.g., the servo system is powered off) to prevent motors from moving unexpectedly after power off due to its own weight or external force.

Servo drive

Fuse

Sevo motor

Surge absorber

Emergency stop button interlock (optional)

Brake unit

Figure 2-15 Layout diagram of band-type brake

	Built-in band-type brake of servo motor is only used for shutdown of motors, and frequent use for emergency stop would shorten its service life. Only if motor speed is less than 20rpm, band-type brake can be powered off;

	It's recommended to use independent power supply to prevent the abnormal voltage drop from other electrical appliances, that could result in misoperation of band-type brake.

	Different power sources are used to power the brake and brake control signal separately in order to prevent  electronic devices from electromagnetic interference.

2.7 Description of Control Signal CN5 Port Wiring

2.7.1 Definition of Control Signal CN5 Port

Table 2-7 Definition of CN5 ports on control signal

| IO Interface Connector (CN5) | Module name | Signal name | Pin No. | Default function |
| --- | --- | --- | --- | --- |
| SCSI 20P | Digital input | DI1 | 5 | Forward limit |
|  |  | DI2 | 7 | Reverse limit |
|  |  | DI3 | 8 | Origin switch |
|  |  | DI4 | 9 | Undefined |
|  |  | DI5 | 10 | Undefined |
|  |  | DI6 | 11 | Undefined |
|  |  | HDI1 | 12 | Probe 1 |
|  |  | HDI2 | 13 | Probe 2 |
|  |  | DI_COM | 6 | DI common port |
|  | Digital output | DO1+ | 1 | Band-type brake signal |
|  |  | DO1- | 2 |  |
|  |  | DO2+ | 14 | Servo operation |
|  |  | DO2- | 15 |  |
|  |  | DO3+ | 3 | Servo fault output |
|  |  | DO3- | 4 |  |
|  | Frequency division output | PAO+ | 17 | A phase frequency division output signal |
|  |  | PAO- | 18 |  |
|  |  | PBO+ | 19 | B phase frequency division output signal |
|  |  | PBO- | 20 |  |
|  |  | GND | 16 | Frequency division output signal ground |
|  | Shell | — | — | Connect the cable shielding layer |

2.7.2 Description of Digital Input Wiring

Servo drive

Figure 2-16 Connection wiring of digital input cable

(1) When upper device is relay output

Relay

External 0V

Relay

Servo drive

Servo drive

External +24V DC

External 0V

External +24V DC

Figure 2-17 Figure 1-1	Digital input wiring diagram (relay output)

(2) When upper device is open collector

External +24V DC

Servo drive

Servo drive

External +24V DC

External 0V

External 0V

Servo drive

Servo drive

External 0V

External 0V

External +24V DC

External +24V DC

Figure 2-18 Wiring diagram of digital input(open collector)

	Mixing input of PNP and NPN is not supported.

2.7.3 Description of Digital Output Wiring

Servo drive

Figure 2-19 Digital output wiring

	The maximum allowable voltage and current of optocoupler output circuit inside servo drive are as follows:

	Voltage: DC 30V(Max);

	Current: DC 50mA(Max.)

(1) When upper device is relay input

| Servo drive Relay External +24V DC External 0V | Servo drive External 0V Relay External +24V DC |
| --- | --- |
| Figure 2-20 Schematic diagram of digital output wiring (relay input, correct wiring) |  |
| Relay Flywheel diode not used External +24V DC Servo drive External 0V | Relay Flywheel diode polarity error External +24V DC Servo drive External 0V |
| Figure 2-21 Schematic diagram of digital output wiring (relay input, error wiring) |  |

(2) When upper device is optocoupler input

| External 24V power supply- Servo drive Optocoupler External 24V power supply+ | External 24V power supply+ External 24V power supply- Servo drive Optocoupler |
| --- | --- |
| Figure 2-22 Wiring diagram of digital output(optical coupling input, correct wiring) |  |
| External 24V power supply- External 24V power supply+ Current limiting resistor not connected Servo drive Optocoupler Figure 2-23 Wiring diagram of digital output(optocoupler input, error wiring) |  |

2.7.4 Description of Frequency Division Output Wiring

frequency divider circuit

Servo drive

RS422 line receiver

Long line driver

Figure 2-24 Frequency division output wiring

	For receiving signal of output pulse, please use RS422 long line receiver (AM26C32 or equivalent);

	Max. output current of frequency division output is 20mA;

	Shielded twisted-pair cable is used. The shielding layer is connected to PE; Ground of frequency division output signal is connected to the receiving terminal.

2.8 Encoder interface definition and wiring description

2.8.1 Motor encoder

Table 2-8 Definition of drive encoder interfaces

| Motor encoder Connector (CN6) | Module name | Signal name | Pin No. | Wiring method |
| --- | --- | --- | --- | --- |
| IEEE 1394 10P | Motor encoder | E5V | 1 | Twisted pair |
|  |  | E0V | 2 |  |
|  |  | — | 3 | — |
|  |  | — | 4 | — |
|  |  | — | 5 | — |
|  |  | — | 6 | — |
|  |  | — | 7 | — |
|  |  | — | 8 | — |
|  |  | PS+ | 9 | Twisted pair |
|  |  | PS- | 10 |  |
|  |  | PE | Shell | Connect the cable shielding layer |

	This port is used for connection between drive and motor encoder. During use, the distance between encoder cable and main circuit wire should be more than 30cm or above. Do not bundle them together with conduit.

	The cable length between drive and motor is less than 20m. In case of the requirement above 20m, please negotiate with sellers.

	Input voltage range of the encoder-end connector is 4.75V to 5.25V DC. Please select the appropriate wire. For 10m or less, use shielded twisted pair with a cross-sectional area of 0.18 mm²(AWG24) or above; For 10m or above, use shielded twisted pair with a cross-sectional area of 0.32 mm²(AWG22) or above.

Servo motor encoder

Servo driveCN6

Figure 2-25 Motor encoder signal connection

2.8.2 Full closed-loop encoder

Table 2-9 Definition of fully closed-loop encoder interface

| Fully Closed loop encoder Connector (CN7) | Module name | Signal name | Pin No. | Wiring method | Functions |
| --- | --- | --- | --- | --- | --- |
| IEEE 1394 10P | Fully closed-loop encoder | 5V | 1 | Twisted pair | Encoder power supply |
|  |  | GND | 2 |  |  |
|  |  | SEC_A+ | 3 | Twisted pair | Second encoder input interface |
|  |  | SEC_A- | 4 |  |  |
|  |  | SEC_B+ | 5 | Twisted pair |  |
|  |  | SEC_B- | 6 |  |  |
|  |  | SEC_Z+ | 7 | Twisted pair |  |
|  |  | SEC_Z- | 8 |  |  |
|  |  | — | 9 |  | — |
|  |  | MTR_TEMP | 10 |  | Input of motor temperature sensor |
|  |  | Shell | — | Connect the cable shielding layer | Shielded |

	This port is used for connection between drive and motor encoder. During use, distance between the cable and main circuit wire should be 30cm;

	Please use shielded twisted pair cable for encoders;

	Input of fully closed-loop encoder is differential input, and its Max. input frequency and Min. pulse width are shown in the table below:

Table 2-10 Input signal features of fully closed loop encoder

| Pulse method | Max. frequency (pps) | Min. pulse width (μs) |
| --- | --- | --- |
| Differential input | 4M | 0.125 |

Ecoder

Servo driveCN7

Figure 2-26 Differential signal input of fully closed loop encoder

2.8.3 Input of motor temperature sensor

Servo driveCN7

Motor temperature sensor

Figure 2-27 Wiring diagram of motor temperature sensor

	Support PT/NTC/switch types of motor temperature sensors. Please ensure that function codes P00.05/P08.31/P08.33/P08.34 are set properly.

2.9 Definition of Communication Ports

2.9.1 Serial communication port

Table 2-11 Definition of serial communication ports

| PC debugging connector | Module name | Signal name | Pin No. | Functions |
| --- | --- | --- | --- | --- |
| Mini USB | Serial communication port | VBUS | 1 | Connect to serial communication port of PC upper computer debugging |
|  |  | D- | 2 |  |
|  |  | D+ | 3 |  |
|  |  | — | 4 |  |
|  |  | GND | 5 |  |
|  |  | Shell | — |  |

	The port is used to connect drive to PC. It can execute test run, adjust parameters and collect waveform by debugging software of upper computer.

	This serial communication port is Mini-USB Type B and compatible with USB 2.0.

2.9.2 EtherCAT Communication Port

Table 2-12 Definitions of EtherCAT communication ports

| EtherCAT Connectors (CN3, CN4) | Pin No. | Signal name | Functions |
| --- | --- | --- | --- |
| RJ45 | 1 | TX+ | Data sending + |
|  | 2 | TX- | Data sending - |
|  | 3 | RX+ | Data receving + |
|  | 4 | - | - |
|  | 5 | - | - |
|  | 6 | RX- | Data receving - |
|  | 7 | - | - |
|  | 8 | - | - |
|  | 9 | TX+ | Data sending + |
|  | 10 | TX- | Data sending - |
|  | 11 | RX+ | Data receving + |
|  | 12 | - | - |
|  | 13 | - | - |
|  | 14 | RX- | Data receving - |
|  | 15 | - | - |
|  | 16 | - | - |

Figure 2-28 Description of EtherCAT communication cables

	Please use shielded twisted-pair wire as CAT5E or above, connect to metal shell of network interface to ensure EMC performance.

	The distribution length of communication cable shall be less than 100m;

	The master communication port is connected to CN1(IN), and CN2(OUT) is connected to the next slave device. If the cables were incorrectly connected, the communication would fail.

2.10 Definition and Wiring Description of Functional Safety Terminals

2.10.1 Definition of Functional Security Terminals

Table 2-13 Definition of functional security terminals

| Functional Safety Connector (CN2) | Module name | Signal name | Pin No. | Functions |
| --- | --- | --- | --- | --- |
|  | Functional safety | Internal power negative | 1 | Built-in power supply (Do not use) |
|  |  | Internal power positive | 2 |  |
|  |  | STO1- | 3 | STO1 |
|  |  | STO1+ | 4 |  |
|  |  | STO2- | 5 | STO2 |
|  |  | STO2+ | 6 |  |
|  |  | STO_OUT- | 7 | STO status output |
|  |  | STO_OUT+ | 8 |  |
|  |  | Shell | — | Shielded |

2.10.2 Function Safety Wiring Description

Switch

Switch

Servo drive

External 0V

External +24V DC

External 0V

Fuse

External +24V DC

Figure 2-29 Description of functional safety wiring

2.10.3 Relationship of STO I/O Signals

Table 2-14 Relationship of STO input and output signal

| STO1 status | STO2 state | STO_OUT status | Power-on status of motor |
| --- | --- | --- | --- |
| Invalid | Invalid | Power-on | No power |
| Invalid | Active | Power off | No power |
| Active | Invalid | Power off | No power |
| Active | Active | Power off | Power on |

2.11 Grounding and anti-interference measures

2.11.1 Grounding Measures

Table 2-15 Precautions for grounding servo drives

Mount the drive on metal shell(control cabinet)

Please connect the ground terminal of servo motor to GND PE of servo drive, and ground PE terminal reliably

Drive must be single-point ground

Use thick cable(≥2.0mm2) for grounding cables as much as possible; Use thick cable(≥3.5mm2) for external grounding as braided copper wire as much as possible

Type D or above ground(grounding resistance below 100Ω) is recommended.

Ensure to connect ground terminal of drive and ground cable(PE) of control cabinet to avoid electric shock

As there are two protective ground terminals, do not connect all cables together

2.11.2 Anti-interference measures

Due to different application of peripheral wiring, grounding and anti-interference devices, switching noise may affect normal operation of servo drives. Therefore, it must adopt the correct grounding method and anti-interference measure. The following figure is schematic diagram of anti-interference measure for servo drive.

Ground wire

Servo drive

Safety Controller

Shield grounding

Shield grounding

Shield grounding

Ferrite core

Shield grounding

PC

Shield grounding

Ferrite core

Metal control cabinet

Noise filter

Circuit breaker

Residual Current Device (RCD)

Pwer

Controller

Isolated IO Interface Power Supply

Surge absorber

Figure 2-30 Schematic diagram of anti-interference measure of servo drive

	The length of command input cable shall be less than 3m. That of encoder cable shall be below 20m and use twisted-pair shielded cable;

	Separate strong current cable from weak current cable and keep a distance more than 30cm. Do not put in the same pipeline or tie together;

	Cables, I/O lines and power lines of drive are equipped with ferrite magnetic rings;

	Install surge suppressors on the coils of relay, solenoid and electromagnetic contactor;

	Please install the noise filter at the input of power line, and do not share the power supply with welding machine, discharge processing equipment, etc.

	Connect shielded wires of all cables to ground wire (PE).

	Please ground both ends of shielded layer of motor encoder cable.

2.11.3 Noise filter

2.11.3.1 Use of noise filter

To prevent the interference of power line and reduce the influence of servo drive on other sensitive equipment. As for selection, installation and wire routing of noise filter, please obey the following guidelines:

AC power supply

Servo drive

Servo drive

Noise filter

Noise filter

Servo drive

Figure 2-31 Schematic diagram for use, installation and ground of noise filters in series

	Please select the corresponding noise filter according to input current;

	Please arrange separately the input and output wires of noise filter; Do not put the both in the same pipeline or tie together;

	Ground wire of noise filter is arranged separately from the output power line.

	Noise filter should use the single-point ground, and grounding wire should be as short and thick as possible.

	While noise filter and servo drive are installed in the same cabinet, it is recommended to fix the filter and the servo drive on the same metal plate, ensure that the contact is conductive and well-joint, and ground the metal plate.

	For filter installation, the wire between filter and drive must be as short as possible, which shall be less than 30cm. At the same time, ensure that the filter and the drive are connected to the same grounding reference plane. Ensure the reliable ground of filter, or the filtering effect can't be achieved.

	While using multiple drives and sharing a noise filter in the power supply department, please consult the noise filter manufacturer. If noises reach the limit, it would be better to use two in series(as shown in the figure above)

2.11.3.2 Model selection of noise filter

In order to meet the requirement of the product per EN/IEC 61800-3 standard on radiation and conducted emission EMC directive, please connect external EMC filter. SCHAFFNER's single-phase FN2090 and three-phase FN3258 filters are recommended.

| SCHAFFNER FN2090 filter | SCHAFFNER FN3258 filter |
| --- | --- |

Figure 2-32 SCHAFFNER EMC filter outline

Servo drive model and recommended EMC filter model are shown in the following table:

Table 2-16 Recommended manufacturers and models of EMC filters

| Series | Drive type | Rated input current (A) | Filter model |
| --- | --- | --- | --- |
|  |  | In | (SCHAFFNER) |
| Single-phase 220V power supply |  |  |  |
| SIZE A | SV3H-ES1R6XX | 2.3 | FN2090-3-06 |
|  | SV3H-ES2R8XX | 4.0 | FN2090-4-06 |
| SIZE B | SV3H-ES5R5XX | 7.9 | FN2090-8-06 |
| SIZE C | SV3H-ES7R6XX | 9.6 | FN2090-10-06 |
|  | SV3H-ES012XX | 12.8 | FN2090-16-06 |
|  | SV3H-ES014XX | 16.0 | FN2090-16-06 |
| Three-phase 220V power supply |  |  |  |
| SIZE A | SV3H-ES001XX | 0.8 | FN3258-7-44 |
|  | SV3H-ES1R6XX | 1.4 | FN3258-7-44 |
|  | SV3H-ES2R8XX | 2.6 | FN3258-7-44 |
| SIZE B | SV3H-ES5R5XX | 4.4 | FN3258-7-44 |
| SIZE C | SV3H-ES7R6XX | 5.6 | FN3258-7-44 |
|  | SV3H-ES012XX | 8.0 | FN3258-16-44 |
|  | SV3H-ES014XX | 10.2 | FN3258-16-44 |
| SIZE D | SV3H-EU018XX | 18.7 | FN3258-30-44 |
|  | SV3H-EU022XX | 20.7 | FN3258-30-44 |
|  | SV3H-EU027XX | 24.4 |  |
| Three-phase 380V power supply |  |  |  |
| SIZE C | SV3H-ET3R5XX | 2.4 | FN 3258-7-44 |
|  | SV3H-ET5R4XX | 3.6 | FN 3258-7-44 |
|  | SV3H-ET8R4XX | 6.6 | FN 3258-7-44 |
|  | SV3H-ET012XX | 8 | FN 3258-16-44 |
| SIZE D | SV3H-ET017XX | 12 | FN 3258-16-44 |
|  | SV3H-ET021XX | 16 | FN 3258-16-44 |
|  | SV3H-ET026XX | 21 | FN 3258-30-33 |

Note for EMC filter installation dimensions recommended:

Figure 2-33 Dimensions of FN 2090 1~20A filters (unit: mm)

Table 2-17 Dimensions of FN 2090 1~20A filters (unit: mm)

| Rated current (A) | A | B | C | D | E | F | G | H | I | J | K | L | M | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 71 | 46.6 | 22.3 | 50.5 | 44.5 | 61 | 21 | 10.8 | 16.8 | 25.25 | 5.3 | 6.3 | 0.7 | 6.3×0.8 |
| 3 | 85 | 54 | 30.3 | 64.8 | 49.8 | 75 | 27 | 12.3 | 20.8 | 19.9 | 5.3 | 6.3 | 0.7 | 6.3×0.8 |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 113.5±1 | 57.5±1 | 45.4±1 | 94±1 | 56 | 103 | 25 | 12.4 | 32.4 | 15.5 | 4.4 | 6 | 0.9 | 6.3×0.8 |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 2-34 Drawing of FN3258 7~30A filter(unit: mm)

Table 2-18 Dimensions of FN3258 7~30A filter size(unit: mm)

| Rated current (A) | A | B | C | D | E | F | G | H | I | J | K | L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 190 | 40 | 70 | 160 | 180 | 20 | 4.5 | 1 | 22 | M5 | 20 | 29.5 |
| 16 | 250 | 45 | 70 | 220 | 235 | 25 | 5.4 | 1 | 22 | M5 | 22.5 | 29.5 |
| 30 | 270 | 50 | 85 | 240 | 255 | 30 | 5.4 | 1 | 25 | M5 | 25 | 39.5 |

2.11.4 Leakage protection circuit breaker

Servo drive will generate high frequency leakage current during operation. The following matters should be noted in the configuration of cirtuit leakage protection while installing the drive:

	The equipment can generate DC leakage current in protective conductor, and must use Type B (delay type) leakage protection circuit breaker;

	If multiple drives are required for installation, each drive should be equipped with a leakage protection circuit breaker;

	Capacity, carrier frequency, type and length of motor cable, and EMI filter of drive would affect leakage current, so the protection threshold should be set reasonably.

	Suc brands as Chint and Schneider are recommended for leakage protection circuit breaker;

	When the leakage current generated by drive results in action of leakage protection circuit breaker, the following measures can be taken:

	Increase the rated operating current of leakage protection circuit breaker;

	Replace the leakage protection circuit breaker to Type B (delay type) with the high-frequency suppression;

	Reduce the carrier frequency;

	Shorten the output drive cable length;

	Install a leakage suppression device.

2.11.5 Cable and wiring Requirements

(1) Power cable requirement

To meet the requirement of CE mark EMC, motor power cable must be shielded witt shielding layer which must be well grounded. Shielded cable consists of a shielded cable with three phase conductors and shielded cable with four phase conductors. If conductive properties of the shielding layer can't meet the requirement, a separate PE line shall be added. Or shielded cable with four phase conductors, herein,one of which is PE line. To effectively suppress the emission and conduction of RF interference, the shielding layer of shielding cable is made of coaxial copper braided tape. To increase the shielding efficiency and conductivity, weaving density of the shielding layer shall be greater than 90%.

Recommended power cable type as shown in the following figure - symmetrical shielded cable:

PE conductor core and shield

Shielding Layer

Shielded

Figure 2-35 Recommended Power Cable Type

I/O cable shielding layer of main loop of servo drive is grounded with PE terminal on drives together(Please refer to the following figure for wiring).

Shield grounding

Shield grounding

Figure 2-36 Wiring of shielding layer of I/O power cable

The following matters should be noted when selecting output cable of servo drive:

	Do not connect any capacitor or surge absorber, otherwise it might activate servo drive regular protection or even damage servo drives;

	When motor cable is too long, it's easy to result in electrical resonance due to distributed capacitance, then it might cause motor insulation damage or large leakage current which could activate servo drive overcurrent protection; When the length of motor cable is greater than 100m, AC output reactor must be installed near the servo drive.

	Shielding cable is recommended for motor output. Shielding layer shall be bonded for 360°in the structure of grounding support, and shielding layer lead-in wire is crimpped to PE terminals.

	The lead-in wire of shielding layer of motor cable should be as short as possible, and width b≥1/5·a (see Figure 2-29).

Figure 2-37 Lead-in diagram of motor cable shielding layer

(2) The requirement on encoder cable

The encoder cable must be shielded twisted pair.

Input voltage range of the encoder-end connector is 4.75V to 5.25V DC. Please select the appropriate wire. For 10m or less, use shielded twisted pair with a cross-sectional area of 0.18 mm²(AWG24) or above; For 10m or above, use shielded twisted pair with a cross-sectional area of 0.32 mm²(AWG22) or above.

(3) The requirement on USB cable

	For connector on driver side, please use the commercially available USB mini-B that meets the specification of computers.

	Use shielded USB cable;

	While using the cable without any filtering measure, install signal ferrite magnetic rings at both ends of cable.

(4) The requirement on wiring layout

Pay attention to the following matters for routing, and it's recommended to use the routing mode and layout spacing:

	Motor cable routing must be far away from other cables. Motor cables of several drives can be parallel wiring;

	It is recommended to arrange the motor cable, input power cable, control cable, and encoder cable in different cable ducts. In order to avoid electromagnetic interference due to rapid changes of drive output voltage, long distance parallel wiring of motor cables and other cables should be avoided.

	When control cable must pass through power cable, ensure that the angle between two cables is 90 degrees as far as possible. Do not pass other cables through drive;

	Power input and output lines and weak current signal lines (e.g., control circuit) of drive shall not be in parallel as far as possible, and vertical layout shall be applied if possible;

	It must be properly connected and grounded among cable ducts. Aluminum duct can be used to improve equipotential.

Brake resistor cable

Control cable

Main circuit cable

Minimum 300mm

Drive

Encoder cable

Power cable

Control cable

Minimum 500mm

Minimum 500mm

Communication cable

Minimum 200mm

Main circuit cable

Minimum 500mm

Communication cable

Power cable

Main circuit cable

Control cable

Minimum 500mm

Figure 2-38 Recommended wiring layout

2.11.6 I/O magnetic ring selection

To reduce interference to adjacent devices, it's recommended to add a filter magnetic ring to three-phase I/O power line of servo drive:

	Input cable should be installed far away from servo drive;

	Output cable should be installed close to servo drive.

The following table shows magnetic ring models recommended of manufacturer.

Table 2-19 Magnetic ring models recommended of manufacturer

| External drawing | Magnetic ring models recommended of manufacturer | Dimensions (outer diameter × inner diameter × thickness) (mm) |
| --- | --- | --- |
|  | DY644020H | 64×40×20 |
|  | DY805020H | 80×50×20 |
|  | DY1207030H | 120×70×30 |

2.11.7 Solutions to common EMC problems

Servo drive is a strong interference equipment, if fault from routing, grounding and protection in the use occurs, it might generate interference. If there is mutual interference with other devices, the following measures can be taken for improvement.

Table 2-20 Common EMC problems and solutions

| Interference type | Improvement measure |
| --- | --- |
| Switch tripping of leakage protection circuit breaker | 1. Under the premise of not affecting the performance, reduce carrier frequency; 2. Shorten the length of drive line; 3. Add magnetic ring around to drive line (not winding PE wire); 4. For power-on instantaneous tripping, it shall disconnect the input larger ground capacitor; (Disconnect the ground of external or built-in filter, and ground of ground Y capacitor at input port); 5. For potential tripping in operation, it shall take the leakage current suppression measure at input port(leakage current filter, safety capacitor + magnetic ring, magnetic ring); |
| Drive operation causes interference | 1. Motor shell is connected to PE end of the driver; 2. Drive PE end is connected to power grid PE; 3. Input power line is equipped with magnetic ring; 4. Mount capacitors or magnetic loops to the interfered signal ports; 5. Add extra common ground among devices; |
| Communication interference | 1. Motor shell is connected to PE end of the driver; 2. Drive PE end is connected to power grid PE; 3. Input power line is equipped with magnetic ring; 4. Communication line source and load end are equipped with matching resistance; 5. Communication line/differential line are equipped with external communication GND; 6. Communication line is shielded, and shielding layer is connected to communication GND; 7. Multi-node communication wiring needs to use Daisy chain; Branch length is less than 30cm; |
| I/O interference | 1. Low speed DI increase capacitance filtering, recommended maximum 0.1uF; 2. AI increase capacitive filter, the recommended maximum 0.22uF; |

2.12 General Wiring Diagram

Second encoder

Motor encoder

STO signal

Fuse

Built-in power supply

(Do not use)

Internal power negative

Internal power positive

SV3H series

EtherCAT servo drive

Single/three-phase power input*

Noise filter

Circuit breaker

Origin switch

Reverse limit

Servo operation

Band-type brake control

Control circuit power terminals are marked as L1C, L2C；

Main circuit power terminals: Single/three-phase 220V terminals are marked as L1, L2, L3；

Three-phase 380V terminals are marked as R, S, T.

PE shield connected to connector housing

BPhase

APhase

Fault output

EtherCAT communication output

EtherCAT communication input

Probe 2

Probe 1

Forward limit

Switch

Switch

External+24Vdc

External 0V

External 0V

External+24Vdc

Contactor

Figure 2-39 SV3H drive configuration diagram

	Internal 24V power supply ranges 20~28V; Max. working current is 200mA.

	DI input power supply shall be external connection. Power supply voltage ranges from DC12 to 24V and can't] exceed 30VDC.

	Use twisted-pair shielded cable for high-speed/low-speed pulse port. Both ends of the shielded layer must be connected to PE. GND is reliably connected to upper machine signal.

	DO output power supply must be external connection and ranges from 5 to 24V. Max. allowable voltage and Max. allowable current of DO port are DC30V and 50mA respectively.

	The frequency division output of encoder should use twisted pair shielded cable. The shielding layer must be connected to PE at both ends. GND is reliably connected to the upper machine signal.

- Chapter 3 Commissioning

3.1 Settings of Basic Operation

3.1.1 Motor Tuning

Before servo system runs, it is necessary to enter the motor parameters and identify magnetic poles.

This section describes the panel operation process. For details about background operations, see 3.3 Servo3 Designer.

（1）Entering motor parameters

The setting mode of motor parameters is shutdown operation, and the effective mode is power-on reset. List of motor parameters is as follows:

Table 3-1 Parameters of the Motor

| Name | Function code | Uit | Initial value | Min. | Max. |
| --- | --- | --- | --- | --- | --- |
| Rated voltage | P00.10 | V | 0-220V | 0-220V | 655.35 |
| Rated current | P00.11 | A | 4.70 | 0 | 655.35 |
| Rated power | P00.12 | kW | 0.75 | 0 | 655.35 |
| Rated torque | P00.13 | N.m | 2.39 | 0 | 42949672.95 |
| Max. torque | P00.15 | N.m | 7.16 | 0 | 42949672.95 |
| Rated speed | P00.17 | rpm | 3000 | 0 | 65535 |
| Max. speed | P00.18 | rpm | 6000 | 0 | 65535 |
| Inertia of motor | P00.19 | kg·cm2 | 1.30 | 0 | 42949672.95 |
| Number of pole pairs | P00.21 | - | 4 | 0 | 65535 |
| Phase resistance | P00.22 | Ω | 0.500 | 0 | 65.535 |
| Inductance Lq | P00.23 | mH | 3.27 | 0 | 655.35 |
| Inductive Ld | P00.24 | mH | 3.87 | 0 | 655.35 |
| Counter potential | P00.25 | mV/rpm | 33.30 | 0 | 655.35 |
| D-axis back potential compensation | P00.31 | % | 60.0 | 0.0 | 6553.5 |
| Q-axis back potential compensation | P00.32 | % | 100.0 | 0.0 | 6553.5 |
| Current sampling and extraction rate | P00.33 | - | 0- Extraction rate 32 | 0- Extraction rate 32 | 3- Extraction rate 256 |
| D axis proportional gain 1 | P00.34 | Hz | 2000 | 0 | 65535 |
| D axis integral gain 1 | P00.35 | % | 2.00 | 0.00 | 655.35 |
| Q-axis proportional gain 1 | P00.36 | Hz | 2000 | 0 | 65535 |
| Q axis integral gain 1 | P00.37 | % | 1.00 | 0.00 | 655.35 |
| D axis proportional gain 2 | P00.38 | Hz | 1000 | 0 | 65535 |
| D axis integral gain 2 | P00.39 | % | 2.00 | 0.00 | 655.35 |
| Q-axis proportional gain 2 | P00.40 | Hz | 1000 | 0 | 65535 |
| Q axis integral gain 2 | P00.41 | % | 1.00 | 0.00 | 655.35 |

Please check the model and parameters of motor used, and enter the motor parameters according to the following process.

Motor parameter input flowchart (panel operation)

Motor parameter input completed

Save motor parameters to encoder ROM

At this time, the drive panel displays: E.910.1 (Data verification error in motor encoder ROM or parameters not saved)

If unclear about the manufacturer password settings, please consult the manufacturer; For panel introduction and operation examples, see section 3.2.1

Check each motor parameter in the first group to ensure consistency with the actual motor parameters; If inconsistent, please modify the parameter values and perform parameter write operation. When modifying parameter values, pay attention to consistent unit conversion; For motor parameter list, see Table 3-1

After modifying all parameters, please write 1 to parameter P20.06 (encoder ROM area read/write); P20.06=1, write ROM

Modify motor parameters and perform write operation

Power on

Enter manufacturer password (P05.00) to obtain read/write permission for motor parameters

Start

Figure 3-1 Panel operation process of motor parameter entering

(2) Magnetic pole identification

Operation process of magnetic pole identification is as follows.

Enable function code P20.11

(pole identification)

After enabling P20.11, the motor will automatically enter pole identification, please wait patiently; The pole identification result will be automatically saved to the encoder ROM area

Perform write 1 operation on function code P20.11 (pole identification): P20.11=1, enable pole identification

If unclear about the manufacturer password settings, please consult the manufacturer; During pole identification, ensure the motor is in no-load condition; For panel introduction and operation examples, see section 3.2.1

Pole identification completed

Wait for pole identification to complete

Enable function code P20.11

(pole identification)

Enter manufacturer password (P05.00) to obtain read/write permission for motor parameters

Power on

Start

Figure 3-2 Operation process of magnetic pole identification

3.1.2 Lock Setting

Band-type brake is used to stop the unexpected movement of moving loads(e.g., falling under gravity) when the servo system is not activated (e.g., servo system is powered off), to prevent servo motor from moving unexpectedly due to its own weight or external force after being powered off.

Note:

	Band-type brake is the non-powered action type special mechanism, which can't be used for braking, and can be used only when servo motor is in the stop state.

	Non-polarity of band-type brake coil;

	After servo motor stops, it shall switch off servo enable.

	When the motor with built-in band-type brake runs, the brake might make the clicking, that doesn't affect any function;

	When the brake coil is energized (the brake is ON), magnetic flux leakage may occur at shaft ends. When using the magnetic sensor near motors, please pay attention to the possibility of this situation.

(1) Band-type brake parameter setting

For applications with band-type brake, it must set band-type brake enable switch(P05.12) ON, and one of servo drive DO terminals must be configured to Function 17 (BK, Band-type brake control), and ensure the valid logic for the corresponding DO terminal.

According to the current state of servo drive, working time sequence of the brake mechanism consists of 2 types: the brake time sequence in normal state of servo drive and the brake time sequence in fault state of tservo drive.

Table 3-2 Index code of P05.12 band-type brake enable switch

| P05.12- Lock band-type brake switch |  |
| --- | --- |
| Index - Subindex | 0x2005-0D |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | - |
| Default value | 1 |
| Min. | 0 |
| Max. | 1 |
| Setting and effective mode | Run settings/Effective immediately |
| Related mode | - |
| Note | - |

Table 3-3 Band-type brake output No.

| Encoding | Name | Function name | Functions |
| --- | --- | --- | --- |
| 17 | BK | Band-type brake control | Invalid, the brake power supply is ON, the brake runs, motor is in the position stalled state; Effective, the brake power supply is OFF, the brake is cancelled, and motor can move; |

(2) Band-type brake sequence of servo drive in normal state

Band-type brake sequence in the normal state can be divided into such 2 situations as motor static and motor dynamic:

	Static: Motor actual speed is below 30rpm;

	Dynamic: Motor actual speed reaches 30rpm and above.

(3) The brake sequence when servo motor is static

	When servo enable is switched from ON to OFF, if the current motor speed is lower than 30rpm, the drive would run according to static brake sequence;

	After the brake output is set from OFF to ON, do not enter any position/speed/torque instruction within P05.13 time, or instructions would be lost or it would run improperly;

	Being used in vertical axis, self-weight or external force of mechanical moving part may cause the slight movement of machine. When servo motor is stationary, the servo enable switches OFF, and the brake output immediately turns OFF, but within P05.14 time, the motor is still in the power-on state to prevent mechanical moving part from moving due to self-weight or external force.

Band-type

brake contact

ON

OFF

OFF

Servo enable

(

S

-

ON

)

Band-type brake output

(

BK

)

OFF

OFF

P05.13

Position/speed /

Torque command

Motor speed

P05.14

ON

OFF

OFF

Motor power-on

ON

*

1

OFF

(

)

OFF

*

2

*

2

*

3

*

4

ON

(

)

Zero speed stop

OFF

ON

OFF

ON(Band-type brake not effective)

ON(Band-type brake effective)

Figure 3-3 Timing diagram of band-type brake when motor is stationary

*1. When servo enable is ON, the brake output is set as ON; At the same time, the motor enters the power-on state;

*2. Please refer to the relevant specification for delay time of action of band-type brake contact part;

*3. From the brake output set as ON to inputing command, please leave an interval above P05.13 time;

*4. When servo motor is static(motor speed below 30rpm), when servo enable is OFF, brake output is set as OFF at the same time. By P05.14, motor enters the delay of non-power-on state after brake output is set as OFF.

Table 3-4 P05.13 Index code from the brake switch-off to receiving command delay(Stop state)

| P05.13 - From the brake switch-off to receiving command delay |  |
| --- | --- |
| Index | 0x2005-0E |
| Data type | Uint16 |
| Accessibility | Readable/writable |
| Unit | ms |
| Default value | 250 |
| Min. | 0 |
| Max. | 500 |
| Setting and effective mode | Run settings/Effective immediately |
| Related mode | - |
| Note | - |

Table 3-5 P05.14 Index code of zero speed holding time of band-type brake switched on (Stop state)

| P05.14- Zero speed holding time for lock suction |  |
| --- | --- |
| Index | 0x2005-0F |
| Data type | Uint16 |
| Accessibility | Readable/writable |
| Unit | ms |
| Default value | 150 |
| Min. | 1 |
| Max. | 1000 |
| Setting and effective mode | Run settings/Effective immediately |
| Related mode | - |
| Note | - |

(4) Sequence of band-type brake when servo motor is moving

	When servo enable is switched from ON to OFF, if the current motor speed is higher than or equal to 30rpm, the drive will operate according to the motion brake sequence.

	When the servo enable is set from OFF to ON, do not enter position/speed/torque command within P05.13 time, or commands will be lost or operates improperly;

	While servo motor is moving, servo enable OFF occurs, and servo motor enters zero speed stop state, but band-type brake output can be set as OFF only if any of the following conditions are met. I. P05.16 time hasn't expired, but it has reduced the motor speed to P05.15; ⅱ. P05.16 time ends, but motor speed is still above P05.15.

ON(Band-type brake not effective)

	Output of band-type brakeONchangeOFFthen50msduring the period, motor is still powered on and prevent mechanical parts from moving due to self-weight or external force.

ON

OFF

OFF

Servo enable

(S-ON)

Band-type brake output

(BK)

OFF

OFF

P05.13

Position/speed /

Torque command

Motor speed

ON

OFF

OFF

Motor power-on

ON

*

1

OFF

(

)

OFF

Band-type brake contact

*

2

*

4

*

3

ON

(

)

P05.16

*

2

P05.15

*

4

Zero speed stop

50

ms

OFF

ON

OFF

ON(Band-type brake effective)

Figure 3-4 Sequence diagram of band-type brake when motor moves

*1. When servo enable is ON, the brake output is set as ON; At the same time, the motor enters the power-on state;

*2. Please refer to the relevant specification for delay time of actions of band-type brake contact part.

*3, From band-type brake output as ON to input command, please set an interval above P05.13.

*4. In the case of servo motor movement, when servo enable is OFF, it can set the delay of band-type brake output OFF after servo enable is OFF. It can be set through P05.15 and P05.16. After band-type brake outputs OFF, delay time is 50ms, then motor would enter the non-powered state.

Table 3-6 P05.15 Band-type brake power-on speed threshold

| P05.15 - Band-type brake power-on speed threshold |  |
| --- | --- |
| Index | 0x2005-10 |
| Data type | Uint16 |
| Accessibility | Readable/writable |
| Unit | rpm |
| Default value | 30 |
| Min. | 0 |
| Max. | 3000 |
| Setting and effective mode | Run settings/Effective immediately |
| Related mode | - |
| Note | - |

Table 3-7 P05.16 - Band-type brake power-on speed threshold

| P05.16 - Band-type brake power-on speed threshold |  |
| --- | --- |
| Index | 0x2005-11 |
| Data type | Uint16 |
| Accessibility | Readable/writable |
| Unit | ms |
| Default value | 500（ms） |
| Min. | 1 |
| Max. | 1000 |
| Setting and effective mode | Run settings/Effective immediately |
| Related mode | - |
| Note | - |

(5) Sequence of band-type brake in servo drive fault state

According to stop modes, servo faults are classified into Type 1 faults(referred to as NO.1) and Type 2 faults (referred to as NO.2). For details, see Chapter 5 Troubleshooting and Warning. Band-type brake sequence of servo drive in fault state can be divided into the following 2 situations:

3.1.3 Selection of Running Direction

By setting the direction, it can reverse motor rotation without changing the polarity of input instruction. After modification, only when powering on the servo again, can it take effect.

When "Direction Selection" is changed, pulse form of servo drive output and positive/negative state of monitoring parameters won't change.

Setting of "Forward drive" in the overrun prevention is identical to that of "Motor operation direction selection (P04.01)".

As for operation direction setting, the selections are available as "2004-02h", or "P04.01" on the panel, or "P04 group - Motor operation direction selection" on background software.

Table 3-8 P04.01- Selection of motor operation reversing

| P04.01- Selection of motor operation reversing |  |
| --- | --- |
| Index - Subindex | 0x2004-02 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | 1 |
| Default value | 0 |
| Min. | 0 |
| Max. | 1 |
| Setting and effective mode | Stop setting/power-on reset |
| Related mode | ALL |
| Note | When viewed from motor shaft side.its rotation direction is defined as positive. |

3.1.4 Absolute Value Function

(1)Instruction of absolute value system

Absolute encoder can simultaneously record the position of servo motor and turns of servo motor rotation. Single-turn resolution  is 1048576, and turns of 16-bit Max. 65535 can be recorded. Absolute encoder will back up position data when the upper servo drive is powered off. After power-on reset, machine absolute position can be calculated without re-starting the original zero point(when servo drive uses the incremental encoder, position feedback is 0 after power-on. By the original zero point, it can find the point where mechanical position feedback is really 0, accordingly servo motor can operate properly on the mechanical equipment; After absolute value encoder is used, the correct position feedback will be calculated according to the back-up data by encoder after power-on reset.

Note:

	SV3 servo drives support absolute position linear mode and absolute position rotation mode, which is suitable for position, speed, and torque mode;

	E.917 encoder battery fault occurs when battery is switched on for the first time. It shall set P20.05=1 to reset the encoder fault, and then perform the origin zero return.

	To modify the direction reservsing selection of P04.01, it shall reset the origin zero return;

	In absolute position mode, the servo automatically detects whether motor number is absolute encoder motor, if the setting is wrong, Fault E.019 occurs(encoder matching fault).

(2) Object related to absolute value system

Function selection object:

Table 3-9 P04.02 - Selection of Position Feedback System

| P04.02 - Selection of Position feedback System |  |
| --- | --- |
| Index - Subindex | 0x2004-03 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 2 |
| Setting and effective mode | Stop setting/power-on reset |
| Related mode | ALL |
| Note | Set absolute value system Settings; Selection of absolute value system. 0; Incremental mode. 1; Absolutely linear model. 2; Absolute rotation mode. |

Table 3-10 P20.05- Encoder reset

| P20.05- Encoder reset |  |
| --- | --- |
| Index - Subindex | 0x2020-06 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 2 |
| Setting and effective mode | Stop setting/power-on reset |
| Related mode | - |
| Note | Encoder reset Settings; Description. 0; No action. 1; Reset fault. 2; Reset fault and multi-turn data. |

Table 3-11 encoder feedback parameter objects

| Parameter index | Parameter | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x2009-2B | P09.42 | Number of encoder turns | Turn | 0~65535 | UINT16 | RO | - |
| 0x2009-2C | P09.43 | Encoder single turn position | Encoder unit | 0~(231-1) | INT32 | RO | - |
| 0x2009-2E | P09.45 | Encoder absolute position Low 32 bits | Encoder unit | (-231)~(231-1) | INT32 | RO | - |
| 0x2009-30 | P09.47 | Encoder absolute position High 32 bits | Encoder unit | (-231)~(231-1) | INT32 | RO | - |

Note:

	P09.43 is the encoder single-turn position, and its range is 0 to encoder resolution. In case of 23-bit encoder, the range is 0~(223-1);

	P09.42/P09.43 are both encoder feedback data;

	As P09.42 is an unsigned number, absolute position of absolute encoder is calculated as follows:

Encoder absolute position = P09.42 × encoder resolution + P09.43(P09.42<32768)

Or

Encoder absolute position = (P09.42-65536) × encoder resolution + P09.43 (P09.42≥ 32768)

	P09.45 and P09.47 are used to display the absolute position of encoder. The formula is as follows: absolute encoder absolute position = P09.47×232+ P09.45.

(3) Precautions for absolute value system

	If the battery is connected for the first time, E.917 (encoder battery fault) will occur. It requires to set P20.05=1 to reset the encoder fault, and then perform absolute position system operation;

	If the detected battery voltage is less than 3.0V, E.921 (encoder battery warning) will occur. Please replace the battery; Ensure that the servo drive is powered on but not running when replacing the battery;

	If the servo drive is powered off, the battery is detached or replaced, please use P20.05=1 to reset the encoder fault after powered-on, and then reset the original zero;

	If the servo drive is powered off, Max. speed of the motor should not exceed 6000rpm, otherwise, encoder position data may be recorded improperly;

	Please ensure that battery SOC and storage conditions do not damage the battery.

3.1.5 Setting of Electronic Gear Ratio

Reducer

Internal command

(encoder units)

Command input

(command units)

Electronic

gear ratio

Ecoder

Motor

Mechanical equipment

Drive

Figure 3–5 Electronic gear ratio

Electronic gear ratio: Electronic gear ratio is a simulated gear that converts control commands (command units) into actual motor displacement (encoder units),  shown as the following formula:

| Encoder unit = Command unit* | Numerator of electronic gear ratio |
| --- | --- |
|  | Denominator of electronic gear ratio |

Note: The control variables (non-state feedback variables) in object dictionary are based on command units. If the electronic gear ratio is set of 1:1, then 1 encoder unit is equal to 1 command unit.

Table 3–12  0x6091 electronic gear ratio

| 0x6091-electronic gear ratio |  |  |
| --- | --- | --- |
| Index - Subindex | 0x6091-01 | 0x6091-02 |
| Data type | UINT32 |  |
| Accessibility | RW | RW |
| Unit | - | - |
| Default value | 1 | 1 |
| Min. | 0 | 0 |
| Max. | 232-1 | 232-1 |
| Setting and effective mode | Operation settings/downtime effective | Operation settings/downtime effective |
| Related mode | CSP/PP/HM/CSV/PV |  |
| Note | 6091-01h：Numerator of electronic gear ratio 6091-02h：Denominator of electronic gear ratio |  |

3.1.6 Time sequence diagram

(1) Power on subsequence diagram

（Brake ineffective）

（Brake effective）

No command

Band-type brake contact

Servo enable

Servo ready output

Main power supply

Control power supply

Position/speed/torque command

Motor power-on

Band-type brake output

With command

Microprocessor action

Not working

Normal operation

Initialization2s

0s以上*2

Reset200 400m s*1

Figure 3-6 Power-on subsequence diagram

*1: The reset time is determined by the establishment time of microprocessor and 5V power supply.

*2: 0s above refers to the time determined by actual time when main power supply is turned on.

*3: When control power supply and main power supply are powered on simultaneously, the time is the same as the period from the completion of microprocessor initialization to Rdy  enabled.

*4: Please refer to the relevant specification for delay time of band-type brake contact action.

*5: When band-type brake enable switch (P05.12) is OFF, P05.13 has no effect.

(2) Stop sequence diagram when warning or fault occurs

A) Fault 1: Free stop, maintain a free running state;

（Brake ineffective）

Non-fault state

Power on

Speed Setting Value

ErrFault state

No power

Approximately 0.1~3m/s

Normal

operation

Band-type brake contact

（Brake effective）

Fault

Delay P05.16 *2 or speed less than P05.15 *2

Band-type brake output

Power-on status of motor

Absolute value of motor speed

Whether a fault occurred

Servo alarm state output

Figure 3-7 Sequence diagram of free stop and free running state during fault 1

*1: Please refer to the relevant specifications for delay time of band-type brake contact action.

*2: When the brake enable switch (P05.12) is OFF, P05.15 and P05.16 have no effect.

B)Fault 2: Not a band-type brake. Free shutdown, maintaining a free running state.

Non-fault state

Approximately 0.1~3m/s

Power on

Servo alarm state output

Power-on status of motor

Whether a fault

occurred

Fault

Power Off

ErrFault state

Absolute value of

motor speed

Normal

operation

Figure 3-8 Sequence diagram of free stop and free running state during fault 2

C) Fault 2: Not band-type brake: DB stop, maintain DB status

Whether a fault occurred

Power-on status of motor

Non-fault state

Normal

operation

DB state

Approximately 0.1~3m/s

Fult

No power

ErrFault state

Servo alarm state output

Power on

Absolute value of motor speed

Figure 3-9 Sequence diagram of DB stop, maintain DB status during fault 2

D) Fault 2: Not band-type brake: Zero speed stop, maintain free running status

Non-fault state

Whether a fault occurred

Zero speed stop

Normal

operation

ErrFault state

No power

Fult

Approximately 0.1~3m/s

Power on

Power-on status of motor

Absolute value of motor speed

Servo alarm state output

Figure 3-10 Sequence diagram of zero speed stop and free run state during fault 2 (not band-type brake)

E) Fault 2 with band-type brake: forced to be zero speed stop, and maintain free run state

（Brake ineffective）

Servo alarm state output

（Brake effective）

Non-fault state

Power on

Fult

No power

Band-type brake contact

Band-type brake output

Power-on status of motor

Zero speed stop

Whether a fault occurred

Normal

operation

Approximately 0.1~3m/s

ErrFault state

Motor energized state

Figure 3-11 Sequence diagram of free stop state in zero speed stop mode during fault 2 (with band-type brake)

*1: When the brake enable switch(P05.12) is OFF, P05.14 has no effect;

*2: Please refer to the relevant specifications for delay time of band-type brake contact action.

When the servo encounters Type 3 warning: A.220 (forward overtravel warning)/A.221 (reverse overtravel warning), the current operating state of servo will be interrupted, and its stop sequency is shown in F).

F) Overtravel stop warning: Zero speed stop, maintain position latched state

Band-type brake contact

Warning

Approximately 0.1~3m/s

Normal

operation

Band-type brake output

Power-on status of motor

Absolute value of motor speed

Whether a warning occurred

Zero speed stop

Warning state

Non-warning state

Non-warning state

Power on

Normal operation

Approximately

0.1~3m/s

Position Lock State

Servo alarm state output

（Brake ineffective）

Figure 3-12 Timing diagram of shutdown warning

Except for the above two types of Class 3 warnings, other warnings have no impact on the current status of the servo, as shown in G).

G) Non shutdown warning:

Band-type brake contact

Servo alarm state output

Absolute value of motor speed

Whether a warning occurred

Non-warning state

Normal operation

Approximately 0.1~3m/s

Non-warning state

Band-type brake output

Maintain constant speed

Normal

operation

Power-on status of motor

（Brake ineffective）

Warning state

Approximately 0.1~3m/s

Power on

Warning

Figure 3-13 Non stop Warning Sequence Diagram

H) Fault reset:

Fault reset

Enable

Disable

Power on

Band-type brake contact

With command

No command

Band-type brake output

Fault stop state

Err ErrFault state

No power

Rdy state

Disable

Approximately 3m s

Servo alarm state output

Run state

Fault stop release

Fault stop

（Brake effective）

（Brake ineffective）

Servo enable

Power-on status of motor

Position/speed/torque command

Figure 3-14 Fault reset timing diagram

*1: The DI fault reset signal (5: fault reset) is valid along the variation.

*2: Please refer to the relevant specifications for delay time of band-type brake contact action.

*3: When the brake enable switch (P05.12) is not turned on, P05.13 has no effect.

3.1.7 Operation

(1) Check before operation

Table 3-13 Check procedure before operation

| Item | Content |
| --- | --- |
| Wiring inspection | Motor power line UVW line sequence (Special attention) Whether GND is loose or short-circuited with UVW Whether encoder cable is loose |
| Mechanical connection inspection | Check if there are strict requirements for the direction of motor rotation by the mechanical part which is connected to motor. It's recommended that the motor should be unloaded prior to 'safe operation' execution |
| Environmental inspection | Do not operate any motor in high temperature/humidity |

(2) Safe operation

Table 3-14 Safe operation procedures

| Steps | Description |
| --- | --- |
| Power on and confirm the Panel display | After the servo controller is powered on, the last 2-digit of panel displays 'ry' under normal conditions. If the panel flashes an alarm code, please troubleshoot it according to Chapter 5 |
| Low-speed rotation | 1) Operate the drive motor according Section 3.2.2 JOG 2) Observe the direction of motor movement. If the direction is wrong, stop driving the motor and check the drive parameters according to Section 3.2.1 Panel Introduction 3) Use Servo3 Designer oscilloscope to observe the speed waveform. If the speed is incorrect, please check the electronic gear ratio setting according to section 3.1.5 or correct the upper computer unit conversion |

(3) Operation

Table 3-15 Operational steps

| Item | Description |
| --- | --- |
| Mechanical connection | Please connect motor to load after 'safe operation' is executed correctly. Multi-diaphragm coupler is recommended |
| Inertia recognition | Set proper inertia ratio by inertia recognition function |
| Gain adjustment | Adjust gain parameters, filter parameters, advanced adjustment parameters to realize high precision, high response speed control |
| Operation under program | Use the drive for devices, write control program, and complete specific functions |

3.1.8 Stop

In order to meet the various working conditions of servo, servo drive supports different stop modes and stop states.

(1) Stop mode

	Free stop: servo motor is not powered on, and the motor is free to reduce its speed to 0 by mechanical friction;

	DB stop: stop the servo motor in motion state by the reverse braking torque provided by UVW three-phase short-circuit;

	Ramp stop: Smoothly stop according to pre-set position/speed/current ramp instructions;

	Zero speed stop: servo drive outputs reverse braking torque, immediately set the target  motor speed to zero and execute stop;

	Emergency torque stop: servo drive outputs reverse braking torque and quickly reduces the motor speed to 0.

(2) Stop state

	Hold position state: after the motor stops, the motor shaft is locked and can't rotate freely;

	Free motion state: after the motor stops, the motor is not powered on, and the motor shaft can rotate freely;

	Hold DB state: after the motor stops, the motor is connected to the drive, UVW three-phase short-circuit, and the motor shaft can slowly rotate by external force.

(3) Stop condition

Servo drive supports the following stop conditions:

Table 3-16 Stop modes of SV3 servo drives

| Stop condition | Settings of related parameter |  | Stop action and status | Description |
| --- | --- | --- | --- | --- |
| Type I fault stop | P04.10 | 0 | Free stop, free movement | Stop conditions while type I failure occurs |
|  |  | 1 | DB down, free movement |  |
|  |  | 2 | DB stop, keep DB |  |
| Type II fault stop | P04.11 | 0 | Free stop, free movement | Stop conditions while type II failure occurs |
|  |  | 1 | Zero speed stop, free movement |  |
|  |  | 2 | Zero speed stop. Keep DB |  |
|  |  | 3 | DB down, free movement |  |
|  |  | 4 | DB stop, keep DB |  |
| Disable stop mode | P04.12 | 0 | Free stop | Stop condition when servo drive stops enabling |
|  |  | 1 | DB stop |  |
|  |  | 2 | Zero speed stop |  |
| Disable stop state | P04.13 | 0 | Free movement |  |
|  |  | 1 | Hold DB |  |
| Power-off stop | P04.14 | 0 | Stop by disabled mode | Stop condition when servo drive is powered off |
|  |  | 1 | Zero speed stop |  |
| Overrun stop | P04.15 | 0 | Free stop, free movement | Stop condition when motor shaft position exceeds the pre-set one which can be the position limit value of the servo internal software or the position at external limit DI triggering time |
|  |  | 1 | Zero speed stop, maintain the position |  |
|  |  | 2 | Zero speed stop, free movement |  |
| Fast stop | 605Ah | The stop method is different for each motion. For details, please refer to the object dictionary 605Ah |  | Stop when control word 6040h fast stop position is valid Condition |
| Pause | 605Dh | The stop method is different for each motion. For details, please refer to the object dictionary 605Dh |  | Stop condition when control word 6040h pause position is valid |

Table 3-17 0x605A - Fast Stop Mode Selection

| 0x605A - Fast Stop Mode Selection |  |
| --- | --- |
| Index - Subindex | 0x605A-00 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | - |
| Default value | 2 |
| Min. | 0 |
| Max. | 7 |
| Setting and effective mode | Operation settings/downtime effective |
| Related mode | ALL |
| Note | Fast stop, when Bit2 of control word 6040h is valid, fast stop will be 6executed. Under the same set value, different stop modes have different stop methods as shown in the following table: PP： Settings; Description. 0; Free shutdown, maintain free run state. 1; Ramp stop by 6084h, maintain free run state. 2; Ramp stop by 6085h, maintain free run state. 3; Emergency stop by P04.23 braking torque, maintain free run state. 4; NA. 5; Ramp stop by 6084h, maintain position latched state. 6; Ramp stop by 6085h, maintain position latched state. 7; Emergency stop by P04.23 braking torque, maintain position latched state. CSP： Settings; Description. 0; Free shutdown, maintain free run state. 1; Emergency stop by P04.23 braking torque, maintain free run state. 2. 3. 4; NA. 5; Emergency stop by P04.23 braking torque, maintain position latched state. 6. 7. PV/CSV/HM Settings; Description. 0; Free shutdown, maintain free run state. 1; Ramp stop by 6084h(HM: 609Ah), maintain free run state. 2; Ramp stop by 6085h, maintain free run state. 3; Emergency stop by P04.23 braking torque, maintain free run state. 4; NA. 5; Ramp stop by 6084h(HM: 609Ah), maintain position latched state. 6; Ramp stop by 6085h, maintain position latched state. 7; Emergency stop by P04.23 braking torque, maintain position latched state. CST/PT Settings; Description. 0; Free shutdown, maintain free run state. 1; Ramp stop by 6087h, maintain free run state. 2. 3; Free shutdown, maintain free run state. 4; NA. 5; Ramp stop by 6087h, maintain position latched state. 6. 7; Free stop, maintain position latched state. |

Table 3-18 0x605D Pause Mode Selection

| 0x605D - Pause mode selection |  |
| --- | --- |
| Index - Subindex | 0x605D-00 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | - |
| Default value | 1 |
| Min. | 1 |
| Max. | 3 |
| Setting and effective mode | Operation settings/downtime effective |
| Related mode | ALL |
| Note | Pause, when Bit8 of control word 6040h is valid, pause will be executed. Under the same setting value, different pause modes have different pause methods as shown in the following table: PP： Settings; Description. 1; Ramp stop by 6084h, maintain position latched state. 2; Ramp stop by 6085h, maintain position latched state. 3; Emergency stop by P04.23 braking torque, maintain free run state. CSP： Settings; Description. 1; Emergency stop by P04.23 braking torque, maintain position latched state. 2. 3. PV/CSV/HM Settings; Description. 1; Ramp stop by 6084h(HM: 609Ah), maintain position latched state. 2; Ramp stop by 6085h, maintain position latched state. 3; Emergency stop by P04.23 braking torque, maintain position latched state. CST/PT Settings; Description. 1; Ramp stop by 6087h, maintain position latched state. 2. 3; Free stop, maintain position latched state. |

3.2 Panel Control Operation

3.2.1 Introduction of Panel

(1) Panel composition

SV3 servo panel consists of buttons and a digital display, which can be used for information and parameter display, parameter setting, user password setting, and general function execution.

(2) Button Introduction

Functions of each button are shown in the following figure:

UP key: Increase the flashing digit value on LED display

DOWN key: Decrease the flashing digit value on LED display

SHIFT key: 1. Change flashing digit 2. View high-order values of data longer than 5 digits

SET key:

- Enter next menu

2. Execute commands like parameter storage

MODE key:

- Mode switch

2. Return to previous menu

Figure 3-15 Introduction of Functions of Button

Take panel jog operation for example, the following case is for button usage:

End

Motor reverse rotation

Motor forward rotation

Long press“廉”

Start/Servo power on

Long press“亳”

Requirement: Use panel keys to jog at 300rpm.

Figure 3-16 Case Operation and Display of Panel Jogging Operation

(3) Introduction of Panel Display

The panel display consists of 5-digit 7-segment digital tubes. When the servo drive is running, the display can be used for servo status display, parameter display, fault display and monitoring display.

(4) Type of Panel Displays

Table 3-19 Type of Panel Displays

| Type of Displays | Function description | Entry method | Sample |
| --- | --- | --- | --- |
| Status display | Display the current status of the servo, e.g., servo ready, servo running, etc | 1. While power on, enter immediately; 2. Under the parameter display, press MODE key to enter; 3. Under the monitoring display, enter when the motor is stationary; | rowwo |
| Parameter display | Display function code and the setting value | 1. Under the status display, press MODE key to enter; 2. Under the monitoring display, press MODE key to enter; 3. Under the fault display, first press SET key, then press MODE key to enter; | P20.00 |
| Fault display | Display the fault/warning codes of servo | 1. Under the parameter display, press MODE key to enter; 2. Enter when a fault occurs; | E.910.1 |
| Monitoring display | Display the current operating parameters of the servo | 1. Under the parameter display, set function code of Group 21 and enter; Under the status display, set function code P05.03, enter after the motor runs; | 200 |

(5) Introduction of panel display content

Table 3-20 Status description

| Type of Displays | Display content |  |  |  |  | Name | Display site | Meaning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Status Display | r | o | w | w | o | rocco: servo initialization | Servo power-on moment | The servo drive is in initialization or reset state, waits for the completion of initialization or reset, and automatically enters other state |
|  | + | 1 | 8 | r | y | 18ry: Servo is ready | Servo is ready | The servo drive is in the runnable state with port 1 established connection, communication initialization status, and periodic synchronization control mode |
|  | The 3rd digit Control mode: Displays the current operating mode of the servo in hexadecimal form, not flashing 1: Profile position control 3: Profile velocity mode 4: Profile torque mode 6: Homing mode 8: Cyclic synchronous position mode 9: Cyclic synchronous velocity mode A: Cyclic synchronous torque mode The 2nd digit Communication status: Displays the state of the slave's EtherCAT state machine in numerical form 1: Initialization state 2: Pre-operational state 4: Safe-operational state 8: Operational state The 1st digit Port connection indication: ^：Port IN has established communication connection -：Port OUT has established communication connection +：Both ports IN and OUT have established communication connection The 4th-5th digits Servo ready result display: ry: Drive ready nr: Servo not ready. Servo initialization complete, but main circuit not powered, servo in non-operational state rn: Servo enable signal active, servo running |  |  |  |  |  |  |  |

Table 3-21 Parameter description

| Type of Displays | Display content |  |  |  |  | Name | Display site | Meaning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter display | P | 2 | 0. | 0 | 0 | Function code: P20.00 |  | P: function code 20: function code group 00: Serial No. in function code group |
|  | 1 | 2 | 3 | 4 | 5 | Data (5 digits or below) Display: 12345 |  |  |
|  | - | 1 | 2 | 3 | 4 | Negative data (4 digits or below) Display: -1234 |  |  |
|  | _ | 7 | 8 | 9 | 0 | Data (above 5 digits) Display: 1234567890 |  | _: The lower four digits of multi-digit data -: The middle four digits of multi-digit data ^The top four digits of multi-digit data |
|  | - | 3 | 4 | 5 | 6 |  |  |  |
|  | ^ |  |  | 1 | 2 |  |  |  |
|  | _. | 7 | 8 | 9 | 0 | Negative data (5 or more digits) Display: -1234567890 |  | -: Indicates a negative sign _: The low four digits of multi-digit negative data -: The middle four digits of multi-digit negative data ^: The top four digits of multi-digit negative data |
|  | -. | 3 | 4 | 5 | 6 |  |  |  |
|  | ^ |  | - | 1 | 2 |  |  |  |
|  |  | 1 | 0 | 0. | 0 | Decimal point Display: 100.0 |  | .: Decimal point, non-flashing |
|  | d | o | n | e |  | Done: Parameter setting completed | Parameter setting successful | Parameter setting completed, and stored into servo drive |
|  | = | = | . | . | . | = = . . .： Parameter restored to factory settings | When using system parameter initialization (P05.01), set of 1 | The servo drive is in the process of parameter initialization. Please wait for the completion of system parameter initialization, and power it on for use |
| Example of parameter display | Requirement: Use panel keys to input negative data -1234567891 to function code Multi-turn absolute position offset (low 32 bits) (P13.23) End Multiple MODE. Start/Servo power on |  |  |  |  |  |  |  |

Table 3-22 Fault Display and Monitoring Display

| Display Type | Display content |  |  |  |  | Name | Display site | Meaning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fault Display | E. | 0 | 1 | 0. | 0 | Fault code: E.010.0 | Software parameter fault | E. : Fault Before decimal point:fault main code After decimal point: fault subcode |
| Warning Display | A. | 2 | 2 | 4. | 0 | Warning code: A.224.0 | Regenerative resistor overload | A. Warning: Before decimal point: Warning main code After decimal point: warning subcode |
| monitor Display | 1 | 2 | 3 | 4 | 5 | Current parameter value: 12345 |  |  |
| monitor Display Sample | Use panel keys to check the current electrical angle (P09.17) value.. End Start/Servo power on Multiple MODE. |  |  |  |  |  |  |  |

3.2.2 Jog Operation

Before performing panel jog operation, please conduct a pre operation check on the servo system to confirm that the system is operating without interference. Connect the power supply, power on the servo system, and perform panel jog operation. The flowchart of panel jog operation is as follows:

1. Press MODE key to enter function code setting;

2. Set function code using 亳/廉/ keys:P20.00;

3. Press SET key to confirm and enter panel jog mode

Power on

Press MODE key to exit jog operation mode

1. Current panel display value: JOG;

2. Long press ▲/▼ keys to achieve forward and reverse jog operation

1. Panel displays current set jog speed value;

2. Adjust jog speed to desired value using ▲/▼/ keys;

3. Press SET key to enter jog state;

Servo in fault-free state, current display value: ry

1. Wiring: Sequence corresponds one-to-one, no short circuit, terminals not loose, ground wire reliably connected, etc.;

2. Power: Rated input power;

3. Others: Motor stably fixed, no interference in operating environment, etc.; Judgment criteria: Manually rotate the motor, motor rotates well

End

Jog operation

Speed adjustment

Enter jog mode

Pre-operation check

start

Figure 3-17 Panel jog operation flowchart

Use panel jog operation to confirm whether the servo system can operate normally, and whether there are any abnormal vibrations or sounds when the motor rotates.

3.3 Use the Servo3 Designer

Servo3 Designer is a debugging software for SV3 servo drives.

3.3.1 Overview

Zone3

Zone2

区域1

Figure 3-18 Main interface

The software is divided into three areas, as shown in the following figure:

Area 1: Toolbar area, entrance for servo debugging function, where users can click on relevant buttons to enter the corresponding function window;

Area 2: Function view layer, debugging function presentation area;

Area 3: Status bar area, displaying device status in real-time; Whether it is online, operating status, fault information, motor operation information, etc;

3.3.2 Operating Environment

This software is a green installation-free version.

Operational requirements

	Hardware environment: PC

	Operating System: Windows 7 x86&64 Windows10 x64、Windows11 x64

	Dependency: Net Framework 4.5 and above

3.3.3 Parameter Management

Click on the toolbar to access the function code parameter setting interface.

Figure 3-19 Parameter List

	Toolbar area

Select all or none of the function codes on the current page;

Open and save recipe file, saving recipe only saves the selected function codes on the current page;

Parameter formula comparison function, shown as in the following figure:

Figure 3-20 Parameter Comparison

The previous editing group and the next editing group;

Restore factory settings;

	View Area

Figure 3-21 Parameter List View

1.Function code grouping area: from user's view, functional groups are divided to make it more convenient for users;

3.3.4 Oscilloscope

	toolbar

Open the waveform file, file format is csv;

Save the current waveform file only, file format is csv;

Take a screenshot of the current waveform display area, picture format is bmp;

Measurement function: Click this button to perform measurement operations on the AB interval waveform by dragging A and B cursors in the waveform area;

Figure 3-22 Vernier measurement

Vernier function: After clicking this button, the mouse hovers over the waveform area to display the values of each channel in the waveform at the current mouse position;

: Waveform amplification. Clicking this button will enable amplification function when the button is pressed down; Click the button again to disable the amplification function; Left click to circle the waveform and zoom in on the circled area;

Waveform Recovery: Click this button to restore the waveform to its original state;

Adaptive coordinate system. Clicking this button will automatically calculate Max./Min. values of the waveform for vertical axis adaptation;

FFT analysis: Click this button and use the left mouse button to circle the waveform area. When the left mouse button is lifted, the software makes FFT analysis and pops up the analysis result, identifies 3 resonance point frequencies shown as in the following figure:

Figure 3-23 FFT resonance point analysis

Waveform comparison: Click this button and select the waveform to be compared, the waveform files in the file will be overlaid onto the existing waveform area

Figure 3-24 Waveform Comparison

: To cancel waveform comparison, click that button, the compared waveform is deleted from waveform area;

	Configuration

Channel configuration: Oscilloscope supports up to four channels for acquisition. Users can quickly generate the corresponding channel configuration of its mode through such 3 buttons as "position acquisition", "speed acquisition" and "torque acquisition".

Sampling mode: Supports 2 ways of trigger sampling and continuous sampling.

Trigger sampling: The accuracy can be 1 times carrier frequency, but number of sampling points is few(1024 points per channel). Users can click Button "trigger condition" to enter trigger condition setting interface to set it;

Continuous sampling: The precision of continuous sampling is millisecond level. It supports continuous sampling for a long time and wave form is automatically saved in Directory wavedata during sampling.

Figure 3-25 Sampling configuration interface

3.3.5 Initialization

(1) Motor parameter setting

Click the toolbar to set the parameters of the motor encoder:

Figure 3-26 Motor parameter management

	Open the file: The formula of SV3 servo-related motor parameters has been integrated in the debugging software, and the user can directly select the formula corresponding to the motor model. Click the "Open" button after selecting the recipe as shown below.

Figure 3-27 Opening the motor parameter formula

	Save the file: by editing the parameter value column, edit the parameters, after editing, click the "save file" button to save the motor formula;

	Upload check: Read all motor parameters from the encoder;

	Download check items: Check the parameters to be downloaded and click "Download" button to download the motor parameters to the encoder.

(2) Magnetic pole identification

This function is used to initialize the Angle of the motor during its initial operation.

Click toolbar to enter the magnetic pole identification interface:

Figure 3-28 Magnetic pole identification

(3) DI/DO monitoring

This function displays DI/DO function, status, and pin wiring information. It also supports forced DI/DO output and can make  DI/DO simulation.

Figure 3-29 DI/DO monitoring

3.3.6 Commissioning

(1) Speed JOG

This function can be used to control motor rotation in jog mode to detect if the motor can run normally and if there is any abnormality during the rotation.

Click toolbar to enter the speed JOG interface:

Figure 3-30 Speed JOG

Steps are as follows:

1. Enter the parameters of motor operating speed and acceleration/deceleration time;

2. Click enable switch to enable the drive;

3. Long press Button "Long press forward" and Button "Long press reverse" with the left mouse key to control the motor to move forward and reverse; After releasing the mouse, stop running.

(2) Position JOG

This function is mainly used to control the motor to run in a reciprocating or fixed distance manner at a specified speed within a specified operating limit.

Click toolbar to enter the JOG interface:

Figure 3-31 Position JOG

Steps are as follows:

1. Enter the parameters of motor operating speed and acceleration/deceleration time, and click Button Enable ON;

Figure 3-32 Enable ON

2. Setting of operating limit position: Long press Button Long-press Forward and Button Long-press Reverse with the left mouse key to set positive/negative limit positions

Figure 3-33 Setting of Positive/Negative Limit Positions

3. Operation curve planning: Set the trajectory parameters on operation curve;

Figure 3-34 Operating Curve Configuration

Click "Trajectory Verification" to generate a simulated  curve;

Figure 3-35: Generation of Running Trajectory

4. Run

Single time: The motor moves a specified distance within the operating limit.

Reciprocating: The motor moves back and forth within the specified limit.

Click "Start Forward" and "Start Reverse" for launch; Click Button "Stop" to stop the motor;

Figure 3-36 Run

(3) Preset position task

This function supports users to preset multiple running trajectories, up to 16 segments.

Click toolbar to enter the preset position task interface:

Figure 3-37 Position JOG

Steps are as follows:

1. Edit trajectory parameters for each segment: Select the specified task segment in the list, edit the running parameters, and click "Apply" to update the parameters to the list;

Figure 3-38 Position Segment Configuration

2. Operation settings

Operation mode: Support single and loop operations. Multi-stage position task can only run once in single mode; Multi-stage position task operates cyclically in loop mode;

Termination segment: The user can choose the number of segments to execute, and the program will run from the first segment to the termination segment;

Instruction type: Support relative displacement instruction and absolute displacement instruction;

Click 'Run' to start the multi-stage trajectory task, and the motor will run according to the preset trajectory;

Figure 3-39 Operation

(4) Return to Zero

This function supports 35 zeroing modes, and users can directly select the mode or generate corresponding zeroing modes based on the origin return method, starting direction, encountering limit trajectory, zeroing completion position, etc.

Figure 3-40 Zeroing

3.3.7 Tuning

(1) Offline inertia identification

This function is an offline inertia identification.

Click toolbar to enter the offline inertia recognition interface:

Figure 3-41 Inertia Identification

Steps are as follows:

1.	Parameter settings: Set identification mode, maximum speed, acceleration time. The software supports 4 modes: "0: speed mode, forward/reverse operation", "1: speed mode, electric operation", "2: position mode, forward/reverse operation", and "3: position mode, unidirectional operation";

2.	Click enable switch to enable the drive;

3.	Long press the "Long press forward" and "Long press reverse" buttons with the left mouse button to identify inertia,

4.	Real time display of the current identification result. If the change in the identification results is small, it can be determined that the identification is complete. Click "Download" button to write the identification result to the drive.

(2) Gain adjustment

The software supports 2 gain adjustment modes: automatic gain adjustment and manual gain adjustment.

	Automatic gain adjustment

Users can drag the slider with mouse or click "-" and "+" buttons to reduce or increase the rigidity level.

Figure 3-42 Rigid table settings

	Manual gain adjustment

It supports both sine and step forms of command, supports the adjustment of the corresponding loop gain under 3 control modes of position/speed/torque.

Figure 3-43 Adjust the speed loop gain

(3) Frequency domain analysis -FRF

Frequency domain analysis supports 3 modes: velocity closed loop, velocity open loop, and mechanical characteristics.

	Velocity closed loop: automatically calculate and label the amplitude bandwidth and phase bandwidth.

Figure 3-44 Velocity closed loop

	Velocity open loop: automatically calculate and mark the amplitude margin and phase margin.

Figure 3-45 Velocity open loop

	Mechanical characteristics: Automatic identification of resonance points and anti-resonance points. At present, the debugging software will automatically identify 2 resonance points. After identification, it will automatically update to the setting parameters of notch filter, and users can directly click download to write the identified resonance point frequency to the drive.

Figure 3-46 Mechanical Characteristics

3.3.8 Troubleshooting

(1) Real time fault

In case of equipment fault, the debugging software status bar will prompt users in real time (as shown below). User can double-click the fault area to view the fault details. Fault is marked in red and warning is marked in yellow.

Figure 3-47 Fault prompt

The fault details include: fault name, level, whether it can be reset, fault cause, detection method, and solution, etc., which facilitates users to quickly troubleshoot the fault.

Figure 3-48 Fault Diagnosis

(2) Fault History

This function supports querying the lastest 10 faults, information shown as above figure.

Figure 3-49 Fault History

Query button: query the recent failure history of the device, as shown in the above figure;

Clear button: Clear the historical fault records in the drive;

Select the historical list row with the mouse, and the relevant parameter information and troubleshooting of the selected fault will be displayed below the list;

(3) Fault Dictionary

This function can query the fault information of all SV3 servos;

Figure 3-50 Fault Dictionary

3.4 EtherCAT control operation

3.4.1 Running Status Control

(1) Status machine

The relationship between control word 6040h, status word 6041h, internal event and status machine is shown in the following figure: status switching can be performed through control words or internal events, and the current status can be read from status word.

Figure 3-51 Overview of CiA402 Protocol

Status machine describes the device status and possible control sequence of drive. A single status represents a particular internal or external behavior. The status of drive also determines which command is received. For example, point-to-point movement can start only when the drive is in the 'operation enabled' state.

Fault

Ready

Fault occurrence

Operation

Figure 3-52 CiA402 Status Machine Switching Diagram

Table 3-23 Status Description

| Status | Description |
| --- | --- |
| Initialization |  Control electricity input of servo drive;  Servo drive is initializing or self-checking;  If band-type brake function exists, it's currently in operation;  Driving function is invalid; |
| No fault, not enabled |  Servo driver initialization is completed;  Parameters of servo drive can be modified;  Power electricity of servo drive is not input;  Driving function is invalid; |
| Servo is ready |  Power electricity input of servo drive;  Parameters of servo drive can be modified;  Driving function is invalid; |
| Servo waiting enable |  Servo drive waits to be enabled; |
| Servo operation |  Servo drive doesn't detect any fault;  Motor power-on;  Part of parameters of servo drive can be modified;  Driving function is valid; |
| Fast stop |  Perform rapid stop action;  Motor power-on;  Driving function is valid; |
| Fault action |  Servo drive detects a fault;  Perform fault stop action;  Motor power-on;  Driving function is valid; |
| Fault |  Servo drive alarm;  Potor power-off;  Driving function is invalid; |

Table 3-24 Control Commands and Status Switching

| CiA402 status switching |  |  | Event | Action |
| --- | --- | --- | --- | --- |
| Serial No. | Initial status | Termination status |  |  |
| 0 | Start | Initialization | Reset | Servo self-test/initialization |
| 1 | Initialization | No fault, not enabled | Self check/initialization successful | Activate communication |
| 2 | No fault, not enabled | Servo is ready | Receive 'Shutdown' command from the master * 1 | No |
| 3 | Servo is ready | Servo waiting enable | Receive 'SwitchOn' command from the master | If no power electricity, input power electricity |
| 4 | Servo waiting enable | Servo operation | Receive 'Enable Operation' command from the master | Driving function is valid |
| 5 | Servo operation | Servo waiting enable | Receive 'Disable Operation' command from the master | Driver function is invalid |
| 6 | Servo waiting enable | Servo is ready | Receive 'Shutdown' command from the master | Power off |
| 7 | Servo is ready | No fault, not enabled | Receiving "Quick Stop" or "Disable Voltage" command from the master | No |
| 8 | Servo operation | Servo is ready | Receive 'Shutdown' command from the master | Immediately power off. If no band-type brake is available, the motor will stop freely |
| 9 | Servo operation | No fault, not enabled | Received 'Disable Voltage' command from the master | Immediately power off. If no band-type brake is available, the motor will stop freely |
| 10 | Servo waiting enable | No fault, not enabled | Receiving "Quick Stop" or "Disable Voltage" command from the master | Immediately power off. If no band-type brake is available, the motor will stop freely |
| 11 | Servo operation | Fast stop | Receive 'Quick Stop' command from the master | Perform rapid stop action |
| 12 | Fast stop | No fault, not enabled | Quick Stop execution is completed or receive 'Disable Voltage' command from the master | Power off |
| 13 | Arbitrary state | Fault action | Error occurrence | Fault execution stop |
| 14 | Fault action | fault | Error complete | Power off |
| 15 | Fault | No fault, not enabled | Fault Reset command received from the host | For fault clearing, set the control word "Fault Reset" of 0 after the fault is cleared |
| 16 | Fast stop | Operation enable | Receive 'Enable Operation' command from the master | Servo enable (need to set the fast stop mode to 5, 6, 7 or 8, please see the fast stop section) |

*1. The master sends stop command through control word. Various logical combinations of 1 to 0 in different bit positions constitue different commands.

(2) Control word 6040h

Table 3-25 Control word 6040h

| 0x6040- Control word |  |
| --- | --- |
| Index - Subindex | 0x6040-00 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | Operation settings/downtime effective |
| Related mode | ALL |
| Note | Bit; Name; Description. 0; Servo operation is available; Setting mode: 1-valid, 0-invalid. 1; Switch on the main circuit; Setting mode: 1-valid, 0-invalid. 2; Fast stop; Setting mode: 0-valid, 1-invalid. 3; Servo operation; Setting mode: 1-valid, 0-invalid. 4~6; For operation mode; Different operating modes have different meanings. 7; Fault reset; Reset faults and warnings that can be reset. Setting mode: Rising edge, if the value is 1, all the other control commands are invalid. 8; Pause; The stop mode is different for each motion mode. For details, see object dictionary 605A. 9; For operation mode; Different operating modes have different meanings. 10; Reserved; Parameter reserved, no meaning temporarily. 11~15; Manufacturer's custom; Manufacturer custom parameters. Control command Command; Control word. Bit7; Bit3; Bit 2; Bit 1; Bit0. Shut down; 0; X; 1; 1; 0. Switch on; 0; 0; 1; 1; 1. Enable operation; 0; 1; 1; 1; 1. Disable voltage; 0; X; X; 0; X. Quick stop; 0; X; 0; 1; X. Disable operation; 0; 0; 1; 1; 1. Fault reset; Rising edge; X; X; X; X. |

(3) Status word 6041h

Table 3-26 Status word 6041h

| 0x6041- Status word |  |
| --- | --- |
| Index - Subindex | 0x6041-00 |
| Data type | UINT16 |
| Accessibility | Readable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 0~65535 |
| Setting and effective mode | - |
| Related mode | ALL |
| Note | Reflect servo state Bit; Name; Description. 0; Servo is ready; Status: 1- Valid, 0- invalid. 1; Servo operation is available; Status: 1- Valid, 0- invalid. 2; Servo operation; Status: 1- Valid, 0- invalid. 3; fault; Status: 1- Valid, 0- invalid. 4; The main circuit is ON; Status: 1- Valid, 0- invalid. 5; Fast stop; Status: 0- valid, 1- invalid. 6; Servo can't run; Status: 1- Valid, 0- invalid. 7; Warning; Status: 1- Valid, 0- invalid. 8; Manufacturer's custom; Manufacturer custom parameters. 9; Remote control; Status: 1- Valid, 0- invalid. 10; Target arrival; Status: 1- Valid, 0- invalid. 11; Internal restriction valid; Status: 1- Valid, 0- invalid. 12~13; For operation mode; Different operating modes have different meanings. 14; Manufacturer's custom; Manufacturer custom parameters. 15; Origin found; Status: 1- Valid, 0- invalid. State feedback Status; Status word. Bit6; Bit5; Bit4; Bit3; Bit2; Bit1; Bit0. initialization; 0; X; X; 0; 0; 0; 0. No fault, not enabled; 1; X; X; 0; 0; 0; 0. Servo is ready; 0; 1; X; 0; 0; 0; 1. Servo waiting enable; 0; 1; X; 0; 0; 1; 1. Servo operation; 0; 1; X; 0; 1; 1; 1. Fast stop; 0; 0; X; 0; 1; 1; 1. Fault action; 0; X; X; 1; 1; 1; 1. fault; 0; X; X; 1; 0; 0; 0. |

3.4.2 PDO Configuration

PDO is divided into RxPDO and TxPDO. The master station sends instructions to the slave station through RxPDO, and the slave station feeds back its own status to the master station through TxPDO, as shown in the following figure.

Table 3-27 PDO communication

|  | Sending side | Receiving side |
| --- | --- | --- |
| RxPDO | Master station | Slave station |
| TxPDO | Slave station | Master station |

In practical applications, SV3 servo drive can only be used as a slave station, and the master station is usually PC or PLC. RxPDO sends control words, operation modes, speed commands,etc.; The servo drive feeds back status words, actual operation modes, speed actual values, and other status variables by TxPDO.

(1) PDO mapping

The mapping from object dictionary to the application object of PDO is called PDO mapping. (For PDO and SDO related, please refer to CANopen protocol manual)

SV3 servo provides 5 sets of fixed RxPDO and 4 sets of fixed TxPDO, as well as 1 set of variable RxPDO and 1 set of variable TxPDO. Max. application object has a data length of 32 bytes for each set of PDO.

The fixed PDO is shown in the following table:

Table 3-28 Fixed PDO Mapping List (RxPDO)

| RxPDO | Total number of bytes | Mapping objects |
| --- | --- | --- |
| 1701h | 12 | 6040h - Control Word 607Ah - Target position 60B8h - probe function 60FEh - digital output |
| 1702h | 19 | 6040h - Control Word 607Ah - Target position 60FFh - target speed 6071h - target torque 6060h - mode selection 60B8h - probe function 607Fh - maximum speed |
| 1703h | 17 | 6040h - Control Word 607Ah - Target position 60FFh - target speed 6060h - mode selection 60B8h - probe function 60E0h - forward torque limit 60E1h - reverse torque limit |
| 1704h | 23 | 6040h - Control Word 607Ah - Target position 60FFh - target speed 6071h - target torque 6060h - mode selection 60B8h - probe function 607Fh - maximum speed 60E0h - forward torque limit 60E1h - reverse torque limit |
| 1705h | 19 | 6040h - Control Word 607Ah - Target position 60FFh - target speed 6060h - mode selection 60B8h - probe function 60E0h - forward torque limit 60E1h - reverse torque limit 60B2h - torque bias |

Table 3-29 Fixed PDO Mapping List (TxPDO)

| TxPDO | Total number of bytes | Mapping objects |
| --- | --- | --- |
| 1B01h | 28 | 603Fh - error code 6041h - status Word 6064h - position feedback 6077h - torque feedback 60F4h - position deviation 60B9h - probe status 60BAh - probe 1 rising edge position 60FDh - DI status |
| 1B02h | 25 | 603Fh - error code 6041h - status Word 6064h - position feedback 6077h - torque feedback 6061h - Mode Display 60B9h - probe status 60BAh - probe 1 rising edge position 60BCh - probe 2 rising edge position 60FDh - DI status |
| 1B03h | 29 | 603Fh - error code 6041h - status Word 6064h - position feedback 6077h - torque feedback 60F4h - position deviation 6061h - Mode Selection 60B9h - probe status 60BAh - probe 1 rising edge position 60BCh - probe 2 rising edge position 60FDh - DI status |
| 1B04h | 29 | 603Fh - error code 6041h - status Word 6064h - position feedback 6077h - torque feedback 60F4h - position deviation 6061h - Mode Selection 60B9h - probe status 60BAh - probe 1 rising edge position 60BCh - probe 2 rising edge position 606Ch - speed feedback |

The variable PDO is shown in the following figure:

Table 3-30 Variable PDO Mapping List

| PDO | Index | Default mapping object | Remarks |
| --- | --- | --- | --- |
| RxPDO | 1600h | 6040h - Control Word 607Ah - Target position 60B8h - probe function 6060h operating mode | Max. mapping objects of 10 The longest byte number is 40 |
| TxPDO | 1A00h | 603Fh - error code 6041h - status Word 6061h - Current operating mode 6064h - position feedback 60BCh - probe 2 rising edge position 60B9h - probe status 60BAh - probe 1 rising edge position 60FDh - DI status |  |

(2) PDO allocation object

SM channel (SyncManager) is a memory segment on the slave control chip. In order to use PDO for data exchange, the list of PDO mapping objects must be switched to SM channel. As mentioned in the above section, SV3 has multiple sets of PDO mapping lists, but in practical application, one RxPDO and one TxPDO are selected for data exchange, as shown in the following table:

Table 3-31 SM Channel configuration

| Index | Subindex | Description |
| --- | --- | --- |
| 0x1C12h | 01h | Select one RxPDO as the actual RxPDO |
| 0x1C13h | 01h | Select one TxPDO as the actual TxPDO |

Note: RxPDO:0x1600h, 0x1701h~0x1705h are mapping lists and can be understood as a collection of partial data objects, 0x1C12h is to select a collection of data objects for the actual master-slave communication. The same goes for TxPDO.

3.4.3 Setting of Operation Mode

(1) Introduction of servo mode

SV3 servo drive supports 7 operating modes, Mode Control 6060h is used to control the servo operation in different control modes, and Mode Display 6061 is used to display the current control mode.

Table 3-32 Supported operating modes of SV3 servo drives

| Control mode | Control Mode 6060h Setting Value | Minimum communication cycle |
| --- | --- | --- |
| Profile position model | 1 | 1ms |
| Profile velocity model | 3 | 500μs |
| Profile torque mode | 4 | 125μs |
| Cyclic synchronization position mode | 8 | 1ms |
| Cyclic synchronization speed mode | 9 | 500μs |
| Cyclic synchronization torque mode | 10 | 125μs |
| The origin return model | 6 | 1ms |

Note: When the value is greater than 1ms, the synchronization cycle is an integer multiple of the position loop control cycle(the position loop control cycle is 250μs).

(2) Cyclic Synchronous Position(CSP)

In CSP mode, motion planning of the servo motor is completed by the master, and then the position command is sent to the servo drive periodically, and the communication cycle and sync mode are set by the master station.

Note:

	Min. communication cycle of CSP is 1ms. If the communication cycle is greater than 1ms, ensure that the communication cycle is an integer multiple of the position loop control cycle(the position loop control cycle is 250μs).

	In CSP mode, use DC synchronization.

	When the CSP mode is switched to other modes, the unexecuted position instruction will be discarded in any state.

	When switching from other modes to cyclic synchronization mode in servo operation state, please wait at least 1ms before sending instructions, otherwise instruction loss or errors might occur.

Control block diagram

Figure 3-53 Overview of cyclic synchronization position mode

Speed unit

conversion

Speed feedforward gain

Speed feedforward filter

Position offset 60 80h

Torque limit, filter, vibration suppression

Position command

60FCh

Position command

6062h

Electronic gear ratio forward transformation

Speed limit

Position limit

Torque feedforward gain

Torque feedforward filter

Electronic gear ratio forward transformation

Speed detection

Torque detection

Actual torque 6077h

Position feedback

6063h

Actual speed 606Ch

Torque command

60 74h

Probe position value

Position deviation

60F4

Speed offset 60B 1h

Torque offset 60B 2h

Target position 60 7Ah

encoder

Motor

Position feedback

60 64h

Current control

Speed control

Position control

Figure 3-54 Block diagram of cyclic synchronization  position mode

Related object (instruction * setting class)

Note: For detailed instructions on related objects, see Chapter 6 Object Dictionary.

Common object:

Table 3-33 0x6040- Control word

| 0x6040- Control word |  |
| --- | --- |
| Index - Subindex | 0x6040-00 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | Operation settings/downtime effective |
| Related mode | ALL |
| Note | In CSP mode, only absolute position instruction is supported Mode correlation: Bit0 to Bit3 are 1, indicating that the system is started Bit; Name; Description. 0; Servo is ready; Setting mode: 1-valid, 0-invalid. 1; Switch on the main circuit; Setting mode: 1-valid, 0-invalid. 2; Fast stop; Setting mode: 0-valid, 1-invalid. 3; Servo operation; Setting mode: 1-valid, 0-invalid. 8; Pause; 0: invalid. 1: The servo is set pause by 605Dh.. |

Table 3-34 Objects related to command Settings in CSP mode

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6040 | 00 | Control word | - | 0~65535 | UINT16 | RW | RxPDO |
| 6060 | 00 | Servo mode selection | - | 0~10 | INT8 | RW | RxPDO |
| 6065 | 00 | Threshold of large position deviation | Instruction unit | 0~(232-1) | UINT32 | RW | RxPDO |
| 6067 | 00 | Threshold of position arrival | Encoder unit | 0~(232-1) | UINT32 | RW | RxPDO |
| 6068 | 00 | Position arrival window time | ms | 0~65535 | UINT16 | RW | RxPDO |
| 6072 | 00 | Maximum torque instruction | 0.1% | 0~5000 | UINT16 | RW | RxPDO |
| 607A | 00 | Target position | Instruction unit | -231~(231-1) | INT32 | RW | RxPDO |
| 6091 | 01 | Motor resolution | - | 1~(232-1) | UINT32 | RW | RxPDO |
|  | 02 | Load shaft resolution | - | 1~(232-1) | UINT32 | RW | RxPDO |
| 60B0 | 00 | Position bias | Instruction unit | -231~(231-1) | INT32 | RW | RxPDO |
| 60B1 | 00 | Velocity bias | Instruction unit /s | -231~(231-1) | INT32 | RW | RxPDO |
| 60B2 | 00 | Torque bias | 0.1% | -5000~5000 | INT32 | RW | RxPDO |
| 2006 | 01 | Velocity proportional gain 1 | 0.1Hz | 1~20000 | UINT16 | RW | - |
|  | 02 | Velocity integral gain 1 | 0.01ms | 15~51200 | UINT16 | RW | - |
|  | 03 | Position proportional gain 1 | 0.1Hz | 0~20000 | UINT16 | RW | - |
|  | 09 | Speed feedforward proportional gain | 0.1% | 0~1000 | UINT16 | RW | - |
|  | 0A | Torque feedforward proportional gain | 0.1% | 0~2000 | UINT16 | RW | - |
| 2007 | 03 | Torque filtering 1 | 0.01ms | 0~3000 | UINT16 | RW | - |
|  | 07 | Velocity feedforward filtering time | 0.01ms | 0~6400 | UINT16 | RW | - |
|  | 08 | Torque feedforward filtering time | 0.01ms | 0~6400 | UINT16 | RW | - |

Related objects (status * monitor class)

Table 3-35 0x6041- Status words

| 0x6041- Status word |  |
| --- | --- |
| Index - Subindex | 0x6041-00 |
| Data type | UINT16 |
| Accessibility | Readable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | - |
| Related mode | ALL |
| Note | Reactive servo state For mode: Bit; Name; Description. 10; Target position arrival; Status display: 1- Arrived, 0- not arrived. 11; The software internal position overrun; Status display: 1- overrun, 0- not overrun. 12; Follow instructions from the slave station; Status: 1- Follow, 0- not follow. 13; Following error; Status display: 1- overrun, 0- not overrun. 15; Return to zero completion; Status display: 1- completed, 0- not completed. |

Table 3-36 Status monitoring objects in CSP mode

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 603F | 00 | Error code | - | 0~65535 | UINT16 | RO | TxPDO |
| 6041 | 00 | Status word | - | 0~65535 | UINT16 | RO | TxPDO |
| 6061 | 00 | Run mode display | - | 0~10 | INT8 | RO | TxPDO |
| 6062 | 00 | Position instruction | Instruction unit | - | DINT32 | RO | TxPDO |
| 6063 | 00 | Position feedback | Encoder unit | - | INT32 | RO | TxPDO |
| 6064 | 00 | Position feedback | Instruction unit | - | INT32 | RO | TxPDO |
| 606C | 00 | Actual velocity | Instruction unit /s | - | INT32 | RO | TxPDO |
| 6077 | 00 | Actual torque | 0.1% | -5000~5000 | INT16 | RO | TxPDO |
| 60F4 | 00 | Position deviation | Instruction unit | - | DINT32 | RO | TxPDO |
| 60FC | 00 | Position instruction | Encoder unit | - | DINT32 | RO | TxPDO |

Related function Settings

A) Positioning is complete

If the difference between the actual position and the target position is within a certain threshold range and maintains for a certain time, the positioning completion DO is valid, and  Bit10=1 of 6041.

Note:

Both positioning completion threshold and completion window time must be met at the same time. The control block diagram is as follows:

Comparison

window

DO position reached output

Status word

Target position 607A

Origin offset 607C

Limit

function

Timer

Positioning completion threshold 6067

Software limit 607D

Actual position 6064

Positioning completion window 6068

Figure 3-55 overview of CSP positioning completion

Position arrival window time

Position arrival window time

Actual velocity

Time

Threshold of position arrival

位置到达输出

Speed

Figure 3-56 diagram of CSP mode positioning completion

Related object parameters are shown in the following table:

Table 3-37 objects related to CSP positioning completion

| Index | Subindex | Name | Setting range |
| --- | --- | --- | --- |
| 2013 | 0C | Unit of position arrival threshold | 0: encoder unit 1: command unit |
| 6067 | 00 | Positioning completion threshold | 0~65535 |
| 6068 | 00 | Positioning completion window time | 0~65535 |

B) Threshold of garge position deviation

When the difference between the target position and the actual position exceeds a certain threshold, the servo drive will alarm.

Position feedback 6064

Er.423Excessive position deviation

Comparison

window

Position command 6062

Excessive position deviation threshold 6065

Figure 3-57 Overview of large CSP position deviation

Related object parameters are shown in the following table:

Table 3-38 Objects related to large CSP position deviation

| Index | Subindex | Name | Setting range |
| --- | --- | --- | --- |
| 6065 | 00 | Threshold of large position deviation | 0~(232-1) |

C) Position alignment

Before the servo is enabled, ensure that 607A (target position) +60B0 (position bias) is consistent with 6064 (actual position) so as to avoid high-speed motor movement due to misalignment, as shown in the following figure.

Solution: The upper software periodically assigns the position feedback value to the target position.

Actual position

Rapid move

Actual position

Servo drive enable

Servo drive disabled

Actual position

Position instruction

External force

Position instruction

Position instruction

Move

Servo drive enable

Figure 3-58 Case for CSP position alignment

(3) Cyclic Synchronization Speed(CSV) Mode

In cyclic synchronous speed mode, speed planning of the servo motor is completed by the master computer, and then the speed command is sent to the servo drive periodically, and the communication cycle and synchronization mode are set by the master station.

Note:

	The minimum communication cycle of CSV mode is 500μs.

	Please use DC synchronization in CSV mode.

	When CSV mode is switched to other modes, perform ramp stop in any state, and when the stop is complete, it may switch to others.

Control block diagram

Figure 3-59 Cyclic Synchronization Velocity (CSV) overview diagram

Torque offset60B 2h

Target speed 60FFh

Speed feedforward gain

Speed feedforward filter

Encoder

Motor

Torque limit, filter, vibration suppression

Torque feedforward gain

Torque feedforward filter

Electronic gear ratio forward transformation

Speed detection

Speed unit

conversion

Torque detection

Actual torque 6077h

Position feedback

6063h

Torque command

60 74h

Position feedback

60 64h

Probe position value

Speed offset 60B 1h

Speed unit conversion

Acceleration/deceleration limit

Speed control

Actual speed 606Ch

Current control

Figure 3-60 CSV block diagram

Related object (instruction * setting class)

Note: For detailed instructions on related objects, see Chapter 6 Object Dictionary.

Common object:

Table 3-39 0x6040- Control word

| 0x6040- Control word |  |
| --- | --- |
| Index - Subindex | 0x6040-00 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | Operation settings/downtime effective |
| Related mode | ALL |
| Note | In CSP mode, only absolute position instruction is supported Mode correlation: Bit0 to Bit3 are 1, indicating that the system is started Bit; Name; Description. 0; Servo is ready; Setting mode: 1-valid, 0-invalid. 1; Switch on the main circuit; Setting mode: 1-valid, 0-invalid. 2; Fast stop; Setting mode: 0-valid, 1-invalid. 3; Servo operation; Setting mode: 1-valid, 0-invalid. 8; Pause; 0: invalid. 1: The servo is set pause by 605Dh.. |

Table 3-40 Commands set objects in CSV mode

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6040 | 00 | Control word | - | 0~65535 | UINT16 | RW | RxPDO |
| 6060 | 00 | Servo mode selection | - | 0~10 | INT8 | RW | RxPDO |
| 607F | 00 | Max. speed | Instruction unit /s | 0~(232-1) | UDINT32 | RW | RxPDO |
| 6083 | 00 | Profile acceleration | Instruction unit /s2 | 0~(232-1) | UDINT32 | RW | RxPDO |
| 6084 | 00 | Profile deceleration | Instruction unit /s2 | 0~(232-1) | UDINT32 | RW | RxPDO |
| 60B1 | 00 | Velocity bias | Instruction unit /s | -231~(231-1) | INT32 | RW | RxPDO |
| 60B2 | 00 | Torque bias | 0.1% | -5000~5000 | INT32 | RW | RxPDO |
| 60E0 | 00 | Forward torque limit | 0.1% | 0~5000 | UINT16 | RW | RxPDO |
| 60E1 | 00 | Reverse torque limit | 0.1% | 0~5000 | UINT16 | RW | RxPDO |
| 60FF | 00 | Target speed | Instruction unit /s | -231~(231-1) | INT32 | RW | RxPDO |
| 2006 | 01 | Velocity proportional gain 1 | 0.1Hz | 1~20000 | UINT16 | RW | - |
|  | 02 | Velocity integral gain 1 | 0.01ms | 15~51200 | UINT16 | RW | - |
|  | 0A | Torque feedforward proportional gain | 0.1% | 0~2000 | UINT16 | RW | - |
| 2007 | 03 | Torque filtering 1 | 0.01ms | 0~3000 | UINT16 | RW | - |
|  | 08 | Torque feedforward filtering time | 0.01ms | 0~6400 | UINT16 | RW | - |

Related objects (status * monitor class)

Table 3-41 0x6041- Status word

| 0x6041- Status word |  |
| --- | --- |
| Index - Subindex | 0x6041-00 |
| Data type | UINT16 |
| Accessibility | Readable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | - |
| Related mode | ALL |
| Note | Reactive servo state For mode: Bit; Name; Description. 10; Target speed arrival; Status display: 1- Arrived, 0- not arrived. 12; Follow instructions from the slave station; Status: 1- Follow, 0- not follow. 15; The origin return to zero is complete; Status display: 1- completed, 0- not completed. |

Table 3-42 Status monitoring objects in CSV mode

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 603F | 00 | Error code | - | 0~65535 | UINT16 | RO | TxPDO |
| 6041 | 00 | Status word | - | 0~65535 | UINT16 | RO | TxPDO |
| 6061 | 00 | Run mode display | - | 0~10 | INT8 | RO | TxPDO |
| 6063 | 00 | Position feedback | Encoder unit | - | INT32 | RO | TxPDO |
| 6064 | 00 | Position feedback | Instruction unit | - | INT32 | RO | TxPDO |
| 606C | 00 | Actual velocity | Instruction unit /s | - | INT32 | RO | TxPDO |
| 6077 | 00 | Actual torque | 0.1% | -5000~5000 | INT16 | RO | TxPDO |

Related function Settings

A) Speed limit

Speed limit of motor is determined by Max. speed;

B) Speed arrival function

If actual speed exceeds the threshold of speed arrival signal and remains for a period, the speed arrival DO is valid and Bit10 =1 of status word 6041.

Related object parameters are shown in the following table:

Table 3-43 CSV mode speed arrival function related objects

| Index | Subindex | Name | Setting range |
| --- | --- | --- | --- |
| 606Dh | 00 | the threshold of speed arrival | 0~65535 |
| 606Eh | 00 | Speed arrives window time | 0~65535 |

Target speed

The threshold of speed arrival

The threshold of speed arrival

Speed arrives window time

Speed reach output

Speed arrives window time

Time

Actual velocity

Target speed

Speed

Figure 3-61 Diagram of CSV speed arrival function

(4) Cyclic Synchronous Torque(CST) Mode

In CST mode, motion planning of the servo motor is completed by the master computer, and then the torque command is periodically sent to the servo drive, and the communication cycle and synchronization mode are set by the master station.

Note:

	Min. communication cycle of CST mode is 125μs.

	Please use DC synchronization in CST mode.

	When CST mode is switched to other modes, perform ramp stop in any state. After the stop is completed, it may switch to other modes;

	In CST mode, the speed will enter the speed control when it reaches the limit.

Control block diagram

Figure 3-62 Overview of cyclic synchronous torque (CST)

Speed unit conversion

Target torque

607 1h

Torque limit, filter, vibration suppression

Speed detection

Torque detection

Actual torque

6077h

Actual speed

606Ch

Position feedback

6063h

Torque command

60 74h

Position feedback

60 64h

Probe position value

Torque offset

60B 2h

Speed

Limiting

Electronic gear ratio forward transformation

Current control

Motor

Encoder

Figure 3-63 Block diagram of CST mode

Related object (instruction * setting class)

Note: For detailed instructions on related objects, see Chapter 6 Object Dictionary.

Common object:

Table 3-44 0x6040- Control word

| 0x6040- Control word |  |
| --- | --- |
| Index - Subindex | 0x6040-00 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | Operation settings/downtime effective |
| Related mode | ALL |
| Note | In CSP mode, only absolute position instruction is supported Mode correlation: Bit0 to Bit3 are 1, indicating that the system is started Bit; Name; Description. 0; Servo is ready; Setting mode: 1-valid, 0-invalid. 1; Switch on the main circuit; Setting mode: 1-valid, 0-invalid. 2; Fast stop; Setting mode: 0-valid, 1-invalid. 3; Servo operation; Setting mode: 1-valid, 0-invalid. 8; Pause; 0: invalid. 1: The servo is set pause by 605Dh.. |

Table 3-45 Objects related to command settings in CST mode

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6040 | 00 | Control word | - | 0~65535 | UINT16 | RW | RxPDO |
| 6060 | 00 | Servo mode selection | - | 0~10 | INT8 | RW | RxPDO |
| 6071 | 00 | Target torque | 0.1% | -5000~5000 | INT16 | RW | RxPDO |
| 607F | 00 | Max. speed | Instruction unit /s | 0~(232-1) | UDINT32 | RW | RxPDO |
| 60B2 | 00 | Torque bias | 0.1% | -5000~5000 | INT32 | RW | RxPDO |
| 60E0 | 00 | Forward torque limit | 0.1% | 0~5000 | UINT16 | RW | RxPDO |
| 60E1 | 00 | Reverse torque limit | 0.1% | 0~5000 | UINT16 | RW | RxPDO |
| 2006 | 01 | Velocity proportional gain 1 | 0.1Hz | 1~20000 | UINT16 | RW | - |
|  | 02 | Velocity integral gain 1 | 0.01ms | 15~51200 | UINT16 | RW | - |
| 2007 | 03 | Torque filtering 1 | 0.01ms | 0~3000 | UINT16 | RW | - |

Related objects (status * monitor class)

Table 3-46 0x6041- Status words

| 0x6041- Status word |  |
| --- | --- |
| Index - Subindex | 0x6041-00 |
| Data type | UINT16 |
| Accessibility | Readable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | - |
| Related mode | ALL |
| Note | Reactive servo state For mode: Bit; Name; Description. 10; Target torque arrival; Status display: 1- Arrived, 0- not arrived. 12; Follow instructions from the slave station; Status: 1- Follow, 0- not follow. 15; The origin return to zero is complete; Status display: 1- completed, 0- not completed. |

Table 3-47 Status monitoring objects in CST mode

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6040 | 00 | Control word | RW | UINT16 | - | 0~65535 | TxPDO |
| 603F | 00 | Error code | RO | UINT16 | - | 0~65535 | TxPDO |
| 6041 | 00 | Status word | RO | UINT16 | - | 0~65535 | TxPDO |
| 6061 | 00 | Run mode display | RO | INT8 | - | 0~10 | TxPDO |
| 606C | 00 | Actual velocity | RO | INT32 | Instruction unit /s | - | TxPDO |
| 6074 | 00 | Torque command | RO | INT16 | 0.1% | -5000~5000 | TxPDO |
| 6077 | 00 | Actual torque | RO | INT16 | 0.1% | -5000~5000 | TxPDO |

Related function Settings

A) Speed limit

The speed limit is determined by the smaller value of 607Fh and Max. motor speed;

Table 3-48 Related objects of the speed limit in CST mode

| Index | Subindex | Name | Setting range |
| --- | --- | --- | --- |
| 607F | 00 | Max. speed | 0~(232-1) |

B) Torque arrival

When the difference between torque and reference value is greater than the value of 2015h:12, it outputs valid arrival signal TOQREACH, and Bit10 of status word 6041 is set of 1.

When the difference between torque and reference value is less than the value of 2015h:13, the output is invalid, and Bit10 of status word 6041 is cleared to zero.

Table 3-49 Objects related to torque arrival in CST mode

| Index | Subindex | Name | Setting range |
| --- | --- | --- | --- |
| 2015 | 11 | Reference value of torque arrival | 0~3000 (unit: 0.1%) |
| 2015 | 12 | Torque arrival valid value | 0~3000 (unit: 0.1%) |
| 2015 | 13 | Torque arrival invalid value | 0~3000 (unit: 0.1%) |

(5) Profile Position(PP) Mode

In PP mode, the upper controller specifies the target position, profile speed, profile acceleration, profile deceleration, etc. Motor motion planning is carried out inside the servo drive, suitable for point-to-point motion.

Note:

	Min. communication cycle in PP mode is 1ms. If the communication cycle is set to more than 1ms, ensure that the communication cycle is an integer multiple of the position loop control cycle(the position loop control cycle is 250μs).

	When the PP mode is switched to other modes, any unexecuted position instruction would be discarded in any state.

Control block diagram

Actual torque

6077h

Position feedback

6063h

Actual speed

606Ch

Torque command

60 74h

Position feedback

60 64h

Probe position value

Position deviation

60F4

Profile acceleration/deceleratio

Profile speed

Speed unit

conversion

Speed

detection

Torque

detection

Encoder

Torque limit, filter, vibration suppression

Speed control

Torque feedforward gain

Torque feedforward filter

Position command

60FCh

Position command

6062h

Position offset

60 80h

Target position

60 7Ah

Trajectory generator

Electronic gear ratio forward transformation

Position control

Speed feedforward gain

Speed feedforward filter

Electronic gear ratio forward transformation

Current control

Motor

Figure 3-64 Block diagram of outline position mode

Related object (instruction * setting class)

Note: For detailed instructions on related objects, see Chapter 6 Object Dictionary.

Common object:

Table 3-50 0x6040- Control word

| 0x6040- Control word |  |
| --- | --- |
| Index - Subindex | 0x6040-00 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | Operation settings/downtime effective |
| Related mode | ALL |
| Note | In CSP mode, only absolute position instruction is supported Mode correlation: Bit0 to Bit3 are 1, indicating that the system is started Bit; Name; Description. 0; Servo is ready; Setting mode: 1-valid, 0-invalid. 1; Switch on the main circuit; Setting mode: 1-valid, 0-invalid. 2; Fast stop; Setting mode: 0-valid, 1-invalid. 3; Servo operation; Setting mode: 1-valid, 0-invalid. 4; New target location: New set-point; Effective mode: Rising edge. 5; Change set immediately; 0: Non-immediate change mode 1: Immediate change mode. 6; Absolute position /relative position abs/rel; 0: Position instruction is an absolute position instruction 1: Position instruction is a relative position instruction. |

Table 3-51 Objects related to instruction setting in PP mode

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6040 | 00 | Control word | - | 0~(232-1) | UINT16 | RW | RxPDO |
| 6060 | 00 | Servo mode selection | - | 0~65535 | INT8 | RW | RxPDO |
| 6065 | 00 | Threshold of large position deviation | Instruction unit | 0~65535 | UDINT32 | RW | RxPDO |
| 6067 | 00 | Threshold of position arrival | Encoder unit | -231~(231-1) | UINT32 | RW | RxPDO |
| 6068 | 00 | Position arrival window | ms | 0~(232-1) | UINT16 | RW | RxPDO |
| 607A | 00 | Target position | Instruction unit | 0~(232-1) | INT32 | RW | RxPDO |
| 6083 | 00 | Profile acceleration | Instruction unit /s2 | 0~(232-1) | UDINT32 | RW | RxPDO |
| 6084 | 00 | Profile deceleration | Instruction unit /s2 | 1~(232-1) | UDINT32 | RW | RxPDO |
| 6091 | 01 | Motor resolution | - | 0~5000 | UINT32 | RW | RxPDO |
| 6091 | 02 | Load shaft resolution | - | 0~5000 | UINT32 | RW | RxPDO |
| 60E0 | 00 | Forward torque limit | 0.1% | 0~3000 | UINT16 | RW | RxPDO |
| 60E1 | 00 | Reverse torque limit | 0.1% | 1~20000 | UINT16 | RW | RxPDO |
| 2006 | 01 | Velocity proportional gain 1 | 0.1Hz | 0~20000 | UINT16 | RW | - |
|  | 02 | Velocity integral gain 1 | 0.01ms | 0~6400 | UINT16 | RW | - |
|  | 03 | Position proportional gain 1 | 0.1Hz | 0~1000 | UINT16 | RW | - |
|  | 09 | Speed feedforward proportional gain | 0.1% | 0~2000 | UINT16 | RW | - |
|  | 0A | Torque feedforward proportional gain | 0.1% | 0~65535 | UINT16 | RW | - |
| 2007 | 03 | Torque filtering 1 | 0.01ms | 15~51200 | UINT16 | RW | - |
|  | 07 | Velocity feedforward filtering time | 0.01ms | 0~6400 | UINT16 | RW | - |
|  | 08 | Torque feedforward filtering time | 0.01ms | 0~(232-1) | UINT16 | RW | - |

Related objects (status * monitor class)

Table 3-52 0x6041 Status word

| 0x6041- Status word |  |
| --- | --- |
| Index - Subindex | 0x6041-00 |
| Data type | UINT16 |
| Accessibility | Readable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | - |
| Related mode | ALL |
| Note | Reactive servo state Mode related: After quick stop ends, Bit10 of status word 6041 is set as 1, and the servo is in the stop\ state. Bit; Name; Description. 10; Target position arrival; Status display: 1- Arrived, 0- not arrived. 12; Follow instructions from the slave station; Status: 1- Follow, 0- not follow. 13; Following error; Status: 1- Error, 0- No error. 15; The origin return to zero is complete; Status display: 1- completed, 0- not completed. |

Table 3-53 Objects related to state monitoring in PP mode

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 603F | 00 | Error code | - | 0~65535 | UINT16 | RO | TxPDO |
| 6041 | 00 | Status word | - | 0~65535 | UINT16 | RO | TxPDO |
| 6061 | 00 | Run mode display | - | 0~10 | INT8 | RO | TxPDO |
| 6062 | 00 | Position instruction | Instruction unit | - | DINT32 | RO | TxPDO |
| 6063 | 00 | Position feedback | Encoder unit | - | INT32 | RO | TxPDO |
| 6064 | 00 | Position feedback | Instruction unit | - | INT32 | RO | TxPDO |
| 606C | 00 | Actual velocity | Instruction unit /s | - | INT32 | RO | TxPDO |
| 6077 | 00 | Actual torque | 0.1% | -5000~5000 | INT16 | RO | TxPDO |
| 60F4 | 00 | Position deviation | Instruction unit | - | DINT32 | RO | TxPDO |
| 60FC | 00 | Position instruction | Encoder unit | - | DINT32 | RO | TxPDO |

Related function Settings

A) Positioning is complete

If the difference between the actual position and the target position is within a certain threshold range and maintains for a certain time, the positioning completion DO is valid, and  Bit10=1 of 6041.

Note:

It must meet simultaneously both positioning completion threshold and completion window time.

Related object parameters are shown in the following table:

Table 3-54 Objects related to positioning completion in PP mode

| Index | Subindex | Name | Setting range |
| --- | --- | --- | --- |
| 2013 | 0C | Unit of position arrival threshold | 0: instruction unit 1: encoder unit |
| 6067 | 00 | Threshold of position arrival | 0~65535 |
| 6068 | 00 | Position arrival window time | 0~65535 |

B) Threshold of garge position deviation

When the difference between the target position and the actual position exceeds a certain threshold, the servo drive will alarm.

Related object parameters are shown in the following table:

Table 3-55 Objects related to excessive position deviation in PP mode

| Index | Subindex | Name | Setting range |
| --- | --- | --- | --- |
| 6065 | 00 | Threshold of large position deviation | 0~(232-1) |

C) Speed limit

The speed limit is determined by the smaller value of 607Fh and Max. motor speed;

Table 3-56 Objects related to speed limit in PP mode

| Index | Subindex | Name | Setting range |
| --- | --- | --- | --- |
| 607F | 00 | Max. speed | 0~(232-1) |

(6) Cases for PP action

A)Case 1: Basic set-point

①: Upper controller inputs new target position instruction;

②: 6040h control word Bit4 (New set-point) is set as 1;

③: Receive position instructions from the rising edge of control word Bit4 at 6040h, and start positioning, then set 6041h status word Bit12 (Set-point acknowledgement) of 1;

④: The master station confirms that 6041h status word Bit12  has been set of 1, and then sets 6040h control word  Bit4 of 0, and it can receive new position instruction;

⑤: The slave statio confirms that 6040h control word Bit4 has been set of 0, and set 6041h status word Bit12 of 0;

⑥: Positioning completed, 6041h status word Bit10 positioning completed set to 1.

Figure 3-65 PP Mode Basic Set-Point

B) Case 2: Set of set-point (not immediate update mode)

①: The upper controller inputs the target position command;

②: 6040h control word Bit4 (New set-point) is set as 1;

③: Receive position instructions from the rising edge of control word Bit4 at 6040h, and start positioning, then set 6041h status word Bit12 (Set-point acknowledgement) of 1;

④: The master station confirms that 6041h status word Bit12  has been set of 1, and then sets 6040h control word  Bit4 of 0, and it can receive new position instruction;

⑤: The slave station confirms that 6040h control word Bit4 has been set of 0, then set  6041h status word Bit12 of 0;

⑥: The upper controller inputs the target position command;

⑦: Set 6040h control word Bit4 (New set-point) of 1;

⑧: The slave station receives position command from the rising edge of 6040h control word Bit4, but not start positioning. Then set 6041h status word Bit12 (Set point acknowledgement) of 1;

⑨ The master station confirms that 6041h status word Bit12 has been set of 1, and then sets 6040h control word Bit4 of 0, and it can receive new position instructions;

A: After the completion of the first position command, the servo motor stops, the slave station sets 6041h status word Bit12 of 0, and starts a new positioning;

B: Positioning completed, 6041h status word Bit10 positioning completed set of 1.

Figure 3-66 PP mode set of set-point (not immediate update mode)

C) Case 3: Single set-poinT (immediate update mode)

①: The upper controller inputs the target position command;

②: 6040h control word Bit4 (New set-point) is set as 1;

③: Receive position instructions from the rising edge of control word Bit4 at 6040h, and start positioning, then set 6041h status word Bit12 (Set-point acknowledgement) of 1;

④: The master station confirms that 6041h status word Bit12  has been set of 1, and then sets 6040h control word  Bit4 of 0, and it can receive new position instruction;

⑤: The slave station confirms that 6040h control word Bit4 has been set of 0, then set  6041h status word Bit12 of 0;

⑥: The upper controller inputs the target position command;

⑦: Set 6040h control word Bit4 (New set-point) of 1;

⑧: The slave station receives position instructions on the rising edge of 6040h control word Bit4, executes new positioning with new instructions (profile speed, acceleration/deceleration, etc.), and then set 6041h status word Bit12 (Set-point acknowledgement) of 1;

⑨ The master station confirms that 6041h status word Bit12 has been set of 1, and then sets 6040h control word Bit4 of 0, and it can receive new position instructions;

A: The slave station confirms that 6040h control word Bit4 has been set of 0. Set 6041h status word Bit12 of 0;

B: Positioning completed, 6041h status word Bit10 positioning completed set of 1.

Figure 3-67 PP mode Single set-Point (immediate update mode)

D) Case 4: Relative motion and absolute motion

CSP mode only supports absolute position commands, while PP mode supports both absolute and relative position commands.

Absolute position command: After receiving the position command, the servo drive will drive the motor to make the actual feedback position consistent with the target position. After positioning is completed, the actual position of the servo motor remains consistent with the target position.

Relative position command: After receiving the position command, the servo drive will drive the motor to make the actual position increment fed back by the motor consistent with the target position. After positioning is completed, the actual position of the servo motor doesn't need to be consistent with the target position.

Shown as in the following figure:

Target position

First segment position command

Second segment position command

Target position

Second segment position command

First segment position command

Relative position mode

Absolute position mode

Figure 3–68  PPSchematic diagram of the difference between relative motion and absolute motion

(7) Profile Velocity(PV)

In PV mode, the upper controller specifies target speed, profile acceleration, profile deceleration, etc., and the servo drive performs motor motion planning internally.

Note:

Min. communication cycle in PV mode is 500 μs;

When switching PV mode to other modes, perform ramp stop in any state, and after the stop is completed, it may switch to other modes.

Control block diagram

Torque limit, filter, vibration suppression

Actual torque 6077h

Trajectory generator

Encoder

Torque detection

Speed detection

Current control

Electronic gear ratio forward transformation

Speed unit conversion

Actual speed 606Ch

Torque command

60 74h

Probe position value

Torque feedforward gain

Torque feedforward filter

Deceleration point

6084h

Acceleration6083h

Maximum speed

607Fh

Target speed 60FFh

Speed control

Position feedback

60 64h

Position feedback

6063h

Motor

Figure 3-69 Control Block Diagram of Profile Velocity(PV)

Related object (instruction * setting class)

Note: For detailed instructions on related objects, see Chapter 6 Object Dictionary.

Common object:

Table 3-57 0x6040- Control word

| 0x6040- Control word |  |
| --- | --- |
| Index - Subindex | 0x6040-00 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | Operation settings/downtime effective |
| Related mode | ALL |
| Note | In CSP mode, only absolute position instruction is supported Mode correlation: Bit0 to Bit3 are 1, indicating that the system is started Bit; Name; Description. 0; Servo is ready; Setting mode: 1-valid, 0-invalid. 1; Switch on the main circuit; Setting mode: 1-valid, 0-invalid. 2; Fast stop; Setting mode: 0-valid, 1-invalid. 3; Servo operation; Setting mode: 1-valid, 0-invalid. 8; Pause; 0: invalid. 1: The servo is set pause by 605Dh.. |

Table 3-58 0x6040- Control word

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6040 | 00 | Control word | - | 0~65535 | UINT16 | RW | RxPDO |
| 6060 | 00 | Servo mode selection | - | 0~10 | INT8 | RW | RxPDO |
| 607F | 00 | Max. profile velocity | Instruction unit /s | 0~(232-1) | UINT32 | RW | RxPDO |
| 60FF | 00 | Target speed | Instruction unit /s | -231~(231-1) | INT32 | RW | RxPDO |
| 60E0 | 00 | Forward torque limit | 0.1% | 0~5000 | UINT16 | RW | RxPDO |
| 60E1 | 00 | Reverse torque limit | 0.1% | 0~5000 | UINT16 | RW | RxPDO |
| 2006 | 01 | Velocity proportional gain 1 | 0.1Hz | 1~20000 | UINT16 | RW | - |
|  | 02 | Velocity integral gain 1 | 0.01ms | 15~51200 | UINT16 | RW | - |
|  | 0A | Torque feedforward proportional gain | 0.1% | 0~2000 | UINT16 | RW | - |
| 2007 | 03 | Torque filtering 1 | 0.01ms | 0~3000 | UINT16 | RW | - |
|  | 08 | Torque feedforward filtering time | 0.01ms | 0~6400 | UINT16 | RW | - |

Related objects (status * monitor class)

Table 3-59 0x6041- Status word

| 0x6041- Status word |  |
| --- | --- |
| Index - Subindex | 0x6041-00 |
| Data type | UINT16 |
| Accessibility | Readable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | - |
| Related mode | PST |
| Note | Reactive servo state For mode: Bit; Name; Description. 10; Target speed arrival; Status display: 1- Arrived, 0- not arrived. 11; The software internal position overrun; Status display: 1- overrun, 0- not overrun. 15; The origin return to zero is complete; Status display: 1- completed, 0- not completed. |

Table 3-60 Objects related to state monitoring in PV mode

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 603F | 00 | Error code | - | 0~65535 | UINT16 | RO | TxPDO |
| 6041 | 00 | Status word | - | 0~65535 | UINT16 | RO | TxPDO |
| 6061 | 00 | Run mode display | - | 0~10 | INT8 | RO | TxPDO |
| 6063 | 00 | Position feedback | Encoder unit | - | INT32 | RO | TxPDO |
| 6064 | 00 | Position feedback | Instruction unit | - | INT32 | RO | TxPDO |
| 606C | 00 | Actual velocity | Instruction unit /s | - | INT32 | RO | TxPDO |
| 6077 | 00 | Actual torque | 0.1% | -5000~5000 | INT16 | RO | TxPDO |

Related function Settings

A) Speed limit

The speed limit is determined by the smaller value of 607Fh and Max. motor speed;

Table 3-61 Objects related to speed limit in PV mode

| Index | Subindex | Name | Setting range |
| --- | --- | --- | --- |
| 607F | 00 | Max. speed | 0-(232-1) |

B) Speed arrival function

If the difference between target speed and actual speed is within a certain threshold and maintained for a period, the speed reaches DO valid and Bit10=1 of status word 6041.

Related object parameters are shown in the following table:

Table 3-62 Objects related to speed arrival function in PV mode

| Index | Subindex | Name | Setting range |
| --- | --- | --- | --- |
| 606D | 00 | the threshold of speed arrival | 0~65535 |
| 606E | 00 | Speed arrives window time | 0~65535 |

Actual velocity

Speed arrives window time

Speed arrives window time

The threshold of speed arrival

The threshold of speed arrival

Time

Speed

Target speed

Speed reach output

Target speed

Figure 3-70 Schematic diagram of speed arrival in PV mode

(8) Profile Torque(PT)

In PT mode, upper controller specifies the target torque, torque slope, etc. Motor motion planning is performed inside servo drive.

Note:

	Min. communication cycle in PT mode is 125μs;

	When PT mode is switched to others, perform ramp stop in any state. After the stop is completed, it can switch to other modes;

	In CST mode, the speed will enter the speed control when it reaches the limit.

Control block diagram

Motor

Current control

Torque limit, filter, vibration suppression

Electronic gear ratio forward transformation

Speed unit conversion

Speed detection

Torque detection

Actual torque6077h

Position feedback

6063h

Actual speed 606Ch

Torque command

60 74h

Position feedback

60 64h

Probe position value

Torque limit

60E0h/60E1h

Torque ramp 6087h

Target torque

607 1h

Maximum torque

6072h

Trajectory generator

Speed Limiting

Encoder

Figure 3-71 Control Block Diagram of Profile Torque(PT)

Related object (instruction * setting class)

Note: For detailed instructions on related objects, see Chapter 6 Object Dictionary.

Common object:

Table 3-63 0x6040- Control word

| 0x6040- Control word |  |
| --- | --- |
| Index - Subindex | 0x6040-00 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | Operation settings/downtime effective |
| Related mode | ALL |
| Note | In CSP mode, only absolute position instruction is supported Mode correlation: Bit0 to Bit3 are 1, indicating that the system is started Bit; Name; Description. 0; Servo is ready; Setting mode: 1-valid, 0-invalid. 1; Switch on the main circuit; Setting mode: 1-valid, 0-invalid. 2; Fast stop; Setting mode: 0-valid, 1-invalid. 3; Servo operation; Setting mode: 1-valid, 0-invalid. 8; Pause; 0: invalid. 1: The servo is set pause by 605Dh.. |

Table 3-64 Objects related to instruction settings in PT mode

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6040 | 00 | Control word | - | 0~65535 | UINT16 | RW | RxPDO |
| 6060 | 00 | Servo mode selection | - | 0~10 | INT8 | RW | RxPDO |
| 6071 | 00 | Target torque | 0.1% | -5000~5000 | INT16 | RW | RxPDO |
| 6072 | 00 | Max. torque | 0.1% | 0~5000 | UINT16 | RW | RxPDO |
| 607F | 00 | Max. profile velocity | Instruction unit /s | 0~(232-1) | UINT32 | RW | RxPDO |
| 6087 | 00 | Torque ramp | 0.1%/s | 0~(232-1) | UDINT32 | RW | RxPDO |
| 2007 | 03 | Torque filtering 1 | 0.01ms | 0~3000 | UINT16 | RW | - |

Related objects (status * monitor class)

Table 3-65 0x6041- Status words

| 0x6041- Status word |  |
| --- | --- |
| Index - Subindex | 0x6041-00 |
| Data type | UINT16 |
| Accessibility | Readable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | - |
| Related mode | ALL |
| Note | Reactive servo state For mode: Bit; Name; Description. 10; Target torque arrival; Status display: 1- Arrived, 0- not arrived. 11; The software internal position overrun; Status display: 1- overrun, 0- not overrun. 15; The origin return to zero is complete; Status display: 1- completed, 0- not completed. |

Table 3-66 Objects related to status monitoring in PT mode

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 603F | 00 | Error code | - | 0~65535 | UINT16 | RO | TxPDO |
| 6041 | 00 | Status word | - | 0~65535 | UINT16 | RO | TxPDO |
| 6061 | 00 | Run mode display | - | 0~10 | INT8 | RO | TxPDO |
| 606C | 00 | Actual velocity | Instruction unit /s | - | INT32 | RO | TxPDO |
| 6074 | 00 | Torque command | 0.1% | - | INT16 | RO | TxPDO |
| 6077 | 00 | Actual torque | 0.1% | - | INT16 | RO | TxPDO |

Related function Settings

A) Speed limit

Speed limit in PT mode is set by 2007-12h.

Table 3-67 Objects related to speed limit in PT mode

| P15.03- Speed limit source selection |  |
| --- | --- |
| Index - Subindex | 0x2015-04 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | 1 |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | Operation settings/downtime effective |
| Related mode | ALL |
| Note | Settings; Description. 0; Internal speed limit; Forward speed limit: P15.05 Reverse speed limit: P15.06. 1; EtherCAT External speed limits; Forward speed limit: min{607Fh, P15.07} Reverse speed limit: min{607Fh, P15.08}. 2; Speed limiting is done in DI function 13; DI(Function 13) invalid: Forward/reverse speed is limited by P15.11 DI(Function 13) valid: Forward and reverse speed is limited by P15.12. |

B) Torque arrival

When the difference between the torque and the reference is greater than P15.17 value, the valid arrival signal TOQREACH is output, and Bit10 of status word 6041 is set of 1. When the difference between the torque and the reference  is less than P15.18 value, the output is invalid, and Bit10 of status word 6041 is cleared to zero.

Table 3-68 Objects related to PT mode torque arrival

| Index | Subindex | Name | Setting range |
| --- | --- | --- | --- |
| 2015 | 11 | Reference value of torque arrival | 0-3000 (unit 0.1%) |
| 2015 | 12 | Torque arrival valid value | 0-3000 (unit 0.1%) |
| 2015 | 13 | Torque arrival invalid value | 0-3000 (unit 0.1%) |

C) Torque limit

Torque limit is the maximum limit of the servo output torque, applicable to position/speed/torque modes.

Table 3-69 P15.03 torque limit source selection

| P15.03 Torque limit source selection |  |
| --- | --- |
| Index - Subindex | 0x2015-04 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | 1 |
| Default value | 2 |
| Min. | 0 |
| Max. | 4 |
| Setting and effective mode | Operation settings/downtime effective |
| Related mode | ALL |
| Note | Torque limiting source selection Settings; Description. 0; Forward internal torque limit: P15.05 Reverse internal torque limit: P15.06. 1; Positive external torque limit: When P-CL is valid: P15.07 When P-CL is invalid: P15.05 Reverse external torque limit: When N-CL is valid: P15.08 When N-CL is invalid: P15.06. 2; Forward torque limit: Min. in 6072h and 60E0h Reverse torque limit: Min. in 6072h, 60E1h. 3; Forward torque limit: When P-CL is valid: Min. in P15.07, 6072h, 60E0h When P-CL is invalid: Min. in 6072h and 60E0h Reverse torque limit: N-CL valid: Min. in P15.08, 6072h, 60E1h When N-CL is invalid: Min. in 6072h and 60E1h. 4; Forward torque limit: When P-CL is valid: Min. in 6072h and 60E0h When P-CL is invalid: P15.05 Reverse torque limit: When N-CL is valid: Min. in 6072h and 60E1h When N-CL is invalid: P15.06. |

(9) Home Mode (HM)

HM refers to the operating mode in which the servo drive performs mechanical origin positioning upon external signal, at the given action speed.

	After returning to zero, actual position feedback of motor = 607Ch (origin bias);

	The mechanical origin can correspond to the origin switch signal, forward/reverse limit switch, and Z signal of motor ;

	There are various mechanical zeroing methods. If it is impossible to disconnect mechanical connection between motor and the equipment in actual application, please refer to "Introduction to Zeroing Methods" to choose the appropriate zeroing method, avoid the equipment damage; If the upper controller is used to return-to-zero, the reset methods in this Chapter will not be applicable. Please refer to the relevant zeroing introduction of the upper controller;

	When the servo is in return-to-zero mode and running, it can't switch to other modes; When return-to-zero is completed or interrupted (fault or disabled), it can switch to other modes;

	Please pay attention to the distance between limit switch and the forward/reverse limit switches; They should not be too close, and appropriate acceleration should be set. Otherwise, it might cause collision!

Control block diagram

Return to zero control

Figure 3-72 HM control block diagram

Related object (instruction * setting class)

Note: For detailed instructions on related objects, see Chapter 6 Object Dictionary.

Common object:

Table 3-70 0x6040- Control word

| 0x6040- Control word |  |
| --- | --- |
| Index - Subindex | 0x6040-00 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | Operation settings/downtime effective |
| Related mode | ALL |
| Note | In CSP mode, only absolute position instruction is supported Mode correlation: Bit0 to Bit3 are 1, indicating that the system is started Bit; Name; Description. 0; Servo is ready; Setting mode: 1-valid, 0-invalid. 1; Switch on the main circuit; Setting mode: 1-valid, 0-invalid. 2; Fast stop; Setting mode: 0-valid, 1-invalid. 3; Servo operation; Setting mode: 1-valid, 0-invalid. 4; Start HM; Start HM: rising edge End HM: falling edge HM in progress: constant as 1. 8; Pause; 0: invalid. 1: The servo is set pause by 605Dh.. |

Table 3-71 Objects related to instruction setting in HM mode

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6040 | 00 | Control word | - | 0~65535 | UINT16 | RW | RxPDO |
| 6060 | 00 | Servo mode selection | - | 0~10 | INT8 | RW | RxPDO |
| 6067 | 00 | Threshold of position arrival | Encoder unit | 0~65535 | UINT32 | RW | RxPDO |
| 6068 | 00 | Position arrival window | ms | 0~65535 | UINT16 | RW | RxPDO |
| 6098 | 00 | Origin return method | - | -2~35 | INT8 | RW | RxPDO |
| 6099 | 01 | Deceleration point of high-speed search | Instruction unit /s | 0~(232-1) | UINT32 | RW | RxPDO |
|  | 02 | Search origin low speed | Instruction unit /s | 10~(232-1) | UINT32 | RW | RxPDO |
| 609A | 00 | acceleration | Instruction unit /s2 | 0~(232-1) | UDINT32 | RW | RxPDO |
| 2013 | 32 | Timeout period | 10ms | 100~65535 | UINT16 | RW | - |

Related objects (status * monitor class)

Table 3-72 0x6041- Status word

| 0x6041- Status word |  |
| --- | --- |
| Index - Subindex | 0x6041-00 |
| Data type | UINT16 |
| Accessibility | Readable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | - |
| Related mode | ALL |
| Note | Reactive servo state For mode: Bit; Name; Description. 10; Target position arrival; Status display: 1- Arrived, 0- not arrived. 12; HM end; Status: 1- Succeeded, 0- failed. 13; HM error; Status: 1- Error, 0- No error. 15; The origin return to zero is complete; Status display: 1- completed, 0- not completed. |

Table 3-73 Objects related to status monitoring in HM mode

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 603F | 00 | Error code | - | 0~65535 | UINT16 | RO | TxPDO |
| 6041 | 00 | Status word | - | 0~65535 | UINT16 | RO | TxPDO |
| 6061 | 00 | Run mode display | - | 0~10 | INT8 | RO | TxPDO |
| 6062 | 00 | Actual position | Instruction unit | - | INT32 | RO | TxPDO |
| 6064 | 00 | Position feedback | Instruction unit | - | INT32 | RO | TxPDO |
| 6077 | 00 | Actual torque | 0.1% | -5000~5000 | INT16 | RO | TxPDO |
| 606C | 00 | Actual velocity | Instruction unit /s | - | INT32 | RO | TxPDO |
| 60F4 | 00 | Position deviation | Instruction unit | - | UINT16 | RO | TxPDO |

Related function Settings

A) HM time limit

The zeroing time limit in zeroing mode is set by P13.49. If zeroing is uncompleted within this period, warning of zero return timeout (A.425) will be reported.

Table 3-74 Objects related to the time limit for the origin return

| Index | Subindex | Name | Setting range |
| --- | --- | --- | --- |
| 2013 | 32 | Zero return timeout | 0~65535 (unit: 10ms) |

B) Calculation methods for position after zeroing completion

After the zero return mode is completed, the servo motor position is the mechanical origin, and the position feedback can be set of different calculation methods by 60E6h, as shown in the below table. Different calculation methods are applicable to different industrial machinery.

Table 3-75 0x60E6- Position Calculation Method

| 0x60E6- Position calculation method |  |
| --- | --- |
| Index - Subindex | 0x60E6-00 |
| Data type | UINT8 |
| Accessibility | Readable/writable |
| Unit | 1 |
| Default value | 0 |
| Min. | 0 |
| Max. | 1 |
| Setting and effective mode | Operation settings/downtime effective |
| Related mode | HM |
| Note | Settings; Description. 0; After return to zero is completed, actual feedback value of the origin position is the original bias 607C. 1; After return to zero is completed, actual feedback value of the origin position is the original position feedback 6064 + the origin bias 607C. |

Introduction on Zero Return Method

Zero Return Speed Setting

Table 3-76 Objects related to Zero Return Speed Setting

| Index | Subindex | Name | Setting range |
| --- | --- | --- | --- |
| 6099 | 01h | Deceleration point of high-speed search | 0~(232-1) |
|  | 02h | Low-speed search for the origin | 0~(232-1) |

Note: In the following action description, high-speed operation refers to running at the speed set by 6099-01h, while low-speed operation refers to running at the speed set by 6099-02h. It can be understood as follows: high-speed operation searches for the deceleration point, after finding the deceleration point, low-speed operation searches for the origin.

I) Method 1:

Table 3-77 Origin return mode 1

| movement Block diagram | Positive limit switch Z signal Low speed High speed Forward Reverse |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Z-phase signal; Negative limit switch. |
| Method 1 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 1.1 - Deceleration point is invalid when returning to zero; While start zero return, run at the reverse high speed. When encountering the rising edge of reverse limit signal, run at the forward low speed. When encountering the first Z-phase signal after the falling edge of reverse limit signal, stop running. 1.2- Deceleration point is valid when returning to zero; While start zero return, directly start the forward low-speed return to zero, and stop running when encountering the first Z-phase signal after the falling edge of reverse limit signal. |

II) Method 2:

Table 3-78 Origin return mode 2

| movement Block diagram | Reverse Forward Home switch signal Z signal High speed Low speed |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Z-phase signal; Forward limit switch. |
| Mode 2 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 2.1 - Deceleration point is invalid when returning to zero; While start zero return, run at the forward high speed. When encountering the rising edge of forward limit signal, run at the reverse low speed. When encountering the first Z-phase signal after the falling edge of forward limit signal, stop running. 2.2- Deceleration point is valid when returning to zero; While start zero return, directly start the reverse low-speed return to zero, and stop running when encountering the first Z-phase signal after the falling edge of forward limit signal. |

III) Methods 3 to 4:

Table 3-79 Origin return modes 3 to 4

| Motion block diagram | Origin switching signal Forward Z signal Reverse Low speed High speed |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Z-phase signal; Origin switching signal. |
| Mode 3 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 3.1 - Deceleration point is invalid when returning to zero; While start zero return, run at the forward high speed. When encountering the rising edge of the original switch signal, run at the reverse low speed. When encountering the first Z-phase signal after the falling edge of the original switch signal, stop running. 3.2- Deceleration point is valid when returning to zero; While start zero return, directly start the reverse low-speed return to zero, and stop running when encountering the first Z-phase signal after the falling edge of the original switch signal, stop running. |
| Method 4 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 4.1- Deceleration point is invalid when returning to zero; While start zero return, run at the forward low speed. When encountering the first Z-phase signal after the rising edge of the original switch signal, stop running. 4.2- Deceleration point is valid when returning to zero; While start zero return, run at the reverse high speed. When encountering the falling edge of the original switch signal, run at the forward low speed. When encountering the first Z-phase signal after the rising edge of the original switch signal, stop running. |

IV) Methods 5-6:

Table 3-80 Origin Return Modes 5-6

| Motion block diagram | Forward Origin switching signal Z signal Reverse High speed Low speed |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Z-phase signal; Origin switching signal. |
| Method 5 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 5.1 - Zeroing deceleration point is valid; While start zeroing, directly start the forward low-speed zeroing and stop when the first Z-phase signal occurs after falling edge of the origin switch signal. 5.2 - Zeroing deceleration point is invalid; While start zeroing, run at the reverse high speed. When encountering rising edge of the origin switch signal, run at the forward low speed. Stop when the first Z-phase signal occurs after falling edge of the origin switch signal. |
| Method 6 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 6.1- Zeroing deceleration point is valid; While start zeroing, run at the forward high speed. When encountering falling edge of the origin switch signal, run at the reverse low speed. Stop when the first Z-phase signal occurs after rising edge of the origin switch signal. 6.2- Zeroing deceleration point is invalid; While start zeroing, directly start the reverse low-speed zeroing and stop when the first Z-phase signal occurs after rising edge of the origin switch signal. |

V) Methods 7-10:

Table 3-81 Origin Return Modes 7-10

| Motion block diagram | Z signal Origin switch Positive limit switch Reverse Forward High speed Low speed |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Z-phase signal; Origin switching signal. |
| Method 7 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 7.1- Deceleration point is invalid upon zeroing, and no forward limit switch was encountered during the process; While start zero return, run at the forward high speed. When encountering the rising edge of the original switch signal, run at the reverse low speed. When encountering the first Z-phase signal after the falling edge of the original switch signal, stop running. 7.2- Deceleration point is valid upon zeroing; While start zero return, directly start the reverse low-speed return to zero, and stop running when encountering the first Z-phase signal after the falling edge of the original switch signal, stop running. 7.3- Deceleration point is invalid upon zeroing, and forward limit switch was encountered during the process; While start zeroing, run at the forward high speed. If encountering forward limit signal prior to the origin switch signal, run at the reverse high speed. Run at reverse low speed when encountering the rising edge of the original switch signal.When encountering the first Z-phase signal after falling edge of the origin switch signal,stop running.. |
| Method 8 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 8.1- The deceleration point is invalid when returning to zero, and no forward limit switch was encountered in the process; While start zeroing, run at the forward high speed. When encountering rising edge of the origin switch signal, run at the reverse low speed. Run at forward low speed when encountering falling edge of the original switch signal.When encountering the first Z-phase signal after rising edge of the origin switch signal,stop running.. 8.2- The deceleration point is valid when returning to zero; While start zeroing, run at the reverse low speed. When encountering falling edge of the origin switch signal, run at the forward low speed. Stop when encountering the first Z-phase signal after rising edge of the origin switch signal. 8.3- The deceleration point is invalid when returning to zero, and forward limit switch was encountered in the process; While start zeroing, run at the forward high speed. If encountering forward limit signal prior to the origin switch signal, run at the reverse high speed. Run at reverse low speed when encountering the rising edge of the original switch signal. Run at forward low speed when encountering the falling edge of the original switch signal. When encountering the first Z-phase signal after rising edge of the origin switch signal,stop running.. |
| Method 9 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 9.1- The deceleration point is invalid when returning to zero, and no forward limit switch was encountered in the process; While start zeroing, run at the forward high speed. When encountering rising edge of the origin switch signal, run at the forward low speed. When encountering falling edge of the original switch signal, run at the reverse low speed. Stop when encountering the first Z-phase signal after rising edge of the origin switch signal. 9.2- Deceleration point is valid when returning to zero; While start zeroing, run at the forward low speed. When encountering falling edge of the origin switch signal, run at the reverse low speed. Stop when encountering the first Z-phase signal after rising edge of the origin switch signal. 9.3- The deceleration point is invalid when returning to zero, and forward limit switch was encountered in the process; While start zeroing, run at the forward high speed. If encountering forward limit signal prior to the origin switch signal, run at the reverse high speed. Run at forward low speed when encountering the rising edge of the original switch signal. Run at reverse low speed when encountering the falling edge of the original switch signal. When encountering the first Z-phase signal after rising edge of the origin switch signal,stop running.. |
| Method 10 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 10.1- The deceleration point is invalid when returning to zero, and no forward limit switch was encountered in the process; While start zeroing, run at the forward high speed. When encountering rising edge of the origin switch signal, run at the forward low speed. When encountering falling edge of the original switch signal, run at the forward low speed. Stop when encountering the first Z-phase signal after falling edge of the origin switch signal. 10.2- Deceleration point is valid when returning to zero; While start zeroing, run at the forward low speed. When encountering falling edge of the origin switch signal, run at the forward low speed. Stop when encountering the first Z-phase signal after falling edge of the origin switch signal. 10.3- The deceleration point is invalid when returning to zero, and forward limit switch was encountered in the process; While start zeroing, run at the forward high speed. If encountering forward limit signal prior to the origin switch signal, run at the reverse high speed. Run at forward low speed when encountering the rising edge of the original switch signal. Run at forward low speed when encountering the falling edge of the original switch signal. When encountering the first Z-phase signal after falling edge of the origin switch signal,stop running.. |

VI) Method 11 to 14:

Table 3-82 Origin return modes 11 to 14

| Motion block diagram | Origin switch Negative limit switch High speed Low speed Z signal Forward Reverse |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Z-phase signal; Origin switching signal. |
| Method 11 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 11.1- Deceleration point is invalid upon zeroing, and no forward limit switch was encountered during the process; While start zeroing, run at the reverse high speed. When encountering rising edge of the origin switch signal, run at the forward low speed. Stop when the first Z-phase signal occurs after falling edge of the origin switch signal. 11.2- Deceleration point is valid upon zeroing; While start zeroing, directly start the forward low-speed zeroing and stop when the first Z-phase signal occurs after falling edge of the origin switch signal. 11.3- Deceleration point is invalid upon zeroing, and reverse limit switch was encountered during the process; While start zeroing, run at the reverse high speed. If encountering reverse limit signal prior to the origin switch signal, run at the forward high speed. Run at forward low speed when encountering the rising edge of the original switch signal. When encountering the first Z-phase signal after falling edge of the origin switch signal,stop running.. |
| Method 12 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 12.1- Deceleration point is invalid upon zeroing, and no reverse limit switch was encountered during the process; While start zeroing, run at the reverse high speed. When encountering rising edge of the origin switch signal, run at the forward low speed. When encountering falling edge of the original switch signal, run at the reverse low speed. Stop when encountering the first Z-phase signal after rising edge of the origin switch signal. 12.2- Deceleration point is valid upon zeroing; While start zeroing, run at the forward low speed. When encountering falling edge of the origin switch signal, run at the reverse low speed. Stop when encountering the first Z-phase signal after rising edge of the origin switch signal. 12.3- Deceleration point is invalid upon zeroing, and reverse limit switch was encountered during the process; While start zeroing, run at the reverse high speed. If encountering reverse limit signal prior to the origin switch signal, run at the forward high speed. Run at forward low speed when encountering the rising edge of the original switch signal. Run at the reverse low speed when encountering the falling edge of the original switch signal. When encountering the first Z-phase signal after rising edge of the origin switch signal,stop running.. |
| Method 13 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 13.1- Deceleration point is invalid upon zeroing, and no reverse limit switch was encountered during the process; While start zeroing, run at the reverse high speed. When encountering rising edge of the origin switch signal, run at the reverse low speed. When encountering falling edge of the original switch signal, run at the forward low speed. Stop when encountering the first Z-phase signal after rising edge of the origin switch signal. 13.2- Zero return deceleration point is valid upon zeroing; While start zeroing, run at the reverse low speed. When encountering falling edge of the origin switch signal, run at the forward low speed. Stop when encountering the first Z-phase signal after rising edge of the origin switch signal. 12.3- Deceleration point is invalid upon zeroing, and reverse limit switch was encountered during the process; While start zeroing, run at the reverse high speed. If encountering reverse limit signal prior to the origin switch signal, run at the forward high speed. Run at reverse low speed when encountering the rising edge of the original switch signal. Run at the forward low speed when encountering the falling edge of the original switch signal. When encountering the first Z-phase signal after rising edge of the origin switch signal,stop running.. |
| Method 14 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 14.1- Deceleration point is invalid upon zeroing, and no reverse limit switch was encountered during the process; While start zeroing, run at the reverse high speed. When encountering rising edge of the origin switch signal, run at the reverse low speed. When encountering falling edge of the original switch signal, run at the reverse low speed. Stop when encountering the first Z-phase signal after falling edge of the origin switch signal. 14.2- Deceleration point is valid when returning to zero; While start zeroing, run at the reverse low speed. When encountering falling edge of the origin switch signal, run at the reverse low speed. Stop when encountering the first Z-phase signal after falling edge of the origin switch signal. 14.3- The deceleration point is invalid when returning to zero, and forward limit switch was encountered in the process; While start zeroing, run at the reverse high speed. If encountering reverse limit signal prior to the origin switch signal, run at the forward high speed. Run at reverse low speed when encountering the rising edge of the original switch signal. Run at reverse low speed when encountering the falling edge of the original switch signal. When encountering the first Z-phase signal after falling edge of the origin switch signal,stop running.. |

VII) Method 15-16: Reserved

Note that methods 17-30 are similar to methods 1-14, except that they no longer rely on the Z-phase signal as the origin, detailed methods as follows.

VIII) Method 17:

Table 3-83 Origin Return Mode 17

| Motion block diagram | Negative limit switch Reverse Forward Low speed High speed |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Negative limit switch; Negative limit switch. |
| Method 17 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 17.1- Deceleration point is invalid when returning to zero; While start zeroing, run at the reverse high speed. When encountering rising edge of the reverse limit signal, run at the forward low speed. When encountering falling edge of the reverse limit signal, stop running.. 17.2- Deceleration point is valid when returning to zero; While start zeroing, directly start the forward low-speed zeroing and stop when encountering the falling edge of reverse limit signal.. |

VIII) Method 18:

Table 3-84 Origin Return Mode 18

| Motion block diagram | Reverse Positive limit switch Forward High speed Low speed |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Forward limit switch; Forward limit switch. |
| Method 18 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 18.1- Deceleration point is invalid when returning to zero; While start zeroing, run at the forward high speed. When encountering rising edge of the forward limit signal, run at the reverse low speed. When encountering falling edge of the forward limit signal, stop running.. 18.2- Deceleration point is valid when returning to zero; While start zeroing, directly start the reverse low-speed zeroing and stop when encountering the falling edge of forward limit signal.. |

X)Method 19~20：

Table 3–85 Origin Return Method 19~20

| Motion block diagram | Origin switching signal Forward Reverse Low speed High speed |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Origin switching signal; Origin switching signal. |
| Method 19 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 19.1- Deceleration point is invalid when returning to zero; While start zeroing, run at the forward high speed. When encountering rising edge of the origin switch signal, run at the reverse low speed. When encountering falling edge of the origin switch signal, stop running.. 19.2- Deceleration point is valid when returning to zero; While start zeroing, directly start the reverse low-speed zeroing and stop when encountering the falling edge of the origin switch signal.. |
| Method 20 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 20.1- Deceleration point is invalid when returning to zero; While start zeroing, run at the forward low speed. When encountering rising edge of the origin switch signal, stop running.. 20.2- Deceleration point is valid when returning to zero; While start zeroing, run at the reverse high speed zeroing and run at the forward low speed when encountering the falling edge of the origin switch signal.When encountering the rising edge of the origin switch signal, stop running.. |

X)Method 21-22：

Table 3–86 Origin Return Method 21~22

| Motion block diagram | Origin switching signal Forward Reverse High speed Low speed |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Origin switching signal; Origin switching signal. |
| Method 21 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 21.1- Deceleration point is valid when returning to zero; While start zeroing, directly start at the forward low speed. When encountering falling edge of the origin switch signal, stop running.. 21.2- Deceleration point is invalid when returning to zero; While start zeroing, run at the reverse high speed. When encountering the rising edge of the origin switch signal, run at the forward low speed. When encountering the falling edge of the origin switch signal, stop running... |
| Method 22 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 22.1- Deceleration point is valid when returning to zero; While start zeroing, run at the forward high speed. When encountering falling edge of the origin switch signal, run at the reverse low speed. When encountering rising edge of the origin switch signal, stop running.. 22.2- Deceleration point is invalid when returning to zero; While start zeroing, run at the reverse high speed. When encountering the rising edge of the origin switch signal, run at the forward low speed. When encountering the falling edge of the origin switch signal, stop running... |

XII) Method 23-26:

Table 3-87 Origin Return Mode 23-26

| movement Block diagram | Origin switch Positive limit switch Low speed High speed Forward Reverse |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Origin switching signal; Origin switching signal. |
| Method 23 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 23.1- The deceleration point is invalid while returning to zero, and no forward limit switch was encountered during the process; While start zeroing, run at the forward high speed. When encountering rising edge of the origin switch signal, run at the reverse low speed. When encountering falling edge of the origin switch signal, stop running.. 23.2- The deceleration point is valid when returning to zero; While start zeroing, directly start the reverse low-speed zeroing and stop when encountering the falling edge of the origin switch signal.. 23.3- The deceleration point is invalid when returning to zero, and forward limit switch is encountered during the process; While start zeroing, run at the forward high speed. When encountering the forward limit signal prior to the origin switch signal, run at the reverse high speed. When encountering the rising edge of the origin switch signal, run at the reverse low speed. When encountering the falling edge of the origin switch signal, stop running... |
| Method 24 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 24.1- The deceleration point is invalid when returning to zero, and no forward limit switch was encountered in the process; While start zeroing, run at the forward high speed. When encountering rising edge of the origin switch signal, run at the reverse low speed. When encountering falling edge of the original switch signal, run at the forward low speed. When encountering the rising edge of the origin switch signal, stop running.. 24.2- Deceleration point is valid when returning to zero; While start zeroing, run at the reverse low speed. When encountering falling edge of the origin switch signal, run at the forward low speed. Stop when encountering the rising edge of the origin switch signal. 24.3- The deceleration point is invalid when returning to zero, and forward limit switch was encountered in the process; While start zeroing, run at the forward high speed. If encountering forward limit signal prior to the origin switch signal, run at the reverse high speed. Run at reverse low speed when encountering the rising edge of the original switch signal. Run at the forward low speed when encountering the falling edge of the original switch signal. When encountering the rising edge of the origin switch signal,stop running.. |
| Method 25 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 25.1- The deceleration point is invalid when returning to zero, and no forward limit switch was encountered in the process; While start zeroing, run at the forward high speed. When encountering rising edge of the origin switch signal, run at the forward low speed. When encountering falling edge of the original switch signal, run at the reverse low speed. When encountering the rising edge of the origin switch signal, stop running.. 25.2- Deceleration point is valid when returning to zero; While start zeroing, run at the forward low speed. When encountering falling edge of the origin switch signal, run at the reverse low speed. Stop when encountering the rising edge of the origin switch signal. 25.3- The deceleration point is invalid when returning to zero, and forward limit switch was encountered in the process; While start zeroing, run at the forward high speed. If encountering forward limit signal prior to the origin switch signal, run at the reverse high speed. Run at forward low speed when encountering the rising edge of the original switch signal. Run at the reverse low speed when encountering the falling edge of the original switch signal. When encountering the rising edge of the origin switch signal,stop running.. |
| Method 26 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 26.1- The deceleration point is invalid when returning to zero, and no forward limit switch was encountered in the process; While start zeroing, run at the forward high speed. When encountering rising edge of the origin switch signal, run at the forward low speed. When encountering falling edge of the original switch signal, run at the forward low speed. When encountering the falling edge of the origin switch signal, stop running.. 26.2- Deceleration point is valid when returning to zero; While start zeroing, run at the forward low speed. When encountering falling edge of the origin switch signal, run at the forward speed. Stop when encountering the falling edge of the origin switch signal. 26.3- The deceleration point is invalid when returning to zero, and forward limit switch was encountered in the process; While start zeroing, run at the forward high speed. If encountering forward limit signal prior to the origin switch signal, run at the reverse high speed. Run at forward low speed when encountering the rising edge of the original switch signal. Run at the forward low speed when encountering the falling edge of the original switch signal. When encountering the falling edge of the origin switch signal,stop running.. |

XIII) Method 27-30:

Table 3-88 Origin Return Mode 27-30

| movement Block diagram | Negative limit switch Origin switch Low speed High speed Forward Reverse |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Origin switching signal; Origin switching signal. |
| Method 27 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 27.1: The deceleration point is invalid when returning to zero, and no forward limit switch was encountered during the process; While start zeroing, run at the reverse high speed. When encountering the rising edge of the origin switch signal, run at the forward low speed. When encountering the falling edge of the origin switch signal, stop running... 27.2: The deceleration point is valid when returning to zero; While start zeroing, directly start at the forward low speed. When encountering falling edge of the origin switch signal, stop running.. 27.3: The deceleration point is invalid when returning to zero, and a reverse limit switch is encountered during the process; When starting to return to zero, run at reverse high speed . If encountering reverse limit signal prior to the origin signal, run at the forward high speed. When encountering the rising edge of the origin switch signal, run at forward low speed. When encountering the falling edge of the origin switch signal, stop running. |
| Method 28 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 28.1: The deceleration point is invalid when returning to zero, and no reverse limit switch was encountered during the process; When starting to return to zero, run at the reverse high speed. When encountering the rising edge of the origin switch signal, run at the forward low speed. When encountering the falling edge of the origin switch signal, run at the reverse low speed. Stop running when encountering the rising edge of the origin switch signal. 28.2: The deceleration point is valid when returning to zero; While start zeroing, run at the forward low speed. When encountering falling edge of the origin switch signal, run at the reverse low speed. Stop when encountering the rising edge of the origin switch signal. 28.3: The deceleration point is invalid when returning to zero, and reverse limit switch is encountered during the process; When starting to return to zero, run at the reverse high speed. If encountering reverse limit signal prior to the origin switch, runs at the forward high speed. When encountering the rising edge of the origin switch signal, it runs at the forward low speed. When encountering the falling edge of the origin switch signal, it runs at the reverse low speed. When encountering the rising edge of the origin switch signal, stop running. |
| Method 29 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 29.1: The deceleration point is invalid when returning to zero, and no reverse limit switch was encountered during the process; When starting to return to zero, run at the reverse high speed in reverse. When encountering the rising edge of the origin switch signal, it runs at the reverse low speed in reverse. When encountering the falling edge of the origin switch signal, it runs at the forward low speed. When encountering the rising edge of the origin switch signal, it stops running. 29.2: The deceleration point is valid when returning to zero; While start zeroing, run at the reverse low speed. When encountering falling edge of the origin switch signal, run at the forward low speed. Stop when encountering the rising edge of the origin switch signal. 29.3: The deceleration point is invalid when returning to zero, and a reverse limit switch is encountered during the process; When starting to return to zero, it runs at the reverse high speed. If encountering reverse limit signal prior to the origin switch signal, run at the forward high speed. When encountering the rising edge of the origin switch signal, it runs at the reverse low speed. When encountering the falling edge of the origin switch signal, run at the forward low speed. When encountering the rising edge of the origin switch signal, stop running. |
| Method 30 |  | Action description |
|  |  | Note: different actions in different initial conditions Initial condition; Return to zero action. 30.1: The deceleration point is invalid when returning to zero, and no reverse limit switch was encountered during the process; While starting to return to zero, run at the reverse high speed. When encountering the rising edge of the origin switch signal, run at the reverse low speed. When encountering the falling edge of the origin switch signal, continue to run at the reverse low speed. Stop running, when encountering the falling edge of the origin switch signal. 30.2: The deceleration point is valid when returning to zero; While starting to return to zero, run at the reverse low speed. When encountering the falling edge of the origin switch signal, continue to run at the reverse low speed. Stop running, when encountering the falling edge of the origin switch signal. 30.3: The deceleration point is invalid when returning to zero, and a reverse limit switch is encountered during the process; While starting to return to zero, run at the reverse high speed. If encountering the reverse limit signal prior to the origin switch, run at the forward high speed. When encountering the rising edge of the origin switch signal, run at the reverse low speed. When encountering the falling edge of the origin switch signal, continue running at reverse low speed. Stop running, when encountering the falling edge of the origin switch signal. |

XIV) Method 31-32: Reserved

XV) Methods 33-34:

Table 3-89 Origin Return Modes 33-34

| movement Block diagram | Z signal High speed Low speed Forward Reverse |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Z-phase signal; No. |
| Method 33 |  | Action description |
|  |  | 33: Running at low speed in reverse, encountering the first Z-phase signal, stop |
| Method 34 |  | Action description |
|  |  | 34: Running forward at low speed, encountering the first Z-phase signal, stop |

XVI) Method 35:

Table 3-90 Origin Return Mode 35

| movement Block diagram | Forward Reverse 负限位开关 Low speed High speed |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Current position; No. |
| Method 35 |  | Action description |
|  |  | 35: Use the current position as the origin |

XVII) Method 1:

Table 3-91 Origin Return Mode-1

| movement Block diagram | Mechanical limit position High speed Low speed Z signal Forward Reverse |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Z-phase signal; Mechanical limit position. |
| Method 1 |  | Action description |
|  |  | Method 1: The motor first runs at high speed in reverse direction. After hitting the mechanical limit, if the torque reaches the torque limit and the speed is near zero speed, besides this state maintains for a period, it proves that the shaft has reached the mechanical limit position. The motor runs at low speed in forward direction; Stop running, when encountering the first Z-phase signal. |

XVIII) Method 2

Table 3-92 Origin Return Mode -2

| movement Block diagram | Z signal Mechanical limit position Forward High speed Low speed Reverse |  |
| --- | --- | --- |
| Positioning signal |  | Origin; Decelerating point. Z-phase signal; Mechanical limit position. |
| Method 2 |  | Action description |
|  |  | Method 2: The motor first runs at the forward high speed. After hitting the mechanical limit position, if the torque reaches the torque limit value and the speed is near zero speed, and this state maintains for a period, it proves to have reached the mechanical limit position. The motor runs at the reverse low speed, stop running when encountering the first Z-phase signal. |

3.4.4 Application Functions

(1) Probe function

Function Overview

SV3 servo drive supports servo motor position recording function, also known as probe function. With this function, motor position (command unit) can be latched when the external signal is valid or during riseing/falling edge of the servo motor Z-phase signal.

Table 3-93 Probe Lock Signal Selection

| Serial No. | Latch enable signal |
| --- | --- |
| Probe 1 | HDI1 |
|  | Z signal |
| Probe 2 | HDI2 |
|  | Z signal |

Note:

	The latch time can be rising or falling edge of the external signal, so SV3 can simultaneously latch 4 positions.

	SV3 servo drive supports 2 types of latching: single latching position and continuous latching.

	If using HDI 1 and HDI 2 as trigger signals for probe functions, please disable the DI and DO forcing function.

	If using HDI 1 and HDI 2 as trigger signals for probe functions, please assign HDI 1 function as 33 probe 1 and HDI 2 function as 34 probe 2. The relevant objects are as follows:

Table 3-94 HDI 1~HDI 2 objects

| Parameter | Name | Set |
| --- | --- | --- |
| P03.14 | HDI Terminal Function Selection | Please set 33-probe 1 |
| P03.16 | HDI2 terminal function selection | Please set 34-probe 2 |

Related object (instruction * setting class)

Table 3-95 Probe Function Command Setting Objects

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x2003 | 0F | HDI 1 Function Settings | - | 0~34 | UINT16 | RW | - |
| 0x2003 | 11 | HDI2 Function Settings | - | 0~34 | UINT16 | RW | - |
| 0x60B8 | 00 | Probe function | - | 0~65535 | UINT16 | RW | RxPDO |

- Table 1-5

Table 3-96 0x60B8- Probe Function

| 0x60B8- Probe Function |  |
| --- | --- |
| Index - Subindex | 0x60B8-00 |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | Operation settings/downtime effective |
| Related mode | ALL |
| Note | Bit; Name; Description. 0; Probe 1 Function; 0-Disable; 1- Enable. 1; Probe 1 mode; 0-Single recording; 1- Continuous recording. 2; Probe 1 Trigger signal; 0-HDI1； 1-Z signal. 3; NA; Reserved. 4; Probe 1 rising edge action*; 0- Not latch; 1- Latch. 5; Probe 1 falling edge action*; 0- Not latch; 1- Latch. 6; NA; Reserved. 7; NA; Reserved. 8; Probe 2 Function; 0-Disable; 1- Enable. 9; Probe 2 Mode; 0-Single recording; 1- Continuous recording. 10; Probe 2 trigger signal; 0-HDI2； 1-Z signal. 11; NA; Reserved. 12; Probe 2 rising edge action*; 0- Not latch; 1- Latch. 13; Probe 2 falling edge action*; 0- Not latch; 1- Latch. 14; NA; Reserved. 15; NA; Reserved. |

Note:

	Please set trigger mode, trigger signal, rising edge action, and falling edge action before enabling probe function.

Related objects (status * monitor class)

Table 3-97 Probe function status monitoring objects

| Index | Subindex | Name | Unit | Range | Data type | Accessibility | PDO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0x60B9 | 00 | Probe status | - | - | UINT16 | RO | TxPDO |
| 0x60BA | 00 | Probe 1 rising edge latch position | Instruction unit | - | INT32 | RO | TxPDO |
| 0x60BB | 00 | Probe 1 falling edge latch position | Instruction unit | - | INT32 | RO | TxPDO |
| 0x60BC | 00 | Probe 2 rising edge latch position | Instruction unit | - | INT32 | RO | TxPDO |
| 0x60BD | 00 | Probe 2 falling edge latch position | Instruction unit | - | INT32 | RO | TxPDO |

- Table 1-6

Table 3-98 0x60B9 Probe status word

| 0x60B9- Probe status word |  |
| --- | --- |
| Index - Subindex | 0x00 |
| Data type | UINT16 |
| Accessibility | Readable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 65535 |
| Setting and effective mode | - |
| Related mode | PST |
| Note | Bit; Name; Description. 0; Probe 1 Function; 0 - Not enabled; 1 - Enable. 1; Probe 1 rising edge latch status; 0- Rising edge not latched, 1- Rising edge latch\ed. 2; Probe 1 falling edge latch status; 0-Falling edge not latched, 1- Falling edge latched. 3~7; NA; Reserved. 8; Probe 2 Function; 0 - Not enabled; 1 - Enable. 9; Probe 2 rising edge latch status; 0- Rising edge not latched, 1- Rising edge latch\ed. 10; Probe 2 falling edge latch status; 0-Falling edge not latched, 1- Falling edge latched. 11~15; NA; Reserved. |

Probe usage

Single rising edge latch

Trigger signal

Position B

Position A

Figure 3-73 Schematic diagram of single rising edge latch

Single falling edge latch

Trigger signal

Position B

Position A

Figure 3-74 Schematic diagram of single falling edge latch

Continuous rising edge latch

Trigger signal

Position E

Position D

Position C

Position B

Position A

Figure 3-75 Continuous rising edge latch

Continuous falling edge latch

Trigger signal

Position A

Position B

Position C

Position D

Position E

Figure 3-76 Continuous falling edge latch

(2) Soft limit function

Function Overview

Software limit refers to the function of limiting the position of servo motor by the built-in of drive so as to protect the equipment.

	The software limit function can be available in any mode, and different overtravel stop modes can be set according to the actual connection method of devices. If it is in position mode and the target position is out of the limit interval, the servo motor moves toward the destination of the endpoint of limit interval. If in other modes, it stops in the specified way when the position feedback is out of the limit interval.

	The software limit function is to restrict the value of position feedback 6064h (instruction unit) in a certain range, pay attention to the unit.

	Please ensure that the lower limit of limit interval is less than the upper limit.

	If both DI overtravel switch and the software limit are enabled simultaneously, the overtravel state is determined by external DI overtravel switch.

Related object (instruction * setting class)

Table 3-99 P05.43 Soft Limit Setting

| P05.43- Soft limit setting |  |
| --- | --- |
| Index - Subindex | 0x2005-2C |
| Data type | UINT16 |
| Accessibility | Readable/writable |
| Unit | - |
| Default value | 0 |
| Min. | 0 |
| Max. | 2 |
| Setting and effective mode | Operation settings/downtime effective |
| Related mode | ALL |
| Note | Set the enable mode for software limit function Settings; Software limit function. 0; Disable software limit function. 1; Enable software limit function. 2; Enable software limit function after the origin return,. |

Table 3-100 0x607D - Software Absolute Position Limit

| 0x607D - Software absolute position limit |  |  |
| --- | --- | --- |
| Index - Subindex | 1h | 2h |
| Data type | UINT32 |  |
| Accessibility | RW | RW |
| Unit | - | - |
| Default value | -231 | 231-1 |
| Min. | -231 | -231 |
| Max. | 231-1 | 231-1 |
| Setting and effective mode | Operation settings/downtime effective |  |
| Related mode | ALL |  |
| Note | 607D-01h: Min. position limit 607D-02h: Max. position limit |  |

- Chapter 4 Gain Adjustment

4.1 Purpose

In order to enable the servo system to quickly and accurately track instructions from the upper computer or internal settings, fully utilize mechanical performance, improve production cycle and efficiency, it is necessary to adjust the gain of the servo control loop reasonably.

Taking common screw loads as an example, such as errors! Reference source not found. As shown, by reasonably increasing the gain related to the speed loop and position loop, ensuring gain matching, and combining with the speed feedforward function, the trajectory tracking effect can be greatly improved.

Position proportional gain:40.0Hz

Speed proportional gain :25.0Hz

Speed integral gain :40.00ms

Speed feedforward proportional gain:0

Load inertia ratio:30

Position proportional gain:80.0Hz

Speed proportional gain :50.0Hz

Speed integral gain :20.00ms

Speed feedforward proportional gain:0

Load inertia ratio:30

Incremental Position Feedback

Incremental position command

Gain setting：Low

Gain setting：High + Feedforward setting

Position proportional gain:80.0Hz

Speed proportional gain :50.0Hz

Speed integral gain :20.00ms

Speed feedforward proportional gain:50%

Load inertia ratio:30

Gain setting：High

Figure 4-1 Example of gain setting

The basic gain parameters of the servo loop include the proportional coefficient Kp of the position loop, the gain coefficient Kv of the velocity loop, the integral coefficient Ki of the velocity loop, the low-pass filtering coefficient τ m of the torque, the inertia ratio Jr of the load, etc. They affect each other, and generally need to ensure that the inner loop bandwidth is higher than the outer loop bandwidth to ensure the stability of the entire system. By setting these parameters in a reasonable combination, the servo system can achieve ideal control effects in both tracking and disturbance rejection dimensions. Therefore, in order to achieve the maximum performance of the servo, the setting of the gain must take into account the balance between the set values of various parameters, which poses high capability requirements for on-site debugging personnel. The SV3 series high-performance servo supports two modes: manual parameter tuning and automatic parameter adjustment. In general application scenarios, such as 3C, lithium battery, TP and other industries, using the automatic adjustment function can meet the needs; In trajectory application scenarios that pursue ultimate performance, such as semiconductor, machine tool, laser and other industries, detailed settings of servo system parameters are required to achieve ideal results; For P2P point application scenarios that require high speed and high precision, such as high-speed glue dispensing machines, solidification machines, and sorting machines, the SV3 series servo provides gain switching function, which can achieve ultimate tracking performance through reasonable gain settings.

Before adjusting the gain, it is necessary to perform the trial operation in Chapter 3 to confirm that the motor can operate normally without interference!

4.2 Tuning Method

The general process of gain adjustment is shown in the following figure:

High-speed motion ok?

Enable gain switching function

Motion ok?

End

Position response ok?

Set second set of gains

Set position loop gain

Low speed motion ok?

Speed response ok?

Increase response level

Set speed loop gain

Set first set of gains

Gain switching?

Mechanical load identification

Start

Write EEPROM

Automatic gain adjustment ?

Figure 4-2 Gain Adjustment Process

The SV3 series servo provides three gain adjustment modes, namely "automatic gain adjustment", "manual gain adjustment", and "gain automatic switching". Among them,

	The "automatic gain adjustment" method only requires setting one parameter, P1A group - "response level setting", to achieve the linkage setting of multiple internal gain parameters and achieve the desired response action. The higher the response level, the faster the response; For detailed introduction, please refer to section 4.2.4 on automatic gain adjustment.

	Manual gain adjustment "requires the user to turn off the automatic gain adjustment mode P1A group -" Real time Self adjustment Setting "set to" 0-off ", and sequentially set P06 gain parameters, including speed loop and position loop gain parameters, from the inner loop to the outer loop, to achieve the desired response performance;

	Manual gain adjustment "requires the user to turn off the automatic gain adjustment mode P1A group -" Real time Self adjustment Setting "set to" 0-off ", and sequentially set P06 gain parameters, including speed loop and position loop gain parameters, from the inner loop to the outer loop, to achieve the desired response performance;

To achieve good tracking performance, the prerequisite for the above three gain adjustment methods is to execute a rigorous "mechanical load recognition" program. The SV3 series servo has a built-in mechanical load recognition algorithm, which can automatically identify the mechanical load situation through forward and reverse operation. The following will introduce the contents of mechanical load recognition, manual gain adjustment, automatic gain adjustment, and gain switching in sequence.4.2.1 Mechanical load identification

For servo systems, mechanical loads are the controlled objects and important components of the system; Mechanical load identification includes parts such as load inertia, friction force, and load mechanical resonance point. Servo can automatically identify key mechanical characteristics, set control loop parameters and compensation parameters reasonably, and achieve dynamic response performance that meets application requirements, greatly reducing the tuning pressure of on-site debugging personnel.

Adaptive processing

Friction torque compensation

Basic gain auto-setting

Position/speed command

Motor current

Motor speed

Resonance frequency measurement

Notch filter

Position/speed control

Servo drive

Load characteristic estimation

Current control

Torque command

encoder

Motor

Torque command generation

Figure 4-3 Mechanical Load Identification

Usually, we do not pay much attention to the absolute value of mechanical load inertia, but rather focus more on the relative size of load inertia and motor inertia, so it usually appears in the form of "inertia ratio" in the control loop.

Load inertia ratio' refers to:

| Load inertia ratio= | Mechanical load inertia |
| --- | --- |
|  | Motor rotor inertia |

Load inertia ratio is a key parameter in the servo system, and the proper setting can help to reduce the debugging time.

The servo drive has a built-in load inertia identification function, which can automatically identify the load inertia by this algorithm.

Inertia identification automatic recognition method:

By the buttons on the servo drive panel, make the motor move, so as to realize inertia identification without the intervention of upper computer;

Note:

Under the following conditions, it migt be impossible to perform mechanical load identification normally. In this case, please adjust the gain manually.

Table 4-1 Factors of Mechanical Load Identification

|  | Factors of mechanical load identification |
| --- | --- |
| Load inertia |  Inertia mismatch, load inertia ratio greater than 100 times  Load inertia is unstable, time-varying or slowly changing |
| Mechanical properties |  Extremely low mechanical rigidity, for example, in belt transmission mechanism where the belt is not tensioned  Nonlinear factors such as excessive tooth clearance in operation or excessive backlash in forward/reverse rotation, e.g., misaligned gear installation in gear transmission mechanism |
| Movement conditions |  Movement speed less than 150rpm  When the torque for acceleration/deceleration is less than the offset load torque or less than the viscous friction torque  When the acceleration is less than 3,000rpm/s |

If the actual load inertia ratio is too large, that causes the motor to run slowly, then increase P1A group - "Response Level Setting" and re-identify the inertia.

If vibration occurs in the identification process, the inertia identification should be stopped immediately and P1A group - "Response Level Setting" should be reduced.

Before conducting offline inertia identification, first confirm the following:

	There is a movable stroke of more than 1 turn in each direction between the mechanical limit switches:

	Before identifying offline inertia, please ensure that limit switches are installed on the machinery to prevent overtravel and accidents in the inertia identification process!

	If the actual load inertia ratio is large, estimated to be over 30 times greater than the motor inertia, inertia mismatch phenomenon may occur, that results in slow motor operation. In this case, the following 2 measures can be taken:

	Preset load inertia ratio of a large initial value, and it's recommended to use the 5.00 times as the starting value, gradually increases until the identification is updated accordingly; The load inertia ratio can be set of '2006-0Bh' by object word, or of "P06.10" through the panel, or of "P06 group load inertia ratio" parameter on the background software

	It can be set by appropriately increasing the driver "P1A Advanced Adjustment - Response Level Setting" on the background software, or be set by object word "201A-02h".

The general process for identifying panel operation inertia is as follows:

End

Press "MODE" key to exit

Long press "SET" key: Save identification result

Start

Panel display result stable ?

Long press "▲" key, or long press "▼" key

Confirm servo is in ry state

Panel displays SAVE, indicating identification result saved successfully

This result is the inertia ratio identification result, stable display indicates successful identification

Long press "▲" key, motor rotates forward about 1 turn, then reverses back to initial position, ensure more than 1 turn effective stroke in forward motion range; long press "▼" key for opposite initial motion direction; release key for zero-speed stop, maintain position lock state

Set panel P20.10=1

Figure 4-4 Flow Chart of Inertia Identification for Panel Operation

Inertia identification adopts the form of forward/backward triangular wave motion, and the program has already defaulted to the optimal motion parameters; Users can adjust the motion parameter settings according to actual application scenarios to realize the on-site demands. The identification program motion curve and parameter settings are as follows.

Max. speed

201A-07h

Speed (rpm)

T(ms)

Inertia identification rotation turns

201A-0Ah

Accelerating

time

Waiting time

201A-09h

Long press "UP" button

The motor rotates forward first, and then reverses

Release the button

Zero speed stop, maintain position latched state

Figure 4-5 Inertia Identification Motion Curve

Table 4-2 Self-tuning Parameter Index Codes

| 0x201A - Advanced Adjustment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Subindex | 0x07- Max. speed estimated by inertia | 0x08- Accelerating time estimated by inertia | 0x09- Waiting time estimated by inertia | 0x0A- Rotation turns estimated by inertia |
| Data type | UINT16 |  |  |  |
| Accessibility | Readable/writable | Readable/writable | Readable/writable | Read-only |
| Unit | rpm | ms | ms | turn |
| Default value | 100 | 125 | 800 | 1.00 |
| Min. | 100 | 20 | 50 | 0 |
| Max. | 1000 | 800 | 10000 | 655.35 |
| Set enable mode | Stop setting/immediate enable | Stop setting/immediate enable | Stop setting/immediate enable | -- |
| Related mode | PST |  |  |  |
| Note | Settings of the parameters related to offline inertia identification have been the internal default optimal values. Generally, there is no need to set them |  |  |  |

### 4.2.2  Manual gain adjustment

（1）Basic gain parameters

In scenarios with extreme performance requirements, the gain can be manually fine tuned. By making more detailed adjustments, optimize the debugging effect.

The servo system consists of 3 control loops, i.e., current loop, speed loop and position loop from the inside out. The basic control block diagram is shown in the following figure.

Speed feedback

Position instruction

Position proportional gain 1

2006-03h

Torque filtering 1

2007-03h

Velocity integral gain 1

2006-02h

Velocity proportional gain 1

2006-01h

Encoder

Current control

Torque control

Position control

Speed control

Motor

Figure 4-6 Basic Explanation Block Diagram of Manual Gain

The more inner the loop, the higher the responsiveness required. Generally, the inner loop bandwidth should be set to at least 4 times the outer loop bandwidth. For example, the current loop bandwidth should be 2,000Hz, the speed loop bandwidth should be set not higher than 500Hz, and the position loop bandwidth should be set not higher than 125Hz. In the debugging process, it should adhere to this principle as much as possible, otherwise it might lead to system instability!

The default current loop gain of servo drive ensures responsiveness and generally doesn't need any adjustment. Only the position loop gain, velocity loop gain, and other auxiliary gains need to be adjusted. Therefore, when adjusting the gain in position control mode, if users want to improve the position response performance, to ensure system stability, first increase the speed loop gain and ensure that the inner loop bandwidth between loops is 4 times higher than the outer loop bandwidth, and then increase the position loop gain and reduce position tracking error. It must ensure the order of loop gain adjustment from the inside out.

The basic gain parameter adjustment method is as follows.

Table 4-3 Adjustment Instructions of Loop Gain Parameters

| Steps | Index codes | Name | Adjustment instructions |
| --- | --- | --- | --- |
| 1 | 2006-01h | Velocity proportional gain 1 | Parameter function: Determine Max. frequency of speed command that can follow speed-loop change. On the premise that the average of load inertia ratio (2006-0Bh) is set properly, it can be considered that: Max. follow-up frequency of speed loop = 2006-01h Actual speed Speed command Increase 2006-01h Adjustment method: In the case of no noise and vibration, increasing this parameter can speed up the positioning time and bring better velocity stability and followability. If noise is generated, reduce the parameter setting value; When mechanical vibration occurs, the vibration can be suppressed by using the notch filter or torque low-pass filter function in Section 4.3.2 Vibration Suppression". |
| 2 | 2006-02h | Velocity integral gain 1 | Parameter function: Eliminate velocity loop deviation. Actual speed Speed command Decrease 2006-02h Adjustment method: Set values may follow these relations recommended: 500≤2006-01h×2006-02h≤1000 For example, if the velocity loop gain 2006-01h=40.0Hz, the time constant of the velocity loop integral shall satisfy: 12.50ms≤2006-02h≤25.00ms. Reducing the settings can strengthen the integration function and speed up the positioning time, but too small the settings are prone to mechanical vibration. If the settings are too high, the velocity loop deviation can't be returned to zero. When 2006-02h=512.00ms, the integral is invalid. |
| 3 | 2006-03h | Position proportional gain 1 | Parameter function: Determines the highest frequency of changes in the position instruction that the position ring can follow. The highest following angular frequency of the position ring =2006-03h Actual speed Position instruction Increase 2006-01h Increase 2006-03h Adjustment method: To ensure system stability, the maximum following frequency of the speed loop should be 3-5 times that of the position loop, so: 3≤; 2×π×2006-01h; ≤5. 2006-03h. For example, when velocity loop gain 2006-01h = 40.0Hz, The position loop gain should satisfy: 50.2Hz ≤ 2006-03h ≤ 83.7Hz. Adjust according to the positioning time. Increasing this parameter can speed up the positioning time and improve the ability of the motor to resist external disturbances when it is stationary. Excessively high settings may cause system instability and oscillation. |
| 4 | 2007-03h | Torque filtering 1 | Parameter function: Eliminate high-frequency noise and suppress mechanical resonance. Actual speed Speed command Increase 2007-03h Adjustment method: It should ensure that the cut-off frequency of the torque command low-pass filter is 4 times higher than the maximum following frequency of the speed loop, so: For example, when velocity loop gain 2006-01h = 40.0Hz, The time constant of torque command filtering should satisfy: 2007-03h ≤ 1.00ms. When vibration results from 2006-01h increasement, it can be suppressed by adjusting from 2007-03h. Please refer to "4.3.2 Vibration Suppression Low Pass Filter" for specific settings Excessively large settings may cause deduction of current ring response To suppress the vibration during stop, try increasing 2006 -01h and decreasin 2007-03h; The motor vibration in the stop state is too large. It may try reducing the setting of 2007-03h. |

The position loop gain Kp, velocity loop gain Kv, velocity loop integral Ti, and torque low-pass filtering time Tf are the basic loop gain parameters of servo control. A certain relationship needs to be maintained between the 4 basic gain parameters to ensure the balance between stability and high performance of the entire servo system. The mathematical relationship between the 4 basic parameters is as follows:

Below, based on experimental waveforms, we will introduce the role of basic gain parameters in position control mode.

Figure 4-7 Waveform of position tracking without velocity integration

Figure 4-8 Waveform of position tracking with velocity integration

By comparing errors! Reference source not found. With errors! Reference source not found., It can be observed that when there are only position gain and velocity gain coefficients without velocity integration, fast positioning can be achieved with steady-state error. Increasing the loop gain can reduce steady-state error; When there is velocity integration, there is no steady-state error, but the tuning time becomes longer.

Figure 4-9 No torque low-pass filtering

Figure 4-10: Wiht torque low-pass filtering Tf=4ms

By comparing errors! Reference source not found. And wrong! Reference source not found. It can be observed that adding torque low-pass filtering can eliminate torque oscillation, but the filtering transition frequency is low, the phase lag increases, and the gain cannot be improved, resulting in longer tuning time.

Figure 4-11 with torque low-pass filter with Tf=0.2ms

By comparing errors! Reference source not found. And wrong! Reference source not found. It can be found that by setting the torque low-pass filter reasonably, torque oscillation can be eliminated while ensuring fast position adjustment, achieving a good balance between speed and stability.

- Table 1-1

- The specific function codes for gain class and torque control parameters are shown in the table below:

Table 4-4 Index codes for gain class and torque control parameters

|  | 0x2006- Gain class parameter |  |  | 0x2007- Torque Control Parameter |
| --- | --- | --- | --- | --- |
| Subindex | 0x01- Speed Proportional Gain 1 | 0x02- Speed Integral Gain 1 | 0x03 Position Proportional Gain 1 | 0x03- Torque Filter 1 |
| Data type | UINT16 |  |  |  |
| Accessibility | Readable/writable | Readable/writable | Readable/writable | Readable/writable |
| Unit | Hz | ms | Hz | ms |
| Default value | 25 | 31.83 | 40 | 0.79 |
| Min. | 1 | 15 | 1 | 0 |
| Max. | 20000 | 51200 | 20000 | 3000 |
| Setting and effective mode | Stop setting/immediate enable | Stop setting/immediate enable | Stop setting/immediate enable | Stop setting/immediate enable |
| Related mode | - |  |  |  |
| Note | - |  |  |  |

(2) Feedforward control

Speed feedforward

Speed command input

Position loop control

Selection of speed feedforward control

2013-0D h

Position deviation

Position feedback

No speed feedforward

Internal speed feedforward

Speed feedback

Use 60B1h as speed feedforward input

Speed feedforward controller

Figure 4-12 Block Diagram of Speed Feedforward Control

In position control mode, the theoretical speed command required for the action is directly calculated through internal position commands, and added to the speed command calculated by the position feedback loop. It is applied to the input of the speed regulator command, which can significantly reduce position tracking errors and improve response performance compared to simple feedback control. Therefore, using the speed feedforward function can improve the speed command response and reduce the position deviation when the speed is fixed.

In theory, the relationship between position deviation and position loop gain, as well as velocity feedforward gain, is shown below. If the speed feedforward gain is set to 100%, theoretically the position deviation will become zero, but an excessively large feedforward gain coefficient will cause excessive speed overshoot during acceleration and deceleration.

When the update cycle of the position command is less than the servo control cycle, the differential operation of speed feedforward will cause significant differential errors, which will be converted into high-frequency torque command components, thereby inducing electromagnetic noise during operation. In this case, please use a position command filter (FIR filter or sliding mean filter), or increase the speed feedforward filter value.

Command speed

Motor speed

Position deviation

Speed FF gain

Position deviation [command units] = Command speed [command units/s] / Position loop gain [1/s] × (100 - Speed feedforward gain) / 100

Time

Position deviation in a certain speed range decreases as speed feedforward gain increases

Figure 4-13 Relationship between speed feedforward gain and position deviation

Operational steps for speed feedforward function:

A) Set the source of speed feedforward signal

Set 2013-0Dh(Speed Feedforward Control Selection) as a non-0 value, enable speed feedforward function, and the speed feedforward signal source can be selected internally and externally, shown as in the following table.

Table 4-5 Speed Feedforward Control and Selection of Index Codes

| Index codes | Name | Settings | Remarks |
| --- | --- | --- | --- |
| 2013-0Dh | Selection of speed feedforward control | 0: No speed feedforward | - |
|  |  | 1: Internal speed feedforward | Use the velocity data corresponding to position command as the source of the velocity feedforward signal. |
|  |  | 2: Use 60B1h as speed feedforward input | Use 60B1h speed bias (instruction unit/second) as the source of the speed feedforward signal. By using bit 6 of 607Eh (polarity), polarity of the velocity feedforward signal can be changed at this time. |

B) Set speed feedforward parameters

Including speed feedforward gain (2006-09h) and speed feedforward filtering time (2007-07h).

Table 4-6 Index codes for speed feedforward parameters

| Index codes | Name | Adjustment instructions |
| --- | --- | --- |
| 2007-07h | Velocity feedforward filtering time | Reducing filtering time can suppress speed overshoot of acceleration and deceleration; Increasing the filtering time can suppress noise in situations where the update cycle of the position command is longer than the control cycle of the driver, and the pulse frequency of the position command is uneven, thus suppressing the jitter of the positioning completion signal; |
| 2006-09h | Speed feedforward proportional gain | See error! Reference source not found. |

Torque feedforward

Current loop control

Use 60B2 as torque feedforward input

Torque feedforward select

2014-0Ch

Speed loop control

Motor

Speed calculation

Speed feedback

Command input

Internal torque feedforward

No torque feedforward

Speed Deviation

Encoder

Torque feedforward controller

Figure 4-14 Torque Feedforward Control Operation Diagram

Position control mode, using torque feedforward can improve dynamic velocity response and reduce position deviation during fixed acceleration/deceleration; To use torque feedforward, it is necessary to set the correct load torque inertia ratio. Please refer to the mechanical load identification results in Section 0. The torque feedforward gain is set to a non-zero value, and the torque feedforward function is enabled. By increasing the torque feedforward gain, the position deviation during constant acceleration /deceleration can be controlled to around 0, and the trapezoidal motion curve can be perfectly tracked without external torque interference.

Position deviation

Command speed

Torque feedforward can reduce position deviation in a fixed acceleration range.

Motor speed

Torque FF gain = 100[%] fixed

Torque FF gain

Figure 4-15 Relationship between torque feedforward gain and position deviation of acceleration and deceleration sections

Operation steps for torque feedforward function:

A) Set the source of torque feedforward signal

Set 2014-0Ch (torque feedforward control selection) as non-zero value, enable the torque feedforward function. The feedforward signal source can be selected from internal and external sources, as shown in the table below.

Table 4-7 Parameter Index Code for Speed Feedforward Control Selection

| Index codes | Name | Settings | Remarks |
| --- | --- | --- | --- |
| 2014-0Ch | Selection of torque feedforward control | 0: No torque feedforward | - |
|  |  | 1: Internal torque feedforward | Use speed command as the source of torque feedforward signal. In position control mode, the speed command comes from the output of the position controller. |
|  |  | 2: Use 60B2h as torque feedforward input | Use 60B2h (torque bias, 0.1%) as the source of torque feedforward signal. By using bit 5 of 607Eh (polarity), the polarity of the torque feedforward signal can be changed at this time. |

B) Set torque feedforward parameters

Including torque feedforward proportional gain (2006-0A) and torque feedforward filtering time (2007-08).

Table 4-8 Index codes for speed feedforward parameters

| Index codes | Name | Adjustment instructions |
| --- | --- | --- |
| 2006-0Ah | Torque feedforward proportional gain | Increasing the proportional gain can improve response, but overshoot may occur during acceleration/deceleration; Reducing filtering time can suppress overshoot during acceleration and deceleration; Increasing the filtering time can suppress noise; Adjustment method: When adjusting, first, keep the filtering time at the default value; Then, gradually increase the proportional gain setting value from 0 until the torque feedforward effect is achieved at a certain setting value. When adjusting, the settings of 2006-0Ah and 2007-08h should be repeatedly adjusted to find a balanced setting |
| 2007-08h | Torque feedforward filtering time |  |

(3) Two-degree-of-freedom control

In non torque control mode, two-degree-of-freedom control coefficient can be used to improve the control effect, set to 100%, which is the normal PI control mode; Setting it other than 100%, i.e.,two-degree-of-freedom control, which can be used to increase resistance to external forces and improve velocity response waveforms.

The following figure shows the improvement of the two-degree-of-freedom control coefficient on slow speed increase and slow positioning completion.

2006-0Dh=80%

Good position tracking, positioning time 90ms (balance between tracking and disturbance rejection)

2006-0Dh=50% Position deviation overshoot, positioning time 20ms (significant overshoot)

2006-0Dh=100% Slow position deviation elimination, positioning time 162ms (significant tailing)

t

Position instruction

Position deviation

90ms

t

Position instruction

Position deviation

160ms

t

Position instruction

Position deviation

20ms

Figure 4-16 Example of two-degree-of-freedom Control

two-degree-of-freedom control enhances the anti-interference ability of the velocity loop and improves its ability to follow speed commands by adjusting the velocity loop control method.

Table 4-9 Index codes for two-degree-of-freedom feedforward coefficients

| Index codes | Name | Adjustment instructions |
| --- | --- | --- |
| 2006-0Dh | Two-degree-of-freedom feedforward coefficient | Parameter function: The control method for changing the speed loop in non torque control mode. Adjustment method: The setting of 2006-0Dh is too small, resulting in slow speed loop response; When overshoot exists in velocity feedback, gradually reduce 2006-0Dh from 100.0 until the two-degree-of-freedom control achieves effect at a certain set value. When 2006-0Dh=100.0, the velocity loop control method remains unchanged and defaults to proportional integral control. |

4.2.3 Gain Switching

The gain switching function is only effective in position and speed control mode and can be triggered by the internal state of the servo or external DI. Using gain switching can have the following effects:

	When the motor is enabled to be stationary, the position latched state can be switched to a lower gain to suppress vibration and reduce static noise;

	During the motor stop process, when the position is set, it can be switched to a higher gain to shorten the positioning time;

	It is possible to switch to a higher gain during motor operation to achieve better command tracking performance;

	Different gain settings can be switched through external signals based on the load device situation.

Status

Gain

Command speed

Low gain

(1st gain)

Drive

Stop

(servo lock)

Gain decrease, vibration suppression

High gain

(2nd gain)

Low gain

(1st gain)

Action

Stop

(servo lock)

Time

figure 4–17 Gain Switching Diagram

give an example:

In the application scenario of LED die bonder, there is a demand for high-speed, high-precision and high response servo, which is a typical application of fast positioning P2P. The gain switching function is adopted to ensure fast setting requirement and reduce the noise when the servo is enabled to be stationary.

Table 4-10 Steps for peed gain adjustment

| Parameter group | Steps for gain switching adjustment | Not using gain switching, adjust the gain manually |  | The second gain is set the same as base gain |  | Enable gain switching function |  | In static state, adjust the first group of gain parameters, eliminate static noise |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| P06 | Velocity proportional gain 1 | 35.0Hz |  |  |  |  |  | 27.0Hz |
| P06 | Velocity integral gain 1 | 16.00ms |  |  |  |  |  |  |
| P06 | Position proportional gain 1 | 63.0Hz |  |  |  |  |  |  |
| P07 | Torque filtering 1 | 0.65ms |  |  |  |  |  | 0.84ms |
| P06 | Velocity proportional gain 2 |  |  | 35.0Hz |  |  |  |  |
| P06 | Velocity integral gain 2 |  |  | 16.00ms |  |  |  |  |
| P06 | Position proportional gain 2 |  |  | 63.0Hz |  |  |  |  |
| P07 | Torque filtering 2 |  |  | 0.65ms |  |  |  |  |
| P06 | Gain switching - Mode selection | 0 |  |  |  | 1 |  |  |
| P06 | Gain switching - Condition selection |  |  |  |  | 10 |  |  |
| P06 | Load inertia ratio | Obtained by inertia identification |  |  |  |  |  |  |

SV3 servo supports the following 10 gain switching methods, including 1 external DI switching and 9 switching methods upon internal motion status of the servo:

0: The first gain is fixed				(PS)

1: Use external DI for switching		(PS)

2: Torque instruction					(PS)

3: Speed instruction					(PS)

4: Speed instruction change rate			(PS)

5: High/low speed threshold of speed instruction		(PS)

6: Position deviation					(P)

7: With position instruction				(P)

8: Positioning is incomplete				(P)

9: Actual speed					(P)

10: With position instruction + actual speed		(P)

Herein, (P) represents that that switching method only supports position control mode; (PS) represents that the switching method supports both position control and speed control modes. Below is the detailed instructions about the 9 internal switching methods of the servo.

Table 4-11 Change Diagram of Internal Switching Method of Servo

| Gain switching method | Switching method change diagram |
| --- | --- |
| 2: Torque instruction | First Second First Torque command Switching level Switching level Switching delay Switching delay Second First |
| 3: Speed instruction | First Speed command First Second Switching delay Switching level |
| 4: Speed instruction change rate | Second First Switching delay Speed command First Switching level |
| 5: High/low speed threshold of speed instruction | Positive switching hysteresis Switching level Speed command Negative switching hysteresis Second First First |
| 6: Position deviation | Speed command Switching delay Second First Switching level First |
| 7: Position instruction | Switching delay First Position instruction Second First |
| 8: Positioning is completed | Positioning complete signal First Switching delay First Second |
| 9: Actual speed | Switching level Speed command First Second First Switching delay |
| 10: With position instruction + actual speed | Combination of method 7 and method 9 is generally chosen to ensure fast and stable setting in place |

Note:

The delay time '06-12h' is only valid when the second gain is switched into the first gain.

Table 4-12 Index codes for gain class parameters

| 0x2006- Gain class parameter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Subindex | 0x10- Gain Switching - Mode Setting | 0x11- Gain Switching - Condition Selection | 0x12- Gain Switching - Delay | 0x13- Gain Switching - Level | 0x14- Gain Switching - Time Delay | 0x15- Gain Switching - Time |
| Data type | UINT16 |  |  |  |  |  |
| Accessibility | Readable/writable | Readable/writable | Readable/writable | read-only |  | read-only |
| Unit | - | - | ms | - | - | ms |
| Default value | 1 | 0 | 5.0 | 50 | 30 | 3.0 |
| Min. | 0: The first gain fixed, use external DI for P/PI switching 1: Switching between the first gain and the second gain is valid, and the switching condition is P06.16 | 0: The first gain fixed (PS) 1: Switching by external DI (PS) 2: High torque instruction(PS) 3: High speed instruction(PS) 4: High change rate of speed instruction(PS) 5: Speed instruction high/low speed threshold (PS) 6: Large positional deviation (P) 7: With position instruction (P) 8: Positioning is uncompleted (P) 9: Actual speed (P) 10: With position instruction+actual speed (P) | 0 | 0 | 0 | 0 |
| Max. |  |  | 1000 | 20000 | 20000 | 1000 |
| Setting and effective mode | Run settings/Effective immediately | Run settings/Effective immediately | Stop setting/immediate enable | Stop setting/immediate enable | Stop setting/immediate enable | Stop setting/immediate enable |
| Related mode | - |  |  |  |  |  |
| Note | - |  |  |  |  |  |

4.2.4 Automatic gain adjustment

Automatic gain adjustment refers that SV3 servo drive automatically generates a set of matching basic gain parameters upon parameter settings of "P1A Group - Advanced Adjustment Function - Response Level Selection" so as to meet the requirement on speed and stability.

SV3 servo provides 2 automatic gain adjustment modes: 1. Basic mode; 2. Positioning mode.

Note:

Before using the automatic gain adjustment function, it is essential to correctly obtain the load inertia ratio!

Vibration occurs ?

Vibration disappears ?

End

Meets requirements?

Increase "P1A group advanced adjustment function - response level selection" parameter setting

Host computer enables, inputs set motion command

Refer to section 4.2.1 for load inertia identification

Servo disable enable, select auto-tuning mode

Start

Save servo parameters

Contact our company personnel

Refer to section 4.3.2 for vibration suppression

During motor operation, observe motion curve through background software, or observe servo response through host controller

Whether inertia identification has been performed

Select through "P1A group advanced adjustment real-time auto-tuning mode selection" parameter setting, default is basic mode

Figure 4-18 Steps for automatic gain adjustment

(1) Standard rigid table mode

Real-time automatic gain adjustment - Standard rigid table mode(201A-01h=1) is suitable for most of situations, with response levels (201A-02h) ranging from 0 to 40 levels. The higher response level means the stronger gain and the faster response. Based on different load types, the following empirical values are for reference:

Table 4-13 Response Level Reference

| Recommended response level | Type of load mechanism |
| --- | --- |
| Level 4 to Level 8 | Some large machinery |
| Level 8 to Level 15 | Applications with low rigidity such as belt |
| Level 15-20 | Applications with high rigidity such as ball screws and direct connections |
| Level 20-40 | Directly connected high rigidity, small inertia load application |

Real-time automatic adjustment of standard rigid table mode (201A-01h=1), basic gain (2006-01h~206-03h, 2007-03h) parameters, automatically updated according to the response level set in 201A-02h and stored in the corresponding index code:

Table 4-14 Real-time automatic adjustment mode with automatic parameter update

| Index codes | Name |
| --- | --- |
| 2006-01h | Velocity proportional gain 1 |
| 2006-02h | Velocity integral gain 1 |
| 2006-03h | Position proportional gain 1 |
| 2007-03h | Torque filtering 1 |

(2) Quick positioning mode

Quick positioning mode (201A-01h=2)

The parameter settings on the background software are as follows:,

Figure 4-19 Quick positioning mode setting

The quick positioning mode inside SV3 servo automatically realizes the gain switching function in Section 0 and the feedforward control function in Section 4.2.2, organically combining the two to realize the quick positioning function. Upon "Automatic Adjustment Standard Rigid Table Mode", the second gain parameter (2006-04h~2006-06h, 2007-04h) is also automatically updated and stored in the corresponding index code according to the response level set in 201A-02h, and the position loop gain of the second gain parameter should be 1 response level higher than the first gain parameter.

Table 4-15: Automatic Parameter Update for Quick Positioning Mode

| Index codes | Name |
| --- | --- |
| 2006-04h | Velocity proportional gain 2 |
| 2006-05h | Velocity integral gain 2 |
| 2006-06h | Position proportional gain 2 |
| 2007-04h | Torque filtering 2 |

The parameters related to speed feedforward are set to fixed values:

Table 4-16 Fixed Parameters for Quick Positioning Mode

| Index codes | Name | Parameters |
| --- | --- | --- |
| 2006-08h | Speed feedforward proportional gain | 30.0% |
| 2006-07h | Velocity feedforward filtering time | 0.50ms |

The parameters related to gain switching are set to fixed values:

In fast positioning mode, gain switching function is automatically enabled.

Table 4-17 Gain Parameters of Fast Positioning Mode

| Index codes | Name | Parameters | Remarks |
| --- | --- | --- | --- |
| 2006-10h | Gain switching - Mode selection | 1 | In fast positioning mode, the switching between the first gain (2006-01h~206-03h, 2007-03h) and the second gain (2006-04h~206-06h, 2007-04h) is valid; Maintain the original settings other than fast positioning mode. |
| 2006-11h | Gain switching - Condition selection | 10 | In fast positioning mode, the gain switching condition is 2008-0Ah=10; Maintain the original settings other than fast positioning mode. |
| 2006-12h | Gain Switching - Delay | 5.0ms | In fast positioning mode, gain switching delay time is 5.0ms; Maintain the original settings other than fast positioning mode. |
| 2006-13h | Gain Switching - Level | 50 | In fast positioning mode, gain switching level is 50; Maintain the original settings other than fast positioning mode. |
| 2006-14h | Gain switching - time delay | 30 | In fast positioning mode, gain switching delay is 30; Maintain the original settings other than fast positioning mode |

Note:

In automatic gain adjustment mode, parameters that are automatically updated with response level selection (201A-02h); Parameters with fixed values can't be manually modified. In order to modify it, it must set 201A-01h of 0 and exit the real-time automatic adjustment mode.

Table 4-18 Advanced Adjustment Index Codes

| 0x201A - Advanced Adjustment |  |  |
| --- | --- | --- |
| Subindex | 0x01- Real time self-adjusting settings | 0x02- Response Level Setting |
| Data type | UINT16 |  |
| Accessibility | Readable/writable | Readable/writable |
| Unit | - | - |
| Default value | 7 | 16 |
| Min. | 0: Invalid 1: Standard rigid table mode 2: Quick positioning mode 5: Adaptive interpolation mode 7: Adaptive positioning mode | 0 |
| Max. |  | 40 |
| Setting and effective mode | Run settings/Effective immediately | Run settings/Effective immediately |
| Related mode | PST |  |
| Note | - |  |

4.3 Vibration suppression

4.3.1 Notch Filter

When the servo gain coefficient is relatively large, high-frequency vibration components can't be effectively attenuated, that would trigger system resonance. In order to maintain the servo response performance and drive the mechanical load smoothly without reducing the gain, it rquires to effectively suppress resonance. Generally, servo manufacturers will install notch filter in the front channel of control loop to attenuate loop gain amplitude at the resonance point by a fixed point so as to realize vibration suppression.

Mechanical resonance point enters system from speed feedback

Notch filter

Figure 4-20 Block Diagram of Resonance Suppression Control

Transfer function of notch filter is

4.3.2 Vibration Suppression

SV3 servo has the built-in adaptive vibration suppression algorithm, which extracts vibration components from motor speed during actual operation, calculates the resonance frequency, and sets relevant parameters of adaptive notch filter automatically so as to realize vibration suppression.

(1) Automatic notch filter

To enable the adaptive notch filter function, just set "P1A Group - Advanced Adjustment - Vibration Suppression Mode Selection" of 1 or 2 in the backend software; SV3 servo supports up to 4 adaptive notch filters. When the system has 5 or more resonance points, it shall manually set the notch filters. It supports up to 4 different notch frequency settings.

Motor current

Motor speed

Resonance frequency inference

Load inertia inference

Motor

Gain auto-setting

Position/speed command

Drive

Real-time auto gain adjustment

Adaptive filter

Torque command

Current control

Torque command

Filter auto-adjustment

Motion command under actual use conditions

Encoder

Figure 4-21 Block diagram of adaptive vibration suppression principle

Note:

Under the following conditions, it might be impossible to perform normal automatic vibration suppression. In this situation, please perform manual vibration suppression.

Table 4-19 Factors of Adaptive Vibration

|  | Factors of adaptive vibration suppression |
| --- | --- |
| Resonance characteristics |  When resonance frequency is lower than the velocity response frequency  When 3 or more resonance points exist  When vibration amplitude is small, or control gain is low, and the impact on motor speed is nonsignificant |
| Mechanical properties |  Nonlinear factors such as excessive tooth clearance in operation or excessive backlash in forward/reverse rotation, e.g., misaligned gear installation in gear transmission mechanism  Vibration components randomly appear and last for a short period |
| Movement conditions |  Rapid acceleration/deceleration, when acceleration is greater than 30000rpm/s  When the servo operates in torque mode |

(2) Manual notch filter

A) Analyze resonance frequency;

Using the manual notch filter, it's necessary to set the notch filter frequency as actual resonance frequency.

The resonance frequency can be obtained by driving the oscilloscope interface of debugging platform, and there are 2 methods as follows:

Obtained by the motor current displayed on oscilloscope interface(phase current/torque command/current feedback). As shown in the figure below, the measured period is 0.625ms, and the calculated resonance frequency is:

f = 1/t =1600 Hz

Figure 4-22 Screenshot of resonance frequency test ,on the background software

①	Obtained by "resonance point identification" function on oscilloscope interface. As shown in the figure below, the measured resonance frequency is 1593.750Hz.

②

Figure 4-23 Screenshot of Resonance Point Identification Function on Backeground Software

B) Input the resonance frequency obtained in Step A) into  notch frequency parameter of a notch filter, and other parameters generally don't need to be set;

Figure 4-24: Parameter Settings of Notch Filter

If resonance is suppressed, it proves that the notch filter is effective and the gain can be further adjusted. If new resonance occurs after the gain increases, repeat steps A) to B);

If vibration can't be eliminated for a long time, please turn off the servo enable in time to reduce the loop gain.

(3) Low-pass filter

It may also attenuate the amplitude of all high-frequency vibration components above the transition frequency to below the sensitive value by means of appropriate torque low-pass filter.

The torque command filtering can be set in 2 ways,

Background debugging software "P07 Group filtering parameters - torque filtering"

Object Dictionary Object Word (2007-03h)

By setting a filtering-time constant, make the high-frequency range above the cutoff frequency in torque command be attenuated so as to suppress mechanical resonance.

The setting parameter for torque low-pass filtering is the filtering-time constant, with a unit of 0.01ms. The conversion relationship between the filtering-time constant τm and the filter cutoff frequency ƒc is:

4.4 Common Application Scenarios

4.4.1 Load of ball screw

Figure 4-25 Ball screw transmission

(1) Feature of ball screw

	Low friction resistance, the difference of dynamic and static friction forces is extremely small, ensure stable movement and avoid low-speed crawling. Low wear, long lifespan, and good precision retention.

	After pre-tightening with double nuts, the clearance can be effectively eliminated and transmission stiffness is high.

	Low friction loss, high transmission efficiency, up to 90% to 96%.

(2) Debugging precautions

	The stiffness of screw load transmission is relatively high, and it is generally easy to adjust. If the inertia ratio is reasonable, only the inertia needs to be identified, and automatic gain adjustment can be performed according to Section 4.2.4 Automatic Gain Adjustment.

	If high response performance is required, it is necessary to manually adjust the gain according to Section 4.2.2 Introduction on Manual Gain Adjustment, and even resonance analysis and suppression are required. Refer to Chapter 4.3 Vibration Suppression.

4.4.2 Synchronous Belt Load

Figure 4-26 Synchronous belt transmission

(1) Feature of synchronous belt transmission

	During operation, convex teeth of the synchronous belt mesh with teeth grooves of the belt to transmit motion and power.

	During operation, as no sliding exists and and it has accurate transmission ratio, it's called synchronous belt.

	High transmission efficiency and good energy-saving effect. It has a high transmission efficiency, generally up to 98%.

	Range of transmission ratio is large, the structure is compact, and the rigidity is weak.

(2) Debugging precautions

	Short synchronous belt is easy to test. In the case of reasonable inertia ratio, it's only necessary to identify the inertia ratio and perform automatic gain adjustment.

	If the synchronous belt is relatively long and has a large inertia, it's prone to overshoot during operation. The requirement for position command forms is high, and position command filtering can be performed under the servo.

4.4.3 Rack and pinion load

Figure 4-27 Gear and Rack Transmission

(1) Feature of Gear and Rack Transmission

	Be able to configure larger reduction ratio, strong load driving capability. Increase output torque by reducing load operating speed.

	Backlash of the transmission is big, that may lose partial transmission accuracy.

(2) Debugging precautions

	Generally, reducer can effectively reduce the load inertia ratio, so such equipment has a relatively small inertia ratio and is easy to test. Generally, automatic gain adjustment is used.

	The installation rigidity of rack at different positions may vary, and the machining accuracy of rack may differ. It's necessary to ensure that no vibration or operating noise occurs in the full-run range.

4.4.4 Inertia disk load

Figure 4-28 Directly connected inertia disk

(1) Load feature of directly connected inertia disk

	Directly connected inertia disk load has good transmission accuracy.

	Highly synchronized with motor, no backlash, high transmission rigidity.

	Anti-resonance frequency is low. Max. bandwidth available of system shall be lower than anti-resonance points, besides being far away from resonance point.

(2) Debugging precautions

	As this connection will inevitably bring a large inertia ratio to motor, speed gain can't be set too high.

	Vertical installation condition of motor is generally used in indexing control applications, requiring fast and accurate start/stop.For adjustmnet, refer to Gain Switching in Section 4.2.3.

	Horizontal installation condition of motor generally has a larger inertia and is more prone to vibration; The gain can't be set too high, that requires resonance analysis and suppression.

4.4.2

4.4.5 Long cantilever load

Workbench

End

Sevo motor

Figure 4-29 Long cantilever load

For long cantilever load, there is usually end jitter or overall equipment swing in high-speed motion. Vibration can be suppressed by eliminating frequency components from position command. SV3 servo provides a damping filter that can effectively suppress end jitter. Parameter "P07 group filtering parameters - position notch frequency A" can be set by the backend software to suppress low-frequency jitter.

Figure 4-30: Screenshot of the upper computer for function code "position notch frequency"

There are 2 ways to obtain jitter frequency:

	Obtain the end jitter frequency of mechanical load by external sensor, if Error! Reference source not found, as shown;

	By waveform of the backend software, obtain end jitter frequency of motor. If Error! Reference source not found. As shown.

Motor

Coupling

Programmable controller

Machine

Ball screw

End vibration

Move

Machine

Position instruction

Position/speed control

Torque command

Torque command

Drive

Load

Current control

Motor current

Motor Position

Drive

Encoder

Motor

Set end vibration frequency

Observe vibration with displacement sensor

Figure 4-31 Method for acquisition and suppression of \Vibration Frequency of Long Cantilever Load

After suppression

Position feedback (encoder units)

Before suppression

Low-frequency vibration period T

Figure 4-32 Vibration frequency acquisition and comparison of before-and-after suppression effects of long cantilever load

After measuring the low-frequency vibration period by the background, the low-frequency resonance frequency can be calculated by f=1/T, and Parameter "P07 group filtering parameters - position notch frequency A" can directly be set. Note that the unit of this parameter is 0.1Hz.

- Chapter 5 Troubleshooting and Warning

5.1 Display and Review

Alarm of SV3 servo consists of fault and warning. The main difference is that when a fault occurs, the servo will stop ; When a warning occurs, the servo can still run normally. According to the display of fault or warning on the panel, detailed description and solutions for corresponding fault or warning can be found in this manual.

After power-on, if the servo panel is off or rdy is not displayed at the last 3 digits, possible causes and solutions are as follows:

Table 5-1 Troubleshooting for the fault that the power-on servo is not ready

| Serial No. | Fault cause | Confirmation method |
| --- | --- | --- |
| 1 | Fault of control power supply voltage | After CN1, CN3, CN4, and CN5 are removed, the fault still exists. Measure AC voltage between L1C and L2C. |
| 2 | Fault of main power supply voltage | Check whether 220VAC/380VAC power supply for drives is normal per specifications. |
| 3 | Servo drive fault | Contact the manufacturer for after-sales service. |

After power-on, when the servo has any fault or warning, the panel will present the corresponding display. Herein, definitions of the display are as follows:

Main fault or warning code

Sub fault or warning code

E:Err

A:Alarm

Figure 5-1 Fault display sample

For example, if E.010.0 is displayed on panel, it presents that servo drive has fault. The fault main code is 0x010, and the fault subcode is 0x0. The panel displays A.218.0, i.e., warning has occurred on the servo drive. Main code of the warning is 0x218 and its sub-code is 0x0.

After above faults are rectified, rdy shall be displayed in the last 3 digits of panel.

SV3 servo drive has fault logging function which can record the last 10 faults and warning names, and the status parameters of servo drive when the fault or warning occurred. If repeated faults or warnings occurred in the last 5 times, the fault or warning code(i.e., drive status) is recorded only once.

After fault or warning is reset, the fault and warning are still saved in the fault record. By Fault & Warning management module in Servo3 Designer, it can review and clear the records. For details, see the Servo3 Designer instruction.

In addition, by Fault & Warning management module in Servo3 Designer, it can review the detailed information of present fault and reset it.

5.2 Fault Code Overview

Table 5-2 List of fault codes

| Fault code | Fault name | Reset or not | CiA402 Protocol Error Code (603Fh) |
| --- | --- | --- | --- |
| E.010 | Software parameter fault | × | 0x6320 |
| E.011 | Fault of software internal communication initialization | × | 0x7500 |
| E.012 | Fault of software version not matching the product model | × | 0x7500 |
| E.013 | Fault of software internal interrupt timeout | × | 0x7500 |
| E.014 | Fault of software internal communication timeout | × | 0x0014 |
| E.015 | Fault of current sampling timeout | × | 0x0015 |
| E.016 | Fault of torque instruction update timeout | × | 0x0016 |
| E.017 | Fault of parameter storage | × | 0x5530 |
| E.018 | Fault of out-of-range parameters | × | 0x6320 |
| E.019 | Product matching fault | × | 0x7122 |
| E.210 | Fault of hardware overcurrent | × | 0x2312 |
| E.211 | Fault of output short-circuit to GND | × | 0x2330 |
| E.212 | Fault of UVW phase sequence | × | 0x0212 |
| E.213 | Overrun fault | × | 0x0213 |
| E.214 | Fault of control power supply undervoltage | × | 0x3120 |
| E.216 | STO fault | √ | 0x0216 |
| E.217 | Fault of input phase loss | √ | 0x3130 |
| E.218 | Fault of servo emergency stop | √ | 0x5442 |
| E.219 | Fault of drive overtemperature | √ | 0x4210 |
| E.227 | Fault of output phase loss | √ | 0x0227 |
| E.228 | Fault of abnormal precharge | √ | 0x0228 |
| E.228 | Fault of abnormal precharge | √ | 0x0228 |
| E.411 | Fault of DI allocation | √ | 0x6320 |
| E.412 | Fault of DO allocation | √ | 0x6320 |
| E.413 | Fault of current calculation overflow | √ | 0x0413 |
| E.414 | Fault of DC bus undervoltage | √ | 0x3220 |
| E.415 | Fault of DC bus overvoltage | √ | 0x3210 |
| E.416 | Overspeed fault | √ | 0x8400 |
| E.417 | Fault of startup overspeed | √ | 0x0417 |
| E.418 | Fault of server repeatedly enabled | √ | 0x5441 |
| E.419 | Fault of drive overload | √ | 0x3230 |
| E.420 | Fault of excessively-high frequency of frequency division output | √ | 0x0420 |
| E.421 | Fault of electronic gear ratio setting | √ | 0x0421 |
| E.422 | Fault of fully closed loop parameters | √ | 0x0422 |
| E.423 | Fault of excessively large position deviation | √ | 0x8611 |
| E.424 | Fault of position instruction overspeed | √ | 0x0424 |
| E.430 | Fault of gantry compensation data overflow | √ | 0x0430 |
| E.710 | Overspeed fault | √ | 0x8400 |
| E.711 | Fault of inertia identification | √ | 0x0711 |
| E.712 | Fault of magnetic pole identification | √ | 0x0712 |
| E.715 | Fault of motor parameter identification | √ | 0x0715 |
| E.716 | Fault of gain self-adjusting | √ | 0x0716 |
| E.910 | Fault of encoder parameters | × | 0x7305 |
| E.911 | Fault of encoder communication | × | 0x7305 |
| E.912 | Fault of encoder parameter verification error | × | 0x7305 |
| E.913 | Fault of encoder three-phase Hall logic error | × | 0x7305 |
| E.914 | Fault of encoder disconnected | × | 0x7305 |
| E.916 | Fault of encoder disconnected | √ | 0x7306 |
| E.917 | Fault of encoder battery | √ | 0x7305 |
| E.918 | Error of rncoder multi-turn count | √ | 0x7305 |
| E.919 | Fault of encoder multi-turn counter overflow | √ | 0x7305 |
| E.922 | Encoder overheating warning | √ | 0x0922 |
| E.B10 | Fault of motor overload | √ | 0x3230 |
| E.B11 | Fault of motor stall | √ | 0x7121 |
| E.B13 | Fault of motor vibration | √ | 0x0B13 |
| E.B14 | Running exception detection | √ | 0x0B14 |
| E.B15 | Fault of motor PTC | √ | 0x0B15 |
| E.D15 | Error of upper/lower soft limit settings | √ | 0x0D15 |
| E.D16 | Origin bias out of soft limit range | √ | 0x0D16 |
| E.D20 | EtherCAT communication disconnected | √ | 0x0D20 |
| E.D21 | EtherCAT communication disabled | √ | 0x0D21 |
| E.D22 | EtherCAT communication connection timeout | √ | 0x0D22 |
| E.D23 | EtherCAT extended card communication timeout | √ | 0x0D23 |
| E.D24 | EtherCAT station name conflict | √ | 0x0D24 |
| E.D25 | EtherCAT station name setting error | √ | 0x0D25 |
| E.D26 | EtherCAT communication exception | √ | 0x0D26 |
| E.D27 | EtherCAT system parameter error | √ | 0x0D27 |
| E.D28 | EtherCAT configuration error | √ | 0x0D28 |
| E.D29 | EtherCAT not programming XML file | √ | 0x0D29 |
| E.D30 | EtherCAT communication initialization failure | √ | 0x0D30 |
| E.D31 | EtherCAT synchronization cycle setting error | √ | 0x0D31 |
| E.D32 | EtherCAT synchronization signal excessive deviation | √ | 0x0D32 |

Note:

√ indicates that this fault can be reset. For details about fault reset, see 5.5 Warning Handling. It should be noted that any fault can be successfully reset only if its source has been removed.

X indicates that this fault can't be reset and must be powered on and off again.

5.3 Warning Code Overview

All warnings can be reset automatically after the warning conditions have been removed. Or press any key on the control panel to reset.

Table 5-3 List of warning codes

| Warning display | Warning name | CiA402 Protocol Error Code (603Fh) |
| --- | --- | --- |
| A.220 | Forward overtravel warning | 0x5443 |
| A.221 | Negative overtravel warning | 0x5444 |
| A.222 | Warning of input phase loss | 0x3130 |
| A.224 | Warning of regenerative resistor overload | 0x3210 |
| A.225 | Warning of brake resistance disconnected | 0x0225 |
| A.226 | Warning of excessively small external regenerative resistance | 0x6320 |
| A.425 | Warning of zero return failure | 0x0425 |
| A.426 | Warning of power ON/OFF reset | 0x6320 |
| A.427 | Warning of parameter storage exception | 0x7600 |
| A.428 | Warning of frequency divider output setting error | 0x0428 |
| A.920 | Encoder exception warning | 0x7305 |
| A.921 | Warning of low voltage of encoder battery | 0x7305 |
| A.928 | ABZ encoder feedback overclock | 0x0928 |
| A.D35 | Warning of synchronous zeroing setting error | 0x6320 |

5.4 Troubleshooting

Table 5-4 Troubleshooting list

| Fault codes and their descriptions | Fault cause | Handling measures |
| --- | --- | --- |
| E.010.0 Factory parameter verification is abnormal | 1. Software has been updated. 2. An instant power-off occurs during parameter storage. 3. Number of write times exceeds Max. value within a certain period. 4. Fault of servo drive. | 1. Restore factory parameters (P0501=1). 2. Power on again, after initialization of system parameters (P0501=1), re-write parameters. 3. Change the parameter writing method. 4. Replace the servo drive. |
| E.011.0 Software internal communication initialization is abnormal. | 1. Software versions of FPGA and MCU are inconsistent. 2. FPGA fault. | 1. Consult our technical support to update the matching FPGA or MCU software. 2. Contact technical support of the manufacturer. |
| E.012.0 Software version doesn't match the product model | 1. FPGA version is too low. 2. FPGA fault. | 1. Update the matching FPGA or MCU software. 2. Contact technical support of the manufacturer. |
| E.013.0 FPGA interruption is lost | 1. FPGA fault. 2. Handshake between FPGA and MCU is abnormal. 3. Dive internal calculation timeout. | 1. Replace the servo drive. 2. Contact technical support of the manufacturer. |
| E.014.0 Communication between MCU and FPGA timeout | 1. MCU communication timeout. 2. Encoder communication timeout. | 1. Use our standard motor cable and encoder cable. 2. Check whether cable connection is damaged. 3. Separate high voltage cable and weak current cable. |
| E.015.0 Current chip sampling timeout | 1. Check if output power line is broken or in poor contact. 2. Current sampling timeout. | 1. Use our standard motor cable and encoder cable. 2. Check whether cable connection is damaged. 3. Separate high voltage cable and weak current cable. |
| E.016.0 Torque instruction update timeout | 1. Servo drive fault. | 1. Replace the servo drive. 2. Contact technical support of the manufacturer. |
| E.017.0 Parameter storage writing is abnormal. | 1. EEPROM data fault. | 1. After system parameters are initialized (P0501=1), power it on again. 2. Replace the servo drive. |
| E.018.0 Factory parameters are out of range | 1. Software is updated; Present value of function code exceeds the allowed range. 2. EEPROM fault. 3. Servo drive fault. | 1. Power it on again, after initialization of system parameters (P0501=1), re-write parameters. 2. Replace the servo drive. |
| E.019.0 Encoder matching fault | 1. Product No.(motor or drive) doesn't exist. 2. Power levels of motor and drive don't match. | 1. Replace the mismatched products according to "Supporting Equipment Specification of Servo System". |
| E.210.0 P/N phase overflow | 1. Brake resistance is too small or short circuit. 2. Motor cable is in poor contact or short-circuited. 3. Motor cable isn't properly grounded. 4. Motor burned out. 5. Improper gain parameter settings, motor vibration. 6. Encoder cable fault. | 1. Select an appropriate brake resistance and set related parameters according to the specification. 2. Check if motor cable is properly connected and if there is short circuit, poor contact, or no grounding. 3. Check if encoder cable is damaged. 4. Re-adjust the gain. 5. Replace the motor. |
| E.211.0 Output short-circuit to GND causes bus overvoltage | 1. Drive power line(UVW) is short-circuited to GND. 2. Motor is short-circuited to GND. 3. Servo drive fault. 4. Bus voltage discharge(P0219) is set too low. | 1. Reconnect or replace drive power cable. 2. Replace the motor. 3. Set the bus voltage release point (P0219) correctly. |
| E.212.0 UVW phase sequence fault | 1. When the drive performs angle identification, UVW phase sequences of driver and motor doesn't match. | 1. Connect UVW cables in the correct phase sequence. |
| E.213.0 Overrun fault | 1. UVW phase sequence wiring fault. 2. Power-on interference causes the rotor initial phase error. 3. The encoder model is incorrect or wiring is wrong. 4. Under working condition of vertical axis, gravity load is excessive. | 1. Connect UVW cables in the correct phase sequence. 2. Power on/off it again and re-start angle self-learning. 3. Replace the driver and motor that match mutually, and set motor model properly. 4. Check if encoder and motor cables are properly connected. 5. Reduce loads on vertical axis, or improve the rigidity, or shield this fault without affecting safety and use. |
| E.214.0 Undervoltage of control power supply | 1. Power supply voltage is unstable or it powers off. 2. Control line in poor contact. | 1. Check the wiring or replace cables. 2. Power on it again. For abnormal power-off, ensure that power supply is stable. 3. Increase the power capacity. |
| E.216.0 STO disconnected | 1. STO disconnected causes STO valid. | 1. STO wiring is correct. 2. Replace the servo drive. |
| E.217.0 One phase loss of three-phase input | 1. Per three-phase specification input drive is connected to single-phase input. 2. Three-phase power input cable is in poor contact. 3. Three-phase voltage is unbalanced or the three-phase voltage is too low. | 1. Check the wiring of three-phase power supply. 2. Three-phase drive and allow single-phase operation (below 1kW), switch off the alarm (set P0800=2). |
| E.218.0 Fault of servo emergency stop | 1.DI function 2: Emergency stop is triggered. 2. Background communication emergency stop is triggered. | 1. Check the operating mode, and on the premise of safety remove the effective signal of DI brake(after the effective scram signal is removed, the fault is automatically reset). |
| E.219.0 Fault of drive overtemperature | 1. Ambient temperature is too high. 2. After overload, power off to reset the overload fault, and repeat it for several times. 3. Fan is damaged and can't work normally. 4. It's unreasonable of installation direction of servo drive and interval between the servo drive and other servo drives. | 1. Improve cooling conditions of servo drive and reduce the ambient temperature. 2. Change fault reset method. Wait 30 seconds after overload, and then reset again. Increase the capacity of drive and motor, increase acceleration/deceleration time, and reduce the load. 3. Install servo drive per installation standard. 4. Replace the servo drive. |
| E.227.0 Phase loss of UVW output | 1. U/V/W phase power line of motor is broken. | 1. Check the power cable connection of the motor, reconnect the cable, and replace the cable if necessary. |
| E.228.0 Preloaded relay is not drawn | 1. The hardware is damaged. | 1. Replace the servo drive. 2. Contact technical support of the manufacturer. |
| E.228.0 Open circuit of precharge resistor | 1. Open circuit of precharge resistor | 1. Contact the factory for technical support. |
| E.411.0 DI function is allocated repeatedly | 1. When DI function is allocated, the same function is allocated to multiple DI terminals. | 1. Re-allocate DI function to avoid duplication. |
| E.412.0 Allocation of DO function is out of range | 1. DO functio codes exceed number of DO functions. | 1. Restore factory parameters and power on/off it again. |
| E.413.0 Fault of current calculation overflow | 1. DQ axis current overflows. | 1. Restore factory parameters and power on/off it again. 2. Replace the servo drive. |
| E.414.0 Fault of DC bus undervoltage | 1. Power supply of main loop is unstable or power=off. 2. Instantaneous power failure occurs. 3. Power supply voltage drops during operation. 4. Per three-phase specification input drive is connected to single-phase input. | 1. Adjust or replace the power supply per the specification of input power supply. 2. Increase the power capacity. 3. Shield phase loss fault detection. |
| E.415.0 Fault of DC bus overvoltage | 1. Input voltage of main loop is excessive. 2. Power supply is unstable or affected by lightning strike. 3. Brake resistance fails. 4. External brake resistance is too large, and Max. braking energy can't be fully absorbed. 5. The sampled bus voltage has a large deviation. 6. The motor runs in high acceleration/deceleration, and Max. braking energy exceeds the absorbable value. | 1. Adjust or replace the power supply per the specification of input power supply. 2. Connect the surge suppressor. 3. Select the appropriate external brake resistance per the specification of external brake resistor. 4. Increase the acceleration/deceleration time if possible. |
| E.416.0 Speed exceeds Max. RPM | 1. Fault of U/V/W phase sequence of motor cable. 2. Motor or encoder parameters are set improperly, e.g., pole-pair number, encoder resolution. 3. Motor Angle identification is not done. 4. Input command exceeds the overspeed threshold. 5. Motor speed closed-loop overshoot. | 1. Perform the wiring in the proper U/V/W phase sequence. 2. Set motor parameters or encoder resolution properly. 3. Increase the power capacity. 4. Re-adjust the gain. 5. On the premise of the demand satisfied, reduce electronic gear ratio. 6. Set speed threshold within overspeed threshold. |
| E.417.0 Starting speed is higher than rated speed | 1. When the drive is enabled, actual motor speed is higher than rated speed. | 1. Reset the fault, reduce the actual speed, and re-operate the motor. |
| E.418.0 The servo is enabled repeatedly | 1. When internal function is enabled, communication servo enable is valid. | 1. Switch off servo enable signal of upper computer. |
| E.419.0 Fault of drive overload | 1. Fault of parameter settings. 2. Drive load rate is excessive, load inertia is too large or the machine is stuck. 3. Fault of motor stall. | 1. Set P0102 per the drive model. 2. Adjust parameters per the current feedback. 3. Replace it with a drive of larger power. 4. Adjust the machine to remove problems of the machine stuck. 5. Refer to E.B11.0 troubleshooting method. |
| E.420.0 Fault of excessively-high frequency of frequency division output | 1. Output pulse frequency exceeds upper limit of hardware frequency allowed(4MHz for a single channel). | 1. Reduce the number of frequency division output pulses (P1316) of encoder. 2. Use twisted-pair shielded wire to prevent interference pulses from superimposing on real pulses, that might result in false alarm. |
| E.421.0 Fault of local electronic gear ratio setting | 1. Electronic gear ratio settings exceed the above range. 2. Parameter change sequence | 1. Set gear ratio according to the range specification of electronic gear ratio. 2. Use the fault reset function or power it on again. |
| E.422.0 Full closed-loop can't switch internal and external loops in the multi-stage absolute position mode | 1. In full closed-loop position mode, the source of position instruction is internal position instruction, but use internal/external ring switching mode. | 1. When the full closed-loop function is used, and the position instruction source is internal, only the external encoder feedback mode can be used, i.e., P2200 can only be 1. |
| E.423.0 Internal-ring position deviation is excessive | 1. Drive U/V/W output loses phase or phase sequence is connected inproperly. 2. Drive U/V/W output is disconnected or the encoder is disconnected. 3. Motor stall from mechanical factors. 4. Low servo drive gain. 5. Increment of position instruction is excessive. 6. Deviation threshold 6065h/P0806 is too small. 7. Fault of servo drive/motor. | 1. Re-connect the cable properly, or replace it. 2. Re-connect the cable, replace it with brand-new one, if necessary, and ensure that it's reliably connected. 3. Exclude mechanical factors. 4. Adjust the gain manually or automatically. 5. Increase acceleration/deceleration ramp of position instruction. 6. Increase the deviation threshold 6065h/P0806. 7. Replace servo drive or motor. |
| E.424.0 Position instruction has double overspeed for several times | 1. Increment of position instruction is excessive. 2. Target position (607A target position) isn't aligned with the present position prior to mode switching or when the servo is enabled. 3. Synchronization loss which causes excessive accumulation of position instructions. 4. Motor speed limit error. | 1. Reduce the increment of target position instruction. 2. Assign the value of present position to target position (607A target position) prior to mode switching or when the servo is enabled. 3. Confirm if Max. speed of motor meets the application requirement. |
| E.430.0 Write overflow of Gantry compensation data | Reserved | Reserved |
| E.710.0 Overflow of FPGA internal speed measured | 1. Overflow of FPGA internal speed measured calculation . | 1. Check and confirm that encoder cable is properly connected. 2. Power it on/off again and re-try it. 3. Replace motor or drive. |
| E.711.0 Fault of offline inertia identification | 1. Offline inertia identification is uncompleted. | 1. Contact the factory for technical support. |
| E.712.0 Fault of magnetic pole identification | 1. Failure of initial magnetic pole angle identification of motor. | 1. Dis-connect motor shaft and re-identify the angle. |
| E.715.0 Exception of parameter identification result | 1. Identification result is 0 | 1. Contact the factory for technical support. |
| E.716.0 Self-adjust gain is too small | 1. Self-adjust gain is too small | 1. Execute self-adjustment again; 2. Contact technical support of the manufacturer. |
| E.910.0 Exception of encoder parameter verification during power-on | 1. Models of drive and motor don't match. 2. Parameter verification error or no parameter stored in bus incremental encoder ROM. | 1. Replace with a matching drive and motor. 2. Check the encoder cable, please use our company standard encoder cable. 3. Separate encoder cable from power cable. |
| E.911.0 (Detailed definition on pending) | 1. ault of encoder wiring. 2. Encoder cable is loose. 3. Encoder Z signal is interfered (EMC issues). 4. Encoder fault. | 1. Re-connect cables per the wiring diagram. 2. Re-connect cables, and ensure that encoder terminals are firmly connected. 3. Use standard encoder cables of our company. 4. Replace the servo motor. |
| E.912.0 Error of encoder parameter verification | 1. Bus incremental encoder cable is broken or loose. 2. Exception of Read/Write parameters of bus incremental encoder. | 1. Check if encoder cable is improperly connected, broken, or in poor contact. 2. Separate encoder cable from power cable. 3. Replace the servo motor. |
| E.913.0 Fault of reading initial angle during power-on initialization | 1. Models of drive and motor don't match. 2. The encoder cable is broken. | 1. Replace with a matching motor and drive. 2. Replace with quality encoder cable and fix it firmly. |
| E.914.0 Z signal line is broken | 1. Encoder fault causes Z signal loss. 2. Poor wiring connection or wrong connection causes encoder Z signal loss. | 1. Replace the servo motor. 2. Check if the encoder cable is in proper contact, and re-connect or replace the cable. |
| E.916.0 Wire of full closed-loop grating ruler is broken | 1. Frequency division output isn't disabled. 2. While using the full closed-loop function or non-standard pulse input, level difference of 2-way signals of any group A+/A-, B+/B-, Z+/Z- does not meet the requirement: Level difference is greater than or equal to 2V. | 1. Set P1315 value as 2 (frequency division or synchronous output disabled). 2. Adjust the level until it meets the specification. |
| E.917.0 Fault of encoder battery | 1. Absolute encoder isn't connected any battery during power-off. 2. Voltage of encoder battery is too low. | 1. Reconnect the battery or replace with a new battery. 2. Set P2005 as 1 and clear the fault. |
| E.918.0 Error of rncoder multi-turn count | 1. Encoder fault. | 1. Set P2005-2 to clear the fault and power on it again. 2. Replace the motor. |
| E.919.0 Overflow of encoder multi-turn counter | 1. As for overflow of absolute value encoder multi-turn count, only if multi-turn value is 32767 or 32768, it would report fault. This fault is reported by servo drive detection, not encoder. | 1. Set P2005=2 to clear multiple-turn data of encoder and power on it again. |
| E.922.0 Encoder overheating warning | 1. Temperature of encoder is too high. | 1. The drive stops for a period. 2. Ensure the encoder is well ventilated to reduce environmental temperature. |
| E.B10.0 Fault of motor overload | 1. Wiring of motor or encoder is wrong and bad contact. 2. Load is too heavy; Effective output torque of motor exceeds the rated torque, and runs continuously for a long time. 3. Acceleration and deceleration are too frequent or load inertia is too large. 4. Gain adjustment is unsuitable or rigidity is too strong. 5. Setting of drive or motor model is wrong. 6. Motor is stalled from mechanical factor, resulting in excessive load during operation. | 1. Set parameters of drive model and motor model properly. 2. Check the wiring by our company's standard wire and according to wiring diagram. 3. Replace large-capacity drive and the matching motor, or reduce load to increase acceleration/deceleration time. 4. Increase acceleration/deceleration time in a single run. 5. Re-adjust the gain. 6. Exclude mechanical factor. |
| E.B11.0 Motor stall protection | 1. Phase missing, broken wire, phase sequence connection error of drive U/V/W output. 2. Motor parameters are set impropery, e.g., number of pole-pairs. 3. Motor Angle identification is not done. 4. Communication command is interfered. 5. Motor is stalled from mechanical factor. | 1. Re-connect the cable properly, or replace it. 2. Set motor parameters properly. 3. Check if communication line between upper computer and the servo is interfered. 4. Exclude mechanical factors, Check if motor stall occurs, occasionally stall and eccentric situation. |
| E.B13.0 Excessive motor vibration | 1. Motor vibration is excessive | 1. Adjust gain parameters 2. Start vibration suppression |
| E.B14.0 Position exception detection |  |  |
| E.B15.0 Thermistor line is broken or thermistor is disconnected | 1. Thermistor line is broken or thermistor is disconnected | 1. Check thermistor and its wires |
| E.D15.0 Error of upper/lower soft limit settings | 1. Software lower limit is equal to or greater than the upper limit. | 1. Reset parameters; Ensure that 607D-01h is smaller than 607D-02h(P0B45 is smaller than P0B47). |
| E.D16.0 Origin bias out of soft limit range | 1. The origin is offset out of soft position limit. | 1. Reset the parameters of 607D-01h and 607D-02h (P0B45/P0B47) reasonably. |
| E.D20.0 EtherCAT communication disconnected | Reserved | Reserved |
| E.D21.0 EtherCAT communication disabled | Reserved | Reserved |
| E.D22.0 EtherCAT communication connection timeout | Reserved | Reserved |
| E.D23.0 EtherCAT extended card communication timeout | Reserved | Reserved |
| E.D24.0 EtherCAT station name conflict | Reserved | Reserved |
| E.D25.0 EtherCAT station name setting error | Reserved | Reserved |
| E.D26.0 EtherCAT communication exception | 1. The server is enabled. Due to misoperation of master station or human misoperation, EtherCAT network status is switched from OP to other status, and network status switchover is abnormal. | 1. Check network status switching program of upper computer. |
| E.D27.0 EtherCAT system parameter error | Reserved | Reserved |
| E.D28.0 EtherCAT configuration error | Reserved | Reserved |
| E.D29.0 EtherCAT not programming XML file | 1. No XML programmed in EEPROM. 2. XML file in EEPROM is modified abnormally. | 1. Programme XML file. |
| E.D30.0 EtherCAT communication initialization failure | 1. FPGA software isn't programmed. 2. Fault of servo drive. | 1. Contact the manufacturer to programme FPGA software. 2. Replace the servo drive. |
| E.D31.0 EtherCAT synchronization cycle setting error | 1. After network is switched to the operation mode, synchronization cycle isn't an integer multiple of 125us or 250us. | 1. Change the synchronization cycle to an integer multiple of 125us or 250us. |
| E.D32.0 EtherCAT synchronization signal excessive deviation | 1. Synchronization cycle error exceeds the threshold; Synchronization cycle error of controller is excessive. | 1. Increase factory parameter P0A32. |

5.5 Warning Handling

Table 5-5 Warning handling list

| Fault codes and their descriptions | Fault cause | Handling measures |
| --- | --- | --- |
| A.220.0 Forward overtravel warning | 1. For DI Function 9, forward limit input is valid and forward drive is disabled. | 1. Check the operation mode, under the premise of safety, send motor reverse instruction or reverse the motor rotation so as to make positive limit invalid. |
| A.221.0 Negative overtravel warning | 1. For DI Function 10, reverse limit input is valid and reverse drive is disabled. | 1. Check the operation mode, under the premise of safety, give the motor forward instruction or rotate the motor forward, so as to make reverse limit invalid. |
| A.222.0 Warning of input phase loss | 1. Phase loss of three-phase power input. | 1. Check the wiring of three-phase power supply. 2. Three-phase drive and allow single-phase operation (below 1kW), switch off the alarm (set P0800=2). |
| A.224.0 Brake resistor overload | 1. Wiring of external brake resistor is improperly. 2. While using built-in brake resistor, power terminals P/D are disconnected. 3. Erros of such parameters as brake resistor type, resistance and power. 4. Input voltage of main loop exceeds the specification. 5. Load moment of inertia ratio is excessive. 6. Motor has been in the deceleration for a long period. 7. Capacity of servo drive or brake resistor capacity is insufficient. | 1. Check if wiring of external brake resistor is proper. 2. While using the built-in brake resistor, connect P/D terminals properly. 3. Set the parameters of brake resistor propertly. 4. Per the specification, select the appropriate brake resistance. 5. Use the appropriate power input per the specification. 6. Reduce the load, or increase acceleration/deceleration time, or increase the operation cycle. |
| A.225.0 Software detects that the brake resistor is disconnected | 1. Brake discharge resistor is disconnected, or P/RB terminals are not short-circuited; | 1. Check the connection of brake resistor. |
| A.226.0 External brake resistance is too small | 1. While using external brake resistor, external brake resistance is less than Min.value by the specification. | 1. Per the specification, select the appropriate brake resistor, and connect it properly between P/C. 2. Set the parameters of external brake resistor properly. |
| A.425.0 Warning of the origin zero timeout | 1. Fault of the origin switch. 2. Search time for the origin is too short. 3. Switch signal speed of high-speed search for the origin is too small. 4. Switch setting is unreasonable. | 1. If hardware DI is used, ensure that DI Function 11 is configured in P03h group, and then check if wiring of DI terminal is normal; Error in the origin return operation was found, and operate this function correctly. If virtual DI is used, check if VDI handling process is proper. 2. Increase the origin search time P1349. 3. Increase return-to-zero high-speed 6099-01h. 4. Set hardware switch position reasonably. |
| A.426.0 Warning of power ON/OFF reset | 1. After the parameters are changed, only when power ON/OFF again, can parameters be valide. | 1. Power it ON/OFF again. |
| A.427.0 Warning of parameter storage exception | 1. Write data into EEPROM very frequently and abundantly. | 1. Reduce unnecessary parameters written into EEPROM. 2. Set 2005-0Bh of 0 and do not store parameters into EEPROM. |
| A.428.0 Error of frequency division output setting | 1. While using output function (P1315=0) of encoder frequency division, pulse number (P1316) setting of encoder frequency division doesn't meet the range requirement. | 1. Reset the pulse number (P1316) of encoder frequency division so as to satisfy the specified range. |
| A.713.2 Sampling error warning | 1. Contact the factory for technical support. | 1. Parameter identification can be performed repeatedly. 2. Contact technical support of the manufacturer. |
| A.713.3 Check the error warning of rising edge | 1. Contact the factory for technical support. | 1. Parameter identification can be performed repeatedly. 2. Contact technical support of the manufacturer. |
| A.713.4 Check overshoot error warning | 1. Contact the factory for technical support. | 1. Parameter identification can be performed repeatedly. 2. Contact technical support of the manufacturer. |
| A.713.5 Warning of Q-axis current steady state error test | 1. Contact the factory for technical support. | 1. Parameter identification can be performed repeatedly. 2. Contact technical support of the manufacturer. |
| A.713.6 Warning of D-axis current steady state error test | 1. Contact the factory for technical support. | 1. Parameter identification can be performed repeatedly. 2. Contact technical support of the manufacturer. |
| A.920.0 Encoder internal algorithm error | 1. The encoder zero search algorithm fails. 2. Error of frequency division counting algorithm of encoder. | 1. Power on/off the servo drive again. 2. Replace the servo motor. |
| A.921.0 Encoder battery voltage is too low | 1. Battery voltage of absolute encoder is lower than 3.0V. | 1. Replace it with the new matching battery. |
| A.D35.0 Error of 6098h parameter settings in return-to-zero mode | 1. In return-to-zero mode, input non-existent return-to-zero modes such as 15/16/31/32 in EtherCAT 6098h (or P0B.30 of PN) | 1. EtherCAT model is properly set to 6098h (PN model is properly set to P0B.30). |
| nr Servo not ready | 1. Control loop voltage is too low; 2. Bus voltage of power loop is too low; 3. Exception of encoder feedback; | 1. Confirm that AC power supply of the drive is normal per the specification. 2. Remove the motor structure being reversed, or replace the motor encoder. |

5.6 Resetting Methods

There are 3 ways as follows to reset faults and warnings of SV3 servo:

	Set parameter P20.03 = 1 to reset.

	Reset via DI input (Function 5, fault reset);

	Reset by setting the rising edge of Bit7 of control word 0x6040 through the upper computer;

Herein, for fault reset, the servo should be disabled first, and then the fault reset signal is sent; For warning reset, the fault reset signal can be directly sent. The premise for fault reset is that fault condition has been removed. After the warning condition is removed, the warning will automatically reset.

- Chapter 6 EtherCAT Communication

The application of Ethernet technology in computer networks to industrial automation constitutes industrial control Ethernet, generally known as industrial Ethernet or Ethernet fieldbus; Those servo drives that add Ethernet fieldbus are called Bus Servo, which is the principal development trend of servo drive. Compared with traditional bus servo and pulse servo, it has the following advantages:

	Industrial Ethernet has fast transmission speed, large packet capacity, and long transmission distance;

	Utilize general Ethernet components, cost-effective;

	Compatible with the standard Ethernet system, can access the standard Ethernet end;

	The network topology is diversified, the lines are simple, and it's easy to extend.

6.1 Overview

6.1.1 Overview of EtherCAT

EtherCAT is the abbreviation of Ethernet for Control Automation Technology, which is an Ethernet-based fieldbus technology proposed by German Beckhoff company in 2003. It is currently managed by ETG(EtherCAT Technology Group). EtherCAT is a high-speed and efficient Ethernet bus, and supports a variety of topologies such as line, tree, star, etc. Slave node uses a special control chip (ESC), and master station uses a standard Ethernet controller. Main features of EtherCAT are as follows:

	Wide applications, any control unit of commercial Ethernet controller can be used as EtherCAT master station.

	Fully compatible with standard Ethernet, both can coexist in the same system;

	Short delay, data transmission of single-axis slave station can't exceed 1us;

	The data refresh cycle is short, and it can reach the data refresh cycle less than 100us.

	Good synchronization, synchronization accuracy less than 1us;

	High efficiency, maximize the use of Ethernet broadband for user data transmission;

Currently, EtherCAT has entered several relevant international standards:

	Type12 in IEC61158;

	CPF12 in IEC61784;

	In IEC61800, EtherCAT supports CANopen DS402 and SERCOS.

	In ISO15745, EtherCAT supports DS301.

The topology of EtherCAT system supports a variety of topologies such as star, tree, and line. At present, mainstream servo drives may reserve 2 network interfaces which are used as IN/OUT port of signal. EtherCAT can select the physical medium of 100Base-TX standard Ethernet cable or optical cable. As for 100Base-TX cable, the distance between stations can reach 100m and the entire network can link up to 65,535 devices.

EtherCAT uses a specific Ethernet data frame type(0x88A4) definition to transfer EtherCAT data packets by Ethernet data frame. EtherCAT packet can also be transferred in UDP/IP protocol format. An EtherCAT packet can consist of multiple EtherCAT sub-messages. EtherCAT slave station doesn't process non-EtherCat data frames, and non-EtherCat data frame can be packaged in segments into EtherCAT data sub-message for transparent transmission in network segment, so EtherCAT slave system and standard Ethernet devices can coexist on the same system through network links and are independent of each other.

6.1.2 Host/Slave System Composition

EtherCAT system follows the principle of one master and multiple slaves. Number of slave stations that a master station can link depends on the processing capacity of the master station, communication cycle, transfer volume, etc., but Max. number of slave stations shall not exceed 65,535.

Master station works on the basis of ENI files which are generated by ESI files provided by our company through EtherCAT Configuration Tool or suppliers of master station in a special way.

Generated by EtherCAT Configuration Tool

EtherCAT Slave Information (ESI):

Files in .xml format provided by our company.

Records information inherent to slave station, including supplier information, product information, profile, data type, object dictionary, process data, synchronization method, SyncManager settings, etc.

EtherCAT Network Information (ENI):

Files generated by master station on basis of information of slave stations.

ENI contains information to identify slave station and initialize each slave station. Master station initializes the network and constructs the system based on the information recorded in ENI.

6.1.3 ESC Overview

ESC, the full name is EtherCAT Slave Controller, i.e., controller of EtherCAT slave station, is the key part of EtherCAT technology. It's the intermediate component of master-and-slave station communication. The following is the mainstream information on ESC in markets.

| Feature | ET1200 | ET1100 | IP Core | ESC20 |
| --- | --- | --- | --- | --- |
| Ports | 2~3 (eachEBUS/ MII,max.1xMII) | 2~4 (eachEBUS/MI) | 1~3 MII/1~3 RGMII/ 1~2 RMII | 2 MII |
| FMMUS | 3 | 8 | 0~8 | 4 |
| SyncManagers | 4 | 8 | 0~8 | 4 |
| RAM [Kbyte] | 1 | 8 | 0~60 | 4 |
| Distributed Clocks | 64bit | 64bit | 32/64bit | 32bit |
| Process Data Interfaces |  |  |  |  |
| Digital I/O | 16bit | 32bit | 8~32bit | 32bit |
| SPI Slave | Yes | Yes | Yes | Yes |
| 8/16 bit μController | - | Async/Sync | Async | Async |
| On-chip bus | - | - | Yes | - |

The internal architecture and external interfaces are shown as follows:

ESC processes EtherCAT data frames. ESC of each EtherCAT slave station reads and writes data frames according to its physical position on the loop. When a message passes through the slave station, the built-in processing unit of ESC extracts sub-message data sent by master station from messages and stores it in the internal storage area. Output data is written from internal storage area to the corresponding sub-message; Data extraction and insertion are completed by the hardware of data link layer.

The number of sending/receiving ports for each ESC is slightly different. Taking ET1100 for example, it has 4 PORTs, each PORT can send and receive Ethernet data frames which has the fixed sequence in ESC transmission. As there is the data processing unit between PORT0 and PORT3, it's better for data to enter ESC from PORT0. If ESC detects that a PORT has no external link, it automatically closes that PORT, and data is looped back and forwarded to the next PORT automatically. Therefore, the servo supports at least 2 ports.

ESC can use 2 physical layer interface modes: MII and EBUS. MII is the standard physical layer interface of Ethernet  that requires external physical layer chip. Transmission delay of one port is about 500us. EBUS is the data transmission standard defined by German Beckford Company using LVDS(Low Voltage Differential Signaling) standard, which can directly link to ESC chip without any extra physical layer chip. Transmission delay of one port is about 100ns. EBUS has a maximum transmission distance of 10m and is suitable for links between I/O devices or servo drives in close proximity.

6.1.4 Structure of EtherCAT Application Layer Protocol

Application Layer (AL) is the highest functional layer of EtherCAT protocol. It's directly oriented to control task, which provides the means for control program to access the network environment, and provides services for control program. EtherCAT protocol structure is as follows:

Physical layer

Data link layer

Application layer

File access

Transceiver

Transceiver

Ethernet physical layer interface

EtherCAT data processing unit (with auto-forwarding loop-back function)

Use PDO mapping or MDT/AT to transmit cyclic data

Physical data interface (microprocessor, synchronous serial/parallel interface, I/O)

Operate object dictionary

Parameter operations, process commands

Read/write registers

COE/SOE application

Application

APP

Process data

Mailbox data（Mailbox）

Registers

Physical layer control

Physical layer control

6.2 EtherCAT Specification

6.2.1 EtherCAT Frame Structure

As EtherCAT uses the standard IEEE 802.3 Ethernet frame,  standard network controller can be used, and no special hardware is required on master station. EtherCAT only extends IEEE 802.3 Ethernet specification and doesn't make any change to basic structure of Ethernet protocol.

EtherType of EtherCAT Header is 0x88A4, which distinguishes it from other Ethernet frames. Subsequently, EtherCAT can run in parallel with other Ethernet protocols.

EtherCAT doesn't require IP protocol, but can encapsulate it into IP/UDP. EtherCAT slave controllers process frames in a hardware method. Therefore, communication performance is independent of processor power.

An EtherCAT frame can be divided into EtherCAT frame header, followed by one or more EtherCAT datagrams. There must be at least one EtherCAT datagram in data frame.  Currently, ESC only processes EtherCAT frames with the type of 1 in EtherCAT header. ESC also supports IEEE802.1Q VLAN tags, although ESC doesn't evaluate the content of VLAN tags.

If EtherCAT frame size doesn't meet Min. size requirement of Ethernet frame(64bytes), padding bytes (typically padding 0) must be added. Size of EtherCAT frame is exactly the sum of all EtherCAT datagrams plus EtherCAT header(i.e., EtherCAT header+datagrams).

The following diagram shows how an Ethernet frame contains EtherCAT data:

Figure 6-1 How does an Ethernet frame contain EtherCAT data

Table 6-1 Description on EtherCAT Header

| Field | Data Type | Value/Description |
| --- | --- | --- |
| Length | 11bit | Length of the EtherCAT datagrams(excl.Fcs) |
| Reserved | 1bit | Reserved,0 |
| Type | 4bit | Protocol type.only EtherCAT commands(type=0x01) are supported by ESCs |

ESC doesn't care about the length of EtherCAT Header(i.e., length), ESC cares about the length of Datagram section.

Structure of EtherCAT frame:

Figure 6-2 Structure of EtherCAT frame

Note: When Ethernet frame is shorter than 64bytes, add 1 to 32bytes(Ethernet Header+Ethernet Data+FCS).

Table 6-2 EtherCAT Datagram

| Field | Data Type | Value/Description |
| --- | --- | --- |
| Cmd | Byte | EtherCAT Command Type; Instruction type, i.e., the way to find; |
| Idx | Byte | The index is a numeric identifier used by the master for identification of duplicates/lost datagrams. It shall not be changed by EtherCAT slaves; Index is a digital identifier used by master station to distinguish duplicate or lost datagrams; Slave station can't modify it. |
| Address | Byte[4] | Address (Auto Increment, Configured Station Address, or Logical Address); Address(auto-addressing, configured site addressing, logical addressing); |
| Len | 11bit | Length of the following data within this datagram; Length of data in Datagram; |
| R | 3bit | Reserved, 0; |
| C | 1bit | Circulating frame: 0: Frame is not circulating; 1: Frame has circulated once; |
| M | 1bit | More EtherCAT datagrams; 0: Last EtherCAT datagram; 1: More EtherCAT datagrams will follow, EtherCAT datagram will follow; |
| IRQ | WORD | EtherCAT Event Request registers of all slaves combined with a logical OR; All EtherCAT Event Request registers (0x210::0x211) of slave station perform logical or. |
| Data | Byte[n] | Read/Write Data; |
| WKC | WORD | Working Counter; Details are as follows: Command; Operate; Increment. Read command; Not successful; No change. Read successfully; +1. Write command; Not successful; No change. Write successfully; +1. Read/Write command; Not successful; No change. Read successfully; +1. Write successfully; +2. The read and write were both successful; +3. |

6.2.2 Addressing Mode

Two modes for EtherCAT device addressing are supported in a network segment: device addressing and logicaladdressing. EtherCAT provides 3 device addressing modes: auto increment addressing, configured station address, and broadcast (broadcast addressing). EtherCAT device can have up to 2 configured station addresses: one is assigned by the master (Configured Station Address) and the other is stored in SII EEPROM. It can be changed by the slave station application (Configured Station Alias address). EEPROM settings of Configured Station Alias address takes over only when  EEPROM is loaded for the first time after power-on/reset.

Table 6-3 Addressing methods

| Mode |  | Field | Data Type | Remarks |
| --- | --- | --- | --- | --- |
| Device addressing | Auto-increment addressing | Position | WORD | Each slave station will increase Position by 1, if position=0, then addressing is successful; |
|  |  | Offset | WORD | ESC logical register or memory address; |
|  | Configured station address | Position | WORD | If Address matches either Configured Station Address (Configured station address) or Configured Station Alias (Configured station alias)(if enabled), the slave station will be addressed. |
|  |  | Offset | WORD | ESC logical register or memory address; |
|  | Broadcast | Position | WORD | Each slave station increase 1 to Position, but not used for addressing; |
|  |  | Offset | WORD | ESC logical register or memory address; |
| Logicaladdressing |  | Address | DWORD | Logical Address (configured by FMMUs). If Address matches logical address configured by FMMU, addressing is successful. |

Table 6-4 Cmd details

| Addressing mode | Cmd | Abbreviation | Name | Remarks |
| --- | --- | --- | --- | --- |
| - | 00h | NOP | No Operation | No operation is executed. |
| Position Addressing | 01h | APRD | Auto Increment Read | Each slave station has an incremental increase of Address. When slave station receives a frame with Address value of 0, the slave station reads data in the specified memory unit and inserts EtherCAT datagram. Position of EtherCAT datagram will be increased by 1,. |
|  | 02h | APWR | Auto Increment Write | Each slave station has an incremental increase of Address. When slave station receives a frame with Address value of 0, slave station receives the data and writes it to the specified local storage unit. Position of EtherCAT datagram will be increased by 1. |
|  | 03h | APRW | Auto Increment Read Write | Each slave station has an incremental increase of Address. When slave station receives a frame with Address value of 0, the specified local storage unit exchanges data (read&write) with the data frame EtherCAT datagram. Position of EtherCAT datagram will be increased by 1. |
| Node Addressing | 04h | FPRD | Configured Address Read | When the address configured for slave station is the same as Address value of EtherCAT datagram, slave station reads data in the specified memory unit and inserts EtherCAT datagram. |
|  | 05h | FPWR | Configured Address Write | When the address configured for slave station is the same as Address value of EtherCAT datagram, slave station receives data and writes it into the specified local storage unit. |
|  | 06h | FPRW | Configured Address Read Write | When the address configured for slave station is the same as Address value of EtherCAT datagram, the specified local storage unit exchanges data (read&write) with the data frame EtherCAT datagram. |
| Broadcast | 07h | BRD | Broadcast Read | All slave stations read data in the specified memory unit, perform logical-or operations with data of EtherCAT datagram, and then store the data to EtherCAT datagram. Position of EtherCAT datagram will be increased by 1. |
|  | 08h | BWR | Broadcast Write | All slave stations store EtherCAT datagram in the specified storage unit. Position of EtherCAT datagram will be increased by 1. |
|  | 09h | BRW | Broadcast Read Write | All slave stations read data in the specified memory unit, perform logical-or operations with data of EtherCAT datagram, then inserts the data to EtherCAT datagram, and store the original EtherCAT datagram in the specified memory unit. Position of EtherCAT datagram will be increased by 1. |
| Logical Addressing | 0Ah | LRD | Logical Memory Read | If the received address matches one of the configured FMMU areas, slave station reads data in the specified memory unit and inserts EtherCAT datagram. |
|  | 0Bh | LWR | Logical Memory Write | If the received address matches one of the configured FMMU areas, slave station receives data and writes it in the specified local storage unit. |
|  | 0Ch | LRW | Logical Memory Read Write | If the received address matches one of the configured FMMU zones, the specified local storage unit performs data exchange (read&write) with the data frame EtherCAT datagram. |
| Position Addressing | 0Dh | ARMW | Auto Increment Read Multiple Write | If the received address is 0, slave station reads data in the specified memory unit and inserts EtherCAT datagram; Otherwise, slave station receives data and writes it into the specified local storage unit.Position of EtherCAT datagram will be increased by 1. |
| Node Addressing | 0Eh | FRMW | Configured Read Multiple Write | If received address is the same as configured address, slave station reads data in the specified memory unit and inserts EtherCAT datagram; Otherwise, slave station receives data and writes it into the specified local storage unit. |
| - | 0Fh~FFh | - | reserved | - |

6.2.3 Frame Processing Sequence

Frame processing sequence by EtherCAT slave station controller depends on logical port No.

Table 6-5 Frame processing sequence

| Port No. | Frame processing sequence |
| --- | --- |
| 1 | 0→EtherCAT Processing Unit→0 |
| 2 | 0→EtherCAT Processing Unit→1 / 1→0 |
| 3 | 0→EtherCAT Processing Unit→1 / 1→2 / 2→0 Or, 0→EtherCAT Processing Unit→3 / 3→1 / 1→0 |
| 4 | 0→EtherCAT Processing Unit→3 / 3→1 / 1→2 / 2→0 |

The direction through ESC that includes EtherCAT processing units is called the "processing" direction, while other directions that don't pass through EtherCAT processing units are called the "forwarding" direction. The behavior of an unimplemented port is similar to that of a closed port; That data frame will be forwarded to the next port.

Figure 6-3 Frame processing sequence

6.2.4 ESC Register

| Address | Length (byte) | Description | Status |
| --- | --- | --- | --- |
| ESC information |  |  |  |
| 0x0000 | 1 | Type | Y |
| 0x0001 | 1 | Revision | Y |
| 0x0002:0x0003 | 2 | Build | Y |
| 0x0004 | 1 | FMMUs Supported | Y |
| 0x0005 | 1 | SyncManagers supported | Y |
| 0x0006 | 1 | RAM Size | Y |
| 0x0007 | 1 | Port Descriptor | Y |
| 0x0008:0x0009 | 2 | ESC Features supported | Y |
| Station address |  |  |  |
| 0x0010:0x0011 | 2 | Configured Station Address | Y |
| 0x0012:0x0013 | 2 | Configured Station Alias | Y |
| write-protect |  |  |  |
| 0x0020 | 1 | Write Register Enable | Y |
| 0x0021 | 1 | Write Register Protection | Y |
| 0x0030 | 1 | ESC Write Enable | Y |
| 0x0031 | 1 | ESC Write Protection | Y |
| Data link layer |  |  |  |
| 0x0040 | 1 | ESC Reset ECAT | Y |
| 0x0041 | 1 | ESC Reset PDI | N |
| 0x0100:0x0101 | 2 | ESC DL Control | Y |
| 0x0102:0x0103 | 2 | Extended ESC DL Control | Y |
| 0x0108:0x0109 | 2 | Physical Read/Write Offset | Y |
| 0x0110:0x0111 | 2 | ESC DL Status | Y |
| Application layer |  |  |  |
| 0x0120 | 5 bits [4:0] | AL Control | Y |
| 0x0120:0x0121 | 2 | AL Control | Y |
| 0x0130 | 5 bits [4:0] | AL Status | Y |
| 0x0130:0x0131 | 2 | AL Status | Y |
| 0x0134:0x0135 | 2 | AL Status Code | Y |
| 0x0138 | 1 | RUN LED Override | N |
| 0x0139 | 1 | ERR LED Override | N |
| PDI (Process Data Interface) |  |  |  |
| 0x0140 | 1 | PDI Control | Y |
| 0x0141 | 1 | ESC Configuration | Y |
| 0x014E:0x014F | 2 | PDI Information | N |
| 0x0150 | 1 | PDI Configuration | Y |
| 0x0151 | 1 | DC Sync/Latch Configuration | Y |
| 0x0152:0x0153 | 2 | Extended PDI Configuration | Y |
| Interrupt |  |  |  |
| 0x0200:0x0201 | 2 | ECAT Event Mask | Y |
| 0x0204:0x0207 | 4 | PDI AL Event Mask | Y |
| 0x0210:0x0211 | 2 | ECAT Event Request | Y |
| 0x0220:0x0223 | 4 | AL Event Request | Y |
| Error counter |  |  |  |
| 0x0300:0x0307 | 4×2 | Rx Error Counter[3:0] | Y |
| 0x0308:0x030B | 4×1 | Forwarded Rx Error counter [3:0] | Y |
| 0x030C | 1 | ECAT Processing Unit Error Counter | Y |
| 0x030D | 1 | PDI Error Counter | Y |
| 0x030E | 1 | PDI Error Code | N |
| 0x0310:0x0313 | 4×1 | Lost Link Counter[3:0] | Y |
| Watchdog |  |  |  |
| 0x0400:0x0401 | 2 | Watchdog Divider | Y |
| 0x0410:0x0411 | 2 | Watchdog Time PDI | Y |
| 0x0420:0x0421 | 2 | Watchdog Time Process Data | Y |
| 0x0440:0x0441 | 2 | Watchdog Time Process data | Y |
| 0x0442 | 1 | Watchdog Counter Process Data | Y |
| 0x0443 | 1 | Watchdog Counter PDI | Y |
| EEPROM interface |  |  |  |
| 0x0500:0x050F | 16 | SII EEPROM Interface | Y |
| MII management interface |  |  |  |
| 0x0510:0x0515 | 6 | MII Management Interface | Y |
| 0x0516:0x0517 | 2 | MII Management Access State | N |
| 0x0518:0x051B | 4 | PHY Port Status[3:0] | N |
| 0x0600:0x06FC | 16×13 | FMMU[15:0] | 8 |
| 0x0800:0x087F | 16×8 | SyncManager[15:0] | 8 |
| Distributed clock,DC |  |  |  |
| 0x0900:0x090F | 4×4 | DC – Receive Times[3:0] | Y |
| 0x0918:0x091F | 8 | DC – Receive Time EPU | S/I |
| 0x0920:0x0935 | 24 | DC – Time Loop Control Unit | S/I |
| 0x0910:0x0917 | 8 | DC – System Time | S/I |
| 0x0936 | 1 | DC – Receive Time Latch mode | N |
| 0x0980 | 1 | DC – Cyclic Unit Control | S |
| 0x0981 | 1 | DC – Activation | S |
| 0x0982:0x0983 | 2 | DC – Pulse length of SyncSignals | S |
| 0x0984 | 1 | DC – Activation Status | N |
| 0x098E:0x09A7 | 26 | DC – SYNC Out Unit | S |
| 0x09A8 | 1 | DC – Latch0 Control | I |
| 0x09A9 | 1 | DC – Latch1 Control | I |
| 0x09AE | 1 | DC – Latch0 Status | I |
| 0x09B0:0x09B7 | 8 | DC – Latch0 Positive Edge | I |
| 0x09B8:0x09BF | 8 | DC – Latch0 Negative Edge | I |
| 0x09C0:0x09C7 | 8 | DC – Latch1 Positive Edge | I |
| 0x09C7:0x09CF | 8 | DC – Latch1 Negative Edge | I |
| 0x09F0:0x09F3 0x09F8:0x09FF | 12 | DC – SyncManager Event Times | S/I |
| ESC specific |  |  |  |
| 0x0E00:0x0E03 | 4 | Power-On Values (Bits) | 16bits |
| 0x0E00:0x0E07 | 8 | Product ID | N |
| 0x0E08:0x0E0F | 8 | Vendor ID | N |
| 0x0E10 | 1 | ESC Health Status | N |
| Digital I/O |  |  |  |
| 0x0F00:0x0F03 | 4 | Digital I/O Output Data | Y |
| 0x0F10:0x0F17 | 8 | General Purpose Outputs [Byte] | 2bytes |
| 0x0F18:0x0F1F | 8 | General Purpose Inputs [Byte] | 2bytes |
| User RAM |  |  |  |
| 0x0F80:0x0FFF | 128 | User RAM | Y |
| Process data RAM |  |  |  |
| 0x1000:0x1003 | 4 | Digital I/O Input Data | IO |
| 0x1000:0x1FFF |  | Process Data RAM [Kbyte] | 4KB |

Note) : Y	   Support

N	   Not support

S	   If 0x0140.10=1, it is valid

I	   If 0x0140.11=1, it is valid

S/I	   If 0x0140.10=1 and/or 0x0140.11=1,, it is valid

6.2.5  ESM(EtherCAT State Machine)

EtherCAT State Machine (ESM) coordinates the state relationship of master/slave stations during application initialization and runtime.

EtherCAT bus servo supports 4 states:

(1)Init: Initialization

(2)Pre-Operational: Pre-operational (PreOP)

(3)Safe-Operational: Safe operation, or SafeOP

(4)Operational: Operation, or OP

Diagram of each state transition:

Switching of EtherCAT state machine must strictly follow the arrow direction in the above figure. When transitioning from initialization state to running state, it must be done in the order of "INIT → PREOP → SAFEOP → OP" and can't be skipped. When returning from the running state, it's possible to bypass the path. All state changes are initiated by the master which sends a state control command to the slave and requests the new state. Slave station responds to this command, executes the requested state transition, and writes the result into the state indication register of slave station. If the requested state transition fails, the slave will give an error flag.

Table 6-6 Executable operations in each status

| Status and transition | Action |
| --- | --- |
| Init | Master station can only read & write ESC registers, and the application layer has no communication |
| Init→PreOP | Master station configures address register (ESC reg: 0x0010~0x0011) of slave station. If email communication is supported, configure mailbox channel parameters(SM channel). If distributed clocks are supported, configure DC-related registers. Control register (ESC reg: 0x0120~0x0121) for master station write-status, requesting PreOP status |
| PreOP | Mailbox communication is activated. Process data communication is disabled |
| PreOP→SafeOP | The master uses mailbox initialization process data mapping; SM channel for process data communication configured by the master; The master configures FMMU. Control register (ESC reg: 0x0120~0x0121) for master station write-status, requesting SafeOP status |
| SafeOP | The application layer supports mailbox data communication. Process data can be transmitted between master station and slave station, but the slave doesn't update the data sent by the master, i.e., the slave doesn't respond to any command of the master . |
| SafeOP→OP | Master station sends valid output data; Control register (ESC reg: 0x0120~0x0121) for master station write-status, requesting OP status |
| OP | The application layer supports mailbox data communication. Process data communication (slave to master) is valid; Process data communication (master to slave) is valid |

Table 6-7 Relationship between PDS status and ESM status

| ESM status PDS status | Init | Preop | Safeop | OP |
| --- | --- | --- | --- | --- |
| Not ready to switch on | Yes | No | No | No |
| Switch on disabled | Yes | Yes | Yes | Yes |
| Ready to switch on*1) | No | Yes | Yes | Yes |
| Switch on*1) | No | Yes | Yes | Yes |
| Operation enabled *2) *5) | No | Yes*4) | Yes*4) | Yes |
| Fault reaction active | Yes | Yes | Yes | Yes |
| Fault*3) | Yes | Yes | Yes | Yes |

*1) : If ESM status is to receive the migration command from PreOP,SafeOP,OP to Init, PDS status is migrated to Switch on disabled.

*2) : PDS status means that in Operation enabled status, if ESM status receives ESM migration command, PDS status is automatically enabled and PDS status is migrated to Switch on disabled.

*3) : Keep ESM state when PDS state is migrated to Fault.

*4) : ESM status is to request PDS status to Operation enabled in OP status.

*5) : Because master station has requirements on ESM, it takes time to complete the state migration. Please pay attention to the timeout setting on master station.

6.2.6  SII(Slave Information Interface) EEPROM

6.2.6.1 EEPROM Data Layout

Table 6-8 EEPROM data layout

| Word Address | +0h | +1h | +2h |  | +3h | +4h | +5h |  | +6h | +7h |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0000h | EtherCAT Slave Controller Configuration Area |  |  |  |  |  |  |  |  |  |
| 0008h | VendorId |  | ProductCode |  |  | RevisionNo |  |  | SerialNo |  |
| 0010h | Hardware Delays |  |  |  |  | Bootstrap Mailbox Config |  |  |  |  |
| 0018h | Mailbox Sync Man Config |  |  |  |  |  |  |  |  |  |
| 0020h … 0030h | Reserved |  |  |  |  |  |  |  |  |  |
| 0038h |  |  |  | Size |  |  |  | Version |  |  |
| 0040h … | Additional Information (Subdivided in Categories) |  |  |  |  |  |  |  |  |  |
|  | Category Strings |  |  |  |  |  |  |  |  |  |
|  | Category Generals |  |  |  |  |  |  |  |  |  |
|  | Category FMMU |  |  |  |  |  |  |  |  |  |
|  | Category SyncManager |  |  |  |  |  |  |  |  |  |
|  | Category Tx- / RxPDO for each PDO |  |  |  |  |  |  |  |  |  |

6.2.6.2 SII Area (0000h to 003Fh)

ESC configuration data is stored in ESC-EEPROM character addresses 0x0000 to 0x0007. During the power-on and initialization of ESC, ESC automatically reads EEPROM data and writes the content of that storage area into the corresponding register of ESC.

Note: Please do not change the content of EEPROM storage area at will.

| SII EEPROM Word Address | Name | Remarks | ESC Register Word Address | Initial value |
| --- | --- | --- | --- | --- |
| 0x0000h | PDI Control/ ESC Configuration | Initial values of PDI control register and ESC configuration register | 0140h 0141h | 0C08h |
| 0x0001h | PDI Configuration | Initial value of PDI configuration register | 0150h 0151h | 6608h |
| 0x0002h | Pulse Length of SYNC Signals | Initial value of pulse length of synchronization signal | 0982h 0983h | 01F4h |
| 0x0003h | Extended PDI Configuration | Extend the initial value of PDI configuration register | 0152h 0153h | 0000h |
| 0x0004h | Configured Station Alias | Initial value of station alias configuration register | 0012h 0013h | 0000h |
| 0x0005h | Reserved | Reserved, it should be 0 | - | 0000h |
| 0x0006h | Reserved | Reserved, it should be 0 | - | 0000h |
| 0x0007h | CheckSum | Character Address 0 to 6h checksum | - | 00D2h |
| 0x0008h | Vendor ID | Manufacturer ID | - | 07FBh |
| 0x0009h |  |  |  | 0000h |
| 0x000Ah | Product Code | Product code | - | - |
| 0x000Bh |  |  |  |  |
| 0x000Ch | Revision Number | Revision No. | - | - |
| 0x000Dh |  |  |  |  |
| 0x000Eh | Serial Number | Serial No. | - | - |
| 0x000Fh |  |  |  |  |
| 0x0010h | Execution Delay | Execution delay | - | 0000h |
| 0x0011h | Port0 Delay | Port 0 delay | - | 0000h |
| 0x0012h | Port1 Delay | Port 1 delay | - | 0000h |
| 0x0013h | Reserved | Reserved | - | 0000h |
| 0x0014h | Bootstrap Receive Mailbox Offset | Bootstrap status receiving Mailbox offset (SM0, MbxOut, master → slave) | - | 0000h |
| 0x0015h | Bootstrap Receive Mailbox Size | Bootstrap status receiving Mailbox size (SM0, MbxOut, master → slave) | - | 0000h |
| 0x0016h | Bootstrap Send Mailbox Offset | Bootstrap Status sending Mailbox offset (SM1, MbxIn, slave → master) | - | 0000h |
| 0x0017h | Bootstrap Send Mailbox Size | Bootstrap Status sending Mailbox size (SM1, MbxIn, slave → master) | - | 0000h |
| 0x0018h | Standard Receive Mailbox Offset | Standard status receiving Mailbox offset (SM0, MbxOut, master → slave) | - | 1000h |
| 0x0019h | Standard Receive Mailbox Size | Standard status receving Mailbox size (SM0, MbxOut, master → slave) | - | 0080h |
| 0x001Ah | Standard Send Mailbox Offset | Standard status sending Mailbox offset (SM1, MbxIn, slave → master) | - | 1400h |
| 0x001Bh | Standard Send Mailbox Size | Standard status sending Mailbox size (SM1, MbxIn, slave → master) | - | 0080h |
| 0x001Ch | Mailbox Protocol | Supported Mailbox protocol | - | 0004h |
| 0x001Dh … 0x003Dh | Reserved | Reserved | - | - |
| 0x003Eh | Size | EEPROM size | - | 000Fh |
| 0x003Fh | Version | edition | - | 0001h |
| 0040h … | Types of data |  |  |  |

6.2.7 Synchronization Mode

The servo supports DC_sync0 synchronization mode.

EtherCAT's distributed clock (DC) uses the DC clock of the first slave station as reference clock, and the master  distributes the reference clock to all slave stations. When EtherCAT master station periodically sends an ARMW command to read the bus time stored in ESC register of the clock master, and writes this value into DC, the corresponding register of slave station to update local time. In order to ensure the accuracy of request, EtherCAT frame delay between slave stations must be additionally compensated. For each slave station, the time from sending to receiving a frame will be measured. Upon the bus topology, master station calculates the delay between slave stations and writes the corresponding delay compensation value into the register 0x928 in ESC.

Figure 6-4 Sequence diagram of DC mode communication

6.2.8 MailBox Mailbox Structure

Mailbox mailbox frame structure is shown as below:

For details, see ETG Specification (ETG1000-4).

秐

n th秐

Figure 6-5 Mailbox frame structure

Table 6-9 EtherCAT mailbox codes

| Frame part | Data area | Data type | Value/Description |
| --- | --- | --- | --- |
| Mailbox Header | Length | WORD | Length of mailbox service data |
|  | Address | WORD | If the master station is a client, it's the source station address; If the slave station is a client, it's the destination station address. |
|  | Channel | Unsigned6 | 0x00(Reserved) |
|  | Priority | Unsigned2 | 0x00: The lowest priority … 0x03: The highest priority |
|  | Type | Unsigned4 | 0x00：Mailbox Error 0x01：(Reserved) 0x02：EOE(Not supported) 0x03：COE 0x04：FOE(Not supported) 0x05：SOE(Not supported) 0x06~0x0E：(Reserved) 0x0F：Manufacturer Specific |
|  | Cnt | Unsigned3 | Mailbox service counter (0 is reserved, 1 is the starting value, and the value after 7 is 1). Slave station increments Cnt value for each new mailbox service, and master station should check that value to prevent mailbox service loss; Slave station should also check that value to find duplicate written services, while slave station should not check the sequence of Cnt values; Cnt values of the master and the slave are independent. |
|  | Reserved | Unsigned1 | 0x00 |
| Service Data | Service Data | OctetString[Length] | Mailbox service data |

6.2.8.1  Mailbox Error

The service data responded by Mailbox Error is shown as below:

Table 6-10 Service data responded by Mailbox Error

| Frame part | Data area | Data type | Value/Description |
| --- | --- | --- | --- |
| Mailbox Header |  | 6byte |  |
| Service Data | Type | Unsigned16 | 0x01：MBXSERVICE_MBXERRORCMD Mail error command. |
|  | Detail | Unsigned16 | 0x01h：MBXERR_SYNTAX (Not Supported) Syntax error of 6byte mailbox header; 0x02h：MBXERR_UNSUPPORTEDPROTOCOL Mailbox protocol is not supported. 0x03h：MBXERR_INVALIDCHANNEL (Not Supported) Channel field contains the error value; 0x04h：MBXERR_SERVICENOTSUPPORTED Services in the mailbox protocol are not supported. 0x05h：MBXERR_INVALIDHEADER Mailbox protocol Header error (excluding 6 bytes of Mailbox Header); 0x06h：MBXERR_SIZETOOSHORT Length of received mailbox data is too short; 0x07h：MBXERR_NOMOREMEMORY Unable to provide sufficient memory for mailbox service due to resource constraints; 0x08h：MBXERR_INVALIDSIZE Data length is inconsistent; 0x09h：MBXERR_SERVICEINWORK (Not Supported) Mailbox service in process; |

Note: The above table is the abstract description on Mailbox Error service. For details, see ETG1000-4.

When Mailbox Error occurs, Mailbox Header.Type = 0x00

6.2.8.2  SDO(Service Data Object)

SV3 servo supports Service Data Objects (SDO) :

Note)	• When PDO modifies data, do not refresh the data by SDO

• SDO response may take some time.

For details, see ETG specifications (ETG1000-5 and ETG1000-6).

秐

n th秐

Figure 6-6 Mailbox/SDO frame structure

Table 6-11 COE mailbox codes

| Frame part | Data area | Data type | Value/Description |
| --- | --- | --- | --- |
| Mailbox Header | Length | WORD | Length of mailbox service data |
|  | Address | WORD | If the master station is a client, it's the source station address; If the slave station is a client, it's the destination station address. |
|  | Channel | Unsigned6 | 0x00 |
|  | Priority | Unsigned2 | Priority 0x00: The lowest priority … 0x03: The highest priority |
|  | Type | Unsigned4 | 0x03：COE |
|  | Cnt | Unsigned3 | Mailbox service counter (0 is reserved, 1 is the starting value, and the value after 7 is 1) |
|  | Reserved | Unsigned1 | 0x00 |
| COE Header | Number | Unsigned9 | Depend on COE service |
|  | Reserved | Unsigned3 | 0x00 |
|  | Service | Unsigned4 | 0x01：Emergency 0x02：SDO Request 0x03：SDO Response 0x04：TxPDO (Not Supported) 0x05：RxPDO (Not Supported) 0x06：TxPDO remote request (Not Supported) 0x07：RxPDO remote request (Not Supported) 0x08：SDO information |
| Cmd Specific | Cmd Specific |  |  |

Note: The above table is the abstract description on COE services. For details, see ETG1000-6.

Supported services:

SDO Download Expedited

SDO Download Normal

Download SDO Segment

SDO Upload Expedited

SDO Upload Normal

Upload SDO Segment

Abort SDO Transfer

Abort Message

When SDO data exchange processing (Read or Write) fails,  Abort Message is returned, which is specified by Abort Code as the error message indicating the reason for SDO termination.

Table 6-12 Abort Message

| Value | Meaning |  |
| --- | --- | --- |
| 0x05030000 | Toggle bit not changed | Toggle bit no change |
| 0x05040000(Not Supported) | SDO protocol timeout | SDO protocol timeout |
| 0x05040001 | Client/Server command specifier not valid or unknown | The client/server command qualifier is invalid or unknown |
| 0x05040005 | Out of memory | Memory overflow |
| 0x06010000 | Unsupported access to an object | Unsupported object access |
| 0x06010001 | Attempt to read to a write only object | Attempt to read a write-only object |
| 0x06010002 | Attempt to write to a read only object | Attempt to write to a read-only object |
| 0x06010003 | Entry can not be written because Subindex0 is not 0 | Unable to write into sub-index because Subindex 0 is not 0 |
| 0x06010004 (Not Supported) | The object can not be accessed via complete access | The specified object can't be accessed in the full access method |
| 0x06020000 | Object not existing | That object does not exist in the object directory |
| 0x06040041 (Not Supported) | Object can not be mapped to PDO | That object cannot be mapped to PDO |
| 0x06040042 (Not Supported) | The number and length of the objects to be mapped would exceed the PDO length | The number and length of mapped objects will exceed PDO length |
| 0x06040043 (Not Supported) | General parameter incompatibility reason | General parameters are not compatible |
| 0x06040047 (Not Supported) | General internal incompatibility in the device | The device has general internal incompatibility |
| 0x06060000 (Not Supported) | Access failed due to a hardware error | Access failed due to a hardware error |
| 0x06070010 | Data type does not match,length of service parameter does not match | The data type is inconsistent, and the length of service parameter are inconsistent |
| 0x06070012 (Not Supported) | Data type does not match,length of service parameter too high | The data type is inconsistent, and the length of service parameter is too long |
| 0x06070013 (Not Supported) | Data type does not match,length of service parameter too low | The data type is inconsistent, and the length of service parameter is too short |
| 0x06090011 | Subindex does not exist | Subindex does not exist |
| 0x06090030 | Value range of parameter exceeded (only for write access) | Parameter value out of range (for write access only) |
| 0x06090031 | Value of parameter written too great | Value of the parameter written is too large |
| 0x06090032 (Not Supported) | Value of parameter written too small | Value of the parameter written is too small |
| 0x06090036 (Not Supported) | Maximum value is less than minimum value | The maximum is less than the minimum |
| 0x08000000 (Not Supported) | General error | General alarm |
| 0x08000020 | Data cannot be transferred or stored the application | Data can't be transferred or stored to the application layer |
| 0x08000021 | Data cannot be transferred or stored to the application because of local control | Data can't be transferred/stored to the application layer due to local control |
| 0x08000022 | Data cannot be transferred or stored to the application because of the present device state | Due to current device state, data can't be transferred/stored to the application layer |
| 0x08000023 | Object dictionary dynamic generation fails or no object dictionary is present | Object dictionary fails to be dynamically generated or does not exist |

6.2.8.3 SDO Information

Table 6-13 COE-SDO Information Service data

| Frame part | Data area | Data type | Value/Description |
| --- | --- | --- | --- |
| Mailbox Header | Length | WORD | n>0x06: Length of mailbox service data |
|  | Address | WORD | If the master station is a client, it's the source station address; If the slave station is a client, it's the destination station address. |
|  | Channel | Unsigned6 | 0x00 |
|  | Priority | Unsigned2 | priority 0x00: The lowest priority … 0x03: The highest priority |
|  | Type | Unsigned4 | 0x03：COE |
|  | Cnt | Unsigned3 | Mailbox service counter (0 is reserved, 1 is the starting value, and the value after 7 is 1) |
|  | Reserved | Unsigned1 | 0x00 |
| COE Header | Number | Unsigned9 | 0x00 |
|  | Reserved | Unsigned3 | 0x00 |
|  | Service | Unsigned4 | 0x08：SDO information |
| SDO Info Header | Opcode | Unsigned7 | 0x01：Get OD List request 0x02：Get OD List response(Not Supported) 0x03：Get Object Description request 0x04：Get Object Description response(Not Supported) 0x05：Get Entry Description request 0x06：Get Entry Description response(Not Supported) 0x07：SDO Info Error(Not Supported) |
|  | Incomplete | Unsigned1 | Whether it is the last SDO Information segment(Not Supported) |
|  | Reserved | Unsigned8 | 0x00 |
|  | Fragments Left | WORD | Number of subsequent segments |
| SDO Info Service Data | Data |  | SDO Information service data |

Note: The above table is the abstract description on SDO Information service, see ETG1000-6 for details

The following services are supported:

Get OD List

OD List Segment

Get Object Description

Get Entry Description

Entry Description Segment

Emergency

Emergency Message

When servo operation fault occurs, the servo actively sends an emergency frame to notify operation controller of the servo fault.

Emergency Message can occur only in Non-Init state.

When Emergency Message occurs, object 0x603F is set of the corresponding fault code.

Table 6–14  COE-SDO Emergency service data

| Frame part | Data area | Data type | Value/Description |
| --- | --- | --- | --- |
| Mailbox Header |  | 6byte |  |
| COE Header | Number | Unsigned9 | 0x00 |
|  | Reserved | Unsigned3 | 0x00 |
|  | Service | Unsigned4 | 0x01：SDO emergency |
| Emergency | Error Code | WORD | Fault code |
|  | Error Register | Byte | Error register |
|  | Data | Byte[5] | Diagnostic data |
|  | Reserved |  |  |

Note: The above table is the abstract description on SDO Information service, see ETG1000-6 for details

Table 6-15 Diagnostic Data[0]

| Data [0] | Data [1~4] | Meaning |
| --- | --- | --- |
| 0x00+channel*4 | Sync Manager Address Error(address is odd) | Address of synchronization manager channel is odd |
| 0x01+channel*4 | Sync Manager Address Error(address invalid) | Address of synchronization manager channel is invalid |
| 0x02+channel*4 | Sync Manager Length Error | Length of synchronization manager channel is invalid |
| 0x03+channel*4 | Sync Manager Setting Error | Setting of synchronization manager channel is invalid |

Table 6-16 Sync Manager address error: Diagnostic Data [1-4]

| Data [1~4] | Data type | Value/Description |
| --- | --- | --- |
| Minimum Address | WORD | Min. value of the physical start address of synchronization manager channel |
| Maximum Address | WORD | Max.value of the physical start address of synchronization manager channel |

Table 6-17 Error of Sync Manager Length: Diagnostic Data [1~4]

| Data [1~4] | Data type | Value/Description |
| --- | --- | --- |
| Minimum Length | WORD | Min. value for the length of sync manager channel |
| Maximum Length | WORD | Max. value for the length of sync manager channel |

Table 6-18 Error of Sync Manager Settings: Diagnostic  Data[1-4]

| Data [1~4] | Data type | Value/Description |
| --- | --- | --- |
| 0x02 + channel *4 | WORD | 0x02 + channel *4 |
| 0x0001 | WORD | Sync Manager setting enable value |

2) Servo operation error

Error code is the same as Object 0x603F.

SDO Emergency is used as follows:

Error code: the same as Object 0x603F

Error register: the corresponding error register

Table 6-19 Diagnostic Data[0~4]

| Data | Numerical value | Data type | Value/Description |
| --- | --- | --- | --- |
| [0] | (err_code-0xFF00)&0xFF | Byte | That's err in the table |
| [1~2] | error data[0]+0x320 | WORD | Panel display data |
| [3~4] | 0 | WORD | Forced to be 0 |

6.2.8.4 Processing Capability

The servo has the cache for the received mailbox data. Max. cache number is 4 pieces of mailbox data. If master station continuously sends over 4 pieces of mailbox data, the subsequent data 3will not be received temporarily. Only when the cache is free, can it continue to receive subsequent mailbox data.

Therefore, in the case of not processing mailbox feedback data, it is recommended that number of consecutive mailbox messages sent by master station should not exceed 4.

6.2.9  PDO(Process Data Object)

EtherCAT bus servo supports Process Data Object (PDO) and can be configured online. Real-time data transmission based on EtherCAT is exchanged by PDO. PDO has RxPDO for transmitting data from master station to slave station and has TxPDO for transmitting data from slave station to master station.

| RxPDO | ≤68byte: Number of allocated objects is 1; Number of mapped application objects ≤20 |
| --- | --- |
| TxPDO | ≤68byte: Number of allocated objects is 1; Number of mapped application objects ≤20 |

SV3 servo supports PDO online dynamic mapping.

Dynamic mapping can be performed in 2 steps: allocating objects and mapping objects.

6.2.9.1 PDO allocates objects

SV3 servo must allocate objects to SyncManager PDO, 0x1C12 used for RxPDO(SyncManager2) and 0x1C13 used for TxPDO(SyncManager2).

Table 6-20 RxPDO allocation objects

| Index | Sub | Default Value | Remarks |
| --- | --- | --- | --- |
| 1C12h | 00h | 01h | Only one object can be configured |
|  | 01h | 1600h | 1600h/1601h/1602h/1603h one out of four 1600h/1601h/1602h/1603h are mutually exclusive |

Table 6-21 TxPDO allocation objects

| Index | Sub | Default Value | Remarks |
| --- | --- | --- | --- |
| 1C13h | 00h | 01h | Only one object can be configured |
|  | 01h | 1A00h | 1A00h/1A01h/1A02h/1A03h one out of four 1A00h/1A01h/1A02h/1A03h are mutually exclusive |

Configuration case: < Take 1C12h configured of 1603h for example>

1) Switch ESM status into PreOP;

Activate mailbox communication and configure 1C12h by SDO.

2) Set 1C12h-00h of 0 by SDO;

Value of 1C12h-01h can be changed only after 1C12h-00h is set of 0.

3) Set 1C12h-01h of 1603h by SDO;

Set the specific allocation object

4) Set 1C12h-00h of 1 by SDO;

Activate the setting of 1C12h.

5) Switch ESM status into SafeOP;

Activate TxPDO.

6) Switch ESM status into OP.

Activate RxPDO.

6.2.9.2 PDO Mapping Objects

The mapping objects used for RxPDO can be 1600h/1601h/1602h/1603h.

The mapping objects used for TxPDO can be 1A00h/1A01h/1A02h/1A03h.

Table 6-22 Mapping object 1600h

| Index | Sub | Default Value | Remarks |
| --- | --- | --- | --- |
| 1600h | 00h | 07h | Max. 20 |
|  | 01h | 60400010h | 1st receive PDO mapped |
|  | 02h | 607A0020h | 2nd receive PDO mapped |
|  | 03h | 60FF0020h | 3rd receive PDO mapped |
|  | 04h | 60710010h | 4th receive PDO mapped |
|  | 05h | 60600008h | 5th receive PDO mapped |
|  | 06h | 5FFE0008h | 6th receive PDO mapped |
|  | 07h | 60B80010h | 7th receive PDO mapped |
|  | 08h | 00000000h | 8th receive PDO mapped |
|  | … | … | … |
|  | 14h | 00000000h | 20th receive PDO mapped |

Other, omit.

Configuration case: < Take 1600h-08h configured of 606Eh for example.

1) Switch ESM status into PreOP;

Activate mailbox communication and configure 1600h by SDO.

2) Set 1600h-00h of 0 by SDO;

It must set 1600h-00h of 0 first, then value of 1600h-08h can be changed.

3) Set 1600h-08h of 606E0010h by SDO;

Set the specific mapping object

4) Set 1600h-00h of 8 by SDO;

Activate 1600h setting.

5) Switch ESM status into SafeOP;

Activate TxPDO.

6) Switch ESM status into OP.

Activate RxPDO.

- Chapter 7 Object Dictionary

7.1 Allocation List of Object Group 1000h

Table 7-1 List of 1000h object group

| Index | Subindex | Name | Accessibility | PDO mapping | Data type | Unit | Data range | Factory setting |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1000 | 0 | Drive type | RO | NO | UINT32 | - | - | 0x00020192 |
| 1008 | 0 | Drive name | RO | NO | - | - | - | SV3H-ECAT |
| 1009 | 0 | Hardware version | RO | NO | - | - | - | Depends on the software version |
| 100A | 0 | Software version | RO | NO | - | - | - | Depends on the hardware version |
| 1018 | ID object |  |  |  |  |  |  |  |
|  | 0 | Max. subindex number included by ID object | RO | NO | UINT8 | - | - | 04 hex |
|  | 1 | Supplier ID | RO | NO | UINT32 | - | - | 0010 0000 hex |
|  | 2 | Product code | RO | NO | UINT32 | - | - | 0x000C0108 |
|  | 3 | Revision No. | RO | NO | UINT32 | - | - | 0x00010001 |
| 1C00 | Manufacturer's software version |  |  |  |  |  |  |  |
|  | 0 | Max. subindex number of synchronous management communication type | RO | NO | UINT8 | - | - | 04 hex |
|  | 1 | SM0 communication type | RO | NO | UINT8 | - | - | 01hex |
|  | 2 | SM1 communication type | RO | NO | UINT8 | - | - | 02hex |
|  | 3 | SM2 communication type | RO | NO | UINT8 | - | - | 03hex |
|  | 4 | SM3 communication type | RO | NO | UINT8 | - | - | 04hex |
| 1600 | RxPDO1 Mapping object 1st |  |  |  |  |  |  |  |
|  | 0 | RxPDO1 Number of supported mapping objects | RW | NO | UINT8 | - | 0~10 | 3 |
|  | 1 | The first mapping object | RW | NO | UINT32 | - | 0~4294967295 | 6040 0010 |
|  | 2 | The second mapping object | RW | NO | UINT32 | - | 0~4294967295 | 607A 0020 |
|  | 3 | The third mapping object | RW | NO | UINT32 | - | 0~4294967295 | 60B8 0010 |
|  | 4 | The fourth mapping object | RW | NO | UINT32 | - | 0~4294967295 | - |
|  | 5 | The fifth mapping object | RW | NO | UINT32 | - | 0~4294967295 | - |
|  | 6 | The sixth mapping object | RW | NO | UINT32 | - | 0~4294967295 | - |
|  | 7 | The seventh mapping object | RW | NO | UINT32 | - | 0~4294967295 | - |
|  | 8 | The eighth mapping object | RW | NO | UINT32 | - | 0~4294967295 | - |
|  | 9 | The ninth mapping object | RW | NO | UINT32 | - | 0~4294967295 | - |
|  | 0A | The tenth mapping object | RW | NO | UINT32 | - | 0~4294967295 | - |
| 1701 | RxPDO1 Mapping object 258th |  |  |  |  |  |  |  |
|  | 0 | RxPDO1 Number of supported mapping objects | RO | NO | UINT8 | - | - | 04hex |
|  | 1 | The first mapping object | RO | NO | UINT32 | - | - | 6040 0010 |
|  | 2 | The second mapping object | RO | NO | UINT32 | - | - | 607A 0020 |
|  | 3 | The third mapping object | RO | NO | UINT32 | - | - | 60B8 0010 |
|  | 4 | The fourth mapping object | RO | NO | UINT32 | - | - | 60FE 0120 |
| 1702 | RxPDO1 Mapping object 259th |  |  |  |  |  |  |  |
|  | 0 | RxPDO259 Number of supported mapping objects | RO | NO | UINT8 | - | - | 07 hex |
|  | 1 | The first mapping object | RO | NO | UINT32 | - | - | 6040 0010 |
|  | 2 | The second mapping object | RO | NO | UINT32 | - | - | 607A 0020 |
|  | 3 | The third mapping object | RO | NO | UINT32 | - | - | 60FF 0020 |
|  | 4 | The fourth mapping object | RO | NO | UINT32 | - | - | 6071 0010 |
|  | 5 | The fifth mapping object | RO | NO | UINT32 | - | - | 6060 0008 |
|  | 6 | The sixth mapping object | RO | NO | UINT32 | - | - | 60B8 0010 |
|  | 7 | The seventh mapping object | RO | NO | UINT32 | - | - | 607F0020 |
| 1703 | RxPDO1 Mapping object 260th |  |  |  |  |  |  |  |
|  | 0 | RxPDO260 Number of supported mapping objects | RO | NO | UINT8 | - | - | 07 hex |
|  | 1 | The first mapping object | RO | NO | UINT32 | - | - | 6040 0010 |
|  | 2 | The second mapping object | RO | NO | UINT32 | - | - | 607A 0020 |
|  | 3 | The third mapping object | RO | NO | UINT32 | - | - | 60FF 0020 |
|  | 4 | The fourth mapping object | RO | NO | UINT32 | - | - | 6060 0008 |
|  | 5 | The fifth mapping object | RO | NO | UINT32 | - | - | 60B8 0010 |
|  | 6 | The sixth mapping object | RO | NO | UINT32 | - | - | 60E0 0010 |
|  | 7 | The seventh mapping object | RO | NO | UINT32 | - | - | 60E1 0010 |
| 1704 | RxPDO1 Mapping object 261st |  |  |  |  |  |  |  |
|  | 0 | RxPDO261 Number of supported mapping objects | RO | NO | UINT8 | - | - | 09 hex |
|  | 1 | The first mapping object | RO | NO | UINT32 | - | - | 6040 0010 |
|  | 2 | The second mapping object | RO | NO | UINT32 | - | - | 607A 0020 |
|  | 3 | The third mapping object | RO | NO | UINT32 | - | - | 60FF 0020 |
|  | 4 | The fourth mapping object | RO | NO | UINT32 | - | - | 6071 0010 |
|  | 5 | The fifth mapping object | RO | NO | UINT32 | - | - | 6060 0008 |
|  | 6 | The sixth mapping object | RO | NO | UINT32 | - | - | 60B8 0010 |
|  | 7 | The seventh mapping object | RO | NO | UINT32 | - | - | 607F0020 |
|  | 8 | The eighth mapping object | RO | NO | UINT32 | - | - | 60E0 0010 |
|  | 9 | The ninth mapping object | RO | NO | UINT32 | - | - | 60E1 0010 |
| 1705 | RxPDO1 Mapping object 262nd |  |  |  |  |  |  |  |
|  | 0 | RxPDO262 Number of supported mapping objects | RO | NO | UINT8 | - | - | 08hex |
|  | 1 | The first mapping object | RO | NO | UINT32 | - | - | 6040 0010 |
|  | 2 | The second mapping object | RO | NO | UINT32 | - | - | 607A 0020 |
|  | 3 | The third mapping object | RO | NO | UINT32 | - | - | 60FF 0020 |
|  | 4 | The fourth mapping object | RO | NO | UINT32 | - | - | 6060 0008 |
|  | 5 | The fifth mapping object | RO | NO | UINT32 | - | - | 60B8 0010 |
|  | 6 | The sixth mapping object | RO | NO | UINT32 | - | - | 60E0 0010 |
|  | 7 | The seventh mapping object | RO | NO | UINT32 | - | - | 60E1 0010 |
|  | 8 | The eighth mapping object | RO | NO | UINT32 | - | - | 60B2 0010 |
| 1A00 | TxPDO1 Mapping object 1st |  |  |  |  |  |  |  |
|  | 0 | TxPDO1 Number of supported mapping objects | RW | NO | UINT8 | - | 0~10 | 7 |
|  | 1 | The first mapping object | RW | NO | UINT32 | - | 0~4294967295 | 6041 0010 |
|  | 2 | The second mapping object | RW | NO | UINT32 | - | 0~4294967295 | 6064 0020 |
|  | 3 | The third mapping object | RW | NO | UINT32 | - | 0~4294967295 | 60B9 0010 |
|  | 4 | The fourth mapping object | RW | NO | UINT32 | - | 0~4294967295 | 60BA 0020 |
|  | 5 | The fifth mapping object | RW | NO | UINT32 | - | 0~4294967295 | 60BC0020 |
|  | 6 | The sixth mapping object | RW | NO | UINT32 | - | 0~4294967295 | 603F0010 |
|  | 7 | The seventh mapping object | RW | NO | UINT32 | - | 0~4294967295 | 60FD0020 |
|  | 8 | The eighth mapping object | RW | NO | UINT32 | - | 0~4294967295 | - |
|  | 9 | The ninth mapping object | RW | NO | UINT32 | - | 0~4294967295 | - |
|  | 0A | The tenth mapping object | RW | NO | UINT32 | - | 0~4294967295 | - |
| 1B01 | TxPDO258 Mapping object |  |  |  |  |  |  |  |
|  | 0 | TxPDO258 Number of supported mapping objects | RO | NO | UINT8 | - | - | 8 |
|  | 1 | The first mapping object | RO | NO | UINT32 | - | - | 603F0010 |
|  | 2 | The second mapping object | RO | NO | UINT32 | - | - | 6041 0010 |
|  | 3 | The third mapping object | RO | NO | UINT32 | - | - | 6064 0020 |
|  | 4 | The fourth mapping object | RO | NO | UINT32 | - | - | 6077 0010 |
|  | 5 | The fifth mapping object | RO | NO | UINT32 | - | - | 60F40020 |
|  | 6 | The sixth mapping object | RO | NO | UINT32 | - | - | 60B90010 |
|  | 7 | The seventh mapping object | RO | NO | UINT32 | - | - | 60BA0020 |
|  | 8 | The eighth mapping object | RO | NO | UINT32 | - | - | 60FD0020 |
| 1B02 | TxPDO259 Mapping object |  |  |  |  |  |  |  |
|  | 0 | TxPDO259 Number of supported mapping objects | RO | NO | UINT8 | - | - | 9 |
|  | 1 | The first mapping object | RO | NO | UINT32 | - | - | 603F0010 |
|  | 2 | The second mapping object | RO | NO | UINT32 | - | - | 6041 0010 |
|  | 3 | The third mapping object | RO | NO | UINT32 | - | - | 6064 0020 |
|  | 4 | The fourth mapping object | RO | NO | UINT32 | - | - | 6077 0010 |
|  | 5 | The fifth mapping object | RO | NO | UINT32 | - | - | 6061 0008 |
|  | 6 | The sixth mapping object | RO | NO | UINT32 | - | - | 60B9 0010 |
|  | 7 | The seventh mapping object | RO | NO | UINT32 | - | - | 60BA 0020 |
|  | 8 | The eighth mapping object | RO | NO | UINT32 | - | - | 60BC0020 |
|  | 9 | The ninth mapping object | RO | NO | UINT32 | - | - | 60FD0020 |
| 1B03 | TxPDO260 Mapping object |  |  |  |  |  |  |  |
|  | 0 | TxPDO260 Number of supported mapping objects | RO | NO | UINT8 | - | - | 10 |
|  | 1 | The first mapping object | RO | NO | UINT32 | - | - | 603F0010 |
|  | 2 | The second mapping object | RO | NO | UINT32 | - | - | 6041 0010 |
|  | 3 | The third mapping object | RO | NO | UINT32 | - | - | 6064 0020 |
|  | 4 | The fourth mapping object | RO | NO | UINT32 | - | - | 6077 0010 |
|  | 5 | The fifth mapping object | RO | NO | UINT32 | - | - | 60F4 0020 |
|  | 6 | The sixth mapping object | RO | NO | UINT32 | - | - | 6061 0008 |
|  | 7 | The seventh mapping object | RO | NO | UINT32 | - | - | 60B9 0010 |
|  | 8 | The eighth mapping object | RO | NO | UINT32 | - | - | 60BA 0020 |
|  | 9 | The ninth mapping object | RO | NO | UINT32 | - | - | 60BC0020 |
|  | 0A | The tenth mapping object | RO | NO | UINT32 | - | - | 60FD0020 |
| 1B04 | TxPDO261 Mapping object |  |  |  |  |  |  |  |
|  | 0 | TxPDO261 Number of supported mapping objects | RO | NO | UINT8 | - | - | 10 |
|  | 1 | The first mapping object | RO | NO | UINT32 | - | - | 603F0010 |
|  | 2 | The second mapping object | RO | NO | UINT32 | - | - | 6041 0010 |
|  | 3 | The third mapping object | RO | NO | UINT32 | - | - | 6064 0020 |
|  | 4 | The fourth mapping object | RO | NO | UINT32 | - | - | 6077 0010 |
|  | 5 | The fifth mapping object | RO | NO | UINT32 | - | - | 6061 0008 |
|  | 6 | The sixth mapping object | RO | NO | UINT32 | - | - | 60F4 0020 |
|  | 7 | The seventh mapping object | RO | NO | UINT32 | - | - | 60B9 0010 |
|  | 8 | The eighth mapping object | RO | NO | UINT32 | - | - | 60BA 0020 |
|  | 9 | The ninth mapping object | RO | NO | UINT32 | - | - | 60BC0020 |
|  | 0A | The tenth mapping object | RO | NO | UINT32 | - | - | 606C0020 |
| 1C12 | Sync management 2_ RxPDO allocation |  |  |  |  |  |  |  |
|  | 0 | Sync management 2_RxPDO allocation Max. subindex No. | RW | NO | UINT8 | - | 0~1 | 1 |
|  | 1 | Index of RxPDO allocation objects | RW | YES | UINT16 | - | 0~65535 | 0x1701 |
| 1C13 | Sync management 2_TxPDO allocation |  | RW | NO | UINTER16 | - | OD data range | OD default value |
|  | 0 | Sync management 2_TxPDO allocation Max. subindex No. | RW | NO | UINT8 | - | 0~1 | 1 |
|  | 1 | Index of TxPDO allocation objects | RW | YES | UINT16 | - | 0~65535 | 0x1B01 |
| 1C32 | Sync management 2 Synchronize output parameters |  |  |  |  |  |  |  |
|  | 0 | Sync management 2 Maximum sub-index number of synchronization parameters | RO | NO | UINT8 | - | - | 0x20 |
|  | 1 | Synchronization type | RO | NO | UINT16 | - | - | 0x0002 |
|  | 2 | Cycle time | RO | NO | UINT32 | ns | - | 0 |
|  | 4 | Supported synchronization types | RO | NO | UINT16 | - | - | 0x0004 |
|  | 5 | Min. cycle time | RO | NO | UINT32 | ns | - | 0x0001E848 |
|  | 6 | Calculate and copy time | RO | NO | UINT32 | ns | - | - |
|  | 9 | Delay time | RO | NO | UINT32 | ns | - | - |
|  | 20 | Sync error | RO | NO | BOOL | - | - | - |
| 1C33 | Sync management 2 Synchronize input parameters |  |  |  |  |  |  |  |
|  | 0 | Sync management 2 Maximum sub-index number of synchronization parameters | RO | NO | UINT8 | - | - | 0x20 |
|  | 1 | Synchronization type | RO | NO | UINT16 |  | - | 0x0002 |
|  | 2 | Cycle time | RO | NO | UINT32 | ns | - | 0 |
|  | 4 | Supported synchronization types | RO | NO | UINT16 | - | - | 0x0004 |
|  | 5 | Min. cycle time | RO | NO | UINT32 | ns | - | 0x0001E848 |
|  | 6 | Calculate and copy time | RO | NO | UINT32 | ns | - | - |
|  | 9 | Delay time | RO | NO | UINT32 | ns | - | - |
|  | 20 | Sync error | RO | NO | BOOL | - | - | - |

7.2 Allocation List of Object Groups 2000h

Table 7-2 List of 2000h object group

| Index | Subindex | Parameter | Name | Property | Unit | Range | Factory default | Setting mode | Enable mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2000 Motor parameters |  |  |  |  |  |  |  |  |  |
| 2000 | 04h | P0003 | Motor property | RW | - | 0~65535 | 0 | Stop change | Immediate effect |
| 2000 | 0Bh | P0010 | Rated voltage of motor | RW | V | 0:220V | 0 | Stop change | Power-on reset |
|  |  |  |  |  |  | 1:380V |  |  |  |
| 2000 | 0Ch | P0011 | Rated current of motor | RW | 0.01A | 0~65535 | 100 | Stop change | Power-on reset |
| 2000 | 0Dh | P0012 | Rated power of motor | RW | 0.01kW | 0~65535 | 75 | Stop change | Power-on reset |
| 2000 | 0Eh | P0013 | Rated torque | RW | 0.01Nm | 0~4294967295 | 239 | Stop change | Power-on reset |
| 2000 | 12h | P0017 | Rated speed | RW | rpm | 0~65535 | 3000 | Stop change | Power-on reset |
| 2001 Encoder parameters |  |  |  |  |  |  |  |  |  |
| 2001 | 01h | P0100 | Encoder communication protocol | RW | - | 0~65535 | 11233 | Stop change | Power-on reset |
| 2001 | 04h | P0103 | Encoder version No. | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2001 | 06h | P0105 | Encoder wire count | RW | - | 0~4294967295 | 1048576 | Stop change | Power-on reset |
| 2002 Drive Parameters |  |  |  |  |  |  |  |  |  |
| 2002 | 01h | P0200 | MCU software version No. | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2002 | 02h | P0201 | FPGA software version No. | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2002 | 03h | P0202 | MCU non-standard No. | RO | - | 0~4294967295 | 0 | Display | Immediate effect |
| 2002 | 05h | P0204 | FPGA non-standard No. | RO | - | 0~4294967295 | 0 | Display | Immediate effect |
| 2002 | 0Dh | P0212 | Drive input voltage | RO | - | 0~65535 | 220 | Display | Immediate effect |
| 2002 | 0Eh | P0213 | Drive rated power | RO | 0.01kW | 1~65535 | 75 | Display | Immediate effect |
| 2002 | 10h | P0215 | Drive rated current | RO | 0.01A | 1~65535 | 550 | Display | Immediate effect |
| 2002 | 23h | P0234 | Regenerative resistance selection | RW | - | 0: Built-in regenerative resistor | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: External regenerative resistance, natural cooling |  |  |  |
|  |  |  |  |  |  | 2: external regenerative resistance, forced air cooling |  |  |  |
|  |  |  |  |  |  | 3: No regenerative resistance |  |  |  |
| 2002 | 24h | P0235 | Heat radiation coefficient of regenerative resistance | RW | - | 10~100 | 30 | Stop change | Immediate effect |
| 2002 | 25h | P0236 | Built-in regenerative resistor power | RO | W | 1~65535 | 40 | Display | Immediate effect |
| 2002 | 26h | P0237 | Built-in regenerative resistance | RO | Ω | 1~1000 | 50 | Display | Immediate effect |
| 2002 | 27h | P0238 | Min. external regenerative resistance | RO | Ω | 1~1000 | 40 | Display | Immediate effect |
| 2002 | 28h | P0239 | External regenerated resistor power | RW | W | 1~65535 | 40 | Stop change | Immediate effect |
| 2002 | 29h | P0240 | External regenerative resistance | RW | Ω | 1~1000 | 50 | Stop change | Immediate effect |
| 2003 IO Parameters |  |  |  |  |  |  |  |  |  |
| 2003 | 01h | P0300 | DI1 function | RW | - | 0: no definition | 9 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Servo enabled |  |  |  |
|  |  |  |  |  |  | 2: Emergency shutdown |  |  |  |
|  |  |  |  |  |  | 3: Command forbidden |  |  |  |
|  |  |  |  |  |  | 4: Position deviation cleared |  |  |  |
|  |  |  |  |  |  | 5: Reset the fault |  |  |  |
|  |  |  |  |  |  | 6: Zero speed retention |  |  |  |
|  |  |  |  |  |  | 7: Forward jogging |  |  |  |
|  |  |  |  |  |  | 8: Reverse jogging |  |  |  |
|  |  |  |  |  |  | 9: Forwar limit |  |  |  |
|  |  |  |  |  |  | 10: Reverse limit |  |  |  |
|  |  |  |  |  |  | 11: Origin switch |  |  |  |
|  |  |  |  |  |  | 12: Zero to return enabled |  |  |  |
|  |  |  |  |  |  | 13: Speed limit selection |  |  |  |
|  |  |  |  |  |  | 14: Forward torque limit selection |  |  |  |
|  |  |  |  |  |  | 15: Reverse torque limit selection |  |  |  |
|  |  |  |  |  |  | 16: Preset position enabled |  |  |  |
|  |  |  |  |  |  | 19: Torque instruction reversing |  |  |  |
|  |  |  |  |  |  | 20: Speed instruction reversing switch |  |  |  |
|  |  |  |  |  |  | 21: Position command reversing switch |  |  |  |
|  |  |  |  |  |  | 22: Gain switch selection |  |  |  |
|  |  |  |  |  |  | 23: Operation command switch |  |  |  |
|  |  |  |  |  |  | 24: Mode switch 1 |  |  |  |
|  |  |  |  |  |  | 25: Mode switch 2 |  |  |  |
|  |  |  |  |  |  | 26: Electronic gear switch |  |  |  |
|  |  |  |  |  |  | 27: Preset command selection 1 |  |  |  |
|  |  |  |  |  |  | 28: Preset command selection 2 |  |  |  |
|  |  |  |  |  |  | 29: Preset command selection 3 |  |  |  |
|  |  |  |  |  |  | 30: Preset command selection 4 |  |  |  |
|  |  |  |  |  |  | 31: Current DI trigger point as the origin |  |  |  |
|  |  |  |  |  |  | 33: Probe 1 |  |  |  |
|  |  |  |  |  |  | 34: Probe 2 |  |  |  |
| 2003 | 02h | P0301 | DI1 Polarity | RW | - | 0: Normally open | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Normally close |  |  |  |
| 2003 | 03h | P0302 | DI2 function | RW | - | Refer to DI1 function | 10 | Stop change | Immediate effect |
| 2003 | 04h | P0303 | DI2 polarity | RW | - | 0: Normally open | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Normally close |  |  |  |
| 2003 | 05h | P0304 | DI3 function | RW | - | Refer to DI1 function | 11 | Stop change | Immediate effect |
| 2003 | 06h | P0305 | DI3 polarity | RW | - | 0: Normally open | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Normally close |  |  |  |
| 2003 | 07h | P0306 | DI4 function | RW | - | Refer to DI1 function | 0 | Stop change | Immediate effect |
| 2003 | 08h | P0307 | DI4 polarity | RW | - | 0: Normally open | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Normally close |  |  |  |
| 2003 | 09h | P0308 | DI5 function | RW | - | Refer to DI1 function | 0 | Stop change | Immediate effect |
| 2003 | 0Ah | P0309 | DI5 polarity | RW | - | 0: Normally open | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Normally close |  |  |  |
| 2003 | 13h | P0318 | Initial effect DI function 1 | RW | - | 0: no definition | 0 | Stop change | Power-on reset |
|  |  |  |  |  |  | 1: Servo enabled |  |  |  |
|  |  |  |  |  |  | 2: Emergency shutdown |  |  |  |
|  |  |  |  |  |  | 3: Command forbidden |  |  |  |
|  |  |  |  |  |  | 4: Position deviation cleared |  |  |  |
|  |  |  |  |  |  | 5: Reset the fault |  |  |  |
|  |  |  |  |  |  | 6: Zero lock |  |  |  |
|  |  |  |  |  |  | 7: Forward jogging |  |  |  |
|  |  |  |  |  |  | 8: Reverse jogging |  |  |  |
|  |  |  |  |  |  | 9: Forwar limit |  |  |  |
|  |  |  |  |  |  | 10: Reverse limit |  |  |  |
|  |  |  |  |  |  | 11: Origin switch |  |  |  |
|  |  |  |  |  |  | 12: Zero to return enabled |  |  |  |
|  |  |  |  |  |  | 13: Speed limit selection |  |  |  |
|  |  |  |  |  |  | 14: Forward torque limit selection |  |  |  |
|  |  |  |  |  |  | 15: Reverse torque limit selection |  |  |  |
|  |  |  |  |  |  | 16: Preset position enabled |  |  |  |
| 2003 | 14h | P0319 | Initial effect DI function 2 | RW | - | 0: no definition | 0 | Stop change | Power-on reset |
|  |  |  |  |  |  | 19: Torque instruction reversing |  |  |  |
|  |  |  |  |  |  | 20: Speed instruction reversing switch |  |  |  |
|  |  |  |  |  |  | 21: Position command reversing switch |  |  |  |
|  |  |  |  |  |  | 22: Gain switch selection |  |  |  |
|  |  |  |  |  |  | 23: Speed command source switch |  |  |  |
|  |  |  |  |  |  | 24: Mode switch 1 |  |  |  |
|  |  |  |  |  |  | 25: Mode switch 2 |  |  |  |
|  |  |  |  |  |  | 26: Electronic gear switch |  |  |  |
|  |  |  |  |  |  | 27: Preset command selection 1 |  |  |  |
|  |  |  |  |  |  | 28: Preset command selection 2 |  |  |  |
|  |  |  |  |  |  | 29: Preset command selection 3 |  |  |  |
|  |  |  |  |  |  | 30: Preset command selection 4 |  |  |  |
|  |  |  |  |  |  | 31: Current DI trigger point as the origin |  |  |  |
| 2003 | 15h | P0320 | DO1 function | RW | - | 0: no definition | 17 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Rdy |  |  |  |
|  |  |  |  |  |  | 2: Run |  |  |  |
|  |  |  |  |  |  | 3: Warn |  |  |  |
|  |  |  |  |  |  | 4: Error |  |  |  |
|  |  |  |  |  |  | 5: TGon |  |  |  |
|  |  |  |  |  |  | 6: Zero |  |  |  |
|  |  |  |  |  |  | 7: VCmp |  |  |  |
|  |  |  |  |  |  | 8: VArr |  |  |  |
|  |  |  |  |  |  | 9: TArr |  |  |  |
|  |  |  |  |  |  | 10: Near |  |  |  |
|  |  |  |  |  |  | 11: Coin |  |  |  |
|  |  |  |  |  |  | 12: Clt |  |  |  |
|  |  |  |  |  |  | 13: Vlt |  |  |  |
|  |  |  |  |  |  | 14: HomeOK |  |  |  |
|  |  |  |  |  |  | 15: eHomeOK |  |  |  |
|  |  |  |  |  |  | 17: BK |  |  |  |
|  |  |  |  |  |  | 18: DB |  |  |  |
|  |  |  |  |  |  | 19: AngRdy |  |  |  |
| 2003 | 16h | P0321 | DO1 polarity | RW | - | 0: Normally open | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Normally close |  |  |  |
| 2003 | 17h | P0322 | DO2 function | RW | - | Refer to DO1 function | 2 | Stop change | Immediate effect |
| 2003 | 18h | P0323 | DO2 polarity | RW | - | 0: Normally open | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Normally close |  |  |  |
| 2003 | 19h | P0324 | DO3 function | RW | - | Refer to DO1 function | 4 | Stop change | Immediate effect |
| 2003 | 1Ah | P0325 | DO3 polarity | RW | - | 0: Normally open | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Normally close |  |  |  |
| 2003 | 1Fh | P0330 | DIDO enforcement | RW | - | 0: no operation is performed | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Forcibly DI enabled |  |  |  |
|  |  |  |  |  |  | 2: Forcibly DO enabled |  |  |  |
|  |  |  |  |  |  | 3: Forcibly DIDO enabled |  |  |  |
|  |  |  |  |  |  | 4: Bus forcibly DO enabled |  |  |  |
| 2003 | 20h | P0331 | DI Enforcement | RW | - | 0~447 | 447 | Arbitrary change | Immediate effect |
| 2003 | 21h | P0332 | DO Channel selection | RW | - | 0~7 | 0 | Arbitrary change | Immediate effect |
| 2003 | 23h | P0334 | Communication forcibly DO output switch | RW | - | 0~7 | 0 | Stop change | Immediate effect |
| 2004 Motion control parameters |  |  |  |  |  |  |  |  |  |
| 2004 | 01h | P0400 | Control mode | RW | - | 0: Speed mode | 10 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Position mode |  |  |  |
|  |  |  |  |  |  | 2: Torque mode |  |  |  |
|  |  |  |  |  |  | 3: Torque mode → speed mode |  |  |  |
|  |  |  |  |  |  | 4: Speed mode → position mode |  |  |  |
|  |  |  |  |  |  | 5: Torque mode → position mode |  |  |  |
|  |  |  |  |  |  | 6: Torque mode → speed mode → position hybrid mode |  |  |  |
|  |  |  |  |  |  | 10: EtherCAT bus mode |  |  |  |
| 2004 | 02h | P0401 | Direction of motor rotation | RW | - | 0: CCW as positive direction | 0 | Stop change | Power-on reset |
|  |  |  |  |  |  | 1: CW as positive direction |  |  |  |
| 2004 | 03h | P0402 | Position feedback system | RW | - | 0: Incremental mode | 0 | Stop change | Power-on reset |
|  |  |  |  |  |  | 1: Absolute linear mode |  |  |  |
|  |  |  |  |  |  | 2: Absolute rotation mode |  |  |  |
| 2004 | 0Bh | P0410 | Type I fault stop mode | RW | - | 0: Free stop, keep free state | 2 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: DB stop, keep free state |  |  |  |
|  |  |  |  |  |  | 2: DB stop, keep DB state |  |  |  |
| 2004 | 0Ch | P0411 | Type II fault stop mode | RW | - | 0: Free stop, keep free state | 2 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Zero speed stop, keep free state |  |  |  |
|  |  |  |  |  |  | 2: Zero speed stop, keep DB state |  |  |  |
|  |  |  |  |  |  | 3: DB stop, keep free state |  |  |  |
|  |  |  |  |  |  | 4: DB stop, keeps DB statte |  |  |  |
| 2004 | 0Dh | P0412 | Selection of disable stop mode | RW | - | 0: Free stop | 2 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: DB stop |  |  |  |
|  |  |  |  |  |  | 2: Zero speed stop |  |  |  |
| 2004 | 0Eh | P0413 | Selection of disable stop state | RW | - | 0: Keep free state | 1 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Keep DB state |  |  |  |
| 2004 | 0Fh | P0414 | Selection of power-off stop mode | RW | - | 0: Disabled mode | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Forced zero speed mode |  |  |  |
| 2004 | 10h | P0415 | Overrun stop mode | RW | - | 0: Free stop, keep free running state | 1 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Zero speed stop, keep position locked state |  |  |  |
|  |  |  |  |  |  | 2: Zero speed stop, keep free running state |  |  |  |
| 2004 | 18h | P0423 | Torque for emergency stop | RW | 0.1% | 0~3000 | 1000 | Stop change | Immediate effect |
| 2005 Function setting parameters |  |  |  |  |  |  |  |  |  |
| 2005 | 01h | P0500 | Manufacturer's password | RW | - | 0~65535 | 0 | Arbitrary change | Immediate effect |
| 2005 | 02h | P0501 | Initialization of system parameters | RW | - | 0: no operation is performed | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Parameter initialization |  |  |  |
| 2005 | 0Bh | P0510 | Save for communication write parameters | RW | - | 0: Not save | 3 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1:2000 group save |  |  |  |
|  |  |  |  |  |  | 2:6000 groups save |  |  |  |
|  |  |  |  |  |  | 3:2000 group and 6000 group save |  |  |  |
| 2005 | 0Ch | P0511 | Save for power-off parameters | RW | - | 0: Not save | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Save |  |  |  |
| 2005 | 0Dh | P0512 | Band-type brake enable switch | RW | - | 0: Disable | 1 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Enable |  |  |  |
| 2005 | 0Eh | P0513 | Band-type brake switch off to receive command delay | RW | ms | 0~500 | 250 | Arbitrary change | Immediate effect |
| 2005 | 0Fh | P0514 | Band-type brake switch on zero speed time | RW | ms | 1~1000 | 150 | Arbitrary change | Immediate effect |
| 2005 | 10h | P0515 | Band-type brake switch on speed threshold | RW | rpm | 0~3000 | 30 | Arbitrary change | Immediate effect |
| 2005 | 11h | P0516 | Band-type brake switch on time threshold | RW | ms | 1~1000 | 500 | Arbitrary change | Immediate effect |
| 2005 | 14h | P0519 | Pre-charge test enabled | RW | - | 0: Disable | 1 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Enable |  |  |  |
|  |  |  |  |  |  | 1:A lags B |  |  |  |
| 2005 | 18h | P0523 | Frequency division output phase | RW | - | 0-A leads B | 0 | Stop change | Power-on reset |
|  |  |  |  |  |  | 1-A lags B |  |  |  |
| 2005 | 2Ch | P0543 | Soft limit Settings | RW | - | 0: no limit | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Limit |  |  |  |
|  |  |  |  |  |  | 2: Limit after zero return |  |  |  |
| 2006 Gain Parameters |  |  |  |  |  |  |  |  |  |
| 2006 | 01h | P0600 | Velocity proportional gain 1 | RW | 0.1Hz | 1~20000 | 250 | Arbitrary change | Immediate effect |
| 2006 | 02h | P0601 | Velocity integral gain 1 | RW | 0.01ms | 15~51200 | 3183 | Arbitrary change | Immediate effect |
| 2006 | 03h | P0602 | Position proportional gain 1 | RW | 0.1Hz | 0~20000 | 400 | Arbitrary change | Immediate effect |
| 2006 | 09h | P0608 | Speed feedforward proportional gain | RW | 0.1% | 0~1000 | 0 | Arbitrary change | Immediate effect |
| 2006 | 0Ah | P0609 | Torque feedforward proportional gain | RW | 0.1% | 0~2000 | 0 | Arbitrary change | Immediate effect |
| 2006 | 0Bh | P0610 | Load inertia ratio | RW | - | 0~12000 | 200 | Arbitrary change | Immediate effect |
| 2007 Filter parameters |  |  |  |  |  |  |  |  |  |
| 2007 | 01h | P0700 | Position instruction FIR filtering | RW | 0.1ms | 0~65535 | 0 | Stop change | Immediate effect |
| 2007 | 02h | P0701 | Position instruction mean filtering | RW | 0.1ms | 0~1280 | 0 | Stop change | Immediate effect |
| 2007 | 03h | P0702 | Torque filtering 1 | RW | 0.01ms | 0~3000 | 79 | Arbitrary change | Immediate effect |
| 2007 | 07h | P0706 | Velocity feedforward filtering time | RW | 0.01ms | 0~6400 | 50 | Arbitrary change | Immediate effect |
| 2007 | 08h | P0707 | Torque feedforward filtering time | RW | 0.01ms | 0~6400 | 50 | Arbitrary change | Immediate effect |
| 2007 | 0Bh | P0710 | Vibration suppression frequency 1 | RW | Hz | 50~5000 | 5000 | Arbitrary change | Immediate effect |
| 2007 | 0Ch | P0711 | Vibration suppression bandwidth 1 | RW | - | 0~20 | 2 | Arbitrary change | Immediate effect |
| 2007 | 0Dh | P0712 | Vibration suppression attenuation 1 | RW | - | 0~99 | 0 | Arbitrary change | Immediate effect |
| 2007 | 0Eh | P0713 | Vibration suppression frequency 2 | RW | Hz | 50~5000 | 5000 | Arbitrary change | Immediate effect |
| 2007 | 0Fh | P0714 | Vibration suppression bandwidth 2 | RW | - | 0~20 | 2 | Arbitrary change | Immediate effect |
| 2007 | 10h | P0715 | Vibration suppression attenuation 2 | RW | - | 0~99 | 0 | Arbitrary change | Immediate effect |
| 2007 | 11h | P0716 | Vibration suppression frequency 3 | RW | Hz | 50~5000 | 5000 | Arbitrary change | Immediate effect |
| 2007 | 12h | P0717 | Vibration suppression bandwidth 3 | RW | - | 0~20 | 2 | Arbitrary change | Immediate effect |
| 2007 | 13h | P0718 | Vibration suppression attenuation 3 | RW | - | 0~99 | 0 | Arbitrary change | Immediate effect |
| 2007 | 14h | P0719 | Vibration suppression frequency 4 | RW | Hz | 50~5000 | 5000 | Arbitrary change | Immediate effect |
| 2007 | 15h | P0720 | Vibration suppression bandwidth 4 | RW | - | 0~20 | 2 | Arbitrary change | Immediate effect |
| 2007 | 16h | P0721 | Vibration suppression attenuation 4 | RW | - | 0~99 | 0 | Arbitrary change | Immediate effect |
| 2007 | 30h | P0747 | Position notch frequency A | RW | Hz | 10~1000 | 1000 | Stop change | Immediate effect |
| 2007 | 49h | P0772 | Probe filter | RW | 25ns | 0~31 | 15 | Stop change | Power-on reset |
| 2007 | 4Dh | P0776 | Speed arrival signal filtering | RW | ms | 0~5000 | 10 | Stop change | Immediate effect |
| 2008 Protection Parameters |  |  |  |  |  |  |  |  |  |
| 2008 | 01h | P0800 | Input phase missing detection | RW | - | 0: Fault detection | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Fault detection and warning |  |  |  |
|  |  |  |  |  |  | 2: No detection |  |  |  |
| 2008 | 02h | P0801 | fault detection for encoder multi-turn overflow | RW | - | 0: No detection | 1 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Detection |  |  |  |
| 2008 | 04h | P0803 | Overspeed threshold | RW | rpm | 0~10000 | 0 | Arbitrary change | Immediate effect |
| 2008 | 09h | P0808 | Position deviation removal mode | RW | - | 0: Remove when the server is not running | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Remove when servo OFF or DI signal is effective |  |  |  |
| 2008 | 0Ah | P0809 | Runaway protection detection | RW | - | 0: No detection | 1 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Detection |  |  |  |
| 2008 | 0Bh | P0810 | Runaway current judgment threshold | RW | 0.1% | 1000~4000 | 2000 | Arbitrary change | Immediate effect |
| 2008 | 0Ch | P0811 | Runaway speed threshold | RW | rpm | 1~1000 | 10 | Arbitrary change | Immediate effect |
| 2008 | 0Dh | P0812 | Runaway speed feedback filtering time | RW | 0.1ms | 1~1000 | 20 | Arbitrary change | Power-on reset |
| 2008 | 0Eh | P0813 | Runaway protection identification time | RW | ms | 10~1000 | 30 | Arbitrary change | Immediate effect |
| 2008 | 0Fh | P0814 | Motor overload protection gain | RW | % | 50~300 | 100 | Stop change | Immediate effect |
| 2008 | 11h | P0816 | Motor overload detection | RW | - | 0: Neither detects | 3 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Motor overload is detected, drive overload is not detected |  |  |  |
|  |  |  |  |  |  | 2: Motor overload is not detected, drive overload ia detected |  |  |  |
|  |  |  |  |  |  | 3: Both detect |  |  |  |
| 2008 | 12h | P0817 | Stall detection | RW | - | 0: No detection | 1 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Detection |  |  |  |
| 2008 | 13h | P0818 | Stall protection time | RW | ms | 10~65535 | 200 | Arbitrary change | Immediate effect |
| 2008 | 16h | P0821 | Overtemperature protection point of drives | RW | °C | 0~100 | 0 | Stop change | Power-on reset |
| 2009 Display parameters |  |  |  |  |  |  |  |  |  |
| 2009 | 01h | P0900 | Position command speed | RO | rpm | -32767~32767 | 0 | Display | Immediate effect |
| 2009 | 02h | P0901 | Speed command | RO | rpm | -32767~32767 | 0 | Display | Immediate effect |
| 2009 | 03h | P0902 | Torque command | RO | 0.1% | -32767~32767 | 0 | Display | Immediate effect |
| 2009 | 04h | P0903 | Position feedback speed | RO | rpm | -32767~32767 | 0 | Display | Immediate effect |
| 2009 | 05h | P0904 | Actual speed | RO | rpm | -32767~32767 | 0 | Display | Immediate effect |
| 2009 | 07h | P0906 | Actual speed (accuracy 0.1rpm) | RO | rpm | -2147483648 ~2147483647 | 0 | Display | Immediate effect |
| 2009 | 0Bh | P0910 | Bus voltage | RO | 0.1V | 0~65535 | 0 | Display | Immediate effect |
| 2009 | 0Ch | P0911 | Control voltage | RO | 0.1V | 0~65535 | 0 | Display | Immediate effect |
| 2009 | 0Dh | P0912 | Output phase current RMS(U phase) of drive | RO | 0.01A | 0~65535 | 0 | Display | Immediate effect |
| 2009 | 0Eh | P0913 | Output line voltage RMS of drive | RO | 0.1V | 0~65535 | 0 | Display | Immediate effect |
| 2009 | 0Fh | P0914 | Average load rate | RO | 0.1% | 0~8000 | 0 | Display | Immediate effect |
| 2009 | 10h | P0915 | Drive temperature | RO | °C | 0~65535 | 0 | Display | Immediate effect |
| 2009 | 12h | P0917 | Electrical Angle | RO | 0.1° | 0~65535 | 0 | Display | Immediate effect |
| 2009 | 13h | P0918 | DI input level monitoring | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2009 | 15h | P0920 | DO output level monitoring | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2009 | 1Ah | P0925 | Total running time | RO | 0.1s | 0~4294967295 | 0 | Display | Immediate effect |
| 2009 | 1Ch | P0927 | Current power-on running time | RO | - | 0~4294967295 | 0 | Display | Immediate effect |
| 2009 | 1Fh | P0930 | Real-time instruction counter | RO | Instruction unit | -2147483648 ~2147483647 | 0 | Display | Immediate effect |
| 2009 | 21h | P0932 | Run instruction counter | RO | Instruction unit | -2147483648 ~2147483647 | 0 | Display | Immediate effect |
| 2009 | 23h | P0934 | Position feedback counter | RO | Instruction unit | -2147483648 ~2147483647 | 0 | Display | Immediate effect |
| 2009 | 25h | P0936 | Position feedback counter | RO | Encoder unit | -2147483648 ~2147483647 | 0 | Display | Immediate effect |
| 2009 | 27h | P0938 | Position tracking deviation | RO | Instruction unit | -2147483648 ~2147483647 | 0 | Display | Immediate effect |
| 2009 | 29h | P0940 | Position tracking deviation | RO | Encoder unit | -2147483648 ~2147483647 | 0 | Display | Immediate effect |
| 2009 | 2Bh | P0942 | Number of encoder turns | RO | turn | 0~65535 | 0 | Display | Immediate effect |
| 2009 | 2Ch | P0943 | Encoder single turn position | RO | p | 0~2147483647 | 0 | Display | Immediate effect |
| 2009 | 2Eh | P0945 | Encoder absolute position (low 32 bits) | RO | Encoder unit | -2147483648 ~2147483647 | 0 | Display | Immediate effect |
| 2009 | 30h | P0947 | Encoder absolute position (high 32 bits) | RO | Encoder unit | -2147483648 ~2147483647 | 0 | Display | Immediate effect |
| 2009 | 32h | P0949 | Mechanical absolute position (low 32 bits) | RO | Encoder unit | -2147483648 ~2147483647 | 0 | Display | Immediate effect |
| 2009 | 34h | P0951 | Mechanical absolute position (high 32 bits) | RO | Encoder unit | -2147483648 ~2147483647 | 0 | Display | Immediate effect |
| 2009 | 36h | P0953 | Rotating load single-turn position (low 32 bits) | RO | Encoder unit | 0~4294967295 | 0 | Display | Immediate effect |
| 2009 | 38h | P0955 | Rotating load single-turn position (high 32 bits) | RO | Encoder unit | 0~4294967295 | 0 | Display | Immediate effect |
| 2009 | 3Ah | P0957 | Rotating load single-turn position | RO | Instruction unit | 0~4294967295 | 0 | Display | Immediate effect |
| 200A Communication parameters |  |  |  |  |  |  |  |  |  |
| 200A | 01h | P0A00 | Slave station No. | RW | - | 1~247 | 1 | Arbitrary change | Immediate effect |
| 200A | 03h | P0A02 | ModBus Baud rate of communication | RW | - | 0:2400bps | 6 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1:4800bps |  |  |  |
|  |  |  |  |  |  | 2:9600bps |  |  |  |
|  |  |  |  |  |  | 3:19200bps |  |  |  |
|  |  |  |  |  |  | 4:38400bps |  |  |  |
|  |  |  |  |  |  | 5:57600bps |  |  |  |
|  |  |  |  |  |  | 6:115200bps |  |  |  |
| 200A | 04h | P0A03 | ModBus communication protocol | RW | - | 0: No check, 2 end bits (8-N-2) | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Even parity check, 1 end bit (8-O-1) |  |  |  |
|  |  |  |  |  |  | 2: Odd parity check, 1 end bit (8-E-1) |  |  |  |
|  |  |  |  |  |  | 3: No check, 1 end bit (8-N-1) |  |  |  |
| 200A | 0Bh | P0A10 | EtherCAT version No. | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 200A | 0Ch | P0A11 | EtherCAT XML version No. | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 200A | 0Dh | P0A12 | EtherCAT Slave station name | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 200A | 0Eh | P0A13 | EtherCAT Slave station alias | RW | - | 0~65535 | 0 | Stop change | Immediate effect |
| 200A | 0Fh | P0A14 | EtherCAT State machine | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 200A | 10h | P0A15 | EtherCAT State code | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 200A | 11h | P0A16 | EtherCAT Sync signal loss count | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 200A | 12h | P0A17 | EtherCAT port 0 frame invalid error count | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 200A | 13h | P0A18 | EtherCAT port 1 frame invalid error count | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 200A | 14h | P0A19 | EtherCAT port 0/1 frame send error count | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 200A | 15h | P0A20 | EtherCAT port 0/1 frame loss error count | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 200A | 16h | P0A21 | EtherCAT PDI interface error count | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 200A | 1Ah | P0A25 | CSP mode instruction cache depth | RW | - | 0~1 | 0 | Stop change | Immediate effect |
| 200A | 20h | P0A31 | EtherCAT Sync Allowed times of signal loss | RW | - | 0~65535 | 9 | Arbitrary change | Immediate effect |
| 200A | 21h | P0A32 | EtherCAT Sync Signal detection deviation threshold | RW | ns | 0~4000 | 3000 | Stop change | Immediate effect |
| 200A | 22h | P0A33 | CSP position instruction increase threshold | RW | times | 1~7 | 3 | Arbitrary change | Immediate effect |
| 200E Communication auxiliary parameters |  |  |  |  |  |  |  |  |  |
| 200E | 12h | P0E17 | Numerator of the 2nd set of communication electronic gear ratio | RW | - | 1~65535 | 1 | Stop change | Immediate effect |
| 200E | 13h | P0E18 | Denominator of the 2nd group of communication electronic gear ratio | RW | - | 1~65535 | 1 | Stop change | Immediate effect |
| 201A Advanced adjustment |  |  |  |  |  |  |  |  |  |
| 201A | 01h | P1A00 | Real-time self-adjusting settings | RW | - | 0: Off | 7 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Standard rigid table mode |  |  |  |
|  |  |  |  |  |  | 2: Quick positioning mode |  |  |  |
|  |  |  |  |  |  | 5: Adaptive interpolation mode |  |  |  |
|  |  |  |  |  |  | 7: Adaptive positioning mode |  |  |  |
| 201A | 02h | P1A01 | Response level setting | RW | level | 0~40 | 16 | Arbitrary change | Immediate effect |
| 201A | 03h | P1A02 | Selection of vibration suppression mode | RW | - | 0: Off | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Vibration suppression 3 effective |  |  |  |
|  |  |  |  |  |  | 2: Vibration suppression 3 and 4 are effective |  |  |  |
|  |  |  |  |  |  | 3: P1A.14 shows the resonance frequency |  |  |  |
|  |  |  |  |  |  | 4: Restore vibration suppression 3 and 4 |  |  |  |
| 201A | 07h | P1A06 | Max. speed of inertia identification | RW | rpm | 100~1000 | 500 | Stop change | Immediate effect |
| 201A | 08h | P1A07 | Acceleration time of inertia identification | RW | ms | 20~800 | 125 | Stop change | Immediate effect |
| 201A | 09h | P1A08 | Waiting time of inertia identification | RW | ms | 50~10000 | 800 | Stop change | Immediate effect |
| 201A | 0Ah | P1A09 | Inertia identification rotation turns | RW | 0.01 ring | 0~65535 | 100 | Arbitrary change | Immediate effect |
| 201A | 13h | P1A18 | Disturbance compensation gain | RW | 0.1% | -1000~1000 | 0 | Arbitrary change | Immediate effect |
| 201A | 14h | P1A19 | Disturbance filtering time | RW | ms | 0~2500 | 50 | Arbitrary change | Immediate effect |
| 201A | 15h | P1A20 | Partial load compensation | RW | 0.1% | -1000~1000 | 0 | Arbitrary change | Immediate effect |
| 201A | 16h | P1A21 | Forward friction compensation | RW | 0.1% | -1000~1000 | 0 | Arbitrary change | Immediate effect |
| 201A | 17h | P1A22 | Reverse friction compensation | RW | 0.1% | -1000~1000 | 0 | Arbitrary change | Immediate effect |
| 201A | 18h | P1A23 | Friction compensation speed | RW | 0.1rpm | 1~300 | 20 | Arbitrary change | Immediate effect |
| 201A | 19h | P1A24 | Selection of friction compensation speed | RW | - | 0~18 | 0 | Arbitrary change | Immediate effect |
| 201A | 1Ah | P1A25 | Low frequency vibration detection enabled | RW | - | 0: Off | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Start |  |  |  |
| 2020 Auxiliary parameters |  |  |  |  |  |  |  |  |  |
| 2020 | 03h | P2002 | Emergency shutdown | RW | - | 0: no operation is performed | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Emergency shutdown |  |  |  |
| 2020 | 04h | P2003 | Fault reset | RW | - | 0: no operation is performed | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Fault reset |  |  |  |
| 2020 | 05h | P2004 | Software reset | RW | - | 0: no operation is performed | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Software reset |  |  |  |
| 2020 | 06h | P2005 | Encoder reset | RW | - | 0: no operation is performed | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Reset the fault |  |  |  |
|  |  |  |  |  |  | 2: Reset the fault and number of rings |  |  |  |
| 2020 | 07h | P2006 | Read/write of encoder parameters | RW | - | 0: no operation is performed | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Write operation |  |  |  |
|  |  |  |  |  |  | 2: Read operation |  |  |  |
| 2020 | 27h | P2038 | Call enable | RW | - | 0~1 | 0 | Arbitrary change | Immediate effect |
| 2020 | 29h | P2040 | Bus read and servo state | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2020 | 2Ah | P2041 | Bus read DO low 16-bit function | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2020 | 2Bh | P2042 | Bus read DO high 16-bit function | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2020 | 33h | P2050 | Bus setting VDI level | RW | - | 0~65535 | 0 | Arbitrary change | Immediate effect |
| 2020 | 34h | P2051 | Bus setting DO output | RW | - | 0~7 | 0 | Arbitrary change | Immediate effect |
| 2021 Fault diagnosis parameters |  |  |  |  |  |  |  |  |  |
| 2021 | 01h | P2100 | Exception parameter group ID | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 02h | P2101 | Exception parameter group bias | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 03h | P2102 | FPGA-side system status information | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 04h | P2103 | FPGA-side system fault information | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 05h | P2104 | FPGA-side timeout fault information | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 06h | P2105 | FPGA-side encoder fault information | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 07h | P2106 | Encoder status information | RW | - | 0~65535 | 0 | Arbitrary change | Immediate effect |
| 2021 | 09h | P2108 | Current fault code | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 1Fh | P2130 | Fault logging query | RW | - | 0: current fault | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: the latest one fault |  |  |  |
|  |  |  |  |  |  | 2: the latest two faults |  |  |  |
|  |  |  |  |  |  | 3: the last three faults |  |  |  |
|  |  |  |  |  |  | 4: the last four faults |  |  |  |
|  |  |  |  |  |  | 5: the last five faults |  |  |  |
|  |  |  |  |  |  | 6: the last six faults |  |  |  |
|  |  |  |  |  |  | 7: the last seven faults |  |  |  |
|  |  |  |  |  |  | 8: the last eight faults |  |  |  |
|  |  |  |  |  |  | 9: the last nine faults |  |  |  |
| 2021 | 20h | P2131 | When the fault is selected,the fault code | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 21h | P2132 | When the fault is selected,internal fault code | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 22h | P2133 | When the fault is selected,time stamp | RO | 0.1s | 0~4294967295 | 0 | Display | Immediate effect |
| 2021 | 24h | P2135 | When the fault is selected,rotation speed | RO | rpm | -32767~32767 | 0 | Display | Immediate effect |
| 2021 | 25h | P2136 | When the fault is selected,U-phase current | RO | 0.01A | -32767~32767 | 0 | Display | Immediate effect |
| 2021 | 26h | P2137 | When the fault is selected,V-phase current | RO | 0.01A | -32767~32767 | 0 | Display | Immediate effect |
| 2021 | 27h | P2138 | When the fault is selected,Bus voltage | RO | 0.1V | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 28h | P2139 | When the fault is selected,DI input state | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 29h | P2140 | When the fault is selected,DO output state | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 2Ah | P2141 | When the fault is selected,FPGA-side system state info. | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 2Bh | P2142 | When the fault is selected,FPGA-side system fault info. | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 2Ch | P2143 | When the fault is selected,FPGA-side system timeout info. | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 2Dh | P2144 | When the fault is selected,FPGA-side system encoder fault info. | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2021 | 2Eh | P2145 | When the fault is selected,encoder state info. | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2022 Fully closed loop parameters |  |  |  |  |  |  |  |  |  |
| 2022 | 01h | P2200 | Encoder selection | RW | - | 0: Internal encoder feedback | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: External encoder feedback |  |  |  |
|  |  |  |  |  |  | 2: When DI switches the electronic gear ratio, internal/external ring encoder feedback is switched |  |  |  |
| 2022 | 02h | P2201 | External encoder Enable selection | RW | - | 0: Disables external encoder | 0 | Stop change | Power-on reset |
|  |  |  |  |  |  | 1: Enable external ABZ encoder |  |  |  |
| 2022 | 03h | P2202 | External encoder Direction selection | RW | - | 0: Use default direction | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Use reverse direction |  |  |  |
| 2022 | 04h | P2203 | Number of external encoder pulses per motor rotation | RW | p | 1~1073741824 | 10000 | Stop change | Immediate effect |
| 2022 | 06h | P2205 | Numerator of the 2nd set of communication electronic gear ratio | RW | - | 1~1073741824 | 4 | Arbitrary change | Immediate effect |
| 2022 | 08h | P2207 | Denominator of the 2nd group of communication electronic gear ratio | RW | - | 1~1073741824 | 1 | Arbitrary change | Immediate effect |
| 2022 | 0Bh | P2210 | Fully closed loop deviation Filtering time | RW | 0.1ms | 0~65535 | 0 | Stop change | Immediate effect |
| 2022 | 0Ch | P2211 | Hybrid control deviation threshold | RW | - | 0~1073741824 | 1000 | Arbitrary change | Immediate effect |
| 2022 | 0Eh | P2213 | Hybrid control deviation Clear setting | RW | - | 0~100 | 1 | Arbitrary change | Immediate effect |
| 2022 | 0Fh | P2214 | Fully closed loop external Position error | RO | p | -1073741824 ~1073741824 | 0 | Display | Immediate effect |
| 2022 | 11h | P2216 | Internal encoder feedback value | RO | p | -1073741824 ~1073741824 | 0 | Display | Immediate effect |
| 2022 | 13h | P2218 | External encoder feedback value | RO | p | -1073741824 ~1073741824 | 0 | Display | Immediate effect |
| 2023 Virtual I/O parameters |  |  |  |  |  |  |  |  |  |
| 2023 | 01h | P2300 | VDI enabled | RW | - | 0: Disable | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Enable |  |  |  |
| 2023 | 02h | P2301 | Initial status of VDI power-on | RW | - | 0~65535 | 0 | Arbitrary change | Power-on reset |
| 2023 | 03h | P2302 | VDO enabled | RW | - | 0: Disable | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 1: Enable |  |  |  |
| 2023 | 04h | P2303 | Default value for undefined VDO | RW | - | 0x0:VDO1 Default value | 0 | Stop change | Immediate effect |
|  |  |  |  |  |  | 0x1:VDO2 Default value |  |  |  |
|  |  |  |  |  |  | 0x2:VDO3 Default value |  |  |  |
|  |  |  |  |  |  | 0x3:VDO4 Default value |  |  |  |
|  |  |  |  |  |  | 0x4:VDO5 Default value |  |  |  |
|  |  |  |  |  |  | 0x5:VDO6 Default value |  |  |  |
|  |  |  |  |  |  | 0x6:VDO7 Default value |  |  |  |
|  |  |  |  |  |  | 0x7:VDO8 Default value |  |  |  |
|  |  |  |  |  |  | 0x8:VDO9 Default value |  |  |  |
|  |  |  |  |  |  | 0x9:VDO10 Default value |  |  |  |
|  |  |  |  |  |  | 0xa:VDO11 Default value |  |  |  |
|  |  |  |  |  |  | 0xb:VDO12 Default value |  |  |  |
|  |  |  |  |  |  | 0xc:VDO13 Default value |  |  |  |
|  |  |  |  |  |  | 0xd:VDO14 Default value |  |  |  |
|  |  |  |  |  |  | 0xe:VDO15 Default value |  |  |  |
|  |  |  |  |  |  | 0xf:VDO16 Default value |  |  |  |
| 2023 | 07h | P2306 | VDI1 function | RW | - | 0: no definition | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Servo enabled |  |  |  |
|  |  |  |  |  |  | 2: Emergency shutdown |  |  |  |
|  |  |  |  |  |  | 3: Command forbidden |  |  |  |
|  |  |  |  |  |  | 4: Position deviation cleared |  |  |  |
|  |  |  |  |  |  | 5: Reset the fault |  |  |  |
|  |  |  |  |  |  | 6: Zero speed retention |  |  |  |
|  |  |  |  |  |  | 7: Forward jogging |  |  |  |
|  |  |  |  |  |  | 8: Reverse jogging |  |  |  |
|  |  |  |  |  |  | 9: Forwar limit |  |  |  |
|  |  |  |  |  |  | 10: Reverse limit |  |  |  |
|  |  |  |  |  |  | 11: Origin switch |  |  |  |
|  |  |  |  |  |  | 12: Zero to return enabled |  |  |  |
|  |  |  |  |  |  | 13: Speed limit selection |  |  |  |
|  |  |  |  |  |  | 14: Forward torque limit selection |  |  |  |
|  |  |  |  |  |  | 15: Reverse torque limit selection |  |  |  |
|  |  |  |  |  |  | 16: Preset position enabled |  |  |  |
|  |  |  |  |  |  | 19: Torque instruction reversing |  |  |  |
|  |  |  |  |  |  | 20: Speed instruction reversing switch |  |  |  |
|  |  |  |  |  |  | 21: Position command reversing switch |  |  |  |
|  |  |  |  |  |  | 22: Gain switch selection |  |  |  |
|  |  |  |  |  |  | 23: Operation command switch |  |  |  |
|  |  |  |  |  |  | 24: Mode switch 1 |  |  |  |
|  |  |  |  |  |  | 25: Mode switch 2 |  |  |  |
|  |  |  |  |  |  | 26: Electronic gear switch |  |  |  |
|  |  |  |  |  |  | 27: Preset command selection 1 |  |  |  |
|  |  |  |  |  |  | 28: Preset command selection 2 |  |  |  |
|  |  |  |  |  |  | 29: Preset command selection 3 |  |  |  |
|  |  |  |  |  |  | 30: Preset command selection 4 |  |  |  |
|  |  |  |  |  |  | 31: Current DI trigger point as the origin |  |  |  |
|  |  |  |  |  |  | 33: Probe 1 |  |  |  |
|  |  |  |  |  |  | 34: Probe 2 |  |  |  |
| 2023 | 08h | P2307 | VDI1 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 09h | P2308 | VDI2 function | RW | - | Refer to VDI1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 0Ah | P2309 | VDI2 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 0Bh | P2310 | VDI3 function | RW | - | Refer to VDI1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 0Ch | P2311 | VDI3 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 0Dh | P2312 | VDI4 function | RW | - | Refer to VDI1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 0Eh | P2313 | VDI4 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 0Fh | P2314 | VDI5 function | RW | - | Refer to VDI1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 10h | P2315 | VDI5 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 11h | P2316 | VDI6 function | RW | - | Refer to VDI1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 12h | P2317 | VDI6 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 13h | P2318 | VDI7 function | RW | - | Refer to VDI1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 14h | P2319 | VDI7 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 15h | P2320 | VDI8 function | RW | - | Refer to VDI1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 16h | P2321 | VDI8 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 17h | P2322 | VDI9 function | RW | - | Refer to VDI1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 18h | P2323 | VDI9 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 19h | P2324 | VDI10 function | RW | - | Refer to VDI1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 1Ah | P2325 | VDI10 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 1Bh | P2326 | VDI11 function | RW | - | Refer to VDI1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 1Ch | P2327 | VDI11 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 1Dh | P2328 | VDI12 function | RW | - | Refer to VDI1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 1Eh | P2329 | VDI12 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 1Fh | P2330 | VDI13 function | RW | - | Refer to VDI1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 20h | P2331 | VDI13 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 21h | P2332 | VDI14 function | RW | - | Refer to VDI1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 22h | P2333 | VDI14 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 23h | P2334 | VDI15 function | RW | - | Refer to VDI1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 24h | P2335 | VDI15 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 25h | P2336 | VDI16 function | RW | - | Refer to VDI1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 26h | P2337 | VDI16 polarity | RW | - | 0: Write 1 valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Write rising edge valid (0→1) |  |  |  |
| 2023 | 2Bh | P2342 | VDO output level | RO | - | 0~65535 | 0 | Display | Immediate effect |
| 2023 | 2Ch | P2343 | VDO1 function | RW | - | 0: no definition | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Rdy |  |  |  |
|  |  |  |  |  |  | 2: Run |  |  |  |
|  |  |  |  |  |  | 3: Warn |  |  |  |
|  |  |  |  |  |  | 4: Error |  |  |  |
|  |  |  |  |  |  | 5: TGon |  |  |  |
|  |  |  |  |  |  | 6: Zero |  |  |  |
|  |  |  |  |  |  | 7: VCmp |  |  |  |
|  |  |  |  |  |  | 8: VArr |  |  |  |
|  |  |  |  |  |  | 9: TArr |  |  |  |
|  |  |  |  |  |  | 10: Near |  |  |  |
|  |  |  |  |  |  | 11: Coin |  |  |  |
|  |  |  |  |  |  | 12: Clt |  |  |  |
|  |  |  |  |  |  | 13: Vlt |  |  |  |
|  |  |  |  |  |  | 14: HomeOK |  |  |  |
|  |  |  |  |  |  | 15: eHomeOK |  |  |  |
|  |  |  |  |  |  | 17: BK |  |  |  |
|  |  |  |  |  |  | 18: DB |  |  |  |
|  |  |  |  |  |  | 19: AngRdy |  |  |  |
| 2023 | 2Dh | P2344 | VDO1 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |
| 2023 | 2Eh | P2345 | VDO2 function | RW | - | Refer to VDO1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 2Fh | P2346 | VDO2 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |
| 2023 | 30h | P2347 | VDO3 function | RW | - | Refer to VDO1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 31h | P2348 | VDO3 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |
| 2023 | 32h | P2349 | VDO4 function | RW | - | Refer to VDO1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 33h | P2350 | VDO4 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |
| 2023 | 34h | P2351 | VDO5 function | RW | - | Refer to VDO1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 35h | P2352 | VDO5 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |
| 2023 | 36h | P2353 | VDO6 function | RW | - | Refer to VDO1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 37h | P2354 | VDO6 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |
| 2023 | 38h | P2355 | VDO7 function | RW | - | Refer to VDO1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 39h | P2356 | VDO7 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |
| 2023 | 3Ah | P2357 | VDO8 function | RW | - | Refer to VDO1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 3Bh | P2358 | VDO8 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |
| 2023 | 3Ch | P2359 | VDO9 function | RW | - | Refer to VDO1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 3Dh | P2360 | VDO9 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |
| 2023 | 3Eh | P2361 | VDO10 function | RW | - | Refer to VDO1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 3Fh | P2362 | VDO10 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |
| 2023 | 40h | P2363 | VDO11 function | RW | - | Refer to VDO1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 41h | P2364 | VDO11 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |
| 2023 | 42h | P2365 | VDO12 function | RW | - | Refer to VDO1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 43h | P2366 | VDO12 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |
| 2023 | 44h | P2367 | VDO13 function | RW | - | Refer to VDO1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 45h | P2368 | VDO13 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |
| 2023 | 46h | P2369 | VDO14 function | RW | - | Refer to VDO1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 47h | P2370 | VDO14 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |
| 2023 | 48h | P2371 | VDO15 function | RW | - | Refer to VDO1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 49h | P2372 | VDO15 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |
| 2023 | 4Ah | P2373 | VDO16 function | RW | - | Refer to VDO1 function | 0 | Arbitrary change | Immediate effect |
| 2023 | 4Bh | P2374 | VDO16 polarity | RW | - | 0: Output 1 when valid | 0 | Arbitrary change | Immediate effect |
|  |  |  |  |  |  | 1: Output 0 when valid |  |  |  |

7.3 Allocation List of Object Group 6000h

6000h object group contains the objects related to supported sub-protocol DSP 402.

Table 7-3 List of  6000h object group

| Index | Subindex | Name | Property | PDO mapping | Data type | Unit | Data range | Factory default | Change mode | Enable mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 603Fh | 0 | Error code | RO | TxPDO | UINT16 | - | 0~65535 | 0 | Display | No |
| 6040h | 0 | Control word | RW | RxPDO | UINT16 | - | 0~65535 | 0 | Arbitrary change | Stop effect |
| 6041h | 0 | Status word | RO | TxPDO | UINT16 | - | 0~65535 | 0 | Display | No |
| 605Ah | 0 | Selection of quick stop mode | RW | NO | INT16 |  | 0~7 | 2 | Arbitrary change | Stop effect |
| 605Dh | 0 | Selection of pause stop mode | RW | NO | INT16 |  | 1~3 | 1 | Arbitrary change | Stop effect |
| 6060h | 0 | Servo mode selection | RW | RxPDO | INT8 | - | 0~10 | 0 | Arbitrary change | Stop effect |
| 6061h | 0 | Run mode display | RO | TxPDO | INT8 | - | 0~10 | 0 | Display | No |
| 6062h | 0 | Position instruction | RO | TxPDO | DINT32 | 1 instruction unit | - | - | Display | No |
| 6063h | 0 | Position feedback | RO | TxPDO | DINT32 | 1 encoder unit | - | - | Display | No |
|  |  |  |  |  |  |  |  |  | Parameter |  |
| 6064h | 0 | Position feedback | RO | TxPDO | DINT32 | 1 instruction unit | - | - | Display | No |
| 6065h | 0 | Threshold of large position deviation | RW | RxPDO | UDINT32 | 1 instruction unit | 20bit motor: 3145728 | 1048576 | Arbitrary change | Stop effect |
|  |  |  |  |  |  |  | 23bit motor: 25165824 |  |  |  |
| 6067h | 0 | Threshold of position arrival | RW | RxPDO | UINT32 | 1 encoder unit | 0~65535 | 734 | Arbitrary change | Immediate effect |
| 6068h | 0 | Position arrival window time | RW | RxPDO | UINT16 | 1ms | 0~65535 | x16 | Arbitrary change | Immediate effect |
| 606Ch | 0 | Actual velocity | RO | TxPDO | INT32 | 1 Command unit /s | - | - | Display | No |
| 606Dh | 0 | the threshold of speed arrival | RW | RxPDO | UINT16 | 1rpm | 0~65535 | 10 | Arbitrary change | Stop effect |
| 606Eh | 0 | Speed arrives window time | RW | RxPDO | UINT16 | 1ms | 0~65535 | 0 | Arbitrary change | Stop effect |
| 6071h | 0 | Target torque | RW | RxPDO | INT16 | 0.10% | -5000~5000 | 0 | Arbitrary change | Stop effect |
| 6072h | 0 | Maximum torque instruction | RW | RxPDO | UINT16 | 0.10% | 0~5000 | 5000 | Arbitrary change | Stop effect |
| 6074h | 0 | Torque command | RO | TxPDO | INT16 | 0.10% | -5000~5000 | 0 | Display | No |
| 6077h | 0 | Actual torque | RO | TxPDO | INT16 | 0.10% | -5000~5000 | 0 | Display | No |
|  |  |  |  |  |  |  |  |  | Parameter |  |
| 607Ah | 0 | Target position | RW | RxPDO | INT32 | 1 instruction unit | -231~(231-1) | 0 | Arbitrary change | Stop effect |
| 607Ch | 0 | Origin offset | RW | RxPDO | INT32 | 1 instruction unit | -231~(231-1) | 0 | Arbitrary change | Stop effect |
| 607Dh | Software absolute location restrictions |  |  |  |  |  |  |  |  |  |
|  | 0 | Number of subindex | RO | NO | UINT8 | - | - | 2 | Display | No |
|  | 1 | Min. position restriction | RW | RxPDO | INT32 | 1 User position unit | -231~(231-1) | -231 | Arbitrary change | Stop effect |
|  | 2 | Max. position restriction | RW | RxPDO | INT32 | 1 User position unit | -231~(231-1) | 231-1 | Arbitrary change | Stop effect |
| 607Eh | 0 | Command polarity | RW | RxPDO | UINT8 | - | 00~FF | 0 | Arbitrary change | Stop effect |
| 607Fh | 0 | Max. speed | RW | RxPDO | UDINT32 | 1 Command unit /s | 0~(232-1) | 108 | Arbitrary change | Stop effect |
| 6081h | 0 | Profile speed | RW | RxPDO | UDINT32 | 1 User speed unit | 0~(232-1) | 0 | Arbitrary change | Stop effect |
| 6083h | 0 | Profile acceleration | RW | RxPDO | UDINT32 | 1 instruction unit /s2 | 0~(232-1) | 100 | Arbitrary change | Stop effect |
| 6084h | 0 | Profile deceleration | RW | RxPDO | UDINT32 | 1 instruction unit /s2 | 0~(232-1) | 100 | Arbitrary change | Stop effect |
| 6085h | 0 | Quick stop deceleration | RW | RxPDO | UDINT32 | 1 User acceleration unit | 0~(232-1) | 100 | Arbitrary change | Stop effect |
| 6086h | 0 | Operation curve selection | RW | RxPDO | INT16 | - | -215~(215-1) | 0 | Arbitrary change | Stop effect |
| 6087h | 0 | Torque ramp | RW | RxPDO | UDINT32 | 0.1%/s | 0~(232-1) | 232-1 | Arbitrary change | Stop effect |
| 6091h | Gear ratio |  |  |  |  |  |  |  |  |  |
|  | 0 | Number of subindex | RO | NO | UINT8 | - | - | 2 | Display | No |
|  | 1 | Motor resolution | RW | RxPDO | UINT32 | - | 1~(232-1) | 1 | Arbitrary change | Immediate effect |
|  | 2 | Load shaft resolution | RW | RxPDO | UINT32 | - | 1~(232-1) | 1 | Arbitrary change | Immediate effect |
| 6098h |  | Origin return method | RW | RxPDO | INT8 | - | -2~35 | 1 | Arbitrary change | Stop effect |
| 6099h | Return-to-zero velocity |  |  |  |  |  |  |  |  |  |
|  | 0 | Number of subindexes of return to zero speed | RO | NO | UINT8 | - | 2 | 2 | Display | No |
|  | 1 | Deceleration point of high-speed search | RW | RxPDO | UINT32 | 1 Command unit /s | 0~(232-1) | 100 | Arbitrary change | Stop effect |
|  | 2 | Search origin low speed | RW | RxPDO | UINT32 | 1 Command unit /s | 10~(232-1) | 100 | Arbitrary change | Stop effect |
| 609Ah | 0 | Return-to-zero acceleration | RW | RxPDO | UDINT32 | 1 instruction unit /s2 | 0~(232-1) | 100 | Arbitrary change | Stop effect |
| 60B0h | 0 | Position bias | RW | RxPDO | INT32 | 1 instruction unit | -231~(231-1) | 0 | Arbitrary change | Stop effect |
| 60B1h | 0 | Velocity bias | RW | RxPDO | INT32 | 1 Command unit /s | -231~(231-1) | 0 | Arbitrary change | Stop effect |
| 60B2h | 0 | Torque bias | RW | RxPDO | INT16 | 0.10% | -5000~5000 | 0 | Arbitrary change | Stop effect |
| 60B8h | 0 | Probe mode | RW | RxPDO | UINT16 | - | 0~65535 | 0 | Arbitrary change | Stop effect |
| 60B9h | 0 | Probe status | RO | TxPDO | UINT16 | - | 0~65535 | 0 | Display | No |
| 60BAh | 0 | Probe 1 rising edge position value | RO | TxPDO | INT32 | 1 instruction unit | -231~(231-1) | 0 | Display | No |
| 60BBh | 0 | Probe 1 falling edge position value | RO | TxPDO | INT32 | 1 instruction unit | -231~(231-1) | 0 | Display | No |
| 60BCh | 0 | Probe 2 rising edge position value | RO | TxPDO | INT32 | 1 instruction unit | -231~(231-1) | 0 | Display | No |
| 60BDh | 0 | Probe 2 falling edge position value | RO | TxPDO | INT32 | 1 instruction unit | -231~(231-1) | 0 | Display | No |
| 60E0h | 0 | Forward torque limit | RW | RxPDO | UINT16 | 0.10% | 0~5000 | 5000 | Arbitrary change | Stop effect |
| 60E1h | 0 | Reverse torque limit | RW | RxPDO | UINT16 | 0.10% | 0~5000 | 5000 | Arbitrary change | Stop effect |
| 60E3h | Supported zero return mode |  |  |  |  |  |  |  |  |  |
|  | 0 | Number of subindexes of supported zero return modes | RO | NO | UINT8 | - | - | 31 | Display | No |
|  | 1 | Zero return mode 1 supported | RO | NO | UINT16 | - | - | 0301h | Display | No |
|  | 2 | Zero return mode 2 supported | RO | NO | UINT16 | - | - | 0302h | Display | No |
|  | 3 | Zero return mode 3 supported | RO | NO | UINT16 | - | - | 0303h | Display | No |
|  | 4 | Zero return mode 4 supported | RO | NO | UINT16 | - | - | 0304h | Display | No |
|  | 5 | Zero return mode 5 supported | RO | NO | UINT16 | - | - | 0305h | Display | No |
|  | 6 | Zero return mode 6 supported | RO | NO | UINT16 | - | - | 0306h | Display | No |
|  | 7 | Supported Zero return mode 7 | RO | NO | UINT16 | - | - | 0307h | Display | No |
|  | 8 | Zero return mode 8 supported | RO | NO | UINT16 | - | - | 0308h | Display | No |
|  | 9 | Zero return mode 9 supported | RO | NO | UINT16 | - | - | 0309h | Display | No |
|  | 0A | Zero return mode 10 supported | RO | NO | UINT16 | - | - | 030Ah | Display | No |
|  | 0B | Zero return mode 11 supported | RO | NO | UINT16 | - | - | 030Bh | Display | No |
|  | 0C | Zero return mode 12 supported | RO | NO | UINT16 | - | - | 030Ch | Display | No |
|  | 0D | Zero return mode 13 supported | RO | NO | UINT16 | - | - | 030Dh | Display | No |
|  | 0E | Zero return mode 14 supported | RO | NO | UINT16 | - | - | 030Eh | Display | No |
|  | 0F | Zero return mode 15 supported | RO | NO | UINT16 | - | - | 030Fh | Display | No |
|  | 10 | Zero return mode 16 supported | RO | NO | UINT16 | - | - | 0310h | Display | No |
|  | 11 | Zero return mode 17 supported | RO | NO | UINT16 | - | - | 0311h | Display | No |
|  | 12 | Zero return mode 18 supported | RO | NO | UINT16 | - | - | 0312h | Display | No |
|  | 13 | Zero return mode 19 supported | RO | NO | UINT16 | - | - | 0313h | Display | No |
|  | 14 | Zero return mode 20 supported | RO | NO | UINT16 | - | - | 0314h | Display | No |
|  | 15 | Zero return mode 21 supported | RO | NO | UINT16 | - | - | 0315h | Display | No |
|  | 16 | Zero return mode 22 supported | RO | NO | UINT16 | - | - | 0316h | Display | No |
|  | 17 | Zero return mode 23 supported | RO | NO | UINT16 | - | - | 0317h | Display | No |
|  | 18 | Zero return mode 24 supported | RO | NO | UINT16 | - | - | 0318h | Display | No |
|  | 19 | Zero return mode 25 supported | RO | NO | UINT16 | - | - | 0319h | Display | No |
|  | 1A | Zero return mode 26 supported | RO | NO | UINT16 | - | - | 031Ah | Display | No |
|  | 1B | Zero return mode 27 supported | RO | NO | UINT16 | - | - | 031Bh | Display | No |
|  | 1C | Zero return mode 28 supported | RO | NO | UINT16 | - | - | 031Ch | Display | No |
|  | 1D | Zero return mode 29 supported | RO | NO | UINT16 | - | - | 031Dh | Display | No |
|  | 1E | Zero return mode 30 supported | RO | NO | UINT16 | - | - | 031Eh | Display | No |
|  | 1F | Zero return mode 31 supported | RO | NO | UINT16 | - | - | 031Fh | Display | No |
| 60E6h | 0 | Actual position calculation | RW | NO | UINT8 | - | 0~1 | 0 | Arbitrary change | Stop effect |
| 60F4h | 0 | Position deviation | RO | RxPDO | DINT32 | 1 instruction unit | - | - | Display | No |
| 60FCh | 0 | Position instruction | RO | TxPDO | DINT32 | 1 encoder unit | - | - | Display | No |
| 60FDh | 0 | DI state | RO | RxPDO | UDINT32 | - | 0~FFFFFFFF | 0 | Display | No |
| Th present DI t-erminal logic of response drive Logic invalid Logic valid Each bit represents the corresponding DI signal as follows: P0A28=2; P0A28=0 P0A28=1 P0A28=3. bit; signal; bit; signal. 0; Reverse overrange switch; 0; Reverse overrange switch. 1; Forward overrange switch; 1; Forward overrange switch. 2; Origin switch; 2; Origin switch. 3~15; NA; 3~15; NA. 16; Z signal; 16; DI1. 17; Probe1; 17; DI2. 18; Probe2; 18; DI3. 19; NA; 19; DI4. 20; DI1; 20; DI5. 21; DI2; 21; DI6. 22; DI3; 22; NA. 23; Forward torque output; 23; HDI1. 24; Reverse torque output; 24; HDI2. 25~31; NA; 25~31; NA.. |
| 60FEh | Digital output |  |  |  |  |  |  |  |  |  |
|  | 0 | DO state | RO | NO | UINT8 | - | - | 1 | Display | No |
|  | 1 | Physical output | RW | RxPDO | UINT32 | - | 0~FFFFFFFF | 0 | Arbitrary change | Stop effect |
|  | 2 | Physical output enable | RW | NO | UINT32 | - | 0~FFFFFFFF | 0 | Arbitrary change | Stop effect |
| 60FFh | 0 | Target speed | RW | RxPDO | INT32 | 1 Command unit /s | -461 | 0 | Arbitrary change | Stop effect |
| 6502h | 0 | Supported drive mode | RO | NO | UDINT32 | - | - | 3A1h | Display | No |

- Chapter 8 Appendix

8.1 Definition of DI/DO functions

| Function No. | Function name | Description |
| --- | --- | --- |
| Description on Input signal functions |  |  |
| 1 | Servo enable | Valid: Servo motor power-on enable Invalid: Servo motor power-on is disabled |
| 2 | Emergency shutdown | Valid: Position locked after zero speed stop; Invalid: Present running status is not affected. |
| 3 | Instruction disable | Valid: Disable position instruction input Invalid: Position instruction input is allowed |
| 4 | Position deviation cleared (edge effective function) | Valid: Position deviation is zero cleared; Invalid: Position deviation is not cleared. |
| 5 | Fault reset (Edge effective function) | Invalid: Disable; Valid: Enable. |
| 6 | Zero speed retention | Valid: Enables the zero fixed function. Invalid: Zero fixed function is disabled. |
| 7 | Forward Jogging | Valid: Input per the given instruction; Invalid: Run instruction to stop input. |
| 8 | Reverse jogging | Valid: Reverse input per the given instruction; Invalid: Run instruction to stop input. |
| 9 | Forward limit | Valid: Forward drive is disabled. Invalid: Forward drive is allowed. |
| 10 | Reverse limit | Valid: Reverse drive is disabled; Invalid: Reverse drive is allowed. |
| 11 | Origin switch | Invalid: Not triggered. Valid: Triggered. |
| 12 | Return-to-zero function is enabled | Invalid: Disabled Valid: Enabled |
| 13 | Speed limit selection | Valid: Torque instruction absolute value reaches the setting value Invalid: Torque instruction absolute value is below the setting value |
| 14 | Forward torque limit selection | According to the selection of 2015:04h, torque limit source is switched. |
| 15 | Reverse torque limit selection | According to the selection of 2015:04h, torque limit source is switched. |
| 16 | The preset position enable | Valid: servo motor runs multi-stage position instructions; Invalid: servo motor is locked; |
| 19 | Torque instruction reversing | Invalid: Positive direction; Effective: Reverse direction. |
| 20 | Speed instruction reversing selection | Invalid: Positive direction; Effective: Reverse direction. |
| 22 | Gain switching selection | 2008-09h=0时： Invalid: Speed control loop is PI control; Valid: Speed control loop is P control. 2008-09h =1 hour: Execute per the settings of 2008-0Ah. |
| 23 | Runing instruction switch | Invalid: The present running instruction is A Valid: The present running instruction is B |
| 24 | Mode switch 1 | According to the selected control mode (3/4/5), switch between speed/position/torque modes |
| 25 | Mode switch 2 | According to the selected control mode (6), switch between speed/position/torque modes |
| 26 | Electronic gear switch | Invalid: Electronic gear ratio 1 Valid: Electronic gear ratio 2 |
| 27 | Default instruction selection 1 | 16 preset instruction selection |
| 28 | Default instruction selection 2 | 16 preset instruction selection |
| 29 | Default instruction selection 3 | 16 preset instruction selection |
| 30 | Default instruction selection 4 | 16 preset instruction selection. |
| 31 | The present DI trigger point used as the origin | Valid: Triggered Invalid: Not triggered |

8.2 SDO Transmission Stop Code

When exception of communication or drive occurs, servo drive would send an emergency message to network as a producer, or send an abort response when SDO transmission is abnormal. There are 4Byte abort codes in SDO abort response data, representing various termination reasons, as shown in the following table:

| Abort code | Function description | Abort code | Function description |
| --- | --- | --- | --- |
| 0x05 03 00 00 | Rollover bit doesn't change in segmented transmission | 0x06 07 00 12 | Data type doesn't match, and length of service parameters is too long |
| 0x05 04 00 00 | SDO transfer timeout | 0x06 07 00 13 | Data type doesn't match, and length of service parameters is too short |
| 0x05 04 00 01 | Command code is invalid or unknown | 0x06 09 00 11 | Subindex does not exist |
| 0x05 04 00 05 | Memory overflow | 0x06 09 00 30 | The written data is out of range |
| 0x06 01 00 00 | The object is not accessible | 0x06 09 00 31 | The value of written data is too large. |
| 0x06 01 00 01 | Read a write-only object | 0x06 09 00 32 | Data value written is too small |
| 0x06 01 00 02 | To write a read-only object | 0x06 09 00 36 | The maximum is less than the minimum |
| 0x06 02 00 00 | The data object doesn't exist in data dictionary | 0x08 00 00 00 | Common error |
| 0x06 04 00 41 | Object can't be mapped to PDO | 0x08 00 00 20 | Data can't be transferred or saved to application |
| 0x06 04 00 42 | Number and length of mapped objects exceed PDO length | 0x08 00 00 21 | Data can't be transferred or saved to application due to local control |

| Function No. | Function name | Description | Remarks |
| --- | --- | --- | --- |
| Description on output signal functions |  |  |  |
| 1 | Servo Ready (rdy) | Servo state is ready to receive S:ON effective signal: Valid: Servo ready; Invalid: Servo is not ready. | Servo is not ready: the servo has a Type I or Type II fault, or DI emergency stop is valid. |
| 2 | Servo Run (Run) | Server is in the RUN state and can receive commands: Valid: Servo can run; Invalid: Servo can't run. | - |
| 3 | Servo Warn Output (Warn) | Warning output signal is valid.(Conductive) | - |
| 4 | Servo fault output (Error) | The status is valid when fault is detected. | - |
| 5 | Motor Motion (TGon) | When motor speed is higher than the threshold 2006:11h: Valid: Motor motion signal is valid; Invalid: Motor motion signal is invalid. | - |
| 6 | Zero speed signal (Zero) | Output signal when servo motor stops Valid: Motor speed is zero; Invalid: Motor speed is not zero. | - |
| 7 | Consistent Speed (VCmp) | Under speed control, absolute value of the difference between motor speed and speed instruction is less than 606Dh; It reaches the speed threshold, and the time meets 606Eh, it's valid. | - |
| 8 | Velocity Arrival (VArr) | Valid: Speed feedback reaches the setting; Invalid: Speed feedback doesn't reach the setting. | - |
| 9 | Torque Arrival (TArr) | Valid: Absolute torque reaches the setting; Invalid: Torque absolute value is less than the setting. | - |
| 10 | Positioning Near | Under position control, it's effective when position deviation pulse reaches the setting of positioning near signal amplitude P13.09. | - |
| 11 | Position arrival(Coin) | Under position control, position deviation pulse reaches the threshold 6067h of positioning completion, and the time reaches 6068h; It is valid. | - |
| 12 | Torque Limit (Clt) | Confirmation signal of torque limit: Valid: Motor torque is limited; Invalid: Motor torque isn't limited. | - |
| 13 | Speed Limit (Vlt) | Confirmation signal of speed limitation in torque control: Valid: Motor speed is limited; Invalid: Motor speed is not limited. | - |
| 14 | Zero return completed, HomeOK | Valid: The origin zeroing is completed; Invalid: The origin zeroing is uncomplete; | - |
| 17 | Band-type brake control (BK) | Brake signal output Valid: Switch off, brake is cancelled; Invalid: Start the brake. | - |
| 18 | Dynamic Brake (DB) | Valid: Dynamic brake relay is OFF, dynamic brake is enabled; Invalid: Dynamic brake relay is ON, dynamic brake is disabled; | - |
| 19 | Magnetic pole identification ready (AngRdy) | Valid: Magnetic pole identification is ready; Invalid: Magnetic pole identification bit is uncomplete; | - |

8.3 Application Cases of Adapting Codesys Host Station

Click Device Repository in Tools drop-down, and click Install XML file SV3H_SINSEGYE_V1.0.xml.

After the installation, on the premise of physical wiring is proper, if scanning EtherCAT master station, slave station data can be scanned. After the configuration, SV3 can be enabled normally.

8.4 Adapting TwinCAT Operation Guide

8.4.1 TwinCAT Jog Operation

TwinCAT is PC-based control software, which transforms control function from hardware to software module, and integrates PLC, motion control and CNC into a PC software solution. In this section, TwinCAT3 will be used for jogging operation of SV3 servo.

Jogging operation of TwinCAT3-NC axis debugging interface

A）	Place description file (SV3H_SINSEGYE_V1.0.xml) of EtherCAT  in the path C:\TwinCAT\3.1\Config\Io\EtherCAT;

B）	Open TwinCAT3;

C）	Install NIC driver:

I)	Click TwinCAT option on the menu bar and select Show Realtime EtherCAT Compatible Decives from the drop-down list as shown below.

Figure 8-1 Open NIC driver and install

II)	The following popup is as follows: Select local connection in Incompatible devices, then click Install to install NIC, and click Enable.

Figure 8-2 NIC driver installation window

D）	New TwinCAT3 project

I)	As shown in the following figure, there are 2 ways to create a TwinCAT3 project:

Method 1: Click File-New-Project in menu bar,

Method 2: Click the middle view window of software and click New TwniCAT Project.

Method 1

Method 2

Figure 8-3 New TwinCAT3 Project

II)	Pop up the following window, select TwinCAT XAE Project(XML format), enter project name, select project path, and click OK

(4) Click OK

(3) Select project path

(2) Enter project name

(1) Select TwinCAT Projects

Figure 8-4 TwinCAT new project interface

E）	Switch TwinCAT3 to Config mode

Click Config mode button as shown below.(Note: If Activate Configuration, Restart TwinCAT System and Config Mode in the menu bar are all gray and unavailable, click TwinCAT icon at the lower right corner of PC, select System option in list, and then click Config so as to switch TwinCAT3 status)

Config mode

Run mode

Activate configuration

Figure 8-5 TwinCAT3 toolbar

F）	Scan the servo and add NC axis

I)	In the left tree list, expand I/O node, right-click Devices, click Scan, click OK in the popup prompt, and then pop up new I/O devices found window in which it lists the scanned EtherCAT devices. Check the servo (usually as Devices* (EtherCAT)) and click OK.

Figure 8-6 TwinCAT3 Scanner

II)	Inquiry window will pop up, as shown below. Click YES

Figure 8-7 ScanBox popup

III)	Inquiry window will pop up, as shown below. Click OK

Figure 8-8 Add NC axis popup

IV)	Inquiry window will pop up as shown below. Click No

Figure 8-9 Switch to the Free Run popup or not

G）	Configure NC axis parameters

PLC axis-NC axis-physical axis, there are control variable output and state variable input between these axis. PLC axis is the one controlled by PLC program; NC axis is the CNC shaft that directly controls physical axis, and  physical axis is the actual one scanned.

I)	Link NC axis to physical axis. If no axis was created in PLC, it's unnecessary to link PLC axis, shown as in the following figure(while scanning the device, a window will pop up to ask if it's linked to NC axis or CNC axis. Click Yes, then NC axis will be automatically linked).

Figure 8-10 Link NC axis and physical axis

II)	To change NC axis Enc parameter, click Enc node in the left tree list, then click on Parameter tab in the expanded view in the middle of the software. In Encoder Evaluation, find the following 2 parameters:

Scaling Factor Number: Electronic gear ratio numerator - NC axis displacment per 1 turn of motor rotation. Here,enter 60, i.e., NC axis moves 60mm per 1 turn of motor rotation.

Scaling Factor Denominator (default: 1.0): Electronic gear ratio denominator - motor encoder resolution,

Figure 8-11 Change the electronic gear ratio of NC axis

III)	To change Axis parameters, click Axis 1 node in the left tree list, click on Parameter tab in the expanded view in the middle of the software. In Manual Motion and Homing and in Monitoring, find the following 5 parameters and set them to the following values:

	Manual Velocity(Fast) : High JOG speed - 600mm/s

	Manual Velocity(Slow) : Low JOG speed - 60mm/s

	Position Lag Monitoring: Position lag monitoring --FALSE

	Position Range Monitoring: Position range monitoring --FALSE

	Target Poistion Monitoring: Target location monitoring --FALSE

Figure 8-12 High/low speed setting of NC axis jog

H）	Activation configuration

Click Activate Configuration, click OK in the pop-up, and pop up query window again, click OK to enter Run Mode

Figure 8-13 Activating configuration and switching to Run mode

I）	TwinCAT3 execute servo JOG

I)	Select Axis 1 node in the left tree list and click Online tab in the middle view window of the software;

II)	Click Set button, then pop up Set Enabling window, click All, and then click OK.

III)	Here, Ready check box under State (log.) should be checked (if not, please check if the servo reports an error and clear NC axis error in TwinCAT3);

IV)	Click F1-F4 at will, and then perform JOG of servo device.

F1: Reverse low-speed jog

F2: Reverse high-speed jog

F3: Forward low-speed jog

F4: Forward high-speed jog

F1: Reverse low-speed jog

F2: Reverse high-speed jog

F3: Forward low-speed jog

F4: Forward high-speed jog

Figure 8-14 NC axis jog

J）	TwinCAT3 performs simple movements of servo device

Shown as below, select Axis 1 node in the left tree list, select Functions tab in the middle view of the software, and select Run mode in Start Mode so as to perform various forms of motion of servo device.

Figure 8-15 NC axis compound movement

8.4.2 TwinCAT3-PDO Control Operation

(1) Change of PDO mapping objects

Application object and PDO allocation object in PDO variable mapping objects can be changed by upper machine software. The specific steps are as follows:

A）	TwinCAT3 changes transmit PDO Maooing: As shown in the following figure

I)	Click the scanned servo device in the left tree list, and click Process Data in the middle view of the software.

II)	Click SV3H in Sync Manager;

III)	In PDO Assignment(0x1C12), uncheck 0x1701 and check 0x1600;

IV)	Click 0x1600 in Index column of PDO List;

V)	Right click in PDO Content (0x1600), remove the existing PDO by Delete option, and add PDO by Add new Item.

Figure 8-16 TwinCAT3 Modify RxPDO mapping list

Figure 8-17 TwinCAT3 Modify TxPDO list

B) TwinCAT3 modify transmit PDO Maooing: As shown in the following figure

I)	Click the scanned servo device in the left tree list, and click Process Data in the middle view of the software.

II)	Click SM3 in Sync Manager;

III)	Under PDO Assignment (0x1B01), uncheck 0x1701 and then check 0x1A00;

IV)	Click 0x1A00 in Index column of PDO List;

V)	Right click in PDO Content (0x1A00); Remove the existing PDO by Delete option, and add PDO by Add new Item.

(2) PDO assignment

Switch TwinCAT3 status to Config mode, select NC axis in the left tree list, click Link To I/O in Setting interface, a window will pop up, select none, click OK, then disconnect the variable link between NC axis and servo drive device.

Figure 8-18 Unlink NC axis and physical axis

After unlinking the variable of PDO and NC axis, click servo drive PDO list, and set Link to of all response objects as empty, and re-activate the configuration.

Figure 8-19 Link interface of physical axis variables

A)	Double click Controlword in PDO list, click Online in the middle view box of the software, and click one by one as shown below, then assignment of control word is successful.

Figure 8-20 RxPDO assignment

8.5 Samples for Various Mode Operation

8.5.1 Samples for CSP Mode

Take TwinCAT3 for example, drive the motor in periodic position mode.

Flowchart as follows:

Activation configuration

Set appropriate motion parameters

Use MC_Power to disable servo

Motor rotates given distance

Servo enable

6040h control word write 6-7-15 in sequence

Use MC_Power to enable servo

Motor rotates given distance

Modify communication cycle

Configure PDO

6040 control word write 6 to disable motor

Troubleshoot

Drive motor

6040h control word write 31

607Ah target position assignment

Servo enable

Use NC axis

6060h mode selection assign value8

Figure 8-21 Flowchart of cases in CSP mode

Detailed steps as follows:

I)	Set communication cycle in upper controller;

II)	Select the appropriate PDO mapping object according to 0 run mode settings and activate the configuration.

III)	Assign 6060h mode as 8;

IV)	Set appropriate motion parameters according to the relevant objects in this section.

V)	If using NC axis, select NC_Power to enable servo drive.

If NC axis link is disconnected, write 6-7-15 to 6040h control word in turn. Here, the servo should be enabled, if not, please check if servo alarm occurs, if the first 2 arrays show 88, if upper computer software alarm occurs;

VI)	If using NC axis, drive function block corresponding to the upper controller is used for positioning;

If NC axis link is disconnected, 607Ah target position is assigned (low-speed run recommended: 607A value = communication cycle (s) * Max. motor speed (r/s) /10), and then assign 6040h control word of 31;

VII)	After operation ends, write 6040h control word as 6 and the test ends.

8.5.2 Samples for CSV Mode

Take TwinCAT3 for example, drive the motor in periodic speed mode.

Flowchart as follows:

Activation configuration

Set appropriate motion parameters

Troubleshoot

Motor rotates given distance

Servo enable

Use NC axis

6040h control word write 6-7-15 in sequenc

60FFh target velocity assignment

Use MC_Power to disable servo

6040 control word write 6 to disable motor

Motor rotates at constant speed

Use MC_Power to enable servo

Servo enable

Configure PDO

Drive motor

6060h mode selection assign value9

Modify communication cycle

Figure 8-22 Flowchart for case in CSV mode

Detailed steps as follows

I)	Set communication cycle in upper controller;

II)	Select the appropriate PDO mapping object according to 0 run mode settings and activate the configuration.

III)	Assign 6060h mode selection of 9;

IV)	Per the relevant objects in this section, set the appropriate motion parameters, but do not assign 60FFh target speed;

V)	If using NC axis, select NC_Power to enable servo drive.

If NC axis link is disconnected, write 6-7-15 to 6040h control word in turn. Here, the servo should be enabled, if not, please check if servo alarm occurs, if the first 2 arrays show 89, if upper computer software alarm occurs;

VI)	If using NC axis, drive function block corresponding to the upper controller is used for positioning;

If NC axis link is disconnected, 60FFh target speed is assigned (note the setting of electronic gear ratio);

VII)	In order to finish the run, first write 60FFh target speed of 0, and then write 6040h control word of 6 and then the test ends.

8.5.3 Samples for CST Mode

Take TwinCAT3 for example, drives the motor in periodic torque mode.

Flowchart as follows:

Use MC_Power to disable servo

Set appropriate motion parameters

Activation configuration

6040h control word write 6-7-15 in sequence

6071h Target torque assignment

6040 control word write 6 to disable motor

Troubleshoot

Use MC_Power to enable servo

6071h Target torque assignment

Motor outputs given torque

Motor outputs given torque

6060h mode selection assign value10

Configure PDO

Servo enable

Servo enable

Use NC axis

Modify communication cycle

Figure 8-23 Flowchart for case in CST mode

Detailed steps as follows

I)	Set communication cycle in upper controller;

II)	According to 3.4.3 Setting of Running Mode, select the appropriate PDO mapping object and activate the configuration.

III)	Assign 6060h mode selection of 10;

IV)	Set appropriate motion parameters according to the relevant objects in this section.

V)	If using NC axis, use NC_Power to enable servo drive,

If NC axis link is disconnected, write 6-7-15 to 6040h control word in turn. Here, the servo should be enabled, if not, please check if servo alarm occurs, if the first 2 arrays show 8A, if upper computer software alarm occurs;

VI)	TwinCAT3 has no functional block supporting torque instruction. Assign 6071h target torque. Note: the unit is  0.1%.

VII)	In order to finish the run, first write 6071h target torque of 0, and then write 6040h control word of 6 and then the test ends.

8.5.4 Samples for PP Mode

Take TwinCAT3 for example, drives the motor in profile position mode.

607Ahtarget position assignment

Motor rotates given distance

6040h control word write 6-7-15 in sequence

Use MC_Power to disable servo

Troubleshoot

Use MC_Power to enable servo

Motor rotates given distance

Servo enable

Servo enable

6040h control word write 31 or 95

Set appropriate motion parameters

Drive motor

Use NC axis

6060h mode selection assign value1

Activation configuration

Configure PDO

Modify communication cycle

6040 control word write 6 to disable motor

Figure 8-24 Flowchart for case in PP mode

Detailed steps as follows

I)	According to 3.4.3 Setting of Running Mode, select the appropriate PDO mapping object and activate the configuration.

II)	Assign 6060h mode selection of 1;

III)	Set appropriate motion parameters according to the relevant objects in this section.

IV)	If using NC axis, select NC_Power to enable servo drive.

If NC axis link is disconnected, write 6-7-15 to 6040h control word in turn. Here, the servo should be enabled,if not, please check if servo alarm occurs, if the first 2 arrays show 81, if upper computer software alarm occurs;

V)	If using NC axis, drive function block corresponding to the upper controller is used for positioning;

If NC axis link is disconnected, assign 607Ah target position, and then write 6040h control word as 31 or 95;

VI)	In order to finish the run, write 6040h control word of 6, and then the test ends.

8.5.5 Samples for PV Mode

Take TwinCAT3 for example, drives the motor in profile speed mode.Flowchart as follows:

Set appropriate motion parameters

Activation configuration

6040h control word write 6-7-15 in sequence

Use MC_Power to enable servo

Use MC_Power to disable servo

Troubleshoot

Motor rotates at constant speed

6040 control word write 6 to disable motor

Motor rotates at constant speed

Drive motor

60FFh target velocity assignment

Servo enable

Servo enable

Use NC axis

6060h mode selection assign value3

Configure PDO

Modify communication cycle

Figure 8-25 Flowchart for case in PV mode

Detailed steps as follows

I)	Select an appropriate PDO mapping object based on 0 Settings, and activate the configuration;

II)	Assign 6060h mode selection of 3;

III)	According to the relevant objects in this section, set the appropriate motion parameters, but do not assign 60FFh target speed;

IV)	If using NC axis, use NC_Power to enable servo drive,

If NC axis link is disconnected, write 6-7-15 to 6040h control word in turn. Here, the servo should be enabled, if not, please check if servo alarm occurs, if the first 2

arrays displays 83, if upper computer software alarm occurs;

V)	If using NC axis, drive function block corresponding to the upper controller is used for positioning;

If NC axis link is disconnected, assign 60FFh target speed(Note for settings of electronic gear ratio here, if it can't be assigned, please check if it's a link variable, if link variable exists, please cancel it; If 60FFh assignment is successful, when the servo motor doesn't run, please check if the acceleration/deceleration are 0);

VI)	In order to finish the run, first write 60FFh target speed of 0, and then write 6040h control word of 6 and then the test ends.

8.5.6 Samples for PT mode

Taking TwinCAT3 for example, drive the motor in profile torque mode.

Flowchart as follows:

Set appropriate motion parameters

Modify communication cycle

Configure PDO

6040h control word write 6-7-15 in sequence

Motor outputs given torque

Troubleshoot

Use MC_Power to disable servo

6060h mode selection assign value4

6071h Target torque assignment

Use MC_Power to enable servo

Servo enable

Servo enable

6071h Target torque assignment

Motor outputs given torque

6040 control word write 6 to disable motor

Use NC axis

Activation configuration

Figure 8-26 Flowchart for case in PT mode

Detailed steps as follows

I)	Select the appropriate PDO mapping object according to 0 run mode settings and activate the configuration.

II)	Assign 6060h mode selection of 4;

III)	Set appropriate motion parameters according to the relevant objects in this section.

IV)	If using NC axis, use NC_Power to enable servo drive,

If NC axis link is disconnected, write 6-7-15 to 6040h control word in turn. Here, the servo should be enabled,if not, please check if servo alarm occurs, if the first 2 arrays show 84, if upper computer software alarm occurs;

V)	TwinCAT3 has no function block supporting torque instruction. Assign 6071h target torque, and note that the unit is 0.1%.(Note that the unit is 0.1%, if assignment fails, please check if it is a link variable, if a link variable exists, please cancel it; If 6071h assignment is successful, when servo motor doesn't run, please check if torque ramp is 0 and if motor speed limit is 0);

VI)	In order to finish the run, first write 6071h target torque of 0, and then write 6040h control word of 6 and then the test ends.

8.5.7 Return to Zero Operation Sample

Take TwinCAT3 for example, drive the motor in the origin zeroing mode.

Flowchart as follows:

Set appropriate motion parameters

Troubleshoot

Servo enable

Activation configuration

6040h control word write 6-7-15 in sequence

Use NC axis

Motor Homing

After homing is complete, write 6 to the 6040 control word to disable the motor

Servo enable

Use MC_Power to enable servo

Motor Homing

6040h control word write 31

Configure PDO

MC_HomeDrive motor

Use MC_Power to Disable the Servo for Returning to Home Position

6060h mode selection assign value6

Modify communication cycle

Figure 8-27 Flowchart of return-to-zero operation

Detailed steps as follows

I)	According to 3.4.3 Setting of Running Mode, select the appropriate PDO mapping object and activate the configuration.

II)	Assign 6060h mode selection of 6;

III)	Set appropriate motion parameters according to the relevant objects in this section.

IV)	If NC axis is used, use NC_Power to enable the servo drive, and then use MC_Home to return to zero. Note that the return-to-zero method of the upper controller, please refer to the corresponding software introduction. It isn't equivalent to the return-to-zero mode of servo drive.

V)	If NC axis link is disconnected, first set control mode of 6, and then write 6-7-15 to 6040h control word in turn. Here, enable the servo to return to zero. If fails, please check if servo alarm occurs, if the first 2-bit arrays show 86;

VI)	After the return to zero ends, write 6040h control word of 6 and the test ends