SV3H Universal Series AC Servo Drive

Company Introduction

SINSEGYE is a product-driven company that leverages research and development technologies from the Institute of Computing Technology at the Chinese Academy of Sciences (CAS). Our founding team consists of core experts from the Institute of Computing Technology, each with nearly 20 years of experience in industrial intelligent computing and control technology R&D. We specialize in PC control, dual-domain operating system development, automation control, and industrial software algorithms, which solidify the foundation for our technological innovation. Our goal is to “achieve industrial intelligence based on advanced PC technology and software-defined approaches”. We have secured multiple national invention patents and utility model patents for our core technologies. Our core competency lies in our continuous and intensive R&D capabilities across the full spectrum of PC technology. This includes expertise in motion control chip design, AI chip tool chain, parallel computing, compilers, runtime, real time micro-kernel, operating system isolation, virtualization, motion control acceleration, industrial bus, IDE, and algorithms. Guided by the concept of "Industrial Intelligent Computers" as the driving force and automation as the core, we provide an intelligent industrial automation solutions that integrates computing + control for PLC, industrial computers, motion controllers, and edge servers. Our flexible product combination of IO/Motion/Acceleration/Digitization offers efficient, stable, and reliable solutions for industrial scenarios. This solution has been successfully applied in advanced manufacturing, new energy, and process industries.

Contents

I. Basic characteristics of servo drive	2

II. Model and specifications of servo drive	4

1. Naming rules of servo drives	4

2. Electrical rules of servo drives	4

3. General rules of servo drives	5

III. Servo drive wiring and port definition	6

1. Pulse type servo driver wiring	6

2. Definition of pulse type servo drive port	7

3. Wiring of RS485 bus type servo driver	8

4. Definition of RS485 bus type servo drive port	9

5. Wiring of CANopen bus type servo drive	10

6. Definition of CANopen bus type servo drive port	11

7. Wiring of EtherCAT bus type servo drive	12

8. Definition of EtherCAT bus type servo drive port	13

9. Wiring of PROFINET bus type servo drive	14

10. Definition of PROFINET bus type servo drive port	15

11. Definition of servo drive power port	16

IV. Servo driver debugging software	18

V. Wiring diagram for servo drive control mode	20

1. Wiring diagram of position mode control	20

2. Wiring diagram of speed mode control	21

3. Wiring diagram of torque mode control	22

4. Wiring diagram of  RS485 bus control	23

5. Wiring diagram of CANopen bus control	24

6. Wiring Diagram of EtherCAT Bus Control	25

7. Wiring Diagram of PROFINET Bus Control	26

VI. Installation dimensions of servo drive	27

VII. Servo motor products	29

1. Naming rules of  SM3-M2 series servo motors	29

2. Wiring standard of SM3-M2 series servo motor connector	30

3. Technical specifications of SM3-M2H040 servo motor 220V/(50W-100W)	31

4. Technical specifications of SM3-M2H060 servo motor 220V/(200W-400W)	32

5. Technical specifications of SM3-M2H080 servo motor 220V/(750W-1000W)	33

6. Technical specifications of SM3-M2H130 servo motor 220V/380V/(850W-2200W)	34

7. Technical specifications of SM3-M2H180 servo motor 380V/(2.9kW-4.4kW)	35

8. Technical specifications of SM3-M2H180 servo motor 380V/(5.5kW-7.5kW)	36

9. Technical specifications of SM3-M2A130 servo motor 220V/380V/(1kW-3kW)	37

10. Configuration specification of  SM3-M2H040 servo motor 220V/(50W-100W)	38

11. SM3-M2H040 servo motor and SV3H servo drive selection comparison table 220V/(50W-100W)	38

12. Configuration specification of SM3-M2H060 servo motor 220V/(200W-400W)	39

13. SM3-M2H060 servo motor and SV3H servo drive selection comparison table 220V/(200W-400W)	39

14. SM3-M2H080 servo motor Configuration specification of 220V/(750W-1000W)	40

15. SM3-M2H080 servo motor and SV3H servo drive selection comparison table 220V/(750W-1000W)	40

16. SM3-M2H130 servo motor configuration specification 220V/(850W-2200W)	41

17. SM3-M2H130 servo motor and SV3H servo driver selection comparison table 220V/(850W-2200W)	42

18. Configuration specification of SM3-M2H130 servo motor 380V/(850W-2200W)	43

19. Comparison Table of SM3-M2H130 Servo Motor and SV3H Servo Driver Selection 380V/(850W-2200W)	44

20. Configuration specification of SM3-M2H180 servo motor 380V/(2.9kW-7.5kW)	45

21. SM3-M2H180 servo motor and SV3H servo drive selection comparison table 380V/(2.9kW-7.5kW)	46

22. SM3-M2A130 servo motor configuration specification 220V/(1kW-3kW)	47

23. SM3-M2A130 servo motor and SV3H servo drive selection comparison table 220V/(1kW-3kW)	48

24. SM3-M2A130 servo motor configuration specification 380V/(1kW-3kW)	49

25. SM3-M2A130 servo motor and SV3H servo drive  selection comparison table 380V/(1kW-3kW)	50

26. Naming rules of SM3-M3 series servo motor	51

27. Wiring standard of SM3-M3 series servo motor connector	51

28. Technical specifications of SM3-M3 series servo motor	52

29. Configuration specifications of SM3-M3 series servo motor	54

30. Comparison Table of SM3-M3 Series Servo Motor and SV3H Series Servo Driver Selection	55

31. Cable naming rules for SM3-M2/M3 series servo motors and SV3H series servo drives	56

32. Model and specifications of compatible cables for SM3-M2 series servo motor and SV3H series servo driver	57

33. Cable specifications of SM3-M3 series servo motor and SV3H series servo driver	59

Universal AC servo drive

Product Introduction

| A new generation of high-performance servo drives can achieve position control accuracy at sub-micron level, greatly improving the accuracy, speed, efficiency, and stability of industrial automation equipment. It has two major characteristics of universality and platformization, supporting communication such as RS485, CANopen, EtherCAT and PROFINET, with power range 100W~7.5kW. |  |
| --- | --- |

Typical application scenarios

Widely used in industrial automation scenarios such as laser, printing, and semiconductor.

| AC servo application scenario I: laser cutting | AC servo application scenario II: printing equipment |
| --- | --- |

# I. Basic characteristics of servo drive

|  | High speed |  | High precision |
| --- | --- | --- | --- |
| Double the maximum operating speed Motor speed [rpm] Normal servo Maximum speed of normal servo Maximum speed of SV3 series servo Time [ms] |  | High-precision trajectory tracking of SV3 series servo When a regular servo tracks a corner command, it rotates inward Built in high-precision trajectory tracking algorithm, high dynamic response, greatly improving trajectory tracking performance; Support 17 bit-23 bit single /multi cycle absolute value encoders, significantly improving device positioning accuracy. |  |
| The maximum operating speed can reach 12000rpm, twice the speed of ordinary servos. |  |  |  |
|  | High Efficiency |  | High synchronization |
| Normal servo operation stops Smooth operation, increased processing speed and production efficiency. Stable, accurate, and fast |  | Synchronous jitter:±20ns Synchronization error: 12ns Improve servo synchronization performance |  |
| Smooth operation, increased processing speed and production efficiency. |  | High synchronization, equipped with distributed clock module. Its synchronization jitter is sub-μs level, and the jitter error r ≤ 1μs, meeting requirements of various on-site multi-axis synchronization control. |  |
|  | High speed communication |  | Control performance |
| EtherCAT operating mode; Minimum synchronization cycle. Contour position mode (pp); 1ms. Periodic synchronous position mode (csp); 1ms. Zeroing mode (hm); 1m. Contour speed mode (pv); 500μs. Contour torque mode (pt); 125μs. Periodic synchronization speed mode (csv); 500μs. Periodic synchronous torque mode (cst); 125μs. | Specifications; Refresh frequency. Carrier frequency; 8kHz. Current loop frequency; 16kHz. Speed oop frequency; 16kHz. Posi ion loop frequency; 4kHz. Adopting new ARM+FPGA architecture, the control delay is smaller, the current loop response bandwidth is as high as 3kHz, the instruction follow-up is faster, and the position timing is effectively shortened. High speed, high precision, and high efficiency, maximizing the performance of mechanical equipment. |
| The entire series adopts higher performance main control chips, further enhancing communication and interaction capabilities. |  |  |  |

|  | Gain self-adjusting |
| --- | --- |

Before self-adjustment

Feedback speed

Position command speed

After self-adjustment

Shorten positioning time and improve production pace

Automatically adjust loop gain

Time [ms]

Positioning completion signal

Positioning completion signal

Feedback speed

Position command speed

Time [ms]

One click adjustment of gain

Equipped with two automatic tuning loop parameter functions of “single parameter” and "self-adjusting", it greatly shortens the servo debugging time and significantly improves usability.

|  | Enhanced vibration suppression |
| --- | --- |

Positioning completion signal

After vibration suppression

Torque command

Position deviation

Before vibration suppression

Position deviation

Torque command

Positioning completion signal

Equipped with enhanced vibration suppression function, it has four built-in resonance suppression filters, second-order torque low-pass filters, input shaping filters, and position notch filters, which can perfectly solve vibration problems in various frequency bands such as low frequency, medium frequency, and high frequency.

# II. Model and specifications of servo drive

## 1. Naming rules of servo drives

| ## ① Product serial number SV3H： SV3 series high-performance servo | ③ Type of Drive Motor S: Single phase/Three phase 220V U: Three phase 220V T: Three phase 380V | ⑤ : Safety features Empty: Standard machine S: Safety Function (STO)* *CANopen and RS485 models do not support optional STO | ⑥ Optional Features Empty: Standard machine C: Fully closed-loop system G: Longmen Gate A: Analog interface* *CANopen, RS485 model optional items |
| --- | --- | --- | --- |
| ## ②: Instruction type P: Pulse command E： EtherCAT R： RS485 C： CANopen F： PROFINET | ④ Rated output Single /three-phase 220V model; 001; 1R6; 2R8; 5R5; 7R6; 012; 014*. Rated current; 1.0A; 1.6A; 2.8A; 5.5A; 7.6A; 11.6A; 14.0A. Rated power; 100W; 200W; 400W; 750W; 1kW; 1.5kW; 2k. Three-phase 220V mode; 018*; 022*; 027*. Rated current; 18.0A; 22.0A; 27.0A. Rated power; 4kW; 4.5kW; 5.5kW. 380V model; 3R5; 5R4; 8R4; 012; 017; 021; 026. Rated current; 3.5A; 5.4A; 8.4A; 11.9A; 16.5A; 20.8A; 26.0A. Rated power; 1kW; 1.5kW; 2kW; 3kW; 5kW; 6kW; 7.5kW. Note: S014, U018, U022, and U027 models are about to be launched |

## 2. Electrical rules of servo drives

| Structure size | SIZE-A |  |  | SIZE-B | SIZE-C |  |  | SIZE-D |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modelof servo drive SV3H-X | S001 | S1R6 | S2R8 | S5R5 | S7R6 | S012 | S0142 | U0182 | U0222 | U0272 |
| Rated output current of servo drive Arms | 1.0 | 1.6 | 2.8 | 5.5 | 7.6 | 11.6 | 14.0 | 18.0 | 22.0 | 27.0 |
| Maximum output current Arms of servo drive Arms | 3.9 | 5.8 | 10.1 | 16.9 | 23.0 | 32.0 | 42.0 | 45.0 | 55.0 | 72.0 |
| Rated input current of servo driveArms | Single phase 1.3/Three phase 0.8 | Single phase 2.3/Three phase 1.4 | Single phase 4.0/Three phase 2.6 | Single phase 7.9/Three phase 4.4 | Single phase 9.6/Three phase 5.6 | Single phase 12.8/Three phase 8.0 | Single phase 16.0/Three phase 10.2 | 18.7 | 20.7 | 24.4 |
| Specification of built-in regenerative resistor 1 | No built-in regenerative resistor |  |  | 50Ω/50W | 25Ω/80W |  |  | 20Ω/100W |  |  |
| Control circuit power supply | Single phase AC200V-240V, -10%~+10%, 50/60Hz |  |  |  |  |  |  |  |  |  |
| Main circuit power supply | Single /three-phase AC200V~240V, -10%~+10%, 50/60Hz |  |  |  |  |  |  | Three phase AC 200V~240V, -10%~+10%， 50/60Hz |  |  |

Note 1: All models support external regenerative resistors			Note 2: S014, U018, U022 and U027 models are about to be launched.

Electrical specifications of 380V model servo drive

| Structure size | SIZE-C |  |  |  | SIZE-D |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Model of servo drive l SV3H-X | T3R5 | T5R4 | T8R4 | T012 | T017 | T021 | T026 |
| Rated output current of servo drive Arms | 3.5 | 5.4 | 8.4 | 11.9 | 16.5 | 20.8 | 26.0 |
| Maximum output current Arms of servo drive Arms | 11.0 | 14.0 | 20.0 | 29.8 | 41.3 | 52.1 | 65.0 |
| Rated input current of servo drive Arms | 2.4 | 3.6 | 6.6 | 8.0 | 12.0 | 16.0 | 21.0 |
| Specification of built-in regenerative resistor 1 | 100Ω/80W |  | 50Ω/80W |  | 35Ω/100W |  |  |
| Control circuit power supply | Single phase AC380V~440, -10%~+10%, 50/60Hz |  |  |  |  |  |  |
| Main circuit power supply | Three phase AC380V~440V, -10%~+10%, 50/60Hz |  |  |  |  |  |  |

Note 1: All models support external regenerative resistors

## 3. General rules of servo drives

| Item |  |  | Description |
| --- | --- | --- | --- |
| Basic specifications | Control mode |  | IGBT SVPWM control, sine wave current drive mode. 220V, 380V: single/three-phase full wave rectification. |
|  | Usage conditions | Usage/Storage Temperature Note 1 | 0 ～+40 ℃/-20～+70℃ |
|  |  | Use/store humidity | Below 90% RH (no condensation) |
|  |  | Vibration/impact resistance strength | 4.9m/s2/19.6m/s2 |
|  |  | Protection level | IP20 |
|  |  | Class of pollution | PD2 level |
|  |  | Altitude | Highest altitude up to 5000m, no need to reduce the rating for use at altitude below 1000m or lower. For every 100m increase above 1000m, reduce the rating by 1%. For altitudes exceeding 2000m, please contact the manufacturer. |
| Position control mode | Performance | Feedforward compensation | Support speed feedforward (0~100.0%) setting to eliminate tracking deviation. |
|  |  | Instruction shaping | Position instruction low-pass filtering and mean filtering. |
|  | Frequency division output | Output form | A-phase, B-phase, Z-phase: differential output. |
|  |  | Frequency division range | The motor rotates once and can split any pulse within the range of 140 to 1048576. |
| Speed torque control mode | Performance | Dynamic characteristics of current loop | Step response: 187.5us (0~100%) Frequency response: -3dB amplitude attenuation bandwidth, 2000Hz (command signal: ± 25%) - 90° phase-shift bandwidth, 3500Hz (instruction signal: ± 25%); |
|  |  | Speed control range | 0~12000rpm, please contact the manufacturer for speeds exceeding 6000rpm. |
|  |  | Dynamic characteristics of speed loop | Step response: 562.5us (0~1000rpm) Frequency response: -3dB amplitude attenuation bandwidth, 1000Hz (command signal: ± 500rpm); - 90 ° phase-shift bandwidth, 630Hz (instruction signal) Number: ± 500rpm); |
|  |  | Torque control accuracy | ±2% |
| Input/output | Digital input signal |  | Function configurable: forward overtravel switch, reverse overtravel switch, origin switch, etc. |
|  | Digital output signal |  | Function configurable: servo ready, zero speed signal, speed arrival, position arrival, positioning proximity signal, torque limit, warning, servo failure, etc. |
| Support functions | Electronic gear ratio |  | Built-in two sets of electronic gear ratios, supporting gear ratio switching function. |
|  | Limit protection |  | Stop immediately when the forward overtravel switch and reverse overtravel switch are activated. |
|  | Fault detection |  | Overcurrent, overvoltage, undervoltage, overload, main circuit detection abnormality, radiator overheating, overspeed, encoder abnormality, parameter abnormality, etc. |
|  | Display function |  | 5-digit LED display, power indicator light CHARGE. |
|  | Vibration suppression |  | It has 4 notch filters, ranging from 50Hz to 5000Hz, all of which can be adaptively set. |
|  | Usability |  | Self tuning, speed observer, model tracking. |
|  | Debugging interface |  | USB |
|  | Other |  | Status display, alarm recording, JOG operation, etc. |
| Attention |  |  |  |
| Note 1: Please install or store the servo drive within this temperature range. |  |  |  |

# III. Servo drive wiring and port definition

## 1. Pulse type servo driver wiring

Single/three-phase power input

Electromagnetic contactor

Connect/disconnect the main power supply of the drive.  Install coil surge absorber during use

Distribution circuit breaker

When overcurrent occurs, cut off the circuit to protect the power supply circuit

Noise filter

Resist external noise generated from power circuits

Control circuit power supply

Connect L1C, L2C

Main circuit power supply * 2

Connect L1, L2, L3

Regenerative resistor * 3

External regenerative resistor

Grounding

Connect the motor PE wire and also connect the system grounding

Motor driven U VW

Connect the motor U VW three-phase cable

Brake power supply

24VDC power supply for brake system

Brake contactor

Connect/disconnect the brake power supply

Install coil surge suppressor during use

Servo motor

Note 1:

Model Text:

SV3H Universal Flagship

P-Pulse Command

Note 2:

This legend is a SIZE-A legend for single-phase/three-phase 220V,

The main circuit power terminals of the three-phase 380V driver are marked as R, S, and T.

Model silk screen printing * 1

CN 1 servo backend debugging cable

Power indicator

(Red LED)

Note 3:

SIZE-A does not have a built-in regenerative resistor. When the bus capacitance is insufficient, an external regenerative resistor should be connected between P and B.

The connection terminals of SIZE-B/C/D regenerative resistor are marked as P, RB and B.

Built in regeneration resistor connection: P and RB default to external short circuit.

External regeneration resistor connection method: Remove the short circuit between P and RB, and connect the resistor between P and B.

Note 4:

The servo bus terminals of SIZE-A/B/C servo are connected to P and N.

The bus terminal of SIZE-D servo is connected to P and N 2.  By default, N 1 and N 2 are short circuited. When using an external reactor, remove the short-circuit wire and connect the reactor between N 1 and N 2.

System grounding

CN4 motor encode cable

CN5 second encoder cable

CN 3 servo input/output cable

CN 2 functional safety (STO) cable

## 2. Definition of pulse type servo drive port

| CN5 | Pin No. | Signal name |  |
| --- | --- | --- | --- |
|  |  | Conventional models | Longmen model |
| Second encoder connector 1394-10P | 1 | 5V | 5V |
|  | 2 | GND | GND |
|  | 3 | SEC_A+ | RS485A+ |
|  | 4 | SEC_A- | RS485A- |
|  | 5 | SEC_B+ | RS485B+ |
|  | 6 | SEC_B- | RS485B- |
|  | 7 | SEC_Z+ | - |
|  | 8 | SEC_Z- | - |
|  | 9 | - | - |
|  | 10 | - | - |

| CN3 | Pin No. | Signal name | Pin No. | Signal name |
| --- | --- | --- | --- | --- |
| IO connector SCSI-50P | 1 | DI1 | 28 | PBO- |
|  | 2 | DI2 | 29 | PBO+ |
|  | 3 | DI3 | 30 | PAO- |
|  | 4 | DI_COM | 31 | PAO+ |
|  | 5 | DI4 | 32 | OCZ |
|  | 6 | DI5 | 33 | GND |
|  | 7 | DI6 | 34 | PULSE+ |
|  | 8 | DI7 | 35 | PULSE- |
|  | 9 | HDI1 | 36 | SIGN+ |
|  | 10 | HDI2 | 37 | SIGN- |
|  | 11 | DO1- | 38 | PULLHI |
|  | 12 | DO1+ | 39 | GND |
|  | 13 | DO2- | 40 | HPULSE+ |
|  | 14 | DO2+ | 41 | HPULSE- |
|  | 15 | DO3- | 42 | HSIGN+ |
|  | 16 | DO3+ | 43 | HSIGN- |
|  | 17 | DO4- | 44 | GND |
|  | 18 | DO4+ | 45 | AI1 |
|  | 19 | DO5- | 46 | AI2 |
|  | 20 | DO5+ | 47 | AO2 |
|  | 26 | PZO- | 48 | AO1 |
|  | 27 | PZO+ | 49 | GND |

| CN1 | Pin No. | Signal name |
| --- | --- | --- |
| MINI USB | 1 | VBUS |
|  | 2 | D- |
|  | 3 | D+ |
|  | 4 | - |
|  | 5 | GND |

| CN4 | Pin No. | Signal name |
| --- | --- | --- |
| Motor encoder connector 1394-10P | 1 | 5V |
|  | 2 | GND |
|  | 3 | - |
|  | 4 | - |
|  | 5 | - |
|  | 6 | - |
|  | 7 | - |
|  | 8 | - |
|  | 9 | SD+ |
|  | 10 | SD- |

| CN2 | Pin No. | Signal name |
| --- | --- | --- |
| STO connector TE: 213595-1 | 1 | Internal power supply negative |
|  | 2 | Internal power supply positive |
|  | 3 | STO1- |
|  | 4 | STO1+ |
|  | 5 | STO2- |
|  | 6 | STO2+ |
|  | 7 | STO_OUT- |
|  | 8 | STO_OUT+ |

## 3. Wiring of RS485 bus type servo driver

Single/three-phase power input

Noise filter

Resist external noise generated from power circuits

Distribution circuit breaker

When overcurrent occurs, cut off the circuit to protect the power supply circuit

Electromagnetic contactor

Connect/disconnect the main power supply of the servo drive.  Install coil surge absorber during use

Control circuit power supply

Connect L1C, L2C

Main circuit power supply *2

Connect L1, L2, L3

Regenerative resistor * 3

External regenerative resistor

Grounding

Connect the motor PE wire and also connect the system grounding

Motor driven U VW

Connect the motor U VW three-phase cable

Brake power supply

24VDC power supply for brake system

Brake contactor

Connect/disconnect the brake power supply

Install coil surge suppressor during use

Servo motor

Note 1:

Model Text:

SV3H Universal Flagship R-RS485 Bus

Note 2:

This legend is a SIZE-A legend for single-phase/three-phase 220V,

The main circuit power terminals of the three-phase 380V driver are marked as R, S, and T.

CN 1 servo backend debugging cable

CN 2 functional safety (STO) Cable

CN 3 communication input cable

Power indicator

(Red LED)

Note 3:

SIZE-A does not have a built-in regenerative resistor. When the bus capacitance is insufficient, an external regenerative resistor should be connected between P and B.

The connection terminals of SIZE-B/C/D regenerative resistor are marked as P, RB and B.

Built in regeneration resistor connection: P and RB default to external short circuit.

External regeneration resistor connection method: Remove the short circuit between P and RB, and connect the resistor between P and B.

Note 4:

The servo bus terminals of SIZE-A/B/C servo are connected to P and N.

The bus terminal of SIZE-D servo is connected to P and N 2.  By default, N 1 and N 2 are short circuited. When using an external reactor, remove the short-circuit wire and connect the reactor between N 1 and N 2.

System grounding

CN 5 servo input/output cable

CN 6 motor encoder cable

CN 7 second encoder cable

CN 4 communication output cable

Model silk screen printing * 1

## 4. Definition of RS485 bus type servo drive port

| CN7 | Pin No. | Signal name |
| --- | --- | --- |
| Second encoder connector 1394-10P | 1 | 5V |
|  | 2 | GND |
|  | 3 | SEC_A+ |
|  | 4 | SEC_A- |
|  | 5 | SEC_B+ |
|  | 6 | SEC_B- |
|  | 7 | SEC_Z+ |
|  | 8 | SEC_Z- |
|  | 9 | - |
|  | 10 | - |

| CN1 | Pin No. | Signal name |
| --- | --- | --- |
| MINI USB | 1 | VBUS |
|  | 2 | D- |
|  | 3 | D+ |
|  | 4 | - |
|  | 5 | GND |

| CN6 | Pin No. | Signal name |
| --- | --- | --- |
| Motor encoder connector 1394-10P | 1 | 5V |
|  | 2 | GND |
|  | 3 | - |
|  | 4 | - |
|  | 5 | - |
|  | 6 | - |
|  | 7 | - |
|  | 8 | - |
|  | 9 | SD+ |
|  | 10 | SD- |

| CN2 | Pin No. | Signal name |
| --- | --- | --- |
| Analog connector TE: 2013595-1 | 1 | GND |
|  | 2 | AI1 |
|  | 3 | AI2 |
|  | 4 | GND |
|  | 5 | AO1 |
|  | 6 | AO2 |
|  | 7 | GND |
|  | 8 | - |

|  | CN3 (IN) |  | CN4(OUT) |  |
| --- | --- | --- | --- | --- |
|  | Pin No. | Signal name | Pin No. | Signal name |
| RS485 communication terminal | 1 | - | 1 | - |
|  | 2 | - | 2 | - |
|  | 3 | - | 3 | - |
|  | 4 | RS485+ | 4 | RS485+ |
|  | 5 | RS485- | 5 | RS485- |
|  | 6 | - | 6 | - |
|  | 7 | - | 7 | - |
|  | 8 | GND | 8 | GND |

| CN5 | Pin No. | Signal name | Pin No. | Signal name |
| --- | --- | --- | --- | --- |
| IO connector SCSI-20P | 1 | DO1+ | 11 | DI6 |
|  | 2 | DO1- | 12 | HDI1 |
|  | 3 | DO3+ | 13 | HDI2 |
|  | 4 | DO3- | 14 | DO2+ |
|  | 5 | DI1 | 15 | DO2- |
|  | 6 | DI_COM | 16 | GND |
|  | 7 | DI2 | 17 | PAO+ |
|  | 8 | DI3 | 18 | PAO- |
|  | 9 | DI4 | 19 | PBO+ |
|  | 10 | DI5 | 20 | PBO- |

## 5. Wiring of CANopen bus type servo drive

Single/three-phase power input

Noise filter

Resist external noise generated from power circuits

Distribution circuit breaker

When overcurrent occurs, cut off the circuit to protect the power supply circuit

Electromagnetic contactor

Connect/disconnect the main power supply of the drive.  Install coil surge absorber during use

Control circuit power supply

Connect L1C, L2C

Main circuit power supply * 2

Connect L1, L2, L3

Regenerative resistor * 3

External regenerative resistor

Grounding

Connect motor PE wire and connect system grounding

Motor driven U VW

Connect the motor U VW three-phase cable

Brake power supply

24VDC power supply for brake system

Brake contactor

Connect/disconnect the brake power supply

Install coil surge suppressor during use

servo motor

Note 1:

Model Text:

SV3H Universal Flagship C-CANopen Bus

Note 2:

This legend is illustrated by SIZE-A for single-phase/three-phase 220V,

The main circuit power terminals of the three-phase 380V driver are marked as R, S, and T.

CN 1 servo backend debugging cable

CN 2 functional safety (STO) cable

CN 3 communication input cable

Power indicator

(Red LED)

Note 3:

SIZE-A does not have a built-in regenerative resistor. When the bus capacitance is insufficient, an external regenerative resistor should be connected between P and B.

The connection terminals of SIZE-B/C/D regenerative resistor are marked as P, RB and B.

Built in regeneration resistor connection: P and RB default to external short circuit.

External regeneration resistor connection method: Remove the short circuit between P and RB, and connect the resistor between P and B.

Note 4:

The servo bus terminals of SIZE-A/B/C servo are connected to P and N.

The bus terminal of SIZE-D servo is connected to P and N 2.  By default, N 1 and N 2 are short circuited. When using an external reactor, remove the short-circuit wire and connect the reactor between N 1 and N 2.

System grounding

CN 5 servo input/output cable

CN 6 motor encoder cable

CN 7 second encoder cable

CN 4 communication output cable

Model silk screen printing * 1

## 6. Definition of CANopen bus type servo drive port

| CN7 | Pin No. | Signal name |
| --- | --- | --- |
| Second encoder connector 1394-10P | 1 | 5V |
|  | 2 | GND |
|  | 3 | SEC_A+ |
|  | 4 | SEC_A- |
|  | 5 | SEC_B+ |
|  | 6 | SEC_B- |
|  | 7 | SEC_Z+ |
|  | 8 | SEC_Z- |
|  | 9 | - |
|  | 10 | - |

| CN1 | Pin No. | Signal name |
| --- | --- | --- |
| MINI USB | 1 | VBUS |
|  | 2 | D- |
|  | 3 | D+ |
|  | 4 | - |
|  | 5 | GND |

| CN6 | Pin No. | Signal name |
| --- | --- | --- |
| Motor encoder connector 1394-10P | 1 | 5V |
|  | 2 | GND |
|  | 3 | - |
|  | 4 | - |
|  | 5 | - |
|  | 6 | - |
|  | 7 | - |
|  | 8 | - |
|  | 9 | SD+ |
|  | 10 | SD- |

| CN2 | Pin No. | Signal name |
| --- | --- | --- |
| Analog connector TE: 2013595-1 | 1 | GND |
|  | 2 | - |
|  | 3 | - |
|  | 4 | GND |
|  | 5 | AO1 |
|  | 6 | AO2 |
|  | 7 | GND |
|  | 8 | - |

|  | CN3 (IN) |  | CN4(OUT) |  |
| --- | --- | --- | --- | --- |
|  | Pin No. | Signal name | Pin No. | Signal name |
| CANopen communication terminal | 1 | CANH | 1 | CANH |
|  | 2 | CANL | 2 | CANL |
|  | 3 | C_GND | 3 | C_GND |
|  | 4 | - | 4 | - |
|  | 5 | - | 5 | - |
|  | 6 | - | 6 | - |
|  | 7 | - | 7 | - |
|  | 8 | - | 8 | - |

| CN5 | Pin No. | Signal name | Pin No. | Signal name |
| --- | --- | --- | --- | --- |
| IO connector SCSI-20P | 1 | DO1+ | 11 | DI6 |
|  | 2 | DO1- | 12 | HDI1 |
|  | 3 | DO3+ | 13 | HDI2 |
|  | 4 | DO3- | 14 | DO2+ |
|  | 5 | DI1 | 15 | DO2- |
|  | 6 | DI_COM | 16 | GND |
|  | 7 | DI2 | 17 | PAO+ |
|  | 8 | DI3 | 18 | PAO- |
|  | 9 | DI4 | 19 | PBO+ |
|  | 10 | DI5 | 20 | PBO- |

## 7. Wiring of EtherCAT bus type servo drive

Single/three-phase power input

Noise filter

Resist external noise generated from power circuits

Distribution circuit breaker

When overcurrent occurs, cut off the circuit to protect the power supply circuit

Electromagnetic contactor

Connect/disconnect the main power supply of the drive.  Install coil surge absorber during use

Control circuit power supply

Connect L1C, L2C

Main circuit power supply * 2

Connect L1, L2, L3

Regenerative resistor * 3

External regenerative resistor

Grounding

Connect the motor PE wire and also connect the system grounding

Motor driven U VW

Connect  motor U VW three-phase cable

Brake power supply

24VDC power supply for brake system

Brake contactor

Connect/disconnect the brake power supply

Install coil surge suppressor during use

servo motor

Note 1:

Model Text:

SV3H Universal Flagship E-Ether CAT Bus

Note 2:

This legend is illustrated by SIZE-A for single-phase/three-phase 220V,

The main circuit power terminals of the three-phase 380V driver are marked as R, S, and T.

CN 1 servo backend debugging cable

CN 2 functional safety (STO) cable

CN 3 communication input cable

Power indicator

(Red LED)

Note 3:

SIZE-A does not have a built-in regenerative resistor. When the bus capacitance is insufficient, an external regenerative resistor should be connected between P and B.

The connection terminals of SIZE-B/C/D regenerative resistor are marked as P, RB and B.

Built in regeneration resistor connection: P and RB default to external short circuit.

External regeneration resistor connection method: Remove the short circuit between P and RB, and connect the resistor between P and B.

Note 4:

The servo bus terminals of SIZE-A/B/C servo are connected to P and N.

The bus terminal of SIZE-D servo is connected to P and N 2.  By default, N 1 and N 2 are short circuited. When using an external reactor, remove the short-circuit wire and connect the reactor between N 1 and N 2.

System grounding

CN 5 servo input/output cable

CN 6 motor encoder cable

CN 7 second encoder cable

CN 4 communication output cable

Model silk screen printing * 1

## 8. Definition of EtherCAT bus type servo drive port

| CN7 | Pin No. | Signal name |  |
| --- | --- | --- | --- |
|  |  | Conventional models | Longmen model |
| Second encoder connector 1394-10P | 1 | 5V | 5V |
|  | 2 | GND | GND |
|  | 3 | SEC_A+ | RS485A+ |
|  | 4 | SEC_A- | RS485A- |
|  | 5 | SEC_B+ | RS485B+ |
|  | 6 | SEC_B- | RS485B- |
|  | 7 | SEC_Z+ | - |
|  | 8 | SEC_Z- | - |
|  | 9 | - | - |
|  | 10 | - | - |

| CN1 | Pin No. | Signal name |
| --- | --- | --- |
| MINI USB | 1 | VBUS |
|  | 2 | D- |
|  | 3 | D+ |
|  | 4 | - |
|  | 5 | GND |

| CN6 | Pin No. | Signal name |
| --- | --- | --- |
| Motor encoder connector 1394-10P | 1 | 5V |
|  | 2 | GND |
|  | 3 | - |
|  | 4 | - |
|  | 5 | - |
|  | 6 | - |
|  | 7 | - |
|  | 8 | - |
|  | 9 | SD+ |
|  | 10 | SD- |

| CN2 | Pin No. | Signal name |
| --- | --- | --- |
| STO connector TE: 2013595-1 | 1 | Internal power supply negative |
|  | 2 | Internal power supply positive |
|  | 3 | STO1- |
|  | 4 | STO1+ |
|  | 5 | STO2- |
|  | 6 | STO2+ |
|  | 7 | STO_OUT- |
|  | 8 | STO_OUT+ |

|  | CN3 (IN) |  | CN4(OUT) |  |
| --- | --- | --- | --- | --- |
|  | Pin No. | Signal name | Pin No. | Signal name |
| EtherCAT communication terminal | 1 | TX+ | 1 | TX+ |
|  | 2 | TX- | 2 | TX- |
|  | 3 | RX+ | 3 | RX+ |
|  | 4 | - | 4 | - |
|  | 5 | - | 5 | - |
|  | 6 | RX- | 6 | RX- |
|  | 7 | - | 7 | - |
|  | 8 | - | 8 | - |

| CN5 | Pin No. | Signal name | Pin No. | Signal name |
| --- | --- | --- | --- | --- |
| IO connector SCSI-20P | 1 | DO1+ | 11 | DI6 |
|  | 2 | DO1- | 12 | HDI1 |
|  | 3 | DO3+ | 13 | HDI2 |
|  | 4 | DO3- | 14 | DO2+ |
|  | 5 | DI1 | 15 | DO2- |
|  | 6 | DI_COM | 16 | GND |
|  | 7 | DI2 | 17 | PAO+ |
|  | 8 | DI3 | 18 | PAO- |
|  | 9 | DI4 | 19 | PBO+ |
|  | 10 | DI5 | 20 | PBO- |

## 9.  Wiring of PROFINET bus type servo drive

Single/three-phase power input

Noise filter

Resist external noise generated from power circuits

Distribution circuit breaker

When overcurrent occurs, cut off the circuit to protect the power supply circuit

Electromagnetic contactor

Connect/disconnect the main power supply of the drive.  Install coil surge absorber during use

Control circuit power supply

Connect L1C, L2C

Main circuit power supply * 2

Connect L1, L2, L3

Regenerative resistor * 3

External regenerative resistor

Grounding

Connect the motor PE wire and also connect the system grounding

Motor driven U VW

Connect the motor U VW three-phase cable

Brake power supply

24VDC power supply for brake system

Brake contactor

Connect/disconnect the brake power supply

Install coil surge suppressor during use

Servo motor

Note 1:

Model Text:

SV3H Universal Flagship F-Profinet Bus

Note 2:

This legend is illustrated by SIZE-A for single-phase/three-phase 220V,

The main circuit power terminals of the three-phase 380V driver are marked as R, S, and T.

CN 1 servo backend debugging cable

CN 2 functional safety (STO) cable

CN 3 communication input cable

Power indicator light

(Red LED)

Note 3:

SIZE-A does not have a built-in regenerative resistor. When the bus capacitance is insufficient, an external regenerative resistor should be connected between P and B.

The connection terminals of SIZE-B/C/D regenerative resistor are marked as P, RB and B.

Built in regeneration resistor connection: P and RB default to external short circuit.

External regeneration resistor connection method: Remove the short circuit between P and RB, and connect the resistor between P and B.

Note 4:

The servo bus terminals of SIZE-A/B/C servo are connected to P and N.

The bus terminal of SIZE-D servo is connected to P and N 2.  By default, N 1 and N 2 are short circuited. When using an external reactor, remove the short-circuit wire and connect the reactor between N 1 and N 2.

System grounding

CN 5 servo input/output cable

CN 6 motor encoder cable

CN 7 second encoder cable

CN 4 communication output cable

Model silk screen printing * 1

## 10. Definition of PROFINET bus type servo drive port

| CN7 | Pin No. | Signal name |
| --- | --- | --- |
| Second encoder connector 1394-10P | 1 | 5V |
|  | 2 | GND |
|  | 3 | SEC_A+ |
|  | 4 | SEC_A- |
|  | 5 | SEC_B+ |
|  | 6 | SEC_B- |
|  | 7 | SEC_Z+ |
|  | 8 | SEC_Z- |
|  | 9 | - |
|  | 10 | - |

| CN1 | Pin No. | Signal name |
| --- | --- | --- |
| MINI USB | 1 | VBUS |
|  | 2 | D- |
|  | 3 | D+ |
|  | 4 | - |
|  | 5 | GND |

| CN6 | Pin No. | Signal name |
| --- | --- | --- |
| Motor encoder connector 1394-10P | 1 | 5V |
|  | 2 | GND |
|  | 3 | - |
|  | 4 | - |
|  | 5 | - |
|  | 6 | - |
|  | 7 | - |
|  | 8 | - |
|  | 9 | SD+ |
|  | 10 | SD- |

| CN2 | Pin No. | Signal name |
| --- | --- | --- |
| STO connector TE: 2013595-1 | 1 | Internal power supply negative |
|  | 2 | Internal power supply positive |
|  | 3 | STO1- |
|  | 4 | STO1+ |
|  | 5 | STO2- |
|  | 6 | STO2+ |
|  | 7 | STO_OUT- |
|  | 8 | STO_OUT+ |

|  | CN3 (IN) |  | CN4(OUT) |  |
| --- | --- | --- | --- | --- |
|  | Pin No. | Signal name | Pin No. | Signal name |
| PROFINET communication terminal | 1 | TX+ | 1 | TX+ |
|  | 2 | TX- | 2 | TX- |
|  | 3 | RX+ | 3 | RX+ |
|  | 4 | - | 4 | - |
|  | 5 | - | 5 | - |
|  | 6 | RX- | 6 | RX- |
|  | 7 | - | 7 | - |
|  | 8 | - | 8 | - |

| CN5 | Pin No. | Signal name | Pin No. | Signal name |
| --- | --- | --- | --- | --- |
| IO connector SCSI-20P | 1 | DO1+ | 11 | DI6 |
|  | 2 | DO1- | 12 | HDI1 |
|  | 3 | DO3+ | 13 | HDI2 |
|  | 4 | DO3- | 14 | DO2 + |
|  | 5 | DI1 | 15 | DO2- |
|  | 6 | DI_COM | 16 | GND |
|  | 7 | DI2 | 17 | PAO + |
|  | 8 | DI3 | 18 | PAO- |
|  | 9 | DI4 | 19 | PBO + |
|  | 10 | DI5 | 20 | PBO- |

## 11. Definition of servo drive power port

Definition of SIZE-A series power port

|  | Terminal markings; Terminal name. L1C, L2C; Control circuit power input terminal. L1, L2, L3; Main circuit power input terminal. P. N; Servo bus terminal. P. B; Regenerative resistor connection terminal. U. V, W; Servo motor connection terminal. |
| --- | --- |

Definition of SIZE-B series power port

|  | Terminal markings; Terminal name. L1C, L2C; Control circuit power input terminal. L1, L2, L3; Main circuit power input terminal. P. N; Servo bus terminal. P. RB, B; Regenerative resistor connection terminal. U. V, W; Servo motor connection terminal. |
| --- | --- |

Definition of SIZE-C series power port

|  | Terminal markings; Terminal name. L1C, L2C; Control circuit power input terminal. L1, L2, L3; Main circuit power input terminal. P. N; Servo bus terminal. P, RB, B; Regenerative resistor connection terminal. U, V, W; Servo motor connection terminal. PE; Grounding terminal. |
| --- | --- |

Definition of SIZE-D series power port

|  | Terminal markings; Terminal name. L1C, L2C; Control circuit power input terminal. R, S, T; Main circuit power input terminal. P,N2; Servo bus terminal. P, RB, B; Regenerative resistor connection terminal. U, V, W; Servo motor connection terminal. N2, N1; External reactor connection terminal. |
| --- | --- |

# IV. Servo driver debugging software

|  | Visualization |  |  |
| --- | --- | --- | --- |
| Zeroing trajectory configurable |  |  | Tune the scene to match each other |
| Multi segment trajectory can be planned |  |  | Location JOG visualization |

|  | Fault diagnosis |  |  |
| --- | --- | --- | --- |
| Remind fault information in real-time and provide troubleshooting methods. Can trace fault history, support to view parameter information of last 10 historical fault occurrences for easy problem localization. The fault dictionary covers all SV3H series fault information, allowing for quick troubleshooting. |  |  |  |
|  |  |  | Trace historical fault |
| Real time fault diagnosis |  |  | Fault dictionary query |

# V. Wiring diagram for servo drive control mode

## 1. Wiring diagram of position mode control

Internal power supply negative

SV3H series pulse type servo driver

Contactor

Noise filter

Circuit breaker

Single/three-phase power input*

High speed pulse position command

Up to 4MHz

Zeroing completed

Fault output

Internal power supply (do not use)

Internal power supply positive

Motor encoder

Second encoder

Positive limit

Reverse limit

Fault reset

Origin switch

Servo enable

Brake control

Servo operation

STO signal

Switch

External 0V

External+24VDC

Fuse

Switch

Phase A

CCW Phase B

Low-speed pulse position command

CW Phase A

External 24V power port with open collector output

PE shielding layer is connected to the connector housing

CW Phase A

CCW Phase B

Phase B

Phase Z

*The power terminals of the control circuit are marked as L1C and L2C;

Power terminal of main circuit: Single/three-phase 220V terminal is marked as L1, L2, and L3;

Three-phase 380V terminals are marked as R, S, and T.

Encoder Z-phase

Open collector output

Encoder frequency division

Pulse differential output

External 0V

External+24VDC

## 2. Wiring diagram of speed mode control

Internal power supply negative

SV3H series pulse type servo driver

Contactor

Noise filter

Circuit breaker

Single/three-phase power input*

Speed signal input -10~10V

Input impedance is about 9k Ω

Zeroing completed

Fault output

Internal power supply (do not use)

Internal power supply positive

Motor encoder

Second encoder

Positive limit

Reverse limit

Fault reset

Origin switch

Servo enable

Brake control

Servo operation

STO signal

Switch

External 0V

External+24VDC

Fuse

Switch

Phase A

Low pass filter

A/D conversion

External 0V

External+24VDC

PE shielding layer is connected to the connector housing

Torque limit input -10~10V

Input impedance is about 9k Ω

Phase B

Phase Z

*The power terminals of the control circuit are marked as L1C and L2C;

Power terminal of main circuit: Single/three-phase 220V terminal is marked as L1, L2, and L3;

Three-phase 380V terminals are marked as R, S, and T.

Encoder Z-phase

Open collector output

Encoder frequency division

Pulse differential output

Low pass filter

## 3. Wiring diagram of torque mode control

Internal power supply negative

SV3H series pulse type servo driver

Contactor

Noise filter

Circuit breaker

Single/three-phase power input*

Torque signal input -10~10V

Input impedance about 9k Ω

Zeroing completed

Fault output

Internal power supply (do not use)

Internal power supply positive

Motor encoder

Second encoder

Positive limit

Reverse limit

Fault reset

Origin switch

Servo enable

Brake control

Servo operation

STO signal

Switch

External OV

External+24VDC

Fuse

Switch

Phase A

Low pass filter

A/D conversion

External OV

External+24VDC

PE shielding layer is connected to the connector housing

Speed limit input -10~10V

Input impedance is about 9k Ω

Phase B

Phase Z

*The power terminals of the control circuit are marked as L1C and L2C;

Power terminal of main circuit: Single/three-phase 220V terminal is marked as L1, L2, and L3;

Three-phase 380V terminals are marked as R, S, and T.

Encoder Z-phase

Open collector output

Encoder frequency division

Pulse differential output

Low pass filter

## 4. Wiring diagram of  RS485 bus control

SV3H series RS485 servo driver

Contactor

Noise filter

Circuit breaker

Single/three-phase power input*

RS485 communication input

Fault output

Motor encoder

Second encoder

Positive limit

Reverse limit

Origin switch

Brake control

Servo operation

Analog input/output signals

Phase A

Bidirectional 1mA gauge outfit

Bidirectional 1mA gauge outfit

PE shielding layer is connected to the connector housing

RS485 communication output

Phase B

*The power terminals of the control circuit are marked as L1C and L2C;

Power terminal of main circuit: Single/three-phase 220V terminal is marked as L1, L2, and L3;

Three-phase 380V terminals are marked as R, S, and T.

## 5. Wiring diagram of CANopen bus control

SV3H series CANopen servo driver

Contactor

Noise filter

Circuit breaker

Single/three-phase power input*

CANopen communication input

Fault output

Motor encoder

Second encoder

Positive limit

Reverse limit

Origin switch

Brake control

Servo operation

Analog output signals

Phase A

Bidirectional 1mA gauge outfit

Bidirectional 1mA gauge outfit

PE shielding layer is connected to the connector housing

CANopen communication output

Phase B

*The power terminals of the control circuit are marked as L1C and L2C;

Power terminal of main circuit: Single/three-phase 220V terminal is marked as L1, L2, and L3;

Three-phase 380V terminals are marked as R, S, and T.

## 6. Wiring Diagram of EtherCAT Bus Control

Internal power supply negative

SV3H series EtherCAT servo driver

Contactor

Noise filter

Circuit breaker

Single/three-phase power input*

EtherCAT communication input

Fault output

Internal power supply (do not use)

Internal power supply positive

Motor encoder

Second encoder

Positive limit

Reverse limit

Probe 1

Origin switch

Probe 2

Brake control

Servo operation

STO signal

Switch

External 0V

External+24Vdc

Fuse

Switch

Phase A

External 0V

External+24Vdc

PE shielding layer is connected to the connector housing

EtherCAT communication output

Phase B

*The power terminals of the control circuit are marked as L1C and L2C;

Power terminal of main circuit: Single/three-phase 220V terminal is marked as L1, L2, and L3;

Three-phase 380V terminals are marked as R, S, and T.

## 7. Wiring Diagram of PROFINET Bus Control

Internal power supply negative

SV3H series PROFINET servo driver

Contactor

Noise filter

Circuit breaker

Single/three-phase power input*

PROF IN ET communication input

Fault output

Internal power supply (do not use)

Internal power supply positive

Motor encoder

Second encoder

Positive limit

Reverse limit

Probe 1

Origin switch

Probe 2

Brake control

Servo operation

STO signal

Switch

External 0V

External+24Vdc

Switch

Phase A

External 0V

External+24Vdc

PE shielding layer is connected to the connector housing

PROF IN ET communication output

Phase B

*The power terminals of the control circuit are marked as L1C and L2C;

Power terminal of main circuit: Single/three-phase 220V terminal is marked as L1, L2, and L3;

Three-phase 380V terminals are marked as R, S, and T.

# VI. Installation dimensions of servo drive

SIZE-A

Vertical view

Back view

Left view

Set screw: M4*2pcs

Suggested torque:1.2N·m

Front View

SIZE-B

Left view

Back view

Vertical view

Set screw: M4*2pcs

Suggested torque:1.2N·m

Front View

SIZE-C

Left view

Back view

Vertical view

Set screw: M4*3pcs

Suggested torque:1.2N·m

Front View

SIZE-D

Left view

Back view

Vertical view

Set screw: M4*4pcs

Suggested torque:1.2N·m

Front View

# VII. Servo motor products

## 1. Naming rules of  SM3-M2 series servo motors

| ① Product Series | ④ Rated voltage | ⑥ Rated speed | ⑨ Axis connection method |
| --- | --- | --- | --- |
| SM3-M2: SM3-M2 series | S： AC 220V | 15C： 1500RPM | 0: Optical axis |
| Servo motor | T： AC 380V | 20C： 2000RPM 30C： 3000RPM | 1: With keys |
| ② Inertia capacity | ⑤ Rated power | ⑦ Encoder type | ⑩: Brake |
| A: Low inertia | 05B： 50W | M: 17 bit absolute value per cycle | N: Without brake |
| M: Medium inertia | 10B： 100W | N: 17 bit multi cycle absolute value | B: with brake |
| H: High inertia | 20B： 200W 40B： 400W 75B： 750W | O: 23 bit absolute value per cycle P: 23 bit multi cycle absolute value |  |
| ③: Motor flange | 80B： 800W | ⑧: Interface type | ⑪: Oil seal |
| 040:40 flange | 85B： 850W | C: Wire type | 0: No oil seal |
| 060:60 flange | 10C： 1.0kW | H: Aviation plug-in type | 1: There is an oil seal |
| 080:80 flange | 12C： 1.2kW |  |  |
| 130:130 flange | 13C： 1.3kW |  |  |
| 180:180 flange | 15C： 1.5kW 18C： 1.8kW 20C： 2.0KW 22C： 2.2kW 29C： 2.9kW 30C： 3.0kW 44C： 4.4kW 55C： 5.5kW 75C： 7.5kW |  |  |

## 2. Wiring standard of SM3-M2 series servo motor connector

40, 60, 80 flanges

| Encoder connector. View direction. AMP 9P cable side. Pin No.; Function. 1; SD+. 2; SD-. 3; E+. 4; -. 5; -. 6; +5V. 7; GND. E-. 9; PE. | Power connector. View direction. AMP 4P cable side. Pin No.; Function. 1; U. 2; V. 3; W. 4; PE. | Brake connector. View direction. AMP 2P cable side. Pin No.; Function. 1; BK+. 2; BK-. |
| --- | --- | --- |
| 130 Flange |  |  |
| Encoder connector. View direction. YDB28K7TSLa cable side. Pin No.; Function. 1; PE. 2; E-. 3; E+. 4; SD-. 5; GND. 6; SD+. 7; +5V. | Power connector (without brake). View direction. YD28K4TSL cable side. Pin No.; Function. 1; PE. 2; U. 3; V. 4; W. | Power connector (with brake). View direction. YD28K7TSL cable side. Pin No.; Function. 1; PE. 2; U. 3; V. 4; W. 5; BK+. 6; BK-. 7; -. |
| 180 Flange |  |  |
| Encoder connector. View direction. YDB28K7TSLa cable side. Pin No.; Function. 1; PE. 2; E-. 3; E+. 4; SD-. 5; GND. 6; SD+. 7; +5V. | Power connector (without brake). View direction. YD32K4TSL cable side. Pin No.; Function. 1; PE. 2; U. 3; V. 4; W. | Brake connector. View direction. XS16K4TM cable side. Pin No.; Function. 1; BK+. 2; BK-. 3; -. 4; -. |

## 3. Technical specifications of SM3-M2H040 servo motor 220V/(50W-100W)

| Motor specifications | SM3-M2H040- S05B30C-xxxxx | SM3-M2H040- S10B30C-xxxxx |
| --- | --- | --- |
| Rated power (W) | 50 | 100 |
| Rated voltage (V) | 220 | 220 |
| Rated current (A) | 0.6 | 1.0 |
| Rated torque (N.m) | 0.16 | 0.32 |
| Rated speed (r/min) | 3000 | 3000 |
| Instantaneous maximum current (A) | 1.8 | 3.0 |
| Instantaneous maximum torque (N.m) | 0.48 | 0.96 |
| Instantaneous maximum speed (r/min) | 6000 | 6000 |
| Reverse electromotive force (V/1000r/min) | 18 | 19 |
| Torque coefficient (N.m/A) | 0.27 | 0.32 |
| Phase resistance (Ω) | 13 | 7.25 |
| Q-axis inductance (mH) | 10 | 6.85 |
| D-axis inductance (mH) | 10 | 6.85 |
| Electrical time constant (ms) | 0.77 | 0.94 |
| Rotor inertia (kg. m2 X 10-4) | 0.035 (0.038) | 0.05 (0.053) |
| Weight (kg) | 0.4 (0.9) | 0.5 (1.0) |
| Extreme logarithm | 5 pairs of poles |  |
| Motor insulation grade | F |  |
| Protection level | IP65 |  |
| Usage environment | Environmental temperature: -20 ℃ - 40 ℃; Environmental humidity: Relative humidity < 90% (No frost) |  |
| Note: () represents the parameters of the motor with holding brake. |  |  |

Continuous work area

Torque [N m]

Instantaneous work area

Continuous work area

Torque [N m]

Instantaneous work area

Speed [r/min]

Speed [r/min]

Torque characteristic diagram:

| SM3-M2H040-S05B30C-xxxxx | SM3-M2H040-S10B30C-xxxxx |
| --- | --- |

Outline dimension diagram:

| Outline dimension diagram | 50W | 100W |
| --- | --- | --- |
| L without brake | 68.5 | 79.5 |
| L with brake | 101.5 | 112.5 |

<Without brake>																	<With brake>

## 4. Technical specifications of SM3-M2H060 servo motor 220V/(200W-400W)

| Motor specifications | SM3-M2H060- S20B30C-xxxxx | SM3-M2H060- S40B30C-xxxxx |
| --- | --- | --- |
| Rated power (W) | 200 | 400 |
| Rated voltage (V) | 220 | 220 |
| Rated current (A) | 1.4 | 2.8 |
| Rated torque (N.m) | 0.64 | 1.27 |
| Rated speed (r/min) | 3000 | 3000 |
| Instantaneous maximum current (A) | 4.2 | 8.4 |
| Instantaneous maximum torque (N.m) | 1.92 | 3.81 |
| Instantaneous maximum speed (r/min) | 6000 | 6000 |
| Reverse electromotive force (V/1000r/min) | 31.7 | 31.4 |
| Torque coefficient (N.m/A) | 0.46 | 0.45 |
| Phase resistance (Ω) | 4 | 1.85 |
| Q-axis inductance (mH) | 7.5 | 3.8 |
| D-axis inductance (mH) | 7.5 | 3.8 |
| Electrical time constant (ms) | 1.88 | 2.05 |
| Rotor inertia (kg. m2 X 10-4) | 0.29 (0.32) | 0.53 (0.56) |
| Weight (kg) | 1.0(1.3) | 1.3 (1.7) |
| Extreme logarithm | 5 pairs of poles |  |
| Motor insulation grade | F |  |
| Protection level | IP65 |  |
| Usage environment | Environmental temperature: -20 ℃ -40 ℃; Environmental humidity: Relative humidity<90% (No frost) |  |
| Note: () represents the parameters of the motor with holding brake. |  |  |

- Torque characteristic diagram:

Continuous work area

Torque [N m]

Instantaneous work area

Continuous work area

Torque [N m]

Instantaneous work area

Speed [r/min]

Speed [r/min]

| SM3-M2H060-S20B30C-xxxxx | SM3-M2H060-S40B30C-xxxx |
| --- | --- |

- Outline dimension diagram:

| Outline dimension diagram | 200W | 400W |
| --- | --- | --- |
| L without brake | 77.2 | 93.7 |
| L with brake | 109.2 | 125.7 |

<Without brake>																	<With brake>

## 5. Technical specifications of SM3-M2H080 servo motor 220V/(750W-1000W)

| Motor specifications | SM3-M2H080- S75B30C-xxxxx | SM3-M2H080- S10C30C-xxxxx |
| --- | --- | --- |
| Rated power (W) | 750 | 1000 |
| Rated voltage (V) | 220 | 220 |
| Rated current (A) | 3.8 | 5.5 |
| Rated torque (N.m) | 2.4 | 3.2 |
| Rated speed (r/min) | 3000 | 3000 |
| Instantaneous maximum current (A) | 11.4 | 16.5 |
| Instantaneous maximum torque (N.m) | 7.2 | 9.6 |
| Instantaneous maximum speed (r/min) | 6000 | 6000 |
| Reverse electromotive force (V/1000r/min) | 38 | 38 |
| Torque coefficient (N.m/A) | 0.63 | 0.6 |
| Phase resistance (Ω) | 0.665 | 0.55 |
| Q-axis inductance (mH) | 2.8 | 2.4 |
| D-axis inductance (mH) | 2.8 | 2.4 |
| Electrical time constant (ms) | 4.2 | 4.4 |
| Rotor inertia (kg. m2 X 10-4) | 1.62 (1.72) | 2.1 (2.2) |
| Weight (kg) | 2.5 (3.5) | 3.2 (4.2) |
| Extreme logarithm | 5 pairs of poles |  |
| Motor insulation grade | F |  |
| Protection level | IP65 |  |
| Usage environment | Environmental temperature: -20 ℃ -40 ℃; Environmental humidity: Relative humidity<90% (No frost) |  |
| Note: () represents the parameters of the motor with holding brake. |  |  |

- Torque characteristic diagram:

Continuous work area

Torque [N m]

Instantaneous work area

Continuous work area

Torque [N m]

Instantaneous work area

Speed [r/min]

Speed [r/min]

| SM3-M2H080-S75B30C-xxxxx | SM3-M2H080-S10B30C-xxxxx |
| --- | --- |

- Outline dimension diagram:

| Outline dimension diagram | 750W | 1000W |
| --- | --- | --- |
| L without brake | 105 | 119 |
| L with brake | 142 | 156 |

<Without brake> 																	<With brake>

## 6. Technical specifications of SM3-M2H130 servo motor 220V/380V/(850W-2200W)

| Motor specifications | SM3-M2H130- x 85B15C-xxxxx |  | SM3-M2H130- x 13C15C-xxxxx |  | SM3-M2H130- x 18C15C-xxxxx |  | SM3-M2H130- x 22C15C-xxxxx |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rated power (W) | 850 |  | 1300 |  | 1800 |  | 2200 |  |
| Rated voltage (V) | 220 | 380 | 220 | 380 | 220 | 380 | 220 | 380 |
| Rated current (A) | 6.9 | 4 | 10.7 | 6 | 13.8 | 8.5 | 14.5 | 11 |
| Rated torque (N.m) | 5.39 | 5.39 | 8.34 | 8.34 | 11.5 | 11.5 | 14.3 | 14.3 |
| Rated speed (r/min) | 1500 | 1500 | 1500 | 1500 | 1500 | 1500 | 1500 | 1500 |
| Instantaneous maximum current (A) | 20.7 | 12 | 32.1 | 18 | 41.4 | 25.5 | 40.6 | 30.7 |
| Instantaneous maximum torque (N.m) | 16.17 | 16.17 | 25.02 | 25.02 | 34.5 | 34.5 | 40 | 40 |
| Instantaneous maximum speed (r/min) | 3000 | 3000 | 3000 | 3000 | 3000 | 3000 | 3000 | 3000 |
| Reverse electromotive force (V/1000r/min) | 50 | 85 | 52 | 92 | 52 | 90 | 67 | 90 |
| Torque coefficient (N.m/A) | 0.78 | 1.35 | 0.78 | 1.39 | 0.83 | 1.35 | 0.98 | 1.3 |
| Phase resistance (Ω) | 0.5 | 1.3 | 0.25 | 0.77 | 0.175 | 0.6 | 0.3 | 0.425 |
| Q-axis inductance (mH) | 2.5 | 8.1 | 1.6 | 5.25 | 1.25 | 4.15 | 1.75 | 2.8 |
| D-axis inductance (mH) | 2.5 | 8.1 | 1.6 | 5.25 | 1.25 | 4.15 | 1.75 | 2.8 |
| Electrical time constant (ms) | 5 | 6.2 | 6.4 | 6.8 | 7.14 | 6.9 | 5.8 | 6.5 |
| Rotor inertia(kg. m2 X 10-4) | 10.9 (12.13) |  | 16.9 (18.13) |  | 21.4 (22.63) |  | 27.1 (28.33) |  |
| Weight (kg) | 5.7 (7.4) |  | 8.0 (9.1) |  | 9.3 (11.3) |  | 10.5 (12.7) |  |
| Extreme logarithm | 5 pairs of poles |  |  |  |  |  |  |  |
| Motor insulation grade | F |  |  |  |  |  |  |  |
| Protection level | IP65 |  |  |  |  |  |  |  |
| Usage environment | Environmental temperature: -20 ℃ -40 ℃; Environmental humidity: Relative humidity<90% (No frost) |  |  |  |  |  |  |  |
| Note: () represents the parameters of the motor with holding brake. |  |  |  |  |  |  |  |  |

Continuous work area

Torque [N m]

Instantaneous work area

Continuous work area

Torque [N m]

Instantaneous work area

Speed [r/min]

Speed [r/min]

Continuous work area

Torque [N m]

Instantaneous work area

Continuous work area

Torque [N m]

Instantaneous work area

Speed [r/min]

Speed [r/min]

Torque characteristic diagram:

| SM3-M2H130-x85B15C- | SM3-M2H130-x13C15C- | SM3-M2H130-x18C15C- | SM3-M2H130-x22C15C- |
| --- | --- | --- | --- |

- Outline dimension diagram:

| Outline dimension diagram | 850W | 1300W | 1800W | 2200W |
| --- | --- | --- | --- | --- |
| L without brake | 135 | 152.5 | 170 | 200 |
| L with brake | 187 | 204.5 | 222 | 252 |

<Without brake>																	<With brake>

## 7. Technical specifications of SM3-M2H180 servo motor 380V/(2.9kW-4.4kW)

| Motor specifications | SM3-M2H180- T29C15C-xxxxx | SM3-M2H180- T44C15C-xxxxx |
| --- | --- | --- |
| Rated power (kW) | 2.9 | 4.4 |
| Rated voltage (V) | 380 | 380 |
| Rated current (A) | 11.9 | 16.5 |
| Rated torque (N.m) | 18.6 | 28.4 |
| Rated speed (r/min) | 1500 | 1500 |
| Instantaneous maximum current (A) | 35.7 | 49.5 |
| Instantaneous maximum torque (N.m) | 55.8 | 85.2 |
| Instantaneous maximum speed (r/min) | 3000 | 3000 |
| Reverse electromotive force (V/1000r/min) | 100 | 104 |
| Torque coefficient (N.m/A) | 1.56 | 1.72 |
| Phase resistance (Ω) | 0.175 | 0.155 |
| Q-axis inductance (mH) | 2.15 | 1.85 |
| D-axis inductance (mH) | 2.15 | 1.85 |
| Electrical time constant (ms) | 12.3 | 11.9 |
| Rotor inertia (kg. m2 X 10-4) | 63.5 (69.5) | 88.5 (94.5) |
| Weight (kg) | 16.7 (21.2) | 21.1 (25.6) |
| Extreme logarithm | 5 pairs of poles |  |
| Motor insulation grade | F |  |
| Protection level | IP65 |  |
| Usage environment | Environmental temperature: -20 ℃ -40 ℃; Environmental humidity: Relative humidity<90% (No frost) |  |
| Note: () represents the parameters of the motor with holding brake. |  |  |

Continuous work area

Torque [N m]

Instantaneous work area

Continuous work area

Torque [N m]

Instantaneous work area

Speed [r/min]

Speed [r/min]

Torque characteristic diagram:

| SM3-M2H180-T29C15C- | SM3-M2H180-T44C15C- |
| --- | --- |

- Outline dimension diagram:

| Outline dimension diagram | 2.9KW | 4.4KW |
| --- | --- | --- |
| L without brake | 205 | 232 |
| L with brake | 252 | 279 |

<Without brake>																	<With brake>

## 8. Technical specifications of SM3-M2H180 servo motor 380V/(5.5kW-7.5kW)

| Motor specifications | SM3-M2H180- T55C15C-xxxxx | SM3-M2H180- T75C15C-xxxxx |
| --- | --- | --- |
| Rated power (kW) | 5.5 | 7.5 |
| Rated voltage (V) | 380 | 380 |
| Rated current (A) | 20.8 | 26 |
| Rated torque (N.m) | 35 | 48 |
| Rated speed (r/min) | 1500 | 1500 |
| Instantaneous maximum current (A) | 62.4 | 65 |
| Instantaneous maximum torque (N.m) | 105 | 120 |
| Instantaneous maximum speed (r/min) | 3000 | 3000 |
| Reverse electromotive force (V/1000r/min) | 100 | 106 |
| Torque coefficient (N.m/A) | 1.68 | 1.85 |
| Phase resistance (Ω) | 0.09 | 0.095 |
| Q-axis inductance (mH) | 1.15 | 1.15 |
| D-axis inductance (mH) | 1.15 | 1.15 |
| Electrical time constant (ms) | 11.9 | 12.1 |
| Rotor inertia (kg. m2 X 10-4) | 114.4 (120.4) | 136.6 (142.6) |
| Weight (kg) | 25.6 (30.1) | 30.8 (35.3) |
| Extreme logarithm | 5 pairs of poles |  |
| Motor insulation grade | F |  |
| Protection level | IP65 |  |
| Usage environment | Environmental temperature: -20 ℃ -40 ℃; Environmental humidity: Relative humidity<90% (No frost) |  |
| Note: () represents the parameters of the motor with holding brake. |  |  |

Continuous work area

Torque [N m]

Instantaneous work area

Continuous work area

Torque [N m]

Instantaneous work area

Speed [r/min]

Speed [r/min]

Torque characteristic diagram:

| SM3-M2H180-T55C15C-xxxxx | SM3-M2H180-T75C15C-xxxxx |
| --- | --- |

- Outline dimension diagram:

| Outline dimension diagram | 5.5KW | 7.5KW |
| --- | --- | --- |
| L without brake | 260 | 284 |
| L with brake | 307 | 331 |

<Without brake>							<With brake> 						<With brake>

## 9. Technical specifications of SM3-M2A130 servo motor 220V/380V/(1kW-3kW)

| Motor specifications | SM3-M2A130- x 10C20C-xxxxx |  | SM3-M2A130- x 15C20C-xxxxx |  | SM3-M2A130- x 20C20C-xxxxx |  | SM3-M2A130- x 30C20C-xxxxx |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rated power (kW) | 1 |  | 1.5 |  | 2 |  | 3 |  |
| Rated voltage (V) | 220 | 380 | 220 | 380 | 220 | 380 | 220 | 380 |
| Rated current (A) | 5.8 | 3.5 | 8.0 | 4.7 | 10.2 | 6.2 | 16.5 | 11 |
| Rated torque (N.m) | 4.77 | 4.77 | 7.16 | 7.16 | 9.55 | 9.55 | 14.3 | 14.3 |
| Rated speed (r/min) | 2000 | 2000 | 2000 | 2000 | 2000 | 2000 | 2000 | 2000 |
| Instantaneous maximum current (A) | 11.6 | 7 | 16.0 | 9.4 | 20.4 | 12.4 | 33.0 | 22 |
| Instantaneous maximum torque (N.m) | 9.6 | 9.6 | 14.32 | 14.32 | 19.1 | 19.1 | 28.6 | 28.6 |
| Instantaneous maximum speed (r/min) | 3000 | 3000 | 3000 | 3000 | 3000 | 3000 | 3000 | 3000 |
| Reverse electromotive force (V/1000r/min) | 53 | 89 | 58.5 | 98 | 60 | 100 | 55 | 89 |
| Torque coefficient (N.m/A) | 0.82 | 1.36 | 0.92 | 1.52 | 0.94 | 1.54 | 0.87 | 1.3 |
| Phase resistance (Ω) | 0.425 | 1.2 | 0.325 | 0.92 | 0.29 | 0.645 | 0.15 | 0.4 |
| Q-axis inductance (mH) | 6.25 | 17.75 | 4.75 | 14.75 | 3.75 | 11.75 | 1.585 | 7 |
| D-axis inductance (mH) | 6.25 | 17.75 | 4.75 | 14.75 | 3.75 | 11.75 | 1.585 | 7 |
| Electrical time constant (ms) | 14.7 | 10.5 | 14.6 | 12 | 12.9 | 12.5 | 10.56 | 16.09 |
| Rotor inertia(kg. m2 X 10-4) | 6.2 (7.41) |  | 9.16 (10.39) |  | 12.3 (13.33) |  | 18.6 (19.83) |  |
| Weight (kg) | 5.4 (7.4) |  | 7.1 (9.1) |  | 8.3 (10.3) |  | 10.7 (12.7) |  |
| Extreme logarithm | 5 pairs of poles |  |  |  |  |  |  |  |
| Motor insulation grade | F |  |  |  |  |  |  |  |
| Protection level | IP65 |  |  |  |  |  |  |  |
| Usage environment | Environmental temperature: -20 ℃ -40 ℃; Environmental humidity: Relative humidity<90% (No frost) |  |  |  |  |  |  |  |
| Note: () represents the parameters of the motor with holding brake. |  |  |  |  |  |  |  |  |

Continuous work area

Torque [N m]

Instantaneous work area

Continuous work area

Torque [N m]

Instantaneous work area

Speed [r/min]

Speed [r/min]

Continuous work area

Torque [N m]

Instantaneous work area

Continuous work area

Torque [N m]

Instantaneous work area

Speed [r/min]

Speed [r/min]

Torque characteristic diagram:

| SM3-M2A130-x10C20C- | SM3-M2A130-x15C20C- | SM3-M2A130-x20C20C- | SM3-M2A130-x30C20C- |
| --- | --- | --- | --- |

- Outline dimension diagram:

| Outline dimension diagram | 1kW | 1.5kW | 2kW | 3kW |
| --- | --- | --- | --- | --- |
| L without brake | 135 | 152.5 | 170 | 200 |
| L with brake | 187 | 204.5 | 222 | 252 |

<Without brake>																	<With brake>

## 10. Configuration specification of  SM3-M2H040 servo motor 220V/(50W-100W)

| Motor power | inertia | Motor model | 23bit |  | 17bit |  | Equipped with brakes |  | Oil seal | Flange | Shaft diameter | Outgoing form | Rated voltage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Single circle | Multiple circles | Single circle | Multiple circles | yes | no |  |  |  |  |  |
| 50W | High inertia | SM3-M2H040-S05B30C-MC1N0 |  |  | ● |  |  | ● |  | 40 | Φ8 | Wire type | AC220V |
|  |  | SM3-M2H040-S05B30C-NC1N0 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H040-S05B30C-OC1N0 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H040-S05B30C-PC1N0 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H040-S05B30C-MC1B0 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H040-S05B30C-NC1B0 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H040-S05B30C-OC1B0 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H040-S05B30C-PC1B0 |  | ● |  |  |  |  |  |  |  |  |  |
| 100 W | High inertia | SM3-M2H040-S10B30C-MC1N0 |  |  | ● |  |  | ● |  | 40 | Φ8 | Wire type | AC220V |
|  |  | SM3-M2H040-S10B30C-NC1N0 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H040-S10B30C-OC1N0 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H040-S10B30C-PC1N0 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H040-S10B30C-MC1B0 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H040-S10B30C-NC1B0 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H040-S10B30C-OC1B0 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H040-S10B30C-PC1B0 |  | ● |  |  |  |  |  |  |  |  |  |

## 11. SM3-M2H040 servo motor and SV3H servo drive selection comparison table 220V/(50W-100W)

| Motor power | inertia | Motor model | Servo drive model |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pulse | EtherCAT | PROFINET | CANopen | RS485 | Power specifications |
| 50W | High inertia | SM3-M2H040-S05B30C-MC1N1 | Standard type: SV3H-PS001 Optional second encoder: SV3H-PS001C Second encoder+STO: SV3H-PS001SC | Standard type: SV3H-ES001 Optional second encoder: SV3H-ES001C Second encoder+STO: SV3H-ES001SC | Standard type: SV3H-FS001 Optional second encoder: SV3H-FS001C Second encoder+STO: SV3H-FS001SC | Standard type: SV3H-CS001 Optional second encoder: SV3H-CS001C Second encoder+analog: SV3H-CS001AC | Standard type: SV3H-RS001 Optional second encoder: SV3H-RS001C Second encoder+analog: SV3H-RS001AC | Single/three-phase AC220V |
|  |  | SM3-M2H040-S05B30C-NC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H040-S05B30C-OC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H040-S05B30C-PC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H040-S05B30C-MC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H040-S05B30C-NC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H040-S05B30C-OC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H040-S05B30C-PC1B1 |  |  |  |  |  |  |
| 100W | High inertia | SM3-M2H040-S10B30C-MC1N1 | Standard type: SV3H-PS001 Optional second encoder: SV3H-PS001C Second encoder+STO: SV3H-PS001SC | Standard type: SV3H-ES001 Optional second encoder: SV3H-ES001C Second encoder+STO: SV3H-ES001SC | Standard type: SV3H-FS001 Optional second encoder: SV3H-FS001C Second encoder+STO: SV3H-FS001SC | Standard type: SV3H-CS001 Optional second encoder: SV3H-CS001C Second encoder+analog: SV3H-CS001AC | Standard type: SV3H-RS001 Optional second encoder: SV3H-RS001C Second encoder+analog: SV3H-RS001AC | Single/three-phase AC220V |
|  |  | SM3-M2H040-S10B30C-NC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H040-S10B30C-OC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H040-S10B30C-PC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H040-S10B30C-MC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H040-S10B30C-NC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H040-S10B30C-OC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H040-S10B30C-PC1B1 |  |  |  |  |  |  |

## 12.  Configuration specification of SM3-M2H060 servo motor 220V/(200W-400W)

| Motor power | inertia | Motor model | 23bit |  | 17bit |  | Equipped with brakes |  | Oil seal | Flange | Shaft diameter | Outgoing form | Rated voltage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Single circle | Multiple circles | Single circle | Multiple circles | yes | no |  |  |  |  |  |
| 200 W | High inertia | SM3-M2H060-S20B30C-MC1N1 |  |  | ● |  |  | ● | ● | 60 | Φ14 | Wire type | AC220V |
|  |  | SM3-M2H060-S20B30C-NC1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H060-S20B30C-OC1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H060-S20B30C-PC1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H060-S20B30C-MC1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H060-S20B30C-NC1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H060-S20B30C-OC1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H060-S20B30C-PC1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 400 W | High inertia | SM3-M2H060-S40B30C-MC1N1 |  |  | ● |  |  | ● | ● | 60 | Φ14 | Wire type | AC220V |
|  |  | SM3-M2H060-S40B30C-NC1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H060-S40B30C-OC1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H060-S40B30C-PC1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H060-S40B30C-MC1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H060-S40B30C-NC1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H060-S40B30C-OC1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H060-S40B30C-PC1B1 |  | ● |  |  |  |  |  |  |  |  |  |

## 13. SM3-M2H060 servo motor and SV3H servo drive selection comparison table 220V/(200W-400W)

| Motor power | inertia | Motor model | Servo drive model |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pulse | EtherCAT | PROFINET | CANopen | RS485 | Power specifications |
| 200W | High inertia | SM3-M2H060-S20B30C-MC1N1 | Standard type: SV3H-PS1R6 Optional second encoder: SV3H-PS1R6C Second encoder+STO: SV3H-PS1R6SC | Standard type: SV3H-ES1R6 Optional second encoder: SV3H-ES1R6C Second encoder+STO: SV3H-ES1R6SC | Standard type: SV3H-FS1R6 Optional second encoder: SV3H-FS1R6C Second encoder+STO: SV3H-FS1R6SC | Standard type: SV3H-CS1R6 Optional second encoder: SV3H-CS1R6C Second encoder+analog signal SV3H-CS1R6AC | Standard type: SV3H-RS1R6 Optional second encoder: SV3H-RS1R6C Second encoder+analog: SV3H-RS1R6AC | Single/three-phase AC220V |
|  |  | SM3-M2H060-S20B30C-NC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H060-S20B30C-OC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H060-S20B30C-PC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H060-S20B30C-MC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H060-S20B30C-NC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H060-S20B30C-OC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H060-S20B30C-PC1B1 |  |  |  |  |  |  |
| 400W | High inertia | SM3-M2H060-S40B30C-MC1N1 | Standard type: SV3H-PS2R8 Optional second encoder: SV3H-PS2R8C Second encoder+STO: SV3H-PS2R8SC | Standard type: SV3H-ES2R8 Optional second encoder: SV3H-ES2R8C Second encoder+STO: SV3H-ES2R8SC | Standard type: SV3H-FS2R8 Optional second encoder: SV3H-FS2R8C Second encoder+STO: SV3H-FS2R8SC | Standard type: SV3H-CS2R8 Optional second encoder: SV3H-CS2R8C Second encoder+analog: SV3H-CS2R8AC | Standard type: SV3H-RS2R8 Optional second encoder: SV3H-RS2R8C Second encoder+analog: SV3H-RS2R8AC | Single/three-phase AC220V |
|  |  | SM3-M2H060-S40B30C-NC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H060-S40B30C-OC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H060-S40B30C-PC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H060-S40B30C-MC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H060-S40B30C-NC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H060-S40B30C-OC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H060-S40B30C-PC1B1 |  |  |  |  |  |  |

## 14. SM3-M2H080 servo motor Configuration specification of 220V/(750W-1000W)

| Motor power | Inertia | Motor model | 23bit |  | 17bit |  | Equipped with brakes |  | Oil seal | Flange | Shaft diameter | Outgoing form | Rated voltage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Single circle | Multiple circles | Single circle | Multiple circles | Yes | no |  |  |  |  |  |
| 750W | High inertia | SM3-M2H080-S75B30C-MC1N1 |  |  | ● |  |  | ● | ● | 80 | Φ19 | Wire type | AC220V |
|  |  | SM3-M2H080-S75B30C-NC1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H080-S75B30C-OC1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H080-S75B30C-PC1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H080-S75B30C-MC1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H080-S75B30C-NC1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H080-S75B30C-OC1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H080-S75B30C-PC1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 1000W | High inertia | SM3-M2H080-S10C30C-MC1N1 |  |  | ● |  |  | ● | ● | 80 | Φ19 | Wire type | AC220V |
|  |  | SM3-M2H080-S10C30C-NC1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H080-S10C30C-OC1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H080-S10C30C-PC1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H080-S10C30C-MC1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H080-S10C30C-NC1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H080-S10C30C-OC1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H080-S10C30C-PC1B1 |  | ● |  |  |  |  |  |  |  |  |  |

## 15. SM3-M2H080 servo motor and SV3H servo drive selection comparison table 220V/(750W-1000W)

| Motor power | inertia | Motor model | Servo drive model |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pulse | EtherCAT | PROFINET | CANopen | RS485 | Power specifications |
| 750W | High inertia | SM3-M2H080-S75B30C-MC1N1 | Standard type: SV3H-PS5R5 Optional second encoder: SV3H-PS5R5C Second encoder+STO: SV3H-PS5R5SC | Standard type: SV3H-ES5R5 Optional second encoder: SV3H-ES5R5C Second encoder+STO: SV3H-ES5R5SC | Standard type: SV3H-FS5R5 Optional second encoder: SV3H-FS5R5C Second encoder+STO: SV3H-FS5R5SC | Standard type: SV3H-CS5R5 Optional second encoder: SV3H-CS5R5C Second encoder+analog: SV3H-CS5R5AC | Standard type: SV3H-RS5R5 Optional second encoder: SV3H-RS5R5C Second encoder+analog: SV3H-RS5R5AC | Single/three-phase AC220V |
|  |  | SM3-M2H080-S75B30C-NC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H080-S75B30C-OC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H080-S75B30C-PC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H080-S75B30C-MC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H080-S75B30C-NC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H080-S75B30C-OC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H080-S75B30C-PC1B1 |  |  |  |  |  |  |
| 1000W | High inertia | SM3-M2H080-S10C30C-MC1N1 | Standard type: SV3H-PS7R6 Optional second encoder: SV3H-PS7R6C Second encoder+STO: SV3H-PS7R6SC | Standard type: SV3H-ES7R6 Optional second encoder: SV3H-ES7R6C Second encoder+STO: SV3H-ES7R6SC | Standard type: SV3H-FS7R6 Optional second encoder: SV3H-FS7R6C Second encoder+STO: SV3H-FS7R6SC | Standard type: SV3H-CS7R6 Optional second encoder: SV3H-CS7R6C Second encoder+analog: SV3H-CS7R6AC | Standard type: SV3H-RS7R6 Optional Second Encoder: SV3H-RS7R6C Second encoder+analog: SV3H-RS7R6AC | Single/three-phase AC220V |
|  |  | SM3-M2H080-S10C30C-NC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H080-S10C30C-OC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H080-S10C30C-PC1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H080-S10C30C-MC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H080-S10C30C-NC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H080-S10C30C-OC1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H080-S10C30C-PC1B1 |  |  |  |  |  |  |

## 16. SM3-M2H130 servo motor configuration specification 220V/(850W-2200W)

| Motor power | Inertia | Motor model | 23bit |  | 17bit |  | Equipped with brakes |  | Oil seal | Flange | Shaft diameter | Outgoing form | Rated voltage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Single circle | Multiple circles | Single circle | Multiple circles | yes | no |  |  |  |  |  |
| 850W | High inertia | SM3-M2H130-S85B15C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation plug | AC220V |
|  |  | SM3-M2H130-S85B15C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S85B15C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S85B15C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S85B15C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H130-S85B15C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S85B15C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S85B15C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 1300 W | High inertia | SM3-M2H130-S13C15C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation plug | AC220V |
|  |  | SM3-M2H130-S13C15C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S13C15C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S13C15C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S13C15C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H130-S13C15C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S13C15C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S13C15C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 1800 W | High inertia | SM3-M2H130-S18C15C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation plug | AC220V |
|  |  | SM3-M2H130-S18C15C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S18C15C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S18C15C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S18C15C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H130-S18C15C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S18C15C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S18C15C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 2200 W | High inertia | SM3-M2H130-S22C15C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation plug | AC220V |
|  |  | SM3-M2H130-S22C15C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S22C15C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S22C15C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S22C15C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H130-S22C15C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S22C15C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-S22C15C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |

## 17. SM3-M2H130 servo motor and SV3H servo driver selection comparison table 220V/(850W-2200W)

| Motor power | inertia | Motor model | Servo drive model |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pulse | EtherCAT | PROFINET | CANopen | RS485 | Power specifications |
| 850W | High inertia | SM3-M2H130-S85B15C-MH1N1 | Standard type: SV3H-PS7R6 Optional second encoder: SV3H-PS7R6C Second encoder+STO: SV3H-PS7R6SC | Standard type: SV3H-ES7R6 Optional second encoder: SV3H-ES7R6C Second encoder+STO: SV3H-ES7R6SC | Standard type: SV3H-FS7R6 Optional second encoder: SV3H-FS7R6C Second encoder+STO: SV3H-FS7R6SC | Standard type: SV3H-CS7R6 Optional second encoder: SV3H-CS7R6C Second encoder+analog: SV3H-CS7R6AC | Standard type: SV3H-RS7R6 Optional Second Encoder: SV3H-RS7R6C Second Encoder+Analog Quantity: SV3H- RS7R6AC | Single/three-phase AC220V |
|  |  | SM3-M2H130-S85B15C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S85B15C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S85B15C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S85B15C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S85B15C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S85B15C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S85B15C-PH1B1 |  |  |  |  |  |  |
| 1300W | High inertia | SM3-M2H130-S13C15C-MH1N1 | Standard type: SV3H-PS012 Optional second encoder: SV3H-PS012C Second encoder+STO: SV3H-PS012SC | Standard type: SV3H-ES012 Optional second encoder: SV3H-ES012C Second encoder+STO: SV3H-ES012SC | Standard type: SV3H-FS012 Optional second encoder: SV3H-FS012C Second encoder+STO: SV3H-FS012SC | Standard type: SV3H-CS012 Optional second encoder: SV3H-CS012C Second encoder+analog: SV3H-CS012AC | Standard type: SV3H-RS012 Optional Second Encoder: SV3H-RS012C Second Encoder+Analog Quantity: SV3H-RS012AC | Single/three-phase AC220V |
|  |  | SM3-M2H130-S13C15C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S13C15C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S13C15C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S13C15C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S13C15C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S13C15C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S13C15C-PH1B1 |  |  |  |  |  |  |
| 1800W | High inertia | SM3-M2H130-S18C15C-MH1N1 | Standard type: SV3H-PS014 Optional Second Encoder: SV3H-PS014C Second encoder+STO: SV3H-PS014SC | Standard type: SV3H-ES014 Optional second encoder: SV3H-ES014C Second encoder+STO: SV3H-ES014SC | Standard type: SV3H-FS014 Optional second encoder: SV3H-FS014C Second encoder+STO: SV3H-FS014SC | Standard type: SV3H-CS014 Optional second encoder: SV3H-CS014C Second encoder+analog: SV3H-CS014AC | Standard type: SV3H-RS014 Optional Second Encoder: SV3H-RS014C Second Encoder+Analog Quantity: SV3H-RS014AC | Single/three-phase AC220V |
|  |  | SM3-M2H130-S18C15C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S18C15C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S18C15C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S18C15C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S18C15C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S18C15C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S18C15C-PH1B1 |  |  |  |  |  |  |
| 2200W | High inertia | SM3-M2H130-S22C15C-MH1N1 | Standard type: SV3H-PS014 Optional Second Encoder: SV3H-PS014C Second encoder+STO: SV3H-PS014SC | Standard type: SV3H-ES014 Optional second encoder: SV3H-ES014C Second encoder+STO: SV3H-ES014SC | Standard type: SV3H-FS014 Optional second encoder: SV3H-FS014C Second encoder+STO: SV3H-FS014SC | Standard type: SV3H-CS014 Optional second encoder: SV3H-CS014C Second encoder+analog Quantity: SV3H-CS014AC | Standard type: SV3H-RS014 Optional second encoder: SV3H-RS014C Second encoder+module Proposed quantity: SV3H-RS014AC | Single/three-phase AC220V |
|  |  | SM3-M2H130-S22C15C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S22C15C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S22C15C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S22C15C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S22C15C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S22C15C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-S22C15C-PH1B1 |  |  |  |  |  |  |

## 18. Configuration specification of SM3-M2H130 servo motor 380V/(850W-2200W)

| Motor power | Inertia | Motor model | 23bit |  | 17bit |  | Equipped with brakes |  | Oil seal | Flange | Shaft diameter | Outgoing form | Rated voltage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Single circle | Multiple circles | Single circle | Multiple circles | Yes | No |  |  |  |  |  |
| 850W | High inertia | SM3-M2H130-T85B15C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation plug | AC380V |
|  |  | SM3-M2H130-T85B15C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T85B15C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T85B15C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T85B15C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H130-T85B15C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T85B15C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T85B15C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 1300 W | High inertia | SM3-M2H130-T13C15C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation plug | AC380V |
|  |  | SM3-M2H130-T13C15C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T13C15C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T13C15C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T13C15C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H130-T13C15C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T13C15C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T13C15C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 1800 W | High inertia | SM3-M2H130-T18C15C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation plug | AC380V |
|  |  | SM3-M2H130-T18C15C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T18C15C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T18C15C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T18C15C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H130-T18C15C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T18C15C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T18C15C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 2200 W | High inertia | SM3-M2H130-T22C15C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation Plug | AC380V |
|  |  | SM3-M2H130-T22C15C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T22C15C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T22C15C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T22C15C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H130-T22C15C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T22C15C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H130-T22C15C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |

## 19. Comparison Table of SM3-M2H130 Servo Motor and SV3H Servo Driver Selection 380V/(850W-2200W)

| Motor power | inertia | Motor model | Servo drive model |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pulse | EtherCAT | PROFINET | CANopen | RS485 | Power specifications |
| 850W | High inertia | SM3-M2H130-T85B15C-MH1N1 | Standard type: SV3H-PT5R4 Optional second encoder: SV3H-PT5R4C Second encoder+STO: SV3H-PT5R4SC | Standard type: SV3H-ET5R4 Optional second encoder: SV3H-ET5R4C Second encoder+STO: SV3H-ET5R4SC | Standard type: SV3H-FT5R4 Optional second encoder: SV3H-FT5R4C Second encoder+STO: SV3H-FT5R4SC | Standard type: SV3H-CT5R4 Optional second encoder: SV3H-CT5R4C Second encoder+analog: SV3H-CT5R4AC | Standard type: SV3H-RT5R4 Optional Second Encoder: SV3H-RT5R4C Second Encoder+Analog Quantity: SV3H-RT5R4AC | Three-phase AC380 V |
|  |  | SM3-M2H130-T85B15C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T85B15C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T85B15C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T85B15C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T85B15C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T85B15C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T85B15C-PH1B1 |  |  |  |  |  |  |
| 1300 W | High inertia | SM3-M2H130-T13C15C-MH1N1 | Standard type: SV3H-PT8R4 Optional second encoder: SV3H-PT8R4C Second encoder+STO: SV3H-PT8R4SC | Standard type: SV3H-ET8R4 Optional second encoder: SV3H-ET8R4C Second encoder+STO: SV3H-ET8R4SC | Standard type: SV3H-FT8R4 Optional second encoder: SV3H-FT8R4C Second encoder+STO: SV3H-FT8R4SC | Standard type: SV3H-CT8R4 Optional second encoder: SV3H-CT8R4C Second encoder+analog: SV3H-CT8R4AC | Standard type: SV3H-RT8R4 Optional second encoder: SV3H-RT8R4C Second encoder+analog: SV3H-RT8R4AC | Three-phase AC380 V |
|  |  | SM3-M2H130-T13C15C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T13C15C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T13C15C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T13C15C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T13C15C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T13C15C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T13C15C-PH1B1 |  |  |  |  |  |  |
| 1800 W | High inertia | SM3-M2H130-T18C15C-MH1N1 | Standard type: SV3H-PT012 Optional second encoder: SV3H-PT012C Second encoder+STO: SV3H-PT012SC | Standard type: SV3H-ET012 Optional second encoder: SV3H-ET012C Second encoder+STO: SV3H-ET012SC | Standard type: SV3H-FT012 Optional second encoder: SV3H-FT012C Second encoder+STO: SV3H-FT012SC | Standard type: SV3H-CT012 Optional second encoder: SV3H-CT012C Second encoder+analog: SV3H-CT012AC | Standard type: SV3H-RT012 Optional second encoder: SV3H-RT012C Second encoder+analog: SV3H-RT012AC | Three-phase AC380 V |
|  |  | SM3-M2H130-T18C15C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T18C15C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T18C15C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T18C15C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T18C15C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T18C15C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T18C15C-PH1B1 |  |  |  |  |  |  |
| 2200 W | High inertia | SM3-M2H130-T22C15C-MH1N1 | Standard type: SV3H-PT012 Optional second encoder: SV3H-PT012C Second encoder+STO: SV3H-PT012SC | Standard type: SV3H-ET012 Optional second encoder: SV3H-ET012C Second encoder+STO: SV3H-ET012SC | Standard type: SV3H-FT012 Optional second encoder: SV3H-FT012C Second encoder+STO: SV3H-FT012SC | Standard type: SV3H-CT012 Optional second encoder: SV3H-CT012C Second encoder+analog Quantity: SV3H-CT012AC | Standard type: SV3H-RT012 Optional second encoder: SV3H-RT012C Second encoder+module Proposed quantity: SV3H-RT012AC | three-phase AC380 V |
|  |  | SM3-M2H130-T22C15C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T22C15C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T22C15C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T22C15C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T22C15C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T22C15C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H130-T22C15C-PH1B1 |  |  |  |  |  |  |

## 20. Configuration specification of SM3-M2H180 servo motor 380V/(2.9kW-7.5kW)

| Motor power | Inertia | Motor model | 23bit |  | 17bit |  | Equipped with brakes |  | Oil seal | Flange | Shaft diameter | Outgoing form | Rated voltage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Single circle | Multiple circles | Single circle | Multiple circles | Yes | No |  |  |  |  |  |
| 2.9kW | High inertia | SM3-M2H180-T29C15C-MH1N1 |  |  | ● |  |  | ● | ● | 180 | Φ35 | Aviation plug | AC380V |
|  |  | SM3-M2H180-T29C15C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T29C15C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T29C15C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T29C15C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H180-T29C15C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T29C15C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T29C15C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 4.4kW | High inertia | SM3-M2H180-T44C15C-MH1N1 |  |  | ● |  |  | ● | ● | 180 | Φ35 | Aviation plug | AC380V |
|  |  | SM3-M2H180-T44C15C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T44C15C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T44C15C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T44C15C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H180-T44C15C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T44C15C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T44C15C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 5.5kW | High inertia | SM3-M2H180-T55C15C-MH1N1 |  |  | ● |  |  | ● | ● | 180 | Φ42 | Aviation plug | AC380V |
|  |  | SM3-M2H180-T55C15C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T55C15C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T55C15C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T55C15C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H180-T55C15C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T55C15C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T55C15C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 7.5kW | High inertia | SM3-M2H180-T75C15C-MH1N1 |  |  | ● |  |  | ● | ● | 180 | Φ42 | Aviation plug | AC380V |
|  |  | SM3-M2H180-T75C15C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T75C15C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T75C15C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T75C15C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2H180-T75C15C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T75C15C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2H180-T75C15C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |

## 21. SM3-M2H180 servo motor and SV3H servo drive selection comparison table 380V/(2.9kW-7.5kW)

| Motor power | Inertia | Motor model | Servo drive model |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pulse | EtherCAT | PROFINET | CANopen | RS485 | Rated voltage |
| 2.9kW | High inertia | SM3-M2H180-T29C15C-MH1N1 | Standard type: SV3H-PT012 Optional second encoder: SV3H-PT012C Second encoder+STO: SV3H-PT012SC | Standard type: SV3H-ET012 Optional second encoder: SV3H-ET012C Second encoder+STO: SV3H-ET012SC | Standard type: SV3H-FT012 Optional second encoder: SV3H-FT012C Second encoder+STO: SV3H-FT012SC | Standard type: SV3H-CT012 Optional second encoder: SV3H-CT012C Second encoder+analog: SV3H-CT012AC | Standard type: SV3H-RT012 Optional second encoder: SV3H-RT012C Second encoder+analog: SV3H-RT012AC | Three phase AC380V |
|  |  | SM3-M2H180-T29C15C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T29C15C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T29C15C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T29C15C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T29C15C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T29C15C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T29C15C-PH1B1 |  |  |  |  |  |  |
| 4.4kW | High inertia | SM3-M2H180-T44C15C-MH1N1 | Standard type: SV3H-PT017 Optional second encoder: SV3H-PT017C Second encoder+STO: SV3H-PT017SC | Standard type: SV3H-ET017 Optional second encoder: SV3H-ET017C Second encoder+STO: SV3H-ET017SC | Standard type: SV3H-FT017 Optional second encoder: SV3H-FT017C Second encoder+STO: SV3H-FT017SC | Standard type: SV3H-CT017 Optional second encoder: SV3H-CT017C Second encoder+analog: SV3H-CT017AC | Standard type: SV3H-RT017 Optional Second Encoder: SV3H-RT017C Second encoder+analog: SV3H-RT017AC | Three phase AC380V |
|  |  | SM3-M2H180-T44C15C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T44C15C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T44C15C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T44C15C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T44C15C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T44C15C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T44C15C-PH1B1 |  |  |  |  |  |  |
| 5.5kW | High inertia | SM3-M2H180-T55C15C-MH1N1 | Standard type: SV3H-PT021 Optional second encoder: SV3H-PT021C Second encoder+STO: SV3H-PT021SC | Standard type: SV3H-ET021 Optional second encoder: SV3H-ET021C Second encoder+STO: SV3H-ET021SC | Standard type: SV3H-FT021 Optional second encoder: SV3H-FT021C Second encoder+STO: SV3H-FT021SC | Standard type: SV3H-CT021 Optional second encoder: SV3H-CT021C Second encoder+analog: SV3H-CT021AC | Standard type: SV3H-RT021 Optional second encoder: SV3H-RT021C Second encoder+analog: SV3H-RT021AC | Three phase AC380V |
|  |  | SM3-M2H180-T55C15C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T55C15C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T55C15C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T55C15C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T55C15C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T55C15C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T55C15C-PH1B1 |  |  |  |  |  |  |
| 7.5kW | High inertia | SM3-M2H180-T75C15C-MH1N1 | Standard type: SV3H-PT026 Optional second encoder: SV3H-PT026C Second encoder+STO: SV3H-PT026SC | Standard type: SV3H-ET026 Optional second encoder: SV3H-ET026C Second encoder+STO: SV3H-ET026SC | Standard type: SV3H-FT026 Optional second encoder: SV3H-FT026C Second encoder+STO: SV3H-FT026SC | Standard type: SV3H-CT026 Optional second encoder: SV3H-CT026C Second encoder+analog: SV3H-CT026AC | Standard type: SV3H-RT026 Optional second encoder: SV3H-RT026C Second encoder+analog: SV3H-RT026AC | AC380V |
|  |  | SM3-M2H180-T75C15C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T75C15C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T75C15C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T75C15C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T75C15C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T75C15C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2H180-T75C15C-PH1B1 |  |  |  |  |  |  |

## 22. SM3-M2A130 servo motor configuration specification 220V/(1kW-3kW)

| Motor power | Inertia | Motor model | 23bit |  | 17bit |  | Equipped with brakes |  | Oil seal | Flange | Shaft diameter | Outgoing form | Rated voltage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Single circle | Multiple circles | Single circle | Multiple circles | Yes | No |  |  |  |  |  |
| 1kW | Low inertia | SM3-M2A130-S10C20C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation plug | AC220V |
|  |  | SM3-M2A130-S10C20C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S10C20C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S10C20C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S10C20C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2A130-S10C20C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S10C20C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S10C20C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 1.5kW | Low inertia | SM3-M2A130-S15C20C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation plug | AC220V |
|  |  | SM3-M2A130-S15C20C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S15C20C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S15C20C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S15C20C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2A130-S15C20C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S15C20C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S15C20C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 2kW | Low inertia | SM3-M2A130-S20C20C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation plug | AC220V |
|  |  | SM3-M2A130-S20C20C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S20C20C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S20C20C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S20C20C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2A130-S20C20C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S20C20C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S20C20C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 3kW | Low inertia | SM3-M2A130-S30C20C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation plug | AC220V |
|  |  | SM3-M2A130-S30C20C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S30C20C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S30C20C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S30C20C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2A130-S30C20C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S30C20C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-S30C20C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |

## 23. SM3-M2A130 servo motor and SV3H servo drive selection comparison table 220V/(1kW-3kW)

| Motor power | Inertia | Motor model | Servo drive model |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pulse | EtherCAT | PROFINET | CANopen | RS485 | Power specifications |
| 1kW | Low inertia | SM3-M2A130-S10C20C-MH1N1 | Standard type: SV3H-PS7R6 Optional second encoder: SV3H-PS7R6C Second encoder+STO: SV3H-PS7R6SC | Standard type: SV3H-ES7R6 Optional second encoder: SV3H-ES7R6C Second encoder+STO: SV3H-ES7R6SC | Standard type: SV3H-FS7R6 Optional second encoder: SV3H-FS7R6C Second encoder+STO: SV3H-FS7R6SC | Standard type: SV3H-CS7R6 Optional second encoder: SV3H-CS7R6C Second encoder+analog: SV3H-CS7R6AC | Standard type: SV3H-RS7R6 Optional Second Encoder: SV3H-RS7R6C Second Encoder+Analog Quantity: SV3H- RS7R6AC | Single/Three phase AC220 V |
|  |  | SM3-M2A130-S10C20C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S10C20C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S10C20C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S10C20C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S10C20C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S10C20C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S10C20C-PH1B1 |  |  |  |  |  |  |
| 1.5kW | Low inertia | SM3-M2A130-S15C20C-MH1N1 | Standard type: SV3H-PS012 Optional second encoder: SV3H-PS012C Second encoder+STO: SV3H-PS012SC | Standard type: SV3H-ES012 Optional second encoder: SV3H-ES012C Second encoder+STO: SV3H-ES012SC | Standard type: SV3H-FS012 Optional second encoder: SV3H-FS012C Second encoder+STO: SV3H-FS012SC | Standard type: SV3H-CS012 Optional second encoder: SV3H-CS012C Second encoder+analog: SV3H-CS012AC | Standard type: SV3H-RS012 Optional second encoder: SV3H-RS012C Second encoder+analog: SV3H-RS012AC | Single/Three phase AC220 V |
|  |  | SM3-M2A130-S15C20C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S15C20C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S15C20C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S15C20C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S15C20C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S15C20C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S15C20C-PH1B1 |  |  |  |  |  |  |
| 2kW | Low inertia | SM3-M2A130-S20C20C-MH1N1 | Standard type: SV3H-PS014 Optional Second Encoder: SV3H-PS014C Second encoder+STO: SV3H-PS014SC | Standard type: SV3H-ES014 Optional second encoder: SV3H-ES014C Second encoder+STO: SV3H-ES014SC | Standard type: SV3H-FS014 Optional second encoder: SV3H-FS014C Second encoder+STO: SV3H-FS014SC | Standard type: SV3H-CS014 Optional second encoder: SV3H-CS014C Second encoder+analog: SV3H-CS014AC | Standard type: SV3H-RS014 Optional Second Encoder: SV3H-RS014C Second encoder+analog: SV3H-RS014AC | Single/Three phase AC220 V |
|  |  | SM3-M2A130-S20C20C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S20C20C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S20C20C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S20C20C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S20C20C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S20C20C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S20C20C-PH1B1 |  |  |  |  |  |  |
| 3kW | Low inertia | SM3-M2A130-S30C20C-MH1N1 | Standard type: SV3H-PU018 Optional second encoder: SV3H-PU018C Second encoder+STO: SV3H-PU018SC | Standard type: SV3H-EU018 Optional second encoder: SV3H-EU018C Second encoder+STO: SV3H-EU018SC | Standard type: SV3H-FU018 Optional second encoder: SV3H-FU018C Second encoder+STO: SV3H-FU018SC | Standard type: SV3H-CU018 Optional second encoder: SV3H-CU018C Second encoder+analog Quantity: SV3H-CU018AC | Standard type: SV3H-RU018 Optional second encoder: SV3H-RU018C Second encoder+module Proposed quantity: SV3H- RU018AC | Three-phase AC220 V |
|  |  | SM3-M2A130-S30C20C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S30C20C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S30C20C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S30C20C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S30C20C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S30C20C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-S30C20C-PH1B1 |  |  |  |  |  |  |

## 24. SM3-M2A130 servo motor configuration specification 380V/(1kW-3kW)

| Motor power | Inertia | Motor model | 23bit |  | 17bit |  | Equipped with brakes |  | Oil seal | Flange | Shaft diameter | Outgoing form | Rated voltage |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Single circle | Multiple circles | Single circle | Multiple circles | Yes | No |  |  |  |  |  |
| 1kW | Low inertia | SM3-M2A130-T10C20C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation plug | C380V |
|  |  | SM3-M2A130-T10C20C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T10C20C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T10C20C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T10C20C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2A130-T10C20C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T10C20C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T10C20C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 1.5kW | Low inertia | SM3-M2A130-T15C20C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation plug | AC380V |
|  |  | SM3-M2A130-T15C20C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T15C20C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T15C20C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T15C20C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2A130-T15C20C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T15C20C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T15C20C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 2kW | Low inertia | SM3-M2A130-T20C20C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation plug | AC380V |
|  |  | SM3-M2A130-T20C20C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T20C20C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T20C20C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T20C20C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2A130-T20C20C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T20C20C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T20C20C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 3kW | Low inertia | SM3-M2A130-T30C20C-MH1N1 |  |  | ● |  |  | ● | ● | 130 | Φ22 | Aviation plug | AC380V |
|  |  | SM3-M2A130-T30C20C-NH1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T30C20C-OH1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T30C20C-PH1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T30C20C-MH1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M2A130-T30C20C-NH1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T30C20C-OH1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M2A130-T30C20C-PH1B1 |  | ● |  |  |  |  |  |  |  |  |  |

## 25. SM3-M2A130 servo motor and SV3H servo drive  selection comparison table 380V/(1kW-3kW)

| Motor power | Inertia | Motor model | Servo drive model |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pulse | EtherCAT | PROFINET | CANopen | RS485 | Power specifications |
| 1kW | Low inertia | SM3-M2A130-T10C20C-MH1N1 | Standard type: SV3H-PT3R5 Optional Second Encoder: SV3H-PT3R5C Second encoder+STO: SV3H-PT3R5SC | Standard type: SV3H-ET3R5 Optional Second Encoder: SV3H-ET3R5C Second encoder+STO: SV3H-ET3R5SC | Standard type: SV3H-FT3R5 Optional Second Encoder: SV3H-FT3R5C Second encoder+STO: SV3H-FT3R5SC | Standard type: SV3H-CT3R5 Optional Second Encoder: SV3H-CT3R5C Second encoder+analog: SV3H-CT3R5AC | Standard type: SV3H-RT3R5 Optional second encoder: SV3H-RT3R52C Second encoder+analog: SV3H-RT3R5AC | Three phase AC380V |
|  |  | SM3-M2A130-T10C20C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T10C20C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T10C20C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T10C20C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T10C20C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T10C20C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T10C20C-PH1B1 |  |  |  |  |  |  |
| 1.5kW | Low inertia | SM3-M2A130-T15C20C-MH1N1 | Standard type: SV3H-PT5R4 Optional second encoder: SV3H-PT5R4C Second encoder+STO: SV3H-PT5R4SC | Standard type: SV3H-ET5R4 Optional second encoder: SV3H-ET5R4C Second encoder+STO: SV3H-ET5R4SC | Standard type: SV3H-FT5R4 Optional second encoder: SV3H-FT5R4C Second encoder+STO: SV3H-FT5R4SC | Standard type: SV3H-CT5R4 Optional second encoder: SV3H-CT5R4C Second encoder+analog: SV3H-CT5R4AC | Standard type: SV3H-RT5R4 Optional second encoder: SV3H-RT5R4C Second encoder+analog: SV3H-RT5R4AC | Three phase AC380V |
|  |  | SM3-M2A130-T15C20C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T15C20C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T15C20C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T15C20C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T15C20C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T15C20C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T15C20C-PH1B1 |  |  |  |  |  |  |
| 2kW | Low inertia | SM3-M2A130-T20C20C-MH1N1 | Standard type: SV3H-PT8R4 Optional second encoder: SV3H-PT8R4C Second encoder+STO: SV3H-PT8R4SC | Standard type: SV3H-ET8R4 Optional second encoder: SV3H-ET8R4C Second encoder+STO: SV3H-ET8R4SC | Standard type: SV3H-FT8R4 Optional second encoder: SV3H-FT8R4C Second encoder+STO: SV3H-FT8R4SC | Standard type: SV3H-CT8R4 Optional second encoder: SV3H-CT8R4C Second encoder+analog: SV3H-CT8R4AC | Standard type: SV3H-RT8R4 Optional second encoder: SV3H-RT8R4C Second encoder+analog: SV3H-RT8R4AC | Three phase AC380V |
|  |  | SM3-M2A130-T20C20C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T20C20C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T20C20C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T20C20C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T20C20C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T20C20C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T20C20C-PH1B1 |  |  |  |  |  |  |
| 3kW | Low inertia | SM3-M2A130-T30C20C-MH1N1 | Standard type: SV3H-PT012 Optional second encoder: SV3H-PT012C Second encoder+STO: SV3H-PT012SC | Standard type: SV3H-ET012 Optional second encoder: SV3H-ET012C Second encoder+STO: SV3H-ET012SC | Standard type: SV3H-FT012 Optional second encoder: SV3H-FT012C Second encoder+STO: SV3H-FT012SC | Standard type: SV3H-CT012 Optional second encoder: SV3H-CT012C Second encoder+analog quantity: SV3H-CT012AC | Standard type: SV3H-RT012 Optional second encoder: SV3H-RT012C Second encoder+analog Quantity: SV3H-RT012AC | Three phase AC380V |
|  |  | SM3-M2A130-T30C20C-NH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T30C20C-OH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T30C20C-PH1N1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T30C20C-MH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T30C20C-NH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T30C20C-OH1B1 |  |  |  |  |  |  |
|  |  | SM3-M2A130-T30C20C-PH1B1 |  |  |  |  |  |  |

## 26. Naming rules of SM3-M3 series servo motor

| ① Product Series | ③ Motor flange | ⑤ Rated power | ⑦ Encoder type | ⑨Axis connection method | ⑪ Oil seal |
| --- | --- | --- | --- | --- | --- |
| SM3-M3 : | 040:40 flange | 10B: 100W | M: 17 bit absolute value per cycle | 0: Optical axis | 0: without oil seal |
| SM3-M3 series | 060:60 flange | 20B: 200W | N: 17 bit multi cycle absolute value | 1: With keys | 1: without oil seal |
| servo motor | 080:80 flange | 40B: 400W 75B: 750W 10C: 1000W | O: 23 bit absolute value per cycle P: 23 bit multi cycle absolute value |  |  |
| ② Inertia capacity | ④ Rated voltage | ⑥ Rated speed | ⑧ Interface type | ⑩ Brakes |  |
| A: Low inertia | S： AC 220V | 30C： 3000RPM | T: Terminal type | N: without brake |  |
| M: Medium inertia |  |  |  | B: with brake |  |
| H: High inertia |  |  |  |  |  |

## 27. Wiring standard of SM3-M3 series servo motor connector

40, 60, 80 flanges

| Encoder connector. Definition of motor side. Pin No.; function. 1; SD+. 2; SD-. 3; E+. 4; E-. 5; +5V. 6; GND. 7; PE. | Power connector (without brake). Definition of motor side. Pin No.; function. 1; V. 2; U. 3; W. 4; PE. | Power connector (with brake). Definition of motor side. Pin No.; function. 1; V. 2; U. 3; W. 4; PE. A; BK+. B; BK-. |
| --- | --- | --- |

## 28. Technical specifications of SM3-M3 series servo motor

| Motor specifications | SM3-M3H040- S10B30C-xxxxx | SM3-M3H060- S20B30C-xxxxx | SM3-M3H060- S40B30C-xxxxx | SM3-M3H080- S75B30C-xxxxx | SM3-M3H080- S10C30C-xxxxx |
| --- | --- | --- | --- | --- | --- |
| Rated power (W) | 100 | 200 | 400 | 750 | 1000 |
| Rated voltage (V) | 220 | 220 | 220 | 220 | 220 |
| Rated current (A) | 0.98 | 1.3 | 2.6 | 4.6 | 6.3 |
| Rated torque (N.m) | 0.32 | 0.64 | 1.27 | 2.39 | 3.18 |
| Rated speed (r/min) | 3000 | 3000 | 3000 | 3000 | 3000 |
| Instantaneous maximum current (A) | 3.9 | 4.4 | 8.6 | 16.3 | 20.9 |
| Instantaneous maximum torque (N.m) | 1.12 | 2.23 | 4.445 | 8.36 | 11.13 |
| Instantaneous maximum speed (r/min) | 6000 | 6000 | 6000 | 6000 | 6000 |
| Reverse electromotive force (V/1000r/min) | 20 | 31.98 | 34.09 | 34.4 | 33.9 |
| Torque coefficient (N.m/A) | 0.327 | 0.492 | 0.49 | 0.52 | 0.505 |
| Phase resistance (Ω) | 13.186 | 8.286 | 3.33 | 0.936 | 0.638 |
| Q-axis inductance (mH) | 7.81 | 9.35 | 5.09 | 2.8 | 2.04 |
| D-axis inductance (mH) | 5.39 | 7.8 | 4.13 | 2.25 | 1.675 |
| Electrical time constant (ms) | 1.01 | 2.07 | 2.77 | 5.4 | 11.646 |
| Rotor inertia (kg. m2 X 10-4) | 0.031(0.034) | 0.34(0.35) | 0.59(0.60) | 1.72(1.77) | 2.23(2.28) |
| Weight (kg) | 0.36(0.55) | 0.8(1.17) | 1.14(1.51) | 2.15(2.83) | 2.59(3.27) |
| Extreme logarithm | 5 |  |  |  |  |
| Motor insulation grade | F |  |  |  |  |
| Protection level | IP67 with oil seal (shaft end installed with oil seal) |  |  |  |  |
| Usage environment | Environmental temperature: 0-40 ℃ (not frozen); Environmental humidity: 20% to 80% RH (No condensation) |  |  |  |  |
| Note: () represents the parameters of the motor with holding brake. |  |  |  |  |  |

- Torque characteristic diagram:

Continuous work area

Torque [N m]

Instantaneous work area

Continuous work area

Torque [N m]

Instantaneous work area

Speed [r/min]

Speed [r/min]

Torque [N m]

Instantaneous work area

Continuous work area

Speed [r/min]

| SM3-M3H040-S10B30C- | SM3-M3H060-S20B30C- | SM3-M3H060-S40B30C- |
| --- | --- | --- |

Continuous work area

Torque [N m]

Instantaneous work area

Continuous work area

Torque [N m]

Instantaneous work area

Speed [r/min]

Speed [r/min]

| SM3-M3H080-S75B30C- | SM3-M3H080-S10C30C- |
| --- | --- |

- Outline dimension diagram:

| Motor model | LC | LL | LR | LA | LZ | MH | LG | LE | Q |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SM3-M3H040-S10B30C-xxxxx | 40 | 73.4 (100.1) | 25 | 46 | 4.3 | 16.8 | 5.5 | 3 | 20 |
| SM3-M3H060-S20B30C-xxxxx | 60 | 70.5 (99.7) | 30 | 70 | 5.5 | 15.8 | 6.5 | 3 | 25.5 |
| SM3-M3H060-S40B30C-xxxxx | 60 | 88 (117.2) | 30 | 70 | 5.5 | 15.8 | 6.5 | 3 | 25.5 |
| SM3-M3H080-S75B30C-xxxxx | 80 | 94.4 (128.6) | 35 | 90 | 6.4 | 15.5 | 8 | 3 | 30.2 |
| SM3-M3H080-S10C30C-xxxxx | 80 | 107.9 (142.1) | 35 | 90 | 6.4 | 15.5 | 8 | 3 | 30.2 |
| Note: () represents the parameters of the motor with holding brake. |  |  |  |  |  |  |  |  |  |
| model | LB | S | TP | LK | KH | KW | W | T |  |
| SM3-M3H040-S10B30C-xxxxx | 30 | 8 | M3 depth 7 | 14 | 6.2 | 3 | 3 | 3 |  |
| SM3-M3H060-S20B30C-xxxxx | 50 | 14 | M5 depth 10 | 14 | 10.5 | 5 | 5 | 5 |  |
| SM3-M3H060-S40B30C-xxxxx | 50 | 14 | M5 depth 10 | 14 | 10.5 | 5 | 5 | 5 |  |
| SM3-M3H080-S75B30C-xxxxx | 70 | 19 | M6 depth12 | 22 | 15 | 6 | 6 | 6 |  |
| SM3-M3H080-S10C30C-xxxxx | 70 | 19 | M6 depth 12 | 22 | 15 | 6 | 6 | 6 |  |

## 29. Configuration specifications of SM3-M3 series servo motor

| Motor power | Inertia | Motor model | 23bit |  | 17bit |  | Equipped with brakes |  | Oil seal | Flanges | Shaft diameter | Outgoing form | Power specifications |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Single circle | Multiple circles | Single circle | Multiple circles | Yes | No |  |  |  |  |  |
| 100W | High inertia | SM3-M3H040-S10B30C-MT1N0 |  |  | ● |  |  | ● |  | 40 | Φ8 | Terminal | AC220V |
|  |  | SM3-M3H040-S10B30C-NT1N0 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M3H040-S10B30C-OT1N0 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M3H040-S10B30C-PT1N0 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M3H040-S10B30C-MT1B0 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M3H040-S10B30C-NT1B0 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M3H040-S10B30C-OT1B0 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M3H040-S10B30C-PT1B0 |  | ● |  |  |  |  |  |  |  |  |  |
| 200W | High inertia | SM3-M3H060-S20B30C-MT1N1 |  |  | ● |  |  | ● | ● | 60 | Φ14 | Terminal | AC220V |
|  |  | SM3-M3H060-S20B30C-NT1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M3H060-S20B30C-OT1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M3H060-S20B30C-PT1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M3H060-S20B30C-MT1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M3H060-S20B30C-NT1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M3H060-S20B30C-OT1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M3H060-S20B30C-PT1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 400W | High inertia | SM3-M3H060-S40B30C-MT1N1 |  |  | ● |  |  | ● | ● | 60 | Φ14 | Terminal | AC220V |
|  |  | SM3-M3H060-S40B30C-NT1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M3H060-S40B30C-OT1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M3H060-S40B30C-PT1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M3H060-S40B30C-MT1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M3H060-S40B30C-NT1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M3H060-S40B30C-OT1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M3H060-S40B30C-PT1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 750W | High inertia | SM3-M3H080-S75B30C-MT1N1 |  |  | ● |  |  | ● | ● | 80 | Φ19 | Terminal | AC220V |
|  |  | SM3-M3H080-S75B30C-NT1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M3H080-S75B30C-OT1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M3H080-S75B30C-PT1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M3H080-S75B30C-MT1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M3H080-S75B30C-NT1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M3H080-S75B30C-OT1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M3H080-S75B30C-PT1B1 |  | ● |  |  |  |  |  |  |  |  |  |
| 1kW | High inertia | SM3-M3H080-S10C30C-MT1N1 |  |  | ● |  |  | ● | ● | 80 | Φ19 | Terminal | AC220V |
|  |  | SM3-M3H080-S10C30C-NT1N1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M3H080-S10C30C-OT1N1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M3H080-S10C30C-PT1N1 |  | ● |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M3H080-S10C30C-MT1B1 |  |  | ● |  | ● |  |  |  |  |  |  |
|  |  | SM3-M3H080-S10C30C-NT1B1 |  |  |  | ● |  |  |  |  |  |  |  |
|  |  | SM3-M3H080-S10C30C-OT1B1 | ● |  |  |  |  |  |  |  |  |  |  |
|  |  | SM3-M3H080-S10C30C-PT1B1 |  | ● |  |  |  |  |  |  |  |  |  |

## 30. Comparison Table of SM3-M3 Series Servo Motor and SV3H Series Servo Driver Selection

| Motor power | Inertia | Motor model | Servo drive model |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Pulse | EtherCAT | PROFINET | CANopen | RS485 | Power specifications |
| 100W | High inertia | SM3-M3H040-S10B30C-MT1N0 | Standard type: SV3H-PS001 Optional second encoder: SV3H-PS001C Second encoder+STO: SV3H-PS001SC | Standard type: SV3H-ES001 Optional second encoder: SV3H-ES001C Second encoder+STO: SV3H-ES001SC | Standard type: SV3H-FS001 Optional second encoder: SV3H-FS001C Second encoder+STO: SV3H-FS001SC | Standard type: SV3H-CS001 Optional second encoder: SV3H-CS001C Second encoder+analog: SV3H-CS001AC | Standard type: SV3H-RS001 Optional second encoder: SV3H-RS001C Second Encoder+Analog Quantity: SV3H- RS001AC | Single/three-phase AC220V |
|  |  | SM3-M3H040-S10B30C-NT1N0 |  |  |  |  |  |  |
|  |  | SM3-M3H040-S10B30C-OT1N0 |  |  |  |  |  |  |
|  |  | SM3-M3H040-S10B30C-PT1N0 |  |  |  |  |  |  |
|  |  | SM3-M3H040-S10B30C-MT1B0 |  |  |  |  |  |  |
|  |  | SM3-M3H040-S10B30C-NT1B0 |  |  |  |  |  |  |
|  |  | SM3-M3H040-S10B30C-OT1B0 |  |  |  |  |  |  |
|  |  | SM3-M3H040-S10B30C-PT1B0 |  |  |  |  |  |  |
| 200W | High inertia | SM3-M3H060-S20B30C-MT1N1 | Standard type: SV3H-PS1R6 Optional second encoder: SV3H-PS1R6C Second encoder+STO: SV3H-PS1R6SC | Standard type: SV3H-ES1R6 Optional second encoder: SV3H-ES1R6C Second encoder+STO: SV3H-ES1R6SC | Standard type: SV3H-FS1R6 Optional second encoder: SV3H-FS1R6C Second encoder+STO: SV3H-FS1R6SC | Standard type: SV3H-CS1R6 Optional second encoder: SV3H-CS1R6C Second encoder+analog: SV3H-CS1R6AC | Standard type: SV3H-RS1R6 Optional second encoder: SV3H-RS1R6C Second Encoder+Analog Quantity: SV3H- RS1R6AC | Single/three-phase AC220V |
|  |  | SM3-M3H060-S20B30C-NT1N1 |  |  |  |  |  |  |
|  |  | SM3-M3H060-S20B30C-OT1N1 |  |  |  |  |  |  |
|  |  | SM3-M3H060-S20B30C-PT1N1 |  |  |  |  |  |  |
|  |  | SM3-M3H060-S20B30C-MT1B1 |  |  |  |  |  |  |
|  |  | SM3-M3H060-S20B30C-NT1B1 |  |  |  |  |  |  |
|  |  | SM3-M3H060-S20B30C-OT1B1 |  |  |  |  |  |  |
|  |  | SM3-M3H060-S20B30C-PT1B1 |  |  |  |  |  |  |
| 400W | High inertia | SM3-M3H060-S40B30C-MT1N1 | Standard type: SV3H-PS2R8 Optional second encoder: SV3H-PS2R8C Second encoder+STO: SV3H-PS2R8SC | Standard type: SV3H-ES2R8 Optional second encoder: SV3H-ES2R8C Second encoder+STO: SV3H-ES2R8SC | Standard type: SV3H-FS2R8 Optional second encoder: SV3H-FS2R8C Second encoder+STO: SV3H-FS2R8SC | Standard type: SV3H-CS2R8 Optional second encoder: SV3H-CS2R8C Second encoder+analog: SV3H-CS2R8AC | Standard type: SV3H-RS2R8 Optional second encoder: SV3H-RS2R8C Second Encoder+Analog Quantity: SV3H- RS2R8AC | Single/three-phase AC220V |
|  |  | SM3-M3H060-S40B30C-NT1N1 |  |  |  |  |  |  |
|  |  | SM3-M3H060-S40B30C-OT1N1 |  |  |  |  |  |  |
|  |  | SM3-M3H060-S40B30C-PT1N1 |  |  |  |  |  |  |
|  |  | SM3-M3H060-S40B30C-MT1B1 |  |  |  |  |  |  |
|  |  | SM3-M3H060-S40B30C-NT1B1 |  |  |  |  |  |  |
|  |  | SM3-M3H060-S40B30C-OT1B1 |  |  |  |  |  |  |
|  |  | SM3-M3H060-S40B30C-PT1B1 |  |  |  |  |  |  |
| 750W | High inertia | SM3-M3H080-S75B30C-MT1N1 | Standard type: SV3H-PS5R5 Optional second encoder: SV3H-PS5R5C Second encoder+STO: SV3H-PS5R5SC | Standard type: SV3H-ES5R5 Optional second encoder: SV3H-ES5R5C Second encoder+STO: SV3H-ES5R5SC | Standard type: SV3H-FS5R5 Optional second encoder: SV3H-FS5R5C Second encoder+STO: SV3H-FS5R5SC | Standard type: SV3H-CS5R5 Optional second encoder: SV3H-CS5R5C Second encoder+analog: SV3H-CS5R5AC | Standard type: SV3H-RS5R5 Optional second encoder: SV3H-RS5R5C Second Encoder+Analog Quantity: SV3H-RS5R5AC | Single/three-phase AC220V |
|  |  | SM3-M3H080-S75B30C-NT1N1 |  |  |  |  |  |  |
|  |  | SM3-M3H080-S75B30C-OT1N1 |  |  |  |  |  |  |
|  |  | SM3-M3H080-S75B30C-PT1N1 |  |  |  |  |  |  |
|  |  | SM3-M3H080-S75B30C-MT1B1 |  |  |  |  |  |  |
|  |  | SM3-M3H080-S75B30C-NT1B1 |  |  |  |  |  |  |
|  |  | SM3-M3H080-S75B30C-OT1B1 |  |  |  |  |  |  |
|  |  | SM3-M3H080-S75B30C-PT1B1 |  |  |  |  |  |  |
| 1kW | High inertia | SM3-M3H080-S10C30C-MT1N1 | Standard type: SV3H-PS7R6 Optional second encoder: SV3H-PS7R6C Second encoder+STO: SV3H-PS7R6SC | Standard type: SV3H-ES7R6 Optional second encoder: SV3H-ES7R6C Second encoder+STO: SV3H-ES7R6SC | Standard type: SV3H-FS7R6 Optional second encoder: SV3H-FS7R6C Second encoder+STO: SV3H-FS7R6SC | Standard type: SV3H-CS7R6 Optional second encoder: SV3H-CS7R6C Second encoder+analog: SV3H-CS7R6AC | Standard type: SV3H-RS7R6 optional second encoder: SV3H-RS7R6C Second Encoder+Analog Quantity: SV3H- RS7R6AC | Single/three-phase AC220V |
|  |  | SM3-M3H080-S10C30C-NT1N1 |  |  |  |  |  |  |
|  |  | SM3-M3H080-S10C30C-OT1N1 |  |  |  |  |  |  |
|  |  | SM3-M3H080-S10C30C-PT1N1 |  |  |  |  |  |  |
|  |  | SM3-M3H080-S10C30C-MT1B1 |  |  |  |  |  |  |
|  |  | SM3-M3H080-S10C30C-NT1B1 |  |  |  |  |  |  |
|  |  | SM3-M3H080-S10C30C-OT1B1 |  |  |  |  |  |  |
|  |  | SM3-M3H080-S10C30C-PT1B1 |  |  |  |  |  |  |

## 31. Cable naming rules for SM3-M2/M3 series servo motors and SV3H series servo drives

| ① Product Series | ②: Wire harness type | ③ Type of servo terminal | ④: Wire harness current specification/encoder type | ⑤ Motor terminal type | ⑥ Flexible wiring harness | ⑦: Cable length |
| --- | --- | --- | --- | --- | --- | --- |
| SC: Servo Cable | M: Non-brake power cable B: Brake power cable | 1: UVW pin, circular PE 2: All terminal are pin types 3: Circular PE, other terminals are U-shaped | 1: Cable current level 1 (18AWG) 2: Cable current level 2 (14AWG) 3: Cable current level 3 (12AWG) | 1: AMP 4P connector 2: 130 flange non-brake aviation plug (4-hole aviation plug YD28K4TSL) 3:180 flange aviation plug - hole aviation plug YD32K4TSL) 5:130 flange brake aviation plug - hole aviation plug YD28K7TSL) 7: Terminal front outlet connector 8: Terminal rear outlet connector | B: Standard cable H: High flexibility cable S: High flexibility cable with shielding layer | 3.0:3 meters 5.0:5 meters 10.0:10 meters |
|  | E: Encoder cable | 1: 1394 - 10P 2: 1394 - 6P | 1: Single cycle absolute value 2: Multiple cycles absolute value (with battery box) 3: Multiple cycles absolute value (without battery box) | 1: AMP 9P connector 4: 130/180 flange aviation plug - hole aviation plug YD28K7TSLa) 7: Terminal front outlet connector 8: Terminal rear outlet connector | B: Standard cable H: High flexibility cable |  |
|  | H: Brake cable | 2: All terminals are pin types 3: All terminals are U-shaped | 1: Cable current level 1 (18AWG) 2: Cable current level 2 (14AWG) 3: Cable current level 3 (12AWG) | 1: AMP 2P connector 2: 180 flange brake aviation plug (4-pole aviation plug XS16K4TM) | B: Standard cable H: High flexibility cable |  |

## 32. Model and specifications of compatible cables for SM3-M2 series servo motor and SV3H series servo driver

| Adapt motor | Type of outgoing wire | Cable type | Cable flexibility | Cable model | Cable schematic diagram |
| --- | --- | --- | --- | --- | --- |
| SM3-M2 series motor (40, 60, 80 flanges) 220V/ (50W-1kW) | AMP connector | Single circle encoder cable | Standard cable | Label Motor end Servo end Welding surface view of servo end Front view of motor end SC-E111B-□ |  |
|  |  |  | High flexibility cable | SC-E111H-□ |  |
|  |  | Multiple circles encoder cable | Standard cable | Label Motor end Servo end Welding surface view of servo end Front view of motor end Battery box SC-E121B-□ |  |
|  |  |  | High flexibility cable | SC-E121H-□ |  |
|  |  | Power cable | Standard cable | SC-M111B-□ | Label Motor end Servo end Front view of motor end |
|  |  |  | High flexibility cable | SC-M111H-□ |  |
|  |  | Brake cable | Standard cable | SC-H311B-□ | Label Motor end Servo end Insertion surface view of motor end |
|  |  |  | High flexibility cable | SC-H311H-□ |  |
| SM3-M2 series motor (130 flange) 220V/380V/ (850W- 2.2kW) | Aviation plug connector | Single circle encoder cable | Standard cable | Label Motor end Servo end Welding surface view of servo end Front view of motor end SC-E114B-□ |  |
|  |  |  | High flexibility cable | SC-E114H-□ |  |
|  |  | Multiple circles encoder cable | Standard cable | Label Motor end Servo end Welding surface view of servo end Front view of motor end Battery box A end SC-E124B-□ |  |
|  |  |  | High flexibility cable | SC-E124H-□ |  |
|  |  | Power cable | Standard cable | SC-M222B-□ | Label Motor end Servo end ★Insertion surface view of motor end |
|  |  |  | High flexibility cable | SC-M222H-□ |  |
|  |  | Equipped with brake power cable | Standard cable | SC-B225B-□ | Label Motor end Servo end ★Insertion surface view of motor end |
|  |  |  | High flexibility cable | SC-B225H-□ |  |
| □ represents the length of the cable: for example, □=3.0, which means the cable length is 3m. Standard specifications: 3.0m/5.0m/10.0m; |  |  |  |  |  |

| Adapt motor | Type of outgoing wire | Cable type | Cable flexibility | Cable model | Cable schematic diagram |
| --- | --- | --- | --- | --- | --- |
| SM3-M2 series motor (180 flange) 380V/2.9kW | Aviation plug connector | Single circle encoder cable | Standard cable | Label Motor end Servo end Welding surface view of servo end Front view of motor end SC-E114B-□ |  |
|  |  |  | High flexibility cable | Label Motor end Servo end Welding surface view of servo end Front view of motor end Battery box A end SC-E114H-□ |  |
|  |  | Multiple circles encodercable | Standard cable | SC-E124B-□ |  |
|  |  |  | High flexibility cable | SC-E124H-□ |  |
|  |  | Power cable | Standard cable | SC-M223B-□ | Label Servo end ★Front view of motor end |
|  |  |  | High flexibility cable | SC-M223H-□ |  |
|  |  | Brake cable | Standard cable | SC-H212B-□ | Label Servo end Insertion surface view of motor end Motor end |
|  |  |  | High flexibility cable | SC-H212H-□ |  |
| SM3-M2 series motor (180 flange) 380V/(4.4k W-7.5kW) | Aviation plug connector | Single circle encoder cable | Standard cable | Label Motor end Servo end Welding surface view of servo end Front view of motor end SC-E114B-□ |  |
|  |  |  | High flexibility cable | Label Motor end Servo end Welding surface view of servo end Front view of motor end Battery box A end SC-E114H-□ |  |
|  |  | Multiple circles encoder cable | Standard cable | SC-E124B-□ |  |
|  |  |  | High flexibility cable | SC-E124H-□ |  |
|  |  | Power cable | Standard cable | SC-M333B-□ | Label Servo end Front view of motor end Motor end |
|  |  |  | High flexibility cable | SC-M333H-□ |  |
|  |  | Brake cable | Standard cable | SC-H212B-□ | Label Servo end Insertion surface view of motor end Motor end |
|  |  |  | High flexibility cable | SC-H212H-□ |  |
| □ represents the length of the cable: for example, □=3.0, which means the cable length is 3m. Standard specifications: 3.0m/5.0m/10.0m; |  |  |  |  |  |

## 33. Cable specifications of SM3-M3 series servo motor and SV3H series servo driver

| Adapt motor | Type of outgoing wire | Cable type | Cable flexibility | Cable model | Cable schematic diagram |
| --- | --- | --- | --- | --- | --- |
| SM3-M3 series motor (40, 60, 80 flange) 220V/ (100W- 1kW) | Terminal front outlet | Single circle encoder cable | Standard cable | SC-E117B-□ | Label Motor end Servo end Welding surface view of servo end |
|  |  |  | High flexibility cable | SC-E117H-□ |  |
|  |  | Multiple circles encoder cable | Standard cable | SC-E127B-□ | Label Motor end Servo end Welding surface view of servo end |
|  |  |  | High flexibility cable | SC-E127H-□ |  |
|  |  | Power cable | Standard cable | SC-M117B-□ | Label Motor end Servo end |
|  |  |  | High flexibility cable | SC-M117H-□ |  |
|  |  | Equipped with brake power cable | Standard cable | SC-B117B-□ | Label Motor end Servo end |
|  |  |  | High flexibility cable | SC-B117H-□ |  |
| SM3-M3 series motor (40, 60, 80 flange) 220V/ (100W- 1kW) | Terminal rear outlet | Single circle encoder cable | Standard cable | SC-E118B-□ | Label Motor end Servo end Welding surface view of servo end |
|  |  |  | High flexibility cable | SC-E118H-□ |  |
|  |  | Multiple circles Encoder cable | Standard cable | SC-E128B-□ | Label Motor end Servo end Welding surface view of servo end Battery box |
|  |  |  | High flexibility cable | SC-E128H-□ |  |
|  |  | Power cable | Standard cable | SC-M118B-□ | Label Motor end Servo end |
|  |  |  | High flexibility cable | SC-M118H-□ |  |
|  |  | Equipped with brake power cable | Standard cable | SC-B118B-□ | Label Motor end Servo end |
|  |  |  | High flexibility cable | SC-B118H-□ |  |
| □ represents the length of the cable: for example, □=3.0, which means the cable length is 3m. Standard specifications: 3.0m/5.0m/10.0m; |  |  |  |  |  |

SINSEGYE (Shenzhen) Computer System Co., Ltd

Hotline: 400-013-2158

E-mail: sales@sinsegye.com.cn

Shenzhen: Room B25, 14th Floor, West Tower, Baidu International Building, No. 8 Haitian 1st Road, Binhai Community, Yuehai Street, Nanshan District, Shenzhen

Beijing: Room 1801, Jinhui Building, Wangjingdong, Chaoyang District, Beijing

Shanghai: Room 1704, No. 235 Yunling East Road, Putuo District, Shanghai (Cross border Procurement Center)

Suzhou: Room 504, Building 2, No. 209 Zhuyuan Road, High-tech Zone, Suzhou City, Jiangsu Province

Taiyuan: No. 10, Huarun Yuefu, Wanbailin District, Taiyuan City, Jiangxi Province

Qingdao: Building 1, No. 119 Chunyang Road, Chengyang District, Qingdao City, Jinan Province

Jinshi: No. 7, Tianlong Auto Parts City, Liaohe South Road, Xinglongtai District, Panjin City, Liaoning Province