HB430

Digital two-phase stepping driver

an instruction manual

All rights reserved

【Please read this manual carefully before use to avoid damaging the drive】

catalogue

1、Product introduction	2

1. summary	2

2. characteristic	3

3. application area	3

2、Electrical, mechanical and environmental indicators	4

1. Electrical index	4

2. Operating environment and parameters	4

3. Mechanical installation drawing	5

4. Strengthen heat dissipation mode	5

3、Driver interface and wiring introduction	6

1. Interface description	6

2. Control signal interface circuit	7

3. Control signal sequence diagram	8

4. Control signal mode setting	9

5. Wiring requirements	9

4、Current, subdivision dial switch setting and parameter self-tuning	9

1．Current setting	10

2.Subdivision settings	10

3.Parameter self-tuning function	11

5、Power supply selection	11

6、Motor selection	11

1.Motor selection	11

2.Motor wiring	12

2）Set value of output current	12

7、Typical wiring cases	13

8、Protection function	15

9、common problem	15

1.Common problems and solutions in application	15

2.Drive FAQ	16

Company product warranty terms	17

HB430

Digital two-phase stepping driver

1、 Product introduction

1. summary

### Hbd430 is a new digital stepper motor driver launched by the company. It adopts the latest 32-bit arm digital processing technology. The driver control algorithm adopts advanced variable current technology and advanced frequency conversion technology. The driver has low heating, low motor vibration and stable operation. Users can set any subdivision within 200 ~ 51200 and any current value within the rated current, which can meet the application needs of most occasions. Due to the built-in micro segmentation technology, even under the condition of low subdivision, it can achieve the effect of high subdivision. The low, medium and high-speed operation is very stable and the noise is ultra-low. The driver integrates the parameter power on automatic setting function, which can automatically generate the optimal operating parameters for different motors to maximize the performance of the motor.

2. characteristic

● new 32-bit ARM Technology

● ultra low vibration and noise

● built in high subdivision

● parameter power on automatic setting function

● variable current control greatly reduces motor heating

● the current will be automatically halved at standstill

● it can drive 4, 6 and 8-wire two-phase stepping motors

● optically isolated differential signal input

● impulse response frequency up to 500KHz (factory default 160kHz)

● the current setting is convenient and can be arbitrarily selected between 0.1-3.0A

● the subdivision setting range is 200-51200

● it has overvoltage, undervoltage, overcurrent and other protection functions

3. application area

Suitable for all kinds of small and medium-sized automation equipment and instruments, such as engraving machine, marking machine, cutting machine, laser Phototypesetting, plotter, CNC machine tool, automatic assembly equipment, etc. The application effect is especially good in the equipment with low noise and high speed expected by users.

2、Electrical, mechanical and environmental indicators

1. Electrical index

| explain | HBD430 |  |  |  |
| --- | --- | --- | --- | --- |
|  | minimum value | Typical value | Maximum | Company |
| Output current | 0.1 | - | 3.0 | A |
| Input power supply voltage | 12 | 24 | 32 | VDC |
| Control signal input current | 6 | 10 | 16 | mA |
| Control signal interface level | 4.5 | 5 | 28 | Vdc |
| Minimum pulse width of input signal | 1.5 | - | - | us |
| Step pulse frequency | 0 | - | 160 | KHz |
| insulation resistance | 500 |  |  | MΩ |

2. Operating environment and parameters

| Cooling mode |  | Natural cooling or forced air cooling |
| --- | --- | --- |
| Use environment | occasion | It shall not be placed next to other heating equipment. Dust, oil mist, corrosive gas, places with too high humidity and strong vibration shall be avoided. Combustible gas and conductive dust are prohibited; |
|  | temperature | -5℃ ～ +50℃ |
|  | humidity | 40 ～ 90%RH |
|  | Vibration | 5.9m/s2MAX |
| Storage temperature |  | -20℃～80℃ |
| Use altitude |  | Below 1000M |
| weight |  | About 90g |

3. Mechanical installation drawing

※Side mounting is recommended for better heat dissipation. When designing the installation size, pay attention to the terminal size and wiring!

4. Strengthen heat dissipation mode

1）The reliable working temperature of the driver is usually within 60 ℃, and the working temperature of the motor is within 80 ℃；

2）It is recommended to select the automatic half current mode when using, and the current will be automatically reduced by half when the motor stops, so as to reduce the heating of the motor and driver；

3）When installing the driver, please install it vertically on the side to make the heat dissipation teeth form strong air convection; If necessary, install a fan near the driver to force heat dissipation to ensure that the driver works within the reliable working temperature range.

3、Driver interface and wiring introduction

1. Interface description

- 1）Control signal interface

| name | function |
| --- | --- |
| PUL+ | Pulse signal: the rising edge of the pulse is effective; Pul is 4.5 ~ 28vdc at high level and 0 ~ 0.5V at low level. In order to reliably respond to the pulse signal, the pulse width shall be greater than 1.5 μ s。 |
| PUL- |  |
| DIR+ | Direction signal: high / low level signal. In order to ensure reliable commutation of the motor, the direction signal shall be at least 2 times ahead of the pulse signal μ S build. The initial running direction of the motor is related to the wiring of the motor. Exchanging any phase winding (such as a +, a-exchange) can change the initial running direction of the motor. Dir is 4.5 ~ 28vdc at high level and 0 ~ 0.5V at low level. |
| DIR- |  |
| ENA+ | Enable signal: this input signal is used to enable or disable. When ENA + is connected to 4.5 ~ 28vdc and ENA - is connected to low level (or the internal optocoupler is on), the driver will cut off the current of each phase of the motor and make the motor in a free state. At this time, the stepping pulse will not be responded. When this function is not needed, the enable signal end can be suspended. |
| ENA- |  |

2）Strong current interface

| name | function |
| --- | --- |
| GND | DC power ground |
| +Vdc | Positive pole of DC power supply, power supply voltage range: DC 12 ~ 32vdc, 24VDC is recommended. |
| A+、A- | Motor phase A coil interface. |
| B+、B- | Motor phase B coil interface. |

3）232 communication USB interface

You can connect PC or stu debugger through special UCB serial port cable, and hot plug is prohibited. Through stu or PC software protuner, the subdivision and current value, effective edge, single and double pulse and other settings required by customers can be set, and the resonance point can be eliminated and adjusted.

| Terminal number | Symbol | name | explain |
| --- | --- | --- | --- |
| 1 | +5V | 5V power supply positive terminal | External STU only |
| 2 | TxD | RS232 Sender |  |
| 3 | RxD | RS232 receiving end |  |
| 4 | GND | 5V Power ground | 0V |
| 5 | NC | RS232 receiving end |  |

Note: the cable connecting HB430 to PC, text display or stu servo debugger must be a special cable (attached according to the user's situation). Please confirm before use to avoid damage.

4） Status indication

The green LED is the power indicator. When the driver is powered on, the LED is always on; The LED goes off when the drive is powered off. The red LED is the fault indicator. When a fault occurs, the indicator flashes in a cycle of 3 seconds; When the fault is cleared by the user, the red LED is always off. The flashing times of red LED in 3 seconds represent different fault information. The specific relationship is shown in the table below:

| Serial number | Flicker times | Red LED flashing waveform | Fault description |
| --- | --- | --- | --- |
| 1 | 1 |  | Overcurrent or phase to phase short circuit fault |
| 2 | 2 |  | Overvoltage fault |
| 3 | 3 |  | undefined |
| 4 | 4 |  | undefined |

2. Control signal interface circuit

The HB430 driver adopts differential interface circuit, which can be applied to interfaces such as differential signal, single ended common cathode and single ended common anode. The built-in high-speed optocoupler allows to receive signals from long-line driver, open collector and PNP output circuit. In harsh environment, we recommend long line driver circuit, which has strong anti-interference ability. Now, taking open collector and PNP output as an example, the interface circuit diagram is as follows:

控制器

270Ω

270Ω

270Ω

Driver

PUL+

PUL-

DIR+

ENA+

DIR-

ENA-

VCC

R

R

R

脉冲信号

方向信号

使能信号

Common anode connection

Common cathode connection

控制器

270Ω

270Ω

270Ω

驱动器

PUL+

PUL-

DIR+

ENA+

DIR-

ENA-

VCC

R

R

R

脉冲信号

VCC

方向信号

VCC

使能信号

Fig. 3 input interface circuit

Note: when VCC value is 4.5 ~ 28vdc, R is short circuited or not connected；

3. Control signal sequence diagram

In order to avoid some misoperation and deviation, pul, dir and ENA shall meet certain requirements, as shown in the figure below：

t2

t4

t1

t3

t2

PUL

DIR

ENA

>2us

>2us

>5us

High level is higher than 3.5V

High level is higher than 3.5V

Low level is lower than 0.5V

Fig. 4 timing chart of control signal

notes：

1）t1：ENA(enable signal) shall advance dir by at least 5 μ s. Determined to be high. In general, ENA + and ENA - are recommended to be suspended.

2）t2：DIR at least advance pul falling edge 2 μ S determines whether its status is high or low.

3）t3：The pulse width shall not be less than 2 μ s。

4）t4：Low level width not less than 2 μ s。

4. Control signal mode setting

- Pulse trigger edge and single and double pulse selection：Set the pulse rising edge or falling edge trigger to be effective through PC software protuner software or stu debugger; Single pulse mode or double pulse mode can also be set. In dual pulse mode, the signal at the direction control terminal must be kept at high level or suspended.

5. Wiring requirements

1）In order to prevent the driver from being disturbed, it is recommended to use shielded cable for the control signal, and the shielding layer is short circuited with the ground wire. Except for special requirements, the shielded wire of the control signal cable is grounded at one end of the upper computer of the shielded wire, and the driver end of the shielded wire is suspended. The same machine is only allowed to be grounded at the same point. If it is not a real grounding wire, the interference may be serious, and the shielding layer is not grounded at this time.

2）Pulse and direction signal lines and motor lines are not allowed to be bound together side by side, and it is better to separate them for at least 10cm, otherwise the motor noise is easy to interfere with the pulse direction signal, resulting in inaccurate motor positioning, system instability and other faults.

3）If one power supply supplies multiple drives, parallel connection shall be adopted at the power supply, and chain connection from one to another is not allowed.

4）It is strictly forbidden to plug and unplug the strong current P2 terminal of the driver. When the charged motor stops, there is still a large current flowing through the coil. Plugging and unplugging the P2 terminal will cause huge instantaneous induced electromotive force and burn out the driver.

5）It is strictly forbidden to connect the conductor head with tin to the wiring terminal, otherwise the terminal may be overheated and damaged due to the increase of contact resistance.

6）The terminal head shall not be exposed outside the terminal to prevent accidental short circuit from damaging the driver.

4、Current, subdivision dial switch setting and parameter self-tuning

The HB430 driver uses an eight bit dial switch to set the subdivision accuracy, dynamic current, static half current and realize the self-tuning of motor parameters and internal adjustment parameters. Detailed description is as follows：

Dynamic current setting

Subdivision precision setting

| SW1 | SW2 | SW3 | SW4 | SW5 | SW6 | SW7 | SW8 |
| --- | --- | --- | --- | --- | --- | --- | --- |

1．Current setting

1）Operating (dynamic) current setting

| Output RMS current | SW1 | SW2 | SW3 |
| --- | --- | --- | --- |
| 0.6A | on | on | on |
| 0.98A | off | on | on |
| 1.26A | on | off | on |
| 1.59A | off | off | on |
| 1.86A | on | on | off |
| 2.17A | off | on | off |
| 2.41A | on | off | off |
| 3A | off | off | off |

2） Quiescent (quiescent) current setting

Quiescent current can be used, and the default is automatic half current.

2.Subdivision settings

| Steps / revolution | SW5 | SW6 | SW7 | SW8 | Subdivision description |
| --- | --- | --- | --- | --- | --- |
| Default[200] | on | on | on | on | When SW5, SW6, SW7 and SW8 are all on, the default subdivision of the drive is adopted: the user sets the subdivision through the PC software protuner or stu debugger. The minimum value is 1, the resolution is 1 and the maximum value is 51200. |
| 400 | off | on | on | on |  |
| 800 | on | off | on | on |  |
| 1600 | off | off | on | on |  |
| 3200 | on | on | off | on |  |
| 3600 | off | on | off | on |  |
| 6400 | on | off | off | on |  |
| 12800 | off | off | off | on |  |
| 1000 | on | on | on | off |  |
| 2000 | off | on | on | off |  |
| 4000 | on | off | on | off |  |
| 5000 | off | off | on | off |  |
| 7200 | on | on | off | off |  |
| 8000 | off | on | off | off |  |
| 10000 | on | off | off | off |  |
| 20000 | off | off | off | off |  |

3.Parameter self-tuning function

The driver can automatically complete the self-tuning of motor parameters and internal adjustment parameters within 200ms of power on; When the motor, power supply voltage and other conditions change, please conduct a self-tuning, otherwise the motor may run abnormally.Note that the pulse cannot be input at this time, and the direction signal should not change.

5、Power supply selection

The power supply voltage can work normally within the specified range. It is best to use non regulated DC power supply for hbd430 driver, or transformer step-down + bridge rectifier + capacitor filter. However, it shall be noted that the peak value of voltage ripple after rectification shall not exceed the specified maximum voltage. It is recommended that users use DC voltage lower than the maximum voltage for power supply to avoid grid fluctuation exceeding the operating range of driver voltage.

If the regulated switching power supply is used for power supply, it shall be noted that the output current range of the switching power supply shall be set to the maximum.

Please note that：

1）When wiring, pay attention to the positive and negative poles of the power supply, and do not reverse connect；

2）It is better to use non regulated power supply；

3）When non regulated power supply is adopted, the output capacity of power supply current shall be greater than 60% of the set current of the driver；

4）When the regulated switching power supply is adopted, the output current of the power supply shall be greater than or equal to the working current of the driver；

5）In order to reduce the cost, two or three drives can share one power supply, but the power supply power should be large enough。

6、Motor selection

HB430 can be used to drive 4, 6 and 8-wire two-phase and four phase hybrid stepping motors, and those with a step angle of 1.8 degrees and 0.9 degrees can be applied. When selecting the motor, it is mainly determined by the torque and rated current of the motor. The torque is mainly determined by the size of the motor. Large motor torque; The current is mainly related to the inductance. The motor with small inductance has good high-speed performance, but the current is large.

1.Motor selection

1）Determine the load torque, transmission ratio and working speed range

- T电机=C（Jε+T负载）

J：Moment of inertia of load     ε：Maximum angular acceleration of load     C：Safety factor, recommended value 1.2-1.4

- T负载：Maximum load torque, including effective load, friction, transmission efficiency and other resistance torque

2）What factors determine the motor output torque

For a given stepper motor and coil connection, the output torque has the following characteristics：

●The greater the actual current of the motor, the greater the output torque, but the more copper loss (P = I2R) of the motor, the more heating of the motor；

●The higher the power supply voltage of the driver, the greater the high-speed torque of the motor；

●According to the torque frequency characteristic diagram of stepping motor, the torque at high speed is smaller than that at medium and low speed.

V(转速)

I

I+∆I

T(力矩)

I-∆I

V-∆V

T(力矩)

V(转速)

V

V+∆V

Fig. 5 torque frequency characteristic diagram

2.Motor wiring

For 6-wire and 8-wire stepper motors, the connection method of different coils has considerable differences in motor performance, as shown in the figure below：

4线电机

8线电机并行接法

高速性能好

8线电机串行接法

低速力矩大

6线电机高力矩模式

6线电机高速模式

Fig. 6 motor wiring diagram

A+

A-

B+

B-

A+

A-

B+

B-

A+

A-

B+

B-

A+

A-

B+

B-

A+

A-

B+

B-

- 3.Selection of input voltage and output current

1）Setting of supply voltage

Generally speaking, the higher the power supply voltage, the greater the torque of the motor at high speed. The more you can avoid dropping at high speed. On the other hand, too high voltage will lead to overvoltage protection, more motor heating, and even damage the driver. When working at high voltage, the vibration of the motor moving at low speed will be greater.

2）Set value of output current

For the same motor, the larger the current setting value, the greater the motor output torque, but the heating of motor and driver is also more serious when the current is large. The specific calorific value is related not only to the current setting value, but also to the type of motion and residence time. The following setting methods use the rated current value of stepping motor as a reference, but the best value in practical application should be adjusted on this basis. In principle, if the temperature is very low (< 40 ℃), appropriately increase the current setting value as necessary to increase the motor output power (torque and high-speed response).

●Four wire motor: the output current is set to be equal to or slightly less than the rated current of the motor；

●High torque mode of six wire motor: the output current is set to 50% of the rated current of motor unipolar connection method；

●High speed mode of six wire motor: the output current is set to 100% of the rated current of motor unipolar connection method；

●Series connection of eight wire motor: the output current can be set to 70% of the rated current of single polarity connection of motor；

●Parallel connection of eight wire motor: the output current can be set to 140% of the rated current of single polarity connection of motor。

△Note: please run the motor for 15-30 minutes after current setting. If the motor temperature rises too high (> 70 ℃), reduce the current setting value. Therefore, generally, set the current to the value when the motor is warm but not hot during long-term operation.

七、典型接线案例

HBD430 is equipped with to42-08 series and parallel connection (if the motor direction is different from the desired direction, only exchange the positions of a +, a -). Hbd430 driver can drive four wire, six wire or eight wire two-phase / four phase motors. The following figure lists the connection methods of four wire, six wire and eight wire stepping motors in detail:

Figure 7 typical connection method of hb430 with 42 stepper motors

控制器

R

R

R

HB430

PUL+

PUL-

DIR+

ENA+

DIR-

ENA-

DC24V

GND

+Vdc

42电机

A+

A-

B-

B+

note：

1）The corresponding colors of different motors are different. When using, the motor data description shall prevail. For example, the color of 57 motor line is different.

2）The phases are relative, but the windings of different phases cannot be connected to the terminals of the same phase of the driver (a +, a - are one phase and B +, B - are the other phase). The definition, series and parallel connection methods of 57hs22 motor lead are shown in the figure below.

（a）引线定义

（b）串联接法

（c）并联接法

57HS22

57HS22

57HS22

蓝S45

红S45

黄S45

绿S45

白S45

橙S45

棕S45

黑S45

Figure 8 57 Series-parallel connection method of motor

3）HB430 driver can only drive two-phase hybrid stepping motor, not three-phase and five phase stepping motor.

4）Method to judge whether the series or parallel connection of stepping motor is correct: directly rotate the motor shaft by hand without connecting to the driver. If it can rotate easily and evenly, it indicates that the wiring is correct. If it encounters large resistance and uneven and accompanied by a certain sound, it indicates that the wiring is wrong.

8、Protection function

- Short circuit protection

In case of phase to phase short circuit or internal Overcurrent of the driver, the red light of the driver flashes once and repeatedly within 3 seconds. At this time, the fault must be eliminated and power on again for reset.

- Overvoltage protection

HB430 when the input voltage is higher than 38V, the red light of the driver flashes twice and repeatedly within 3 seconds. At this time, the fault must be eliminated and power on again for reset.

- Motor open circuit protection

When the motor is open circuit or not connected, the red light of the driver flashes 4 times and flashes repeatedly within 3 seconds. At this time, the fault must be eliminated and power on again for reset.

- note：Since the driver does not have the reverse connection protection function of the positive and negative poles of the power supply, please reconfirm that the positive and negative poles of the power supply are connected correctly before power on. If the positive and negative electrodes are connected reversely, the safety tube in the driver will be burned!

9、common problem

1.Common problems and solutions in application

| phenomenon | Possible problems | Solutions |
| --- | --- | --- |
| Motor does not rotate | The power light is not on | Check the power supply circuit for normal power supply |
|  | Motor shaft force | The pulse signal is weak, and the signal current increases to 7-16mA |
|  | Subdivision too small | Select pair subdivision |
|  | Is the current setting too low | Selective current |
|  | Drive protected | Power on again |
|  | The enable signal is low | This signal is pulled high or not connected |
|  | No response to control signal | Not powered on |
| Motor steering error | Wrong connection of motor line | Arbitrarily exchange two wires of the same phase of the motor (e.g. A +, A-exchange wiring position) |
|  | There is an open circuit in the motor line | Check and connect |
| The alarm indicator is on | Wrong connection of motor line | Check the wiring |
|  | Voltage too high or too low | Check the power supply |
|  | Motor or drive damaged | Replace the motor or drive |
| Inaccurate position | Signal disturbed | Eliminate interference |
|  | The shielding ground is not connected or not connected well | Reliable grounding |
|  | There is an open circuit in the motor line | Check and connect |
|  | Subdivision error | Set pair subdivision |
|  | Low current | Increase current |
| Locked rotor during motor acceleration | Acceleration time is too short | Longer acceleration time |
|  | Motor torque too small | Select large torque motor |
|  | Low voltage or low current | Increase the voltage or current appropriately |

2.Drive FAQ

1）What are stepper motors and stepper drivers？

Stepping motor is a special motor specially used for accurate speed and position control, It rotates at a fixed angle (called "step angle") step by step, so it is called stepping motor. It is characterized by no cumulative error, receives each pulse signal from the controller, and the motor runs at a fixed angle under the drive of the driver, so it is widely used in various open-loop control.

The stepping driver is a power amplifier that can make the stepping motor run. It can convert the pulse signal sent by the controller into the power signal of the stepping motor. The speed of the motor is directly proportional to the pulse frequency. Therefore, the control pulse frequency can be adjusted accurately, and the control pulse number can be positioned accurately.

2）What is drive segmentation? What is the relationship between the speed of stepping motor and pulse frequency?

Due to its unique structure, the stepper motor is marked with "inherent step angle of the motor" when leaving the factory (for example, 0.9 °/1.8 °, which means that the rotation angle of each step of half-step work is 0.9 °, and that of the whole step is 1.8 °). However, in many precision control and occasions, the angle of the whole step is too large, which affects the control accuracy, and the vibration is too large, so it is required to complete a motor's inherent step angle in many steps, which is called subdivision drive. The electronic device that can achieve this function is called subdivision drive.

V=

P*θe

360*m

V：motor speed（r/s）        P：Pulse frequency（Hz）

θe：Motor inherent step angle        m：Subdivision (1 for the whole step and 2 for the half step)

3）What are the advantages of subdividing drives?

●Because the step angle of each step is reduced and the step uniformity is improved, the control accuracy can be improved.

●It can greatly reduce motor vibration. Low-frequency oscillation is the inherent characteristic of stepping motor, and subdivision is the best way to eliminate it.

●It can effectively reduce torque ripple and improve output torque.

The above advantages are generally recognized by users and bring benefits to them, so it is recommended that you choose subdivision drives.

4）Why does my motor run in only one direction?

●Maybe the direction signal is too weak, or the wiring polarity is wrong.

●The pulse mode does not match, the signal is pulse/direction, and the driver must be set to this mode; If the signal is CW/CCW (double pulse mode), the driver must also be in this mode, otherwise the motor will only run in one direction.

If there are other problems, please contact the company's application engineer

Company product warranty

1 One-year warranty period

The company provides a one-year warranty for the defects in raw materials and workmanship of its products from the date of shipment. During the warranty period, the company provides free maintenance services for defective products.

2 Not covered by warranty

●Improper wiring, such as reverse connection of positive and negative poles of power supply and live plugging

●Change internal components without permission

●Use beyond electrical and environmental requirements

●Poor environmental heat dissipation

3 Maintenance process

If the product needs to be repaired, the following process will be followed：

- 1）Call the company's customer service personnel to obtain the repair permit number before shipment；

2）A written description is attached to the goods to explain the failure phenomenon of the drive to be repaired; Voltage, current and service environment when the fault occurs; Contact person's name, telephone number, mailing address and other information.

4 Warranty limitations

●The warranty scope of the company's products is limited to the components and processes of the products (i.e., consistency).

●The company does not guarantee that its products can be suitable for the specific use of customers, because the suitability is also related to the technical index requirements, use conditions and environment of the use.