HB556DC

Operation manual of digital two phase stepping driver

All rights reserved

[please read this manual carefully before use to avoid damaging the drive]

catalog

1、 Product introduction	2

1. summary	2

2. characteristic	2

3.application area	3

2、 Electrical, mechanical and environmental indicators	3

1. Electrical indicators	3

2.Operating environment and parameters 	3

3. Mechanical installation drawing	4

4. Strengthen heat dissipation	5

3、 Introduction of driver interface and wiring	5

1. Interface description	5

2. Control signal interface circuit	6

3. Control signal sequence diagram	6

4. Control signal mode setting	7

5. Wiring requirements	7

4、 Setting of current, subdivision dial switch and parameter self-tuning	8

1．Current setting	8

2.Subdivision setting	9

3.Parameter self-tuning function	9

5、 Power supply selection	9

6、 Motor selection	10

1.Motor selection	10

2.Motor wiring	10

2）Set value of output current	10

7、 Typical wiring cases	12

8、 Protection function	13

9、 Frequently asked questions	14

1.Common problems and solutions in application	14

2.Drive FAQs answer user questions	15

Warranty terms of the company's products	16

HB556DC

Digital two phase stepping driver

1、 Product introduction

1. summary

### HB556DC is a new digital stepping motor driver launched by our company. It adopts the latest 32-bit DSP digital processing technology. The driver control algorithm adopts advanced variable current technology and advanced frequency conversion technology. The driver has small heating, small motor vibration and stable operation. Users can set any subdivision within 200 ~ 51200 and any current value within the rated current, which can meet the application needs in most occasions. Due to the use of built-in micro separation technology, even under the condition of low subdivision, it can achieve the effect of high subdivision. The low, medium and high-speed operation is very stable, and the noise is very small. The driver is integrated with parameter power on automatic setting function, which can automatically generate optimal operation parameters for different motors to maximize the performance of the motor.

2. characteristic

- New 32-bit DSP Technology

- Ultra low vibration noise

- Built in high subdivision

- Automatic setting function of parameter power on

- The variable current control greatly reduces the motor heating

- Automatic half reduction of current at rest

- It can drive 4,6,8 wire two-phase stepping motor

- Optical isolation differential signal input

- Pulse response frequency up to 500KHz (factory default 200kHz)

- The current setting is convenient and can be selected from 0.1-5.0a

### The subdivision setting range is 200-51200

- It has over-voltage, under voltage and over-current protection functions

- application area

### Suitable for all kinds of small and medium-sized automation equipment and instruments, such as: engraving machine, marking machine, cutting machine, laser Phototypesetting, plotter, CNC machine tool, automatic assembly equipment, etc. The application effect is very good in the equipment with low noise and high speed expected by users.

- Electrical, mechanical and environmental indicators

- 1. Electrical indicators

| explain | HB556DC |  |  |  |
| --- | --- | --- | --- | --- |
|  | minimum value | Typical value | Maximum | Company |
| Output current | 0.1 | - | 5.0 | A |
| Input power voltage | 24 | 36 | 50 | VDC |
| Control signal input current | 6 | 10 | 16 | mA |
| Control signal interface level | 4.5 | 5 | 28 | Vdc |
| Minimum pulse width of input signal | 1.5 | - | - | us |
| Step pulse frequency | 0 | - | 200 | KHz |
| insulation resistance | 100 |  |  | MΩ |

2. Operating environment and parameters

| Cooling mode |  | Natural cooling or forced air cooling |
| --- | --- | --- |
| Usage environment | occasion | It should not be placed next to other heating equipment. Dust, oil mist and corrosive gas should be avoided. Combustible gas and conductive dust are not allowed in places with high humidity and strong vibration; |
|  | temperature | -5℃ ～ +50℃ |
|  | humidity | 40 ～ 90%RH |
|  | Vibration | 5.9m/s2MAX |
| Storage temperature |  | -20℃～80℃ |
| Use altitude |  | Below 1000m |
| weight |  | About 280 g |

3. Mechanical installation drawing

118

112

34

112

22.5

4.5

4-Ф3.5

75.5

25.3

47.8

Figure 1 installation dimension drawing (unit：mm)

※It is recommended to adopt side installation for better heat dissipation effect. When designing the installation size, the terminal size and wiring should be considered！

4. Strengthen heat dissipation

1）The reliable working temperature of the driver is usually within 60 ℃, and that of the motor is within 80 ℃；

2）It is suggested that the automatic half current mode should be selected when using, and the current will be automatically reduced by half when the motor stops, so as to reduce the heating of motor and driver；

3）When installing the driver, please install it vertically on the side to make the cooling teeth form strong air convection; if necessary, install a fan close to the driver to force heat dissipation, so as to ensure that the driver works within the reliable working temperature range。

3、 Introduction of driver interface and wiring

1. Interface description

- 1）Control signal interface

| name | function |
| --- | --- |
| PUL+ | Pulse signal: the rising edge of pulse is effective; pul is 4.5 ~ 28vdc at high level and 0 ~ 0.5V at low level. In order to respond to the pulse signal reliably, the pulse width should be greater than 1.5 μ s. |
| PUL- |  |
| DIR+ | Direction signal: high / low level signal. In order to ensure reliable commutation of the motor, the directional signal should be established at least 2 μ s prior to the pulse signal. The initial running direction of the motor is related to the wiring of the motor. Exchanging any phase winding (such as a + and a-exchange) can change the initial running direction of the motor. Dir is 4.5 ~ 28vdc at high level and 0 ~ 0.5V at low level. |
| DIR- |  |
| ENA+ | Enable signal: this input signal is used to enable or disable. When ENA + is connected to 4.5 ~ 28vdc, and ENA - is connected to low level (or internal optocoupler is on), the driver will cut off the current of each phase of the motor to make the motor in free state. At this time, the step pulse is not responded. When this function is not needed, enable the signal end to hang. |
| ENA- |  |

2）Strong current interface

| name | function |
| --- | --- |
| GND | DC power ground |
| +Vdc | DC power supply positive pole, supply voltage range DC 20 ~ 50 VDC, recommended 36 VDC operation. |
| A+、A- | Motor A phase coil interface. |
| B+、B- | Motor B phase coil interface. |

3） Status indication

The green LED is the power indicator. When the driver is powered on, the LED is always on; when the driver is powered off, the LED is off. The red LED is the fault indicator light. When there is a fault, the indicator light flashes in a cycle of 3 seconds; when the fault is cleared by the user, the red LED is always off. The number of times the red LED flashes in 3 seconds represents different fault messages,

The specific relationship is shown in the following table:

| Serial number | Flicker times | Red LED flashing waveform | Fault description |
| --- | --- | --- | --- |
| 1 | 1 |  | Over current or phase to phase short circuit fault |
| 2 | 2 |  | Over voltage fault |
| 3 | 3 |  | undefined |
| 4 | 4 |  | Motor open circuit or poor contact fault |

2. Control signal interface circuit

HB542C driver adopts differential interface circuit, which can be used for differential signal, single end common cathode and single end common anode interface. It has built-in high-speed optocoupler, which allows receiving signals from long-term driver, collector open circuit and PNP output circuit. In harsh environment, we recommend using long line driver circuit, which has strong anti-interference ability. Taking the collector open circuit and PNP output as an example, the interface circuit diagram is as follows:

控制器

270Ω

270Ω

270Ω

驱动器

PUL+

PUL-

DIR+

ENA+

DIR-

ENA-

VCC

R

R

R

脉冲信号

方向信号

使能信号

Common anode connection

Common cathode connection

控制器

270Ω

270Ω

270Ω

驱动器

PUL+

PUL-

DIR+

ENA+

DIR-

ENA-

VCC

R

R

R

脉冲信号

VCC

方向信号

VCC

使能信号

Figure 3 input interface circuit

Note: when the VCC value is 4.5 ~ 28vdc, R is short circuited or not connected;

3. Control signal sequence diagram

In order to avoid some misoperation and deviation, pul, dir and ENA should meet certain requirements, as shown in the following figure:

t2

t4

t1

t3

t2

PUL

DIR

ENA

>2us

>2us

>5us

高电平高于3.5V

高电平高于3.5V

低电平低于0.5V

低电平低于0.5V

Fig. 4 control signal sequence diagram

notes:

1) T1: ENA (enable signal) should be at least 5 μ s ahead of dir, determined as high. In general, it is suggested that ENA + and ENA - should be suspended.

2) T2: dir was at least 2 μ s ahead of pul.

3) T3: pulse width not less than 2 μ s.

4) T4: low level width not less than 2 μ s.

4. Control signal mode setting

- Pulse trigger edge and single and double pulse selection：Through PC software protuner software or stu debugger, it is effective to set the rising edge or falling edge trigger of pulse, and can also set single pulse mode or double pulse mode. In dual pulse mode, the signal at the direction control terminal must be kept at high level or suspended.

5. Wiring requirements

1）In order to prevent the driver from being interfered, it is suggested that the control signal should be shielded cable, and the shielding layer should be short circuited with the ground wire. In addition to special requirements, the shielding wire of the control signal cable should be single ended grounded: one end of the upper computer of the shielding wire is grounded, and one end of the driver of the shielded wire is suspended. The same machine can only be grounded at the same point. If it is not the real grounding wire, the interference may be serious, and the shielding layer will not be connected.

2）The pulse and direction signal lines and motor lines are not allowed to be wrapped together side by side. It is better to separate them at least 10 cm, otherwise the motor noise will easily interfere with the pulse direction signal, resulting in inaccurate motor positioning and system instability.

3）If one power supply provides multiple drivers, parallel connection should be adopted at the power supply, and chain connection is not allowed from one to the other.

4）It is strictly forbidden to plug and plug the strong current P2 terminal of the driver. When the charged motor stops, there is still a large current flowing through the coil. Pulling and plugging the P2 terminal will lead to a huge instantaneous induced electromotive force that will burn the driver.

5）It is strictly forbidden to connect the wire head into the terminal after adding tin, otherwise the terminal may be damaged due to overheating due to increased contact resistance.

6）The terminal should not be exposed outside the terminal to prevent accidental short circuit and damage to the driver.

4、 Setting of current, subdivision dial switch and parameter self-tuning

HB542CThe driver uses 8-bit dial switch to set subdivision accuracy, dynamic current, static half current, and realize self-tuning of motor parameters and internal adjustment parameters. The detailed description is as follows:

Dynamic current setting

Subdivision precision setting

| SW1 | SW2 | SW3 | SW4 | SW5 | SW6 | SW7 | SW8 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Single and double pulse selection

1．Current setting

1）Working (dynamic) current setting

| Output peak current | Output average current | SW1 | SW2 | SW3 | Current self setting |
| --- | --- | --- | --- | --- | --- |
| Default[1.0A,PK] |  | on | on | on | When SW1, SW2 and SW3 are set to off off off, the required current can be set by PC software, with the maximum value of 4.2a and resolution of 0.1A. If it is not set, the default current is 1.0A. |
| 1.46A | 1.04A | off | on | on |  |
| 1.91A | 1.36A | on | off | on |  |
| 2.37A | 1.69A | off | off | on |  |
| 2.84A | 2.03A | on | on | off |  |
| 3.31A | 2.36A | off | on | off |  |
| 3.76A | 2.69A | on | off | off |  |
| 5.00A | 3.50A | off | off | off |  |

- Static current setting

- The static current can be set by SW4 dial switch. Off means that the static current is set to half of the dynamic current, and on means that the static current is the same as the dynamic current. In general use, SW4 should be set to off to reduce the heating of motor and driver and improve the reliability. About 400ms after the pulse train stops, the current will automatically reduce to about half (60% of the actual value) and the calorific value will be reduced to 30% theoretically.

2.Subdivision setting

| Steps / revolution | SW5 | SW6 | SW7 | SW8 | Detailed description |
| --- | --- | --- | --- | --- | --- |
| Default | on | on | on | on | When SW5, SW6, SW7, and SW8 are all on, the driver subdivision adopts the default subdivision in the driver: the user can set the subdivision by PC software protuner or stu debugger. The minimum value is 1, the resolution is 1, and the maximum value is 51200. |
| 400 | off | on | on | on |  |
| 800 | on | off | on | on |  |
| 1600 | off | off | on | on |  |
| 3200 | on | on | off | on |  |
| 6400 | off | on | off | on |  |
| 12800 | on | off | off | on |  |
| 25600 | off | off | off | on |  |
| 1000 | on | on | on | off |  |
| 2000 | off | on | on | off |  |
| 4000 | on | off | on | off |  |
| 5000 | off | off | on | off |  |
| 8000 | on | on | off | off |  |
| 10000 | off | on | off | off |  |
| 20000 | on | off | off | off |  |
| 25000 | off | off | off | off |  |

3.Parameter self-tuning function

When the driver is an open-loop step drive, each parameter of the motor can be automatically identified once at the moment of power on, and the driver can automatically complete the self-tuning of the motor parameters and internal adjustment parameters; when the motor, supply voltage and other conditions change, please power on again for self-tuning, otherwise, the motor may not run in the optimal state.Note that the pulse can not be input at this time, and the direction signal should not change, so that the enable signal can not be connected.

5、 Power supply selection

The power supply voltage can work normally within the specified range. The HB542C driver is best supplied by non stabilized DC power supply, or transformer step-down + bridge rectifier + capacitor filter. However, it should be noted that the peak value of voltage ripple after rectification shall not exceed the specified maximum voltage. It is suggested that users should use DC voltage lower than the maximum voltage to avoid grid fluctuation exceeding the operating range of driver voltage.

If switching power supply is used, it should be noted that the output current range of switching power supply should be set to the maximum.Please note that:

1）When wiring, pay attention to the positive and negative poles of the power supply and do not connect in reverse;

2）It is better to use non voltage stabilized power supply;

3）When the non regulated power supply is used, the output current capacity of the power supply should be greater than 60% of the set current of the driver;

4）When switching power supply is used, the output current of the power supply should be greater than or equal to the working current of the driver;

5）In order to reduce the cost, two or three drivers can share one power supply, but the power supply should be large enough。

6、 Motor selection

HB542CIt can be used to drive two-phase and four-phase hybrid stepper motors with 4,6,8 wires. The step angle of 1.8 degrees and 0.9 degrees can be applied. Motor selection is mainly determined by the torque and rated current of the motor. The torque is mainly determined by the motor size. The larger the size of the motor torque is, and the current is mainly related to the inductance. The small inductance motor has good high-speed performance, but the current is larger.

1.Motor selection

1）Determine the load torque, transmission ratio, operating speed range

- T电机=C（Jε+T负载）

J：Moment of inertia of load     ε：Maximum angular acceleration of load     C：Safety factor, the recommended value is 1.2-1.4

- T负载：Maximum load torque, including payload, friction, transmission efficiency and other resistance torque

2）What factors determine the motor output torque

For a given step motor and coil connection, the output torque has the following characteristics:

●The larger the actual current, the greater the output torque, but the more copper loss (P = I2R) of the motor, the more heat generated by the motor;

●The higher the supply voltage of the driver, the greater the high-speed torque of the motor;

●According to the torque frequency characteristic diagram of stepping motor, the torque at high speed is smaller than that at medium and low speed.

V(转速)

I

I+∆I

T(力矩)

I-∆I

V-∆V

T(力矩)

V(转速)

V

V+∆V

Figure 5 torque frequency characteristic diagram

2.Motor wiring

For 6-wire and 8-wire stepper motors, the performance of motors with different coil connections is quite different, as shown in the following figure:

4线电机

8线电机并行接法

高速性能好

8线电机串行接法

低速力矩大

6线电机高力矩模式

6线电机高速模式

Figure 6 motor wiring diagram

A+

A-

B+

B-

A+

A-

B+

B-

A+

A-

B+

B-

A+

A-

B+

B-

A+

A-

B+

B-

- 3.Selection of input voltage and output current

1）Setting of power supply voltage

Generally speaking, the higher the supply voltage, the greater the torque of the motor at high speed. The more you can avoid falling off at high speed. But on the other hand, too high voltage will lead to over-voltage protection, motor heating more, and may even damage the driver. When the motor works under high voltage, the vibration of the motor at low speed will be greater.

2）Set value of output current

For the same motor, the higher the current setting value, the greater the motor output torque, but when the current is high, the motor and driver heating is more serious. The specific heat output is not only related to the current setting value, but also related to the movement type and residence time. The following setting mode uses the rated current value of stepping motor as reference, but the optimal value in practical application should be adjusted on this basis. In principle, if the temperature is very low (< 40 ℃), the current setting value can be appropriately increased to increase the motor output power (torque and high-speed response).

●Four wire motor: the output current is set to be equal to or slightly less than the rated current value of the motor；

●Six wire motor high torque mode: the output current is set to 50% of the rated current of single polarity connection；

●Six wire motor high speed mode: the output current is set to 100% of the rated current of single polarity connection；

●Series connection of eight wire motor: the output current can be set as 70% of the rated current of single polarity connection；

●Parallel connection of eight wire motor: the output current can be set as 140% of rated current of single polarity connection。

△Note: please run the motor for 15-30 minutes after setting the current. If the temperature rise of the motor is too high (> 70 ℃), the current setting value should be reduced. Therefore, the general situation is to set the current to the value when the motor is warm but not hot for a long time.

7、 Typical wiring cases

HB542C is equipped with 57 motors in series and connected in parallel (if the rotation direction of the motor is different from the expected direction, only the position of a + and a - can be exchanged). Hb542c driver can drive four wire, six wire or eight wire two-phase / four phase motor. The following figure lists the connection of 4-wire, 6-wire and 8-wire stepping motor in detail

Figure 7 typical connection of HB556DC with 57 motor

控制器

R

R

R

HB542C

PUL+

PUL-

DIR+

ENA+

DIR-

ENA-

DC36V

GND

+Vdc

57电机

A+

A-

B-

B+

be careful:

1) The colors of different motors are different. When using, the motor information shall prevail. For example, the color of 57 type and 86 type motor wire is different.

2) The phases are relative, but the windings of different phases cannot be connected to the terminals of the same phase of the driver (a + and a - are one phase, B + and B - are the other phase). The definition, series and parallel connection of 57 motor leads are shown in the figure below.

3) HB542C driver can only drive two-phase hybrid stepping motor, but not three-phase and five-phase stepping motor.

4) The method to judge whether the serial or parallel connection of stepping motor is correct or not: directly rotate the motor shaft by hand without connecting the driver. If the motor can rotate easily and evenly, the wiring is correct; if the resistance is large and uneven and accompanied with certain sound, the wiring is wrong.

（a）引线定义

（b）串联接法

（c）并联接法

电机

电机

电机

蓝S45

红S45

黄S45

绿S45

白S45

橙S45

棕S45

黑S45

Figure 8 57 series parallel connection method of motor

8、 Protection function

1) Short circuit protection

In case of phase to phase short circuit or internal over-current of the driver, the red light of the driver flashes once and repeatedly within 3 seconds. At this time, the fault must be eliminated and power on reset again.

2) Over voltage protection

When the input voltage of hb542c is higher than 60V, the red light of driver will flash twice, and it will flash repeatedly within 3 seconds. At this time, the fault must be eliminated and power on reset again.

3) Motor open circuit protection

When the motor is open or not connected, the driver red light flashes 4 times, and repeatedly within 3 seconds. At this time, the fault must be eliminated and power on reset again.

Note: since the driver does not have the protection function of reverse connection of positive and negative poles of power supply, please confirm that the positive and negative wiring of power supply is correct before power on. Reverse connection of positive and negative poles will cause the fuse in the driver to be burnt out!

9、 Frequently asked questions

1. common problems and solutions in application

| phenomenon | Possible problems | Solutions |
| --- | --- | --- |
| The motor doesn't work | The power light doesn't work | Check the power supply circuit and supply power normally |
|  | The motor shaft is powerful | The pulse signal is weak and the signal current increases to 7-16mA |
|  | The subdivision is too small | Select the right subdivision |
|  | Is the current setting too small | Select the right current |
|  | Drive protected | Power on again |
|  | Enable signal is low | The signal is pulled high or not connected |
|  | No response to control signal | No power on |
| Motor steering error | Wrong connection of motor wire | Arbitrary exchange of two wires of the same phase of the motor (e.g. A + and A-exchange connection position) |
|  | There is an open circuit in the motor line | Check and connect |
| The alarm indicator is on | Wrong connection of motor wire | Check wiring |
|  | The voltage is too high or too low | Check the power supply |
|  | Motor or driver damaged | Replace the motor or driver |
| Inaccuracy of position | Signal disturbed | Eliminate interference |
|  | The shielding ground is not connected or well connected | Reliable grounding |
|  | There is an open circuit in the motor line | Check and connect |
|  | Subdivision error | Set pair subdivision |
|  | The current is too small | Increase the current |
| Motor stalling when accelerating | Acceleration time is too short | Longer acceleration time |
|  | Motor torque too small | Select high torque motor |
|  | Low voltage or low current | Increase the voltage or current appropriately |

2.Drive FAQs answer user questions

1）What are stepping motor and stepping driver?

Stepper motor is a special kind of motor which is used for precise control of speed and position. It rotates step by step at a fixed angle (called "step angle"), so it is called stepping motor. Its characteristic is that there is no cumulative error, each pulse signal from the controller is received, and the motor runs at a fixed angle under the drive of the driver, so it is widely used in various open-loop control.

Step driver is a kind of power amplifier which can make the stepper motor run. It can convert the pulse signal sent by the controller into the power signal of the stepping motor. The speed of the motor is proportional to the pulse frequency, so the control pulse frequency can be accurately adjusted, and the number of control pulses can be accurately positioned.

- What is drive segmentation? What is the relationship between the speed of stepping motor and pulse frequency?

- Due to its unique structure, stepper motors are marked with "inherent step angle of motor" (for example, 0.9 ° / 1.8 ° indicates that the rotation angle of each step is 0.9 ° for half step operation and 1.8 ° for whole step). However, in many precision control and occasions, the angle of the whole step is too large, which affects the control accuracy, and the vibration is too large. Therefore, it is required to complete an inherent step angle of the motor in many steps, which is the so-called subdivision drive. The electronic device that can realize this function is called subdivision driver.

V=

P*θe

360*m

V：motor speed（r/s）              P：Pulse frequency（Hz）

θe：Inherent step angle of motor        m：Subdivision (1 for whole step and 2 for half step)

3）What are the advantages of subdivided drives?

Because the step angle of each step is reduced and the step uniformity is improved, the control accuracy can be improved.

It can greatly reduce motor vibration. Low frequency oscillation is the inherent characteristic of stepping motor, and subdivision is the best way to eliminate it.

It can effectively reduce the torque ripple and improve the output torque.

These advantages are generally recognized by users and bring them substantial benefits, so it is recommended that you choose subdivision drive.

4）Why does my motor only run in one direction?

It is possible that the directional signal is too weak, or the polarity of the wiring is wrong, or the signal voltage is too high, and the directional current limiting resistor is burnt out.

If the signal is CW / CCW (double pulse mode), the driver must also be in this mode; otherwise, the motor will only run in one direction.

If there are other problems, please contact our application engineer:

TEL：18688012205

Warranty terms of the company's products

1 One year warranty

The company provides a one-year warranty for the defects in raw materials and workmanship of its products from the date of delivery. During the warranty period, the company provides free maintenance service for defective products.

2 Not covered by warranty

Improper wiring, such as reverse connection of positive and negative poles of power supply and charged plug-in

Change internal components without permission

Use in excess of electrical and environmental requirements

Poor environmental heat dissipation

3 Maintenance process

If the product needs to be repaired, it will be handled according to the following process:

1) Call our customer service personnel to obtain the repair permit number before delivery;

2) A written instruction is attached with the goods to explain the fault phenomenon of the drive to be repaired; the voltage, current and operating environment at the time of failure; the name, telephone number and mailing address of the contact person.

4 Warranty restrictions

The warranty scope of the product is limited to the device and process of the product (i.e. consistency).

There is no guarantee that its products can be suitable for the specific use of the customer, because the suitability is also related to the technical index requirements, service conditions and environment of the application.