MY58 Mobile  printer

| project |  | model |
| --- | --- | --- |
| parameter name |  | MY58 |
| Printer data | Print type | Thermal line printer |
|  | Fast Print speed | 50mm/s |
|  | Resolution | 203dpi |
|  | Point distance | 0.125mm |
|  | Valid print width | 48mm |
|  | Paper | Thermal receipt roll paper, thermal sticker paper |
| Print material | Paper width | Bill: 58mm Label: 50mm, 40mm, 30mm |
|  | Paper thickness | 0.05-0.18mm |
|  | Paper roll outer diameter | Max 40mm |
|  | Paper loading | Clamshell paper loading, easy loading paper |
|  | Out paper type | Top exit; thermal side facing out |
|  | Cutter type | Plastic tear knife, manual tear off |
| Print comment | Font English | Height：16， 24 |
|  | Font Chinese | Null |
|  | Encoding | ASCII；GBK（GB18030 double）；Unicode |
|  | Print characters | English, Chinese characters, numbers, various symbols |
|  | Rotation print | 0 degree，90 degree，180 degree，270 degree |
|  | Bar code | EAN－8、EAN－13、ITF、UPC－A、UPC－E、Code bar、Code93、Code39、Code128 |
|  | 2D bar code | QR Code, PDF417, Data Matrix, Grid Matrix |
|  | Bitmap | Black and white bitmap (line, curve, table, picture) |
| Physical characteristics | Machine Size | 101.8mm*86.7mm*48mm |
|  | Bare metal weight | 188g(Including battery without paper) |
|  | Package Size | 128.5mm*93mm*53.5mm |
|  | With package weight | 276g |
|  | Carton Size | 413mm*303mm*360mm |
|  | Carton Quantity | 54 |
|  | Carton Weight | 15.5kg |
| Interactive interface | Input | 2 pc Key button |
|  | Output | Status indicator, battery indicator |
|  | USB Communication Interface | USB-Type-C |
|  | Wireless communication interface | Bluetooth 4.2(SPP+BLE) |
|  | Label detect | Reflective label detection, no virtual slit, label slit >=2mm |
|  | Black mark detect | The middle black mark detection, the black mark is on the printing surface, the position reference picture |
|  | Warning | Out of paper prompt; open cover detection; low battery prompt; overheat prompt |
|  | Command | ESC/POS |
| Power | Battery capacity | 3.7V 1800mAh Lithium |
|  | Battery removable capacity | Detachable |
|  | Adapter | Input: AC 100-240V 0.5A，Output：DC 5V1A |
|  | Data line | USB-Type-C |
| Environmental parameters | Operating temperature | 0～50 °C |
|  | Storage temperature | －20～60°C |
|  | Operating humidity | 35%～85% No condensation |
|  | Storage humidity | 10%～90% No condensation |
| Reliability | Print life | 50Km（1 2.5% Print density）/ 100million pulse |
|  | Battery lift | 300 cycles of charge and discharge > 60% battery |
|  | One charge print length | About 100m (12.5% printing density) |
|  | Certification | CCC |
| Other data | Print data setup | Use button settings |
|  | Charging time | 4 hours |
|  | Standby time | 60 hours |